3GPP TSG-RAN WG3 Meeting #125                                                              R3-244146

Maastricht, The Netherlands, August 19 - 23, 2024

Agenda item: 		17.3

Source: 		Qualcomm Incorporated

Title: 	Aspects of on-demand SIB1 for NES enhancements

Document for:		Discussion

# 1	Introduction

In the last RAN3 meeting, the chair notes captured the following agreements and follow-up discussion points related to on-demand SIB1 for NES [1]:

Focus on case2 in RAN3 first. Which node decides the WUS configuration and On-demand SIB1 mode? DU or CU or OAM? NES-cell gNB decides on-demand SIB1 operation activation and deactivation? Focused on Case 2, and continue the discussion on the issues raised in SoD in next meeting.

The SoD, referred to in the chair nodes, identified the following issues in R3-243868:

Focused on Case 2, the below issues are proposed by the companies to be further discussed at the next meeting: Related to WUS provision: Issue 1: The WUS configuration signalling to support NES Cell to request WUS configuration provision in Cell A and to support NES cell to receive WUS configuration provision status feedback from Cell A? Issue 2: The WUS configuration signalling between NES gNB-DU and NES gNB-CU? Issue 3: Which network node entity decides the content of the WUS configuration? Issue 4: The UL WUS configuration signalling provision and update from NES cell to Cell A? Issue 5: The SIB1 broadcast status indication or On-demand SIB1 operation activation/deactivation indication? Issue 6: There might be multiple NES cells under a single Cell A? Issue 7: There might be multiple Cell A for a single NES cell? Related to NES Cell on demand SIB activation/deactivation: Issue 8: Which network node entity decides operating a cell in periodic or on-demand SIB1 transmission mode? Issue 9: Signal the SIB1 broadcast status indication or On-demand SIB1 operation activation/deactivation indication from the NES cell to the Cell A over Xn? Issue 10: On-demand transmission activation/deactivation, include the split architecture? Issue 11: Cell A needs to be aware of the SIB transmission mode (current ES state) of all the NES cells under its coverage. Accordingly, it can decide which WUS configurations are to be broadcast? Example of questions: (OD SIB1 == On Demand SIB1) - Is it NES gNB-CU who decides to activate OD SIB1 mode for NES gNB-DU? - or it is NES gNB-CU who indicates which NES gNB-DU can go to OD SIB1 mode, and NES gNB-DU determines when it should enter OD SIB1 mode, and when to go out of that mode? - Is it NES gNB-DU who determines the content of WUS configuration(s) and sends it to NES gNB-CU over F1AP? - NES gNB-CU sends WUS configuration to Cell A gNB-CU over XnAP before entering on-demand SIB1 mode in the NES Cell or when the UL WUS configuration is changed. (Note: this is the agreement from RAN3#123bis.) - Cell A gNB-CU may accept or reject the WUS configuration broadcast? - If Cell A gNB-CU accepts this task, it sends ACK to NES gNB-CU, and NES gNB-CU activates OD-SIB1 mode? NES gNB-CU informs NES-gNB-DU of this decision, and NES-gNB-DU stops transmission of periodic SIB1? - or if Cell A gNB-CU accepts this task, it sends ACK to NES gNB-CU who passes the ACK to NES gNB-DU. It is the NES gNB-DU determines when to enter OD-SIB1 mode from the periodic SIB1 transmission mode? - Does NES-gNB-CU need to inform Cell A gNB-CU of its SIB1-transmission-mode? Or any change of the SIB1-transmission-mode?

This contribution aims to make further progress for the Case-2 OD-SIB1 scenario based on the issues raised in last meeting.

# 2	Discussion

## High Level Aspects

Many are many issues related to OD-SIB1 that were raised in last meeting’s discussion and captured in the SoD. Based on this discussion and the SoD, the following high-level aspects can be identified for case-2 OD-SIB1:

UL WUS configuration:

- Aspect 1-1: Which nodes need to know the UL WUS configuration

- Aspect 1-2: How these nodes obtain the UL WUS configuration

OD-SIB1 activation/deactivation:

- Aspect 2: Whether and how dynamic OD-SIB1 activation/deactivation should be supported

## Aspect 1-2: Which nodes need to know the UL WUS configuration

The following candidates can be considered:

- NES-cell CU

- NES-cell DU

- Cell-A CU

- Cell-A DU

Context of UL WUS configuration:

The NES-cell DU needs to know the context of the UL WUS configuration since it needs to be able to decode the UL WUS. All other nodes, i.e., NES-cell CU, Cell-A CU, and Cell-A DU do not have to know the context of the UL WUS configuration unless they themselves generate this configuration or in case they receive it via OAM.

Observation 1-1: The NES-cell DU needs to know the context of the UL WUS configuration since it needs to be able to decode the UL WUS.

Observation 1-2: The NES-cell CU, Cell-A CU, and Cell-A DU only need to know the context of the UL WUS configuration if they generate this configuration or obtain this configuration via OAM.

Encoded UL WUS configuration (i.e, as a byte string):

The Cell-A DU needs to know the encoded UL WUS configuration (byte string) since it transmits this byte string OTA.

All other nodes, i.e., NES-cell DU, NES-cell CU, and Cell-A CU, do not have to know the encoded UL WUS configuration unless they themselves encode this information. They also do not need to know this encoded information if they simply pass it on.

Observation 1-3: The Cell-A DU needs to know the encoded form of the UL WUS configuration, which it transmits OTA.

Observation 1-4: The NES-cell DU, NES-cell CU and Cell-A CU only need to know the encoded form of the UL WUS configuration if they themselves encode the UL WUS configuration.

## Aspect 1-2: How these nodes obtain the UL WUS configuration

The first question is whether the UL WUS configuration is expected to be provisioned via OAM, or whether some network node, e.g., a DU or a CU, should be allowed to dynamically generate or change the UL WUS configuration.

The UL WUS configuration is not assumed to change frequently, and it is not expected to change due to radio-, propagation-, or traffic conditions. It can therefore be expected that the UL WUS configuration is provisioned via OAM.

Proposal 1-1: Since the UL WUS configuration is not expected to change frequently and since it does not rely on the dynamic nature of radio-, propagation- or traffic conditions, it can be semi-statically provisioned via OAM.

The UL WUS further represents cell-specific PHY/MAC signaling conducted at the NES-cell DU. It UL WUS configuration should therefore be owned by the NES-cell DU.

Proposal 1-2: Since the UL WUS represents cell-specific PHY/MAC signaling conducted at the NES-cell DU, the UL WUS configuration should be owned by the NES-cell DU.

Proposal 1-1 and 1-2 imply that the UL WUS configuration is OAM-provisioned on the NES-cell DU. To make the UL information available at the Cell-A DU for transmission, the following options can be considered:

Option 1: The information of the UL WUS configuration is OAM-provisioned on Cell-A.

Option 2: The NES-cell DU forwards the information of the UL WUS configuration to the Cell-A DU via NES-cell CU and Cell-A CU.

These two options presently leave it open, where the information encoding of the UL WUS configuration should occur.

Option 1 has lower inter-network-node signaling overhead than Option 2, which comes at the cost of higher operational complexity. In other scenarios, where inter-gNB information exchange is needed, RAN3 has decided in favor of higher signaling overhead to lower operational overhead. ANR, for instance, could be supported by provisioning all gNBs with their respective neighbor-gNB IP addresses and cell configuration via OAM. However, RAN3 has defined inter-gNB signaling procedures to allow autonomous exchange of this information across gNBs. Therefore, to be consistent with such legacy principles, Option 2 should be supported.

Proposal 1-3: In alignment with legacy principles for inter-gNB configuration exchange, such as ANR, the information of UL WUS configuration should be forwarded from the NES-cell DU to the Cell-A DU via NES-cell CU and Cell-A CU.

It remains to be discussed where the UL WUS configuration information is encoded. The following options could be considered:

Option A: UL WUS configuration is encoded by NES-cell DU.

Option B: UL WUS configuration is encoded by Cell-A CU.

Option C: UL WUS configuration is encoded by Cell-A DU.

Option A make sense since the NES-cell DU is the owner of the UL WUS configuration. Option B makes sense in case the UL WUS configuration information is included in a SIB, which is owned by the Cell-A CU. Option C makes sense in case the UL WUS configuration information is included in  SIB, which is owned by the Cell-A DU. Since RAN2 is still debating how the UL WUS configuration is provided by Cell-A, the down-selection among options A to C should wait until RAN2 has made more progress.

Proposal 1-4: The decision on where the UL WUS configuration is encoded for transmission depends on the SIB used for this transmission, and this is pending RAN2 progress.

## Aspect 2: Whether and how dynamic OD-SIB1 activation/deactivation should be supported

In last meeting, RAN3 discussed whether the NES cell could dynamically switch between OD SIB1 and a legacy periodic SIB1 broadcast. The benefit of such dynamic switching could be the following:

- At low UE traffic, the NES cell can use OD SIB1 delivery since the OD SIB1 request rate is rather low.

- At high traffic, the number of OD SIB1 requests may be more frequent than the periodic SIB1 broadcast transmissions. In this case, the NES cell may prefer to use periodic SIB1 broadcast.

RAN3 further discussed whether and how the NES cell should communicate the change of this operating mode to cell-A.

The dynamic switching between these two operating modes raises the following questions:

- Since the legacy UE cannot access a NES cell operating in OD SIB1 mode, would this UE be able to access this NES cell when periodically broadcasting SIB1?

- If this is the case, what would be the expected behavior for a legacy UE that is camping on the NES cell when the operation mode switches from SIB-broadcast to OD-SIB1 mode?

- If this is not the case, how to prevent the legacy UE from accessing the NES cell while operating in SIB1-broadcast mode?

- How frequently would the NES cell be able to switch the operating mode?

- What would be the impact on the legacy and Rel-19 UE due to such switching (e.g., SIB update, reselection of cell-A, etc).

These issues have not been addressed by RAN2. Therefore, RAN3 should not further discuss dynamic activation/deactivation of OD-SIB1 until RAN2 has explicitly agreed to support such functionality.

Proposal 2: RAN3 to not further discuss dynamic activation/deactivation of OD-SIB1 unless RAN2 has explicitly agreed to support such functionality.

# 3	Conclusion

This contribution discussed RAN3-related aspects of on-demand SIB1 for network energy savings. The following observations and proposals have been made:

Aspect 1-2: Which nodes need to know the UL WUS configuration

Observation 1-1: The NES-cell DU needs to know the context of the UL WUS configuration since it needs to be able to decode the UL WUS.

Observation 1-2: The NES-cell CU, Cell-A CU, and Cell-A DU only need to know the context of the UL WUS configuration if they generate this configuration or obtain this configuration via OAM.

Observation 1-3: The Cell-A DU needs to know the encoded form of the UL WUS configuration, which it transmits OTA.

Observation 1-4: The NES-cell DU, NES-cell CU and Cell-A CU only need to know the encoded form of the UL WUS configuration if they themselves encode the UL WUS configuration.

Aspect 1-2: How these nodes obtain the UL WUS configuration

Proposal 1-1: Since the UL WUS configuration is not expected to change frequently and since it does not rely on the dynamic nature of radio-, propagation- or traffic conditions, it can be semi-statically provisioned via OAM.

Proposal 1-2: Since the UL WUS represents cell-specific PHY/MAC signaling conducted at the NES-cell DU, it should be owned by the NES-cell DU.

Proposal 1-3: In alignment with legacy inter-gNB configuration exchange, the information of UL WUS configuration should be forwarded from the NES-cell DU to the Cell-A DU via NES-cell CU and Cell-A CU.

Proposal 1-4: The decision on where the UL WUS configuration is encoded for transmission depends on the SIB used for this transmission, and this is pending on RAN2 progress.

Aspect 2: SIB1Whether and how dynamic OD-SIB1 activation/deactivation should be supported

Proposal 2: RAN3 to not further discuss dynamic activation/deactivation of OD-SIB1 unless RAN2 has explicitly agreed to support such functionality.

# Reference

[1]  Chair notes, TSG RAN WG3 meeting #124, Fukuoka, Japan, May 2024