3GPP TSG-RAN3 Meeting #125	R3-244147

Maastricht, The Netherlands, August 19th – 23rd 2024

Agenda Item:	18.2

Source: 	Qualcomm Incorporated

Title: 	Support LP-WUS subgrouping

Document for:	Discussion/Decision

# Background

WID of Low-power wake-up signal and receiver for NR (LP-WUS/WUR) (RP-240801) has the below objective for RAN3 impacts:

- For IDLE/INACTIVE modes

  - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4)

In this contribution, we discuss the details on supporting LP-WUS subgrouping to indicate paging via LP-WUS.

# Discussion

## LP-WUS subgrouping methods

RAN study report for "Study on low-power wake up signal and receiver for NR" [1] captured the below for support of LP-WUS subgrouping:

For RRC_IDLE/INACTIVE mode:

-	RAN2 has studied and concluded to support subgrouping for LP-WUS, detailed design depends on the payload of LP-WUS.

Similar as PEI, the subgrouping methods for LP-WUS could include the CN assigned, and UE_ID based subgrouping. Details, e.g. which one/both should be supported, how to support/provide the configuration, etc., could be further determined during WI phase.

Observation 1: LP-WUS/WUR RAN study [1] concluded to support subgrouping for LP-WUS.

Observation 2: LP-WUS/WUR RAN study [1] did not conclude on whether to support one/both of CN assigned and UE_ID based subgrouping.

The decision to support one/both of CN assigned subgrouping and UE_ID based subgrouping for LP-WUS should be made by RAN3 in the same manner as it was done for PEI subgrouping.

In Rel-17 PEI subgrouping, for CN-assigned subgrouping, the UE's subgroup is assigned by CN taking into account the UE's characteristics, such as mobility pattern, paging probabilities, etc. For UEID-based subgrouping, the UE's subgroup is formed based on UEID and the subgroup number in RAN. Supporting both types of subgrouping methods for LP-WUS, provides the flexibility for the network to choose the type of subgrouping method based on the experienced false paging rates and types of UEs for which pages are to be sent in different cells.

Proposal 1: RAN3 to agree to support both CN-assigned subgrouping and UEID-based subgrouping for LP-WUS.

Proposal 2: RAN3 assumes that analogue to PEI, the RAN provides LP-WUS configurations to the UE, including the indication of support for at least one of LP-WUS core-network-assigned subgrouping identifier and UE-assigned subgrouping identifier.

Similar to PEI CN-assigned subgrouping, support of LP-WUS CN-assigned subgrouping requires AMF to assign the CN-assigned subgrouping identifier and include the identifier in the messages sent to RAN on the NG interface. Thus, RAN3 should confirm with SA2 on whether the AMF can support LP-WUS CN-assigned subgrouping.

Proposal 3: Send an LS to SA2 to confirm whether AMF can support CN-assigned subgrouping for LP-WUS.

## 2.2 LP-WUS subgrouping identifier assignment

The RAN study report ,[1], has the below table summarizing that the UE and RAN may decide to support either LP-WUS without PEI or LP-WUS with PEI along with paging. From the table, even though RAN1/RAN2 have agreed to support LP-WUS subgrouping, they have not concluded on the relation between PEI subgrouping and LP-WUS subgrouping.

Table 7.3.1.2-1: Pros and Cons of different alternatives for wake-up procedures

|  | Alt1.1: LP-WUS+ Paging | Alt 1.2: LP-WUS+PEI+paging |
| --- | --- | --- |
| Access latency | LP-WUS reception+ paging reception+ random access procedure | LP-WUS reception + PEI reception + paging reception +random access procedure |
| False Alarm | Can be reduced by use of UE subgrouping for LP-WUS. | Unclear if and how it can be further reduced if UE subgrouping mechanism is used in LP-WUS and PEI together. |
| Complexity of LP-WUS Design | Low | It is higher if UE subgrouping mechanism is used in LP-WUS and PEI together |
| Integration with legacy | Smooth as LP-WUS replaces PEI | Unclear what PEI functionality adds on top of LP-WUS |

Observation 3: Even though RAN1/RAN2 have agreed to support LP-WUS subgrouping, there is no conclusion yet on the relation between PEI subgrouping and LP-WUS subgrouping.

When the network supports both LP-WUS and PEI subgrouping, below options are possible on how the LP-WUS subgrouping identifier is assigned:

- Option 1: Use PEI subgrouping identifier also for LP-WUS subgrouping

- Option 2: Introduce a separate identifier for LP-WUS subgrouping

The decision on which option to be introduced is in the domain of RAN1/RAN2 as it must be concluded based on the false paging alarm, and other performance impacts. However, the chosen option has impacts on the RAN3 design for introducing any new fields for LP-WUS subgrouping identifier indication in NGAP.

Proposal 4: RAN3 to wait for RAN1/RAN2 decision on whether LP-WUS subgrouping uses the same ID as PID (Option 1) of whether a separate subgrouping ID is introduced for LP-WUS (Option 2).

## 2.3 LP-WUS CN-assigned subgrouping identifier

In case RAN1/2 decide to pursue Option 1, enhancements due to LP-WUS have no RAN3 impact.

Observation 4: In case RAN1/2 decide to pursue Option 1, enhancements due to LP-WUS have no RAN3 impact.

In case RAN1/2 decide to pursue option 2, new fields need to be introduced in the paging messages and in the messages received from core network, AMF, to assign new LP-WUS CN-assigned subgrouping identifier.

Observation 5: In case RAN1/2 decide to pursue Option 2, enhancements are necessary to NGAP, XnAP and F1AP .

## 2.4 LP-WUS UEID-based subgrouping identifier

UEID-based subgrouping identifier determination formula and the configuration parameters to use for determination must be concluded by RAN1/RAN2 before making progress in RAN3. We can discuss this in next meeting.

# Summary

Below are the observations and proposals discussed in this document.

Observation 1: LP-WUS/WUR RAN study [1] concluded to support subgrouping for LP-WUS.

Observation 2: LP-WUS/WUR RAN study [1] did not conclude on whether to support one/both of CN assigned and UE_ID based subgrouping.

Observation 3: Even though RAN1/RAN2 have agreed to support LP-WUS subgrouping, there is no conclusion yet on the relation between PEI subgrouping and LP-WUS subgrouping.

Observation 4: In case RAN1/2 decide to pursue Option 1, enhancements due to LP-WUS have no RAN3 impact.

Observation 5: In case RAN1/2 decide to pursue Option 2, enhancements are necessary to NGAP, XnAP and F1AP .

Proposal 1: RAN3 to agree to support both CN-assigned subgrouping and UEID-based subgrouping for LP-WUS.

Proposal 2: RAN3 assumes that analogue to PEI, the RAN provides LP-WUS configurations to the UE, including the indication of support for at least one of LP-WUS core-network-assigned subgrouping identifier and UE-assigned subgrouping identifier

Proposal 3: Send an LS to SA2 to confirm whether AMF can support CN-assigned subgrouping for LP-WUS.

Proposal 4: RAN3 to wait for RAN1/RAN2 decision on whether LP-WUS subgrouping uses the same ID as PID (Option 1) of whether a separate subgrouping ID is introduced for LP-WUS (Option 2).

# References

[1] RP-240801 - WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), vivo, NTT DOCOMO, Ericsson, MediaTek, Samsung, Sony