3GPP TSG-RAN WG3 Meeting #125	R3-244148

Maastricht, Netherlands, 19-23 August 2024

Agenda Item:	16.2

Source:	MediaTek Inc.

Title:	A-IoT Architecture

Document for:	Discussions & Approval

# 1	Introduction

Ambient IoT involves the following considerations for the reader:

- It is important to allow implementation of a small portable A-IoT reader entity, especially since wide coverage cannot be expected even with high performance reader implementations.

- It is important to allow such portable reader entity to be implemented as a consumer device, based on a UE platform.

Opinion: The above is the main purpose of A-IoT Topology 2. This paper discusses the related architectural consequences.

# 2	Reader as a Consumer Device in Topology 2

This discussion relates to topology 2, in which the reader entity is an intermediate node, intended as described in section 1 to be realised in a small portable device on a consumer UE platform.

For topology 2:

A portable reader entity being implemented as a consumer device, based on a UE platform, involves minimising impact especially to the Uu interface and related layers of the UE.  In particular, the existing Uu functionality should be reused wherever possible, with the expectation that there are protocol impacts at layer 3 to introduce control of the UE reader by the gNB.  No RAN network protocol should be extended to the reader, as this would impose significant impacts on the UE platform specific to A-IoT, preventing a clean implementation of the reader functionality on a consumer platform.  We thus have the following proposals:

Proposal 1: [RAN2/RAN3] For communication between the 3GPP network and the portable A-IoT reader entity in topology 2 (AIoT-enabled UE), reuse of Rel-18 Uu functionality, protocols and principles is assumed, with no requirement to use any Rel-18 network protocol, neither CP nor UP.

Proposal 2: [RAN2/RAN3] For communication between the 3GPP network and the portable A-IoT reader entity in topology 2 (AIoT-enabled UE), Uu L1/L2 is re-used as-is, including e.g. Uu AS security in PDCP.

The AIoT RAN node function is assumed to do mainly resource handling for AIoT Radio, and thus there is a need for control information exchange between AIoT RAN node function and the Common Reader function. In topology 2, such communication between AIoT RAN node function and Common reader function can be carried by existing Uu protocols (RRC).

Proposal 3: [RAN2/RAN3] Uu L3 (RRC) may be extended for A-IoT radio resource handling between common reader and A-IoT-enabled gNB. No new protocol is foreseen.

Proposal 4: [RAN2/RAN3] Uu L3 (NAS) or Uu user plane may be used to carry communication between A-IoT common reader and A-IoT Core Network.  Whether a new protocol is introduced for this purpose is outside RAN scope.

The model shown in figure 1 below was proposed at RAN3#124, introducing the concept of an A-IoT “RAS” (Radio Access System) comprising the A-IoT-specific functions of the gNB and the common reader function housed with the AIoT-enabled UE.

Figure 1: Deployment scenario for topology 2

Proposal 5: [RAN2/RAN3] WG Responsibilites to be confirmed as follows:

- The Uu interface is reused under RAN2 responsibility between the A-IoT-enabled gNB and the A-IoT-enabled UE.

- RAN3 is expected to, in cooperation with other relevant groups, specify the role and responsibility of the AIoT RAN Node Function.

- The interface between the CN and the A-IoT-enabled gNB (labelled as “XX” in the figure) is specified under RAN3 responsibility between the A-IoT CN and the A-IoT-enabled gNB.

- The A-IoT radio is specified under RAN2/RAN1 responsibility between the common reader function and the A-IoT device (i.e., no change from current specification responsibilities and processes).

A generic version of the architecture, excluding the topology 2-specific aspects, is shown in figure 2.

Figure 2: Logical system architecture for A-IoT, common for topology 1 and topology 2

This architecture is compatible with both topologies and should be workable in both RAN2 and RAN3 scope.

Proposal 6: [RAN3] Capture the logical architecture of figure 2 and the topology 2 deployment model in figure 1, and send an LS to SA2/RAN2 notifying them of the RAN3 view of the architecture.

# 3	Conclusions

This document promulgated the following proposals:

Proposal 1: [RAN2/RAN3] For communication between the 3GPP network and the portable A-IoT reader entity in topology 2 (AIoT-enabled UE), reuse of Rel-18 Uu functionality, protocols and principles is assumed, with no requirement to use any Rel-18 network protocol, neither CP nor UP.

Proposal 2: [RAN2/RAN3] For communication between the 3GPP network and the portable A-IoT reader entity in topology 2 (AIoT-enabled UE), Uu L1/L2 is re-used as-is, including e.g. Uu AS security in PDCP.

Proposal 3: [RAN2/RAN3] Uu L3 (RRC) may be extended for A-IoT radio resource handling between common reader and A-IoT-enabled gNB. No new protocol is foreseen.

Proposal 4: [RAN2/RAN3] Uu L3 (NAS) or Uu user plane may be used to carry communication between A-IoT common reader and A-IoT Core Network.  Whether a new protocol is introduced for this purpose is outside RAN scope.

Proposal 5: [RAN2/RAN3] WG Responsibilites to be confirmed as follows:

- The Uu interface is reused under RAN2 responsibility between the A-IoT-enabled gNB and the A-IoT-enabled UE.

- RAN3 is expected to, in cooperation with other relevant groups, specify the role and responsibility of the AIoT RAN Node Function.

- The interface between the CN and the A-IoT-enabled gNB (labelled as “XX” in the figure) is specified under RAN3 responsibility between the A-IoT CN and the A-IoT-enabled gNB.

- The A-IoT radio is specified under RAN2/RAN1 responsibility between the common reader function and the A-IoT device (i.e., no change from current specification responsibilities and processes).

Proposal 6: [RAN3] Capture the logical architecture of figure 2 and the topology 2 deployment model in figure 1, and send an LS to SA2/RAN2 notifying them of the RAN3 view of the architecture.