3GPP TSG-RAN WG3 Meeting #125	R3-244149

Maastricht, The Netherlands, 19 - 23 August 2024

Agenda item:		9.2

Title:		Security Indication behavior

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

In 33.501 it is described that e.g. “The SMF shall provide UP security policy for a PDU session to the ng-eNB/gNB during the PDU session establishment procedure”, however in 38.413 the Security Indication IE is present as optional during the PDU session establishment procedure i.e. the Security Indication IE is optional PDU Session Resource Setup Request Transfer IE, there is an ambiguity how would the gNB behave if the Security Indication IE is not present.

Similar issue also happens in the 38.413 Path Swich Request procedure, and also in the 38.423 Handover Preparation procedure.

# 2. Discussion

As the 33.501 has a requirement that

- “The SMF shall provide UP security policy for a PDU session to the ng-eNB/gNB during the PDU session establishment procedure”

- “in the Path-Switch message, the target ng-eNB/gNB shall send the UE's UP security policy and corresponding PDU session ID received from the source gNB to the SMF”

- “At an N2-handover the SMF shall send the UE's UP security policy to the target ng-eNB/gNB via the target AMF.”

There is an ambiguity how would the gNB behave if the Security Indication IE is not present, how would the SMF behave if the Security Indication IE is not present.

This issue was detected and discussed in RAN3#101 ([1], [2]), once was proposed to change the presence of the Security Indication IE from optional to mandatory but was not agreeable. Later in RAN3#101bis a proposal [3] to clarify if the Security Indication IE is not present, the receiving side (the gNB) should consider that the user plane security is not needed. This proposal was only given in the chair note.

We believe that this is a very important clarification which should not just ended in the chair note, this should be given in the specification, simply only because we are in the 3GPP to make the world class specifications that are able for the 3GPP mobile system to work properly, without causing any unnecessary interoperability problems.

Therefore, though it was already discussed and agreed, we still propose to give the wording in the specification,

# 3. Conclusion

In order to avoid unnecessary interoperability issues, though it has been once discussed and agreed, it is still needed to specify the gNB behavior when the Security Indication IE is not present during the PDU session establishment procedure.

Proposal : In order to avoid conflict understanding with the requirement in 33.501,

- For NGAP:

  - To clarify that when the Security Indication IE is not present in the PDU Session Resource Setup Request Transfer IE of the PDU SESSION RESOURCE SETUP REQUEST message, the NG-RAN node shall interpret that the UP security policy as “not needed” for both Integrity Protection Indication and Confidentiality Protection Indication.

  - (Note: the behavior in the Handover Resource Allocation procedure for the PDU session setup is referring to the PDU Session Resource Setup procedure, therefore no additional text is needed for Handover Resource Allocation procedure)

  - To clarify that when the User Plane Security Information IE is not present in the Path Switch Request Transfer IE of the PATH SWITCH REQUEST message, the SMF shall interpret that the UP security policy as “not needed” for both Integrity Protection Indication and Confidentiality Protection Indication, and security result as “not performed” for both Integrity Protection Result and Confidentiality Protection Result.

- For XnAP:

  - To clarify that when the Security Indication IE is not present in the PDU Session Resource To Be Setup List IE of the HANDOVER REQUEST message, the target NG-RAN node shall interpret that the UP security policy as “not needed” for both Integrity Protection Indication and Confidentiality Protection Indication.

Proposal 2: to agree the corresponding CRs to 38.413 and 38.423.

# References

[1] R3-184575,

[2] R3-244629

[3] R3-245785

[4] RAN3#101 chair note

[5] RAN3#101bis chair note

[6] 33.501

[7] 38.413

[8] 38.423

Annex: Quoted from 33.501

## 6.6	UP security mechanisms

### 6.6.1	UP security policy

The SMF shall provide UP security policy for a PDU session to the ng-eNB/gNB during the PDU session establishment procedure as specified in TS 23.502 [8].

The UP security policy shall indicate whether UP confidentiality and/or UP integrity protection shall be activated or not for all DRBs belonging to that PDU session. The UP security policy shall be used to activate UP confidentiality and/or UP integrity for all DRBs belonging to the PDU session.

The ng-eNB/gNB shall activate UP confidentiality and/or UP integrity protection per each DRB, according to the received UP security policy, using RRC signalling as defined in clause 6.6.2. If the user plane security policy indicates "Required" or "Not needed", the ng-eNB/gNB shall not overrule the UP security policy provided by the SMF. If the ng-eNB/gNB cannot activate UP confidentiality and/or UP integrity protection when the received UP security policy is "Required", the ng-eNB/gNB shall reject establishment of UP resources for the PDU Session and indicate reject-cause to the SMF. If the received UP security policy is "Not needed ", then the establishment of the PDU Session shall proceed as described in TS 23.502 [8]. Only if the UE indicates that it supports use of integrity protection with ng-eNB, the ng-eNB can activate UP integrity protection.

NOTE 1: 	Local SMF can override the confidentiality option in the UP security policy received from the home SMF based on its local policy, roaming agreement and/or regulatory requirements.

At an Xn-handover from the source ng-eNB/gNB to the target ng-eNB/gNB, the source ng-eNB/gNB shall include in the HANDOVER REQUEST message, the UE's UP security policy. If the UP security policy is ‘Required’, the target ng-eNB/gNB shall reject all PDU sessions for which it cannot comply with the corresponding received UP security policy and indicate the reject-cause to the SMF. For the accepted PDU sessions, the target ng-eNB/gNB shall activate UP confidentiality and/or UP integrity protection per DRB according to the received UE's UP security policy and shall indicate that to the UE in the HANDOVER COMMAND by the source ng-eNB/gNB. Only if the UE indicates that it supports use of integrity protection with ng-eNB, the target ng-eNB can activate UP integrity protection.

If the UE receives an indication in the HANDOVER COMMAND that UP integrity protection and/or UP encryption for a PDU session is enabled at the target ng-eNB/gNB, the UE shall generate or update the UP encryption key and/or UP integrity protection key and shall activate UP encryption and/or UP integrity protection for the respective PDU session.

NOTE 2:	If the security policy is ‘Preferred’, it is possible to have a change in activation or deactivation of UP integrity after the handover.

Further, in the Path-Switch message, the target ng-eNB/gNB shall send the UE's UP security policy and corresponding PDU session ID received from the source ng-eNB/gNB to the SMF. The SMF shall verify that the UE's UP security policy received from the target ng-eNB/gNB is the same as the UE's UP security policy that the SMF has locally stored. If there is a mismatch, the SMF shall send its locally stored UE's UP security policy of the corresponding PDU sessions to the target ng-eNB/gNB. This UP security policy information, if included by the SMF, is delivered to the target ng-eNB/gNB in the Path-Switch Acknowledge message. The SMF shall support logging capabilities for this  event and may take additional measures, such as raising an alarm.

If the target ng-eNB/gNB receives UE's UP security policy from the SMF in the Path-Switch Acknowledge message, the target ng-eNB/gNB shall update the UE's UP security policy with the received UE's UP security policy. If UE's current UP confidentiality and/or UP integrity protection activation is different from the received UE's UP security policy, then the target ng-eNB/gNB shall initiate intra-cell handover procedure which includes RRC Connection Reconfiguration procedure to reconfigure the DRBs to activate or de-activate the UP integrity/confidentiality as per the received policy from SMF.

In case of the target ng-eNB/gNB receives both UE security capability and UP security policy, then ng-eNB/gNB initiates the intra-cell handover procedure which contains selected algorithm and an NCC to the UE.  New UP keys shall be derived and used at both the UE and the target ng-eNB/gNB.

At an N2-handover the SMF shall send the UE's UP security policy to the target ng-eNB/gNB via the target AMF. The target ng-eNB/gNB shall reject all PDU sessions for which it cannot comply with the corresponding received UP security policy and indicate the reject-cause to the SMF via the target AMF. For all other PDU sessions, the target ng-eNB/gNB shall activate UP confidentiality and/or UP integrity protection per DRB according to the received UE's UP security policy. Only if the UE indicates that it supports use of integrity protection with ng-eNB, the target ng-eNB can activate UP integrity protection.

At interworking-handover from EPS to 5GS, the SMF+PGW-C provides the UE's UP security policy to the target ng-eNB/gNB via the target AMF. The target ng-eNB shall determine from the UP security policy received from the AMF together with the UE indication that it supports user plane integrity protection with ng-eNB in UE EPS security capabilities (i.e. bit EIA7), whether to activate user plane integrity protection with the UE or not. The target ng-eNB/gNB shall reject all DRBs for which it cannot comply with the corresponding UP integrity protection policy in the UP security policy and indicate the reject-cause to the source MME via the target AMF. For all other DRBs, the target ng-eNB/gNB shall activate UP integrity protection per DRB according to the used UP security policy. Only if the UE indicates that it supports use of user plane integrity protection with ng-eNB, the target ng-eNB can activate UP integrity protection. If the target AMF detects in a Registration procedure following interworking-handover from EPS to 5GS, and becomes aware of that there is a mismatch between the UE EPS security capabilities received from the source MME and the one received from the UE, and that the target ng-eNB may not have the UE capability indicating UP IP support in UE EPS security capabilities, then the AMF shall send an N2 CONTEXT MODIFICATION REQUEST message to inform the target ng-eNB about the correct UE EPS security capabilities and target ng-eNB shall take the new UE EPS security capabilities into account.