3GPP TSG-RAN WG3 #125	R3-244152

Maastricht, The Netherlands, 19th - 23rd August 2024

Agenda item:		13.2 Support for inter-CU LTM

Title:		inter-CU LTM issues

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

For the Rel-19 Inter-CU LTM, last RAN3#124 has reached some basic principles, which is a good progress for further work in RAN3. Here continue to discuss and proposal that can a base for further work.

13. NR Mobility Enhancements WI WID [NR_Mob_Ph4-Core]: RP-241515 (target: RAN #109) [TU: 1 ( 1, 1, 1, 1)] QUOTA: 3

13.1. General Time plan, skeletons, BLs

13.2. Support for inter-CU LTM Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - As secondary priority, support the case when NR-DC is configured and CU is acting as SN and MCG is unchanged - As secondary priority, support the case when NR-DC is configured, CU is acting as MN and SCG is unchanged or SCG is released Note: The case that LTM is configured in both MCG and SCG is excluded - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM Coordination with SA3 needed with respect to security key handling Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support RAN3#123bis: Prioritize to support inter-CU LTM over Xn interface, and RAN3 specify the inter-CU LTM solutions for standalone scenario first. Reuse existing Xn Handover Request and Handover Request ACK for Inter-CU LTM initial preparation. Confirm the case that inter-CU LTM is not configured in both MCG and SCG at the same time. Early data forwarding can be supported for inter-CU LTM. Cell Switch Notification from source DU to target DU (in different gNB from source) for LTM execution. RAN3#124: Source gNB-CU initiates the handover preparation procedure for inter-gNB-CU LTM. Introduce a new procedure on Xn to transfer the TA information. A new XnAP class 2 procedure, namely LTM Cell Switch Notification is introduced on Xn to forward the target Cell ID and target TCI state ID(s) from the source gNB-CU to the target gNB-CU. Reuse Handover Success procedure over Xn for Rel-19 Inter-CU LTM, to tell the source CU that the UE has accessed to the target Cell. The Handover Cancel message is reused by the source gNB to release the reserved resource for LTM candidate cells in the candidate gNBs. Early data forwarding can be triggered before the Source gNB triggers a MAC CE Command to the UE to change cells, timing is left to implementation. Early sync configuration (TCI state and RACH configuration) can be obtained during the LTM preparation phase through the handover request and handover request acknowledge messages. A candidate gNB can initiate cancellation of configured LTM candidate cell(s) of its own. Details are FFS. FFS on the LTM modification procedures. FFS on whether to reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exist in the source gNB.

13.3. Support for Conditional LTM Specify support of conditional LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Prioritize intra-CU LTM - Checkpoint to review objective at RAN#105. RAN WG work to not start before this checkpoint

# 2. Discussion

2.1	Information in LTM Preparation Phase

It has been agreed to reuse existing Xn Handover Request and Handover Request Acknowledge message for inter-CU LTM initial preparation, last meeting also agreed that some information will be needed to be exchanged between source CU and candidate CUs, i.e. “Early sync configuration (TCI state and RACH configuration) can be obtained during the LTM preparation phase through the handover request and handover request acknowledge messages.”.

For the inter-CU LTM, the configuration information exchanged between the source CU to the target CU, here we need to confirm the LTM Configuration ID is allocated by the source CU or target CU. The LTM Configuration ID is the RRC LTM-CandidateID which indicates the index of the candidate configuration of the target cell, it may be easy just for the source CU to allocate the LTM Configuration ID and the target CU simply follow and then forward to the candidate cells in the target DUs.

Proposal 1: To confirm that the LTM Configuration ID is allocated by the source CU and transfer from the source CU to candidate CU, target CU simply forward to the candidate cells in candidate DUs.

2.2	Inter-CU LTM modification before LTM Execution Phase

Last meeting has the following FFS: FFS on the LTM modification procedures.

Modification from source CU to candidate CU:

The motivation of the LTM modification from source CU to candidate CU before LTM Execution phase, has been discussed as “For LTM, the source node may decide to modify an already prepared LTM candidate cell in the candidate gNB, e.g., the source gNB may update the LTM reference configuration and provide the updates to the candidate gNB.”.

This is a considerable use case, we need to introduce Modification function from source CU to candidate CU before LTM Execution Phase.

The next question is what kind of procedures in Xn to use? Two thinkable options are

•	Option 1: Reuse the current HANDOVER REQUEST/HANDOVER REQUEST ACKNOWLEDGE message to trigger the LTM modification.

•	Option 2: Introduce new class-1 XnAP message (i.e. LTM Modification Request and LTM Modification Request Acknowledge) to trigger the LTM modification.

If existing signalling procedure can be reused, it would be preferable to do so. On the other hand, it also needs to consider whether reusing of existing signalling procedure can cause significant complexity of the existing signalling message. For the option 1, it would be needed to differentiate whether the HANDOVER REQUEST is requesting for newly preparing candidate cell or modification of already prepared candidate cell, this is not so difficult as the mandatory Target Cell Global ID IE can be used to differentiate so if the Target Cell Global ID has been already existing with the same NG-RAN node UE XnAP ID, and similar to existing Conditional HO, further indicator can be added e.g. to add new LTM Indicator IE with values of “LTM-initiation” and “LTM-modification”. With this availability there will be no need to introduce new procedure, while introduce new procedure (option 2) may be clearer way.

Proposal 2: introduce Modification from source CU to candidate CU before LTM Execution phase, and use of existing the current HANDOVER REQUEST/HANDOVER REQUEST ACKNOWLEDGE message to trigger the LTM modification.

LTM Modification from candidate CU to source CU:

Whether need to have modification from candidate CU? One motivation of the LTM modification from candidate CU to source CU before LTM Execution phase, may be because the situation in candidate is changed e.g. the load status of the candidate cell becomes worse. If this will be main motivation, then the candidate CU can trigger cancel request towards the source CU. Consider the LTM is controlled mainly in source CU, we would think there is no need for Modification from candidate CU to source CU.

Proposal 3: No need to introduce LTM modification from candidate CU to source CU, if only request to release the prepared cells, the candidate CU can trigger cancel request towards the source CU.

2.3	LTM Cancel before LTM Execution Phase

Last meeting has the following agreement and FFS:

The Handover Cancel message is reused by the source gNB to release the reserved resource for LTM candidate cells in the candidate gNBs.

A candidate gNB can initiate cancellation of configured LTM candidate cell(s) of its own. Details are FFS.

LTM cancel from Source CU to candidate CU:

There will be a situation that the source CU decide to cancel the prepared candidate cells e.g. due to sudden change of radio condition in UE or any other local policy in source CU, so LTM cancel from source CU to candidate CU will be needed, and RAN3 has already agreed The Handover Cancel message is reused by the source gNB to release the reserved resource for LTM candidate cells in the candidate gNBs..

LTM cancel from candidate CU to source CU:

The motivation to have LTM cancel from candidate CU to source CU. may be because the situation in candidate is changed e.g. the load status of the candidate cell becomes worse and becomes no further capability for LTM purpose. RAN3 has agreed and FFS that A candidate gNB can initiate cancellation of configured LTM candidate cell(s) of its own. Details are FFS..

Then further discuss which signalling procedure to use for this LTM cancel from candidate CU to source CU.

•	Option 1: Reuse the CHO Cancel procedure, which include the Candidate Cells To Be Cancelled List IE.

•	Option 2: Introduce new XnAP message for the candidate node(s) to release the resources of prepared cells.

Again that If existing signalling procedure can be reused, it would be preferable to do so. The existing Conditional Handover Cancel procedure is not so complicated, which is easy to be reused.

Proposal 4: to use existing Conditional Handover Cancel procedure for LTM cancel from candidate CU to source CU.

2.4	LTM Release of ex-serving CU after the LTM Completion Phase

Last meeting has the following FFS: FFS on whether to reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exist in the source gNB.

The condition of this issue is after the LTM Completion Phase i.e. after the UE has been cell switched to target CU, the ex-serving cells (of the ex-serving CU) can be kept for subsequent LTM. However at the time the UE successfully access to the new serving cell, if the new serving CU decides that there is no need to keep the ex-serving CU, whether to use UE Context Release message?

For the legacy handover procedure, after the target gNB has completed Path Switch towards CN, it initiates UE Context Release towards the source gNB. If we will follow this procedure sequence, then it would be natural to use UE Context Release procedure. There is also one other possibility to use Handover Cancel procedure later. We simply just need to decide one principle, e.g. Handover Cancel procedure is used for the source gNB (or source CU) to cancel some of prepared Cells that was requested by the source gNB, the UE Context Release procedure is used to release the UE context of a gNB (i.e. release all prepared cells).

Proposal 5: To reuse the existing XnAP UE Context Release procedure at the source gNB if no LTM candidate cell(s) exist.

# 3. Conclusion

Proposal 1: To confirm that the LTM Configuration ID is allocated by the source CU and transfer from the source CU to candidate CU, target CU simply forward to the candidate cells in candidate DUs.

Proposal 2: introduce Modification from source CU to candidate CU before LTM Execution phase, and use of existing the current HANDOVER REQUEST/HANDOVER REQUEST ACKNOWLEDGE message to trigger the LTM modification.

Proposal 3: No need to introduce LTM modification from candidate CU to source CU, if only request to release the prepared cells, the candidate CU can trigger cancel request towards the source CU.

Proposal 4: to use existing Conditional Handover Cancel procedure for LTM cancel from candidate CU to source CU.

Proposal 5: To reuse the existing XnAP UE Context Release procedure at the source gNB if no LTM candidate cell(s) exist.

# References

[1] RP-241515, Revised WID: NR mobility enhancements Phase 4

[2] R3-243836, Summary of offline for R19 LTM procedure

The following LTM related IEs for Rel-18 intra-CU LTM in F1AP UE CONTEXT SETUP REQUEST message:

| LTM InformationSetup |  | 0..1 |  |  | YES | reject |
| --- | --- | --- | --- | --- | --- | --- |
| >LTM Indicator | M |  | ENUMERATED (true, …) |  | - |  |
| >Reference Configuration | O |  | 9.3.1.292 |  | - |  |
| >CSI Resource Configuration | O |  | 9.3.1.330 |  | - |  |
| LTM Configuration ID Mapping List | O |  | 9.3.1.294 |  | YES | reject |
| Early Sync Information Request |  | 0..1 |  |  | YES | ignore |
| >Request for RACH Configuration | M |  | ENUMERATED (true, …) |  | - |  |
| >LTM gNB-DUs List |  | 1 |  | This IE contains the IDs of the source gNB-DU and candidate gNB-DU(s). | YES | reject |
| >>LTM gNB-DUs Item IEs |  | 1..< maxnoofLTMgNBDUs> |  |  | - |  |
| >>>LTM gNB-DU ID | M |  | gNB-DU ID 9.3.1.9 |  | - |  |

The following LTM related IEs for Rel-18 intra-CU LTM in F1AP UE CONTEXT SETUP RESPONSE message:

| Early Sync Information |  | 0..1 |  |  | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |
| >TCI States Configurations List | M |  | OCTET STRING | Includes the LTM-TCI-Info IE, as defined in TS 38.331 [8]. | - |  |
| >Early UL Sync Configuration | O |  | 9.3.1.328 |  | - |  |
| >Early UL Sync Configuration for SUL | O |  | Early UL Sync Configuration 9.3.1.328 | This IE applies for SUL carrier. | - |  |
| LTM Configuration |  | 0..1 |  |  | YES | ignore |
| >SSB Information | M |  | 9.3.1.202 | Includes the SSB Information for the requested target cell. | - |  |
| >Reference Configuration Information | O |  | OCTET STRING | Includes the CellGroupConfig IE, as defined in TS 38.331 [8]. | - |  |
| >Complete Configuration Indicator | O |  | ENUMERATED (complete, ...) |  | - |  |
| >LTM CFRA Resource Configuration | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. | - |  |
| >LTM CFRA Resource Configuration for SUL | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. This IE applies for SUL carrier. | - |  |