3GPP TSG-RAN WG3 #125 meeting	R3-244154

# Maastricht, The Netherlands, 19th Aug – 23rd Aug, 2024

Agenda item:		12.2

Source:		NEC

Title:		Discussion on the architecture and mobility for WAB

Document for:		Discussion and decision

# 1. Introduction

In this contribution, we shall continue to discuss the architecture and mobility for WAB based on the agreements and open issues from the last RAN3 meeting [1] and provide the TP to TR38.799.

Whether a BH RAN node can be unaware of WAB needs to be further studied. Establishment of Xn Connections of the WAB-gNB with BH-RAN nodes, as well as with surrounding RAN nodes, is supported and can follow legacy procedures. Addition of new information in the legacy procedures is not precluded If needed, the WAB-gNB may power up one or more new cells with new configuration parameters related to its current location and handover UEs between the old and new cell served by the WAB-gNB. FFS if this procedure is needed for AMF relocation. Discuss whether in case this procedure is used for AMF relocation, new cells have to belong to different logical gNBs or can belong to the same gNB. Finalize the TP and conclusion.

# 2. Discussion

2.1 WAB functionalities aspects.

The last meeting discussed whether a BH-RAN-node can be unaware of WAB. From our understanding, any WAB-specific enhancement in RAN requires upgrade of legacy RAN node, for example, WAB-specific information may be exchanged between WAB-MT and BH-RAN-node or between RAN nodes. If a WAB-MT can access a legacy RAN node that is unaware of WAB, we should discuss the functionalities limitation and also whether the performance is acceptable.

Proposal 1: RAN 3 to discuss whether a WAB-MT can access a legacy RAN node by considering functionalities limitation and acceptable performance.

Meanwhile, if WAB-specific enhancement is introduced in the Uu interface, it’s beneficial for BH-RAN-node to broadcast a WAB-support indication (e.g., wab-Support) via system information. The WAB-MT may prioritize a WAB-support cell as a candidate for cell selection, which provides better performance for WAB. Furthermore, we think broadcasting of this indication is not mandatory for a RAN node.

Proposal 2: BH-RAN-node may broadcast a WAB-support indication via system information.

For the access of WAB-MT, it can use the same way as legacy UEs to connect to the network. To let the network know that the connected UE is a WAB-MT and to assist the network in selecting an AMF supporting WAB, the WAB-MT may indicate a WAB-node indication to the network in an early phase, for example, during random access procedure or in Msg5 (e.g., RRCSetupComplete message).

Proposal 3: WAB-MT to indicate a WAB-node indication to the network during random access procedure or in Msg5.

A WAB-node may serve one or more cells and all the data scheduled by the cells shall be aggregated on WAB-MT, as a result, the Uu link between WAB-MT and BH-RAN-node is highly likely to be the bottleneck for data transmission, and congestion may happen if the data load is huge. Hence, flow control for WAB-MT should be considered to improve user experience. For example, the traffic load or estimated traffic load of the cells served by a WAB-node may be reported to BH-RAN-node.

Proposal 4: Flow control for WAB-MT should be considered to avoid congestion on the Uu link between WAB-MT and BH-RAN-node.

2.2 Protocol stacks for Xn.

It was agreed in the last RAN3 meeting that the establishment of the Xn Connections of WAB-node with BH-RAN-nodes, as well as with surrounding RAN nodes, shall be supported. Although protocol stacks for the Xn-C/Xn-U interface over PDU session have been captured in TR 38.799 [2], it was not fully discussed in the last meeting.

For Xn-C/Xn-U transmission between WAB-node and BH-RAN-node, there is a direct Xn connection. If a PDU session-based solution is used, the Xn data shall be forwarded to the BH-UPF via the BH-RAN-node and then back to the BH-RAN-node. The whole path may be redundant and bring extra delay.

Figure 1: Protocol stacks for Xn-C/Xn-U interfaces between WAB-node and BH-RAN-node

To overcome the drawback, the protocol stacks of Xn-C/Xn-U interfaces illustrated in Figure 1 can be considered, for which the Xn-C/Xn-U data is exchanged directly between WAB-node and BH-RAN-node without the participation of UPF. As illustrated in Figure 1, XnAP or GTP-U encapsulation is performed by WAB-node or BH-RAN-node. Based on the IP address of the packet, the WAB-node or BH-RAN-node can determine whether the destination of the packet is itself.

Proposal 5: RAN3 to consider the candidate protocol stacks in Figure 1 for the Xn interface between WAB-node and BH-RAN-node, which uses the direct Xn connection instead of the PDU session.

For the Xn interface between WAB-node and Neighbour NG-RAN node (i.e., RAN node other than BH-RAN-node), backhauling of XnAP or GTP-U packets over BH-RAN-node is required. Different from Xn transmission over PDU session as captured in the TR 38.799, another alternative (i.e., Alt1) of the protocol stack is illustrated in Figure 2, which considers the connection between BH-RAN-node and Neighbour NG-RAN node. In this alternative, XnAP or GTP-U encapsulation is performed by WAB-node or Neighbour NG-RAN node. The IP packets with XnAP or GTP-U data are forwarded as user-plane data over the Uu interface between WAB-node and BH-RAN-node, and then forwarded as user-plane data over the Xn-U interface between BH-RAN-node and Neighbour NG-RAN node.

Figure 2: Protocol stacks for Xn interface between WAB-node and Neighbour NG-RAN node (Alt 1)

Figure 3: Protocol stack for Xn-C interface between WAB-node and Neighbour NG-RAN node (Alt 2)

To provide higher reliability for the transmission of XnAP message over the Uu interface, the alternative 2 of protocol stack is proposed to enable the Xn-C interface between WAB-node and Neighbour NG-RAN node, which is illustrated in Figure 3. In this alternative, The XnAP message (encapsulated in IP) shall be transmitted via an RRC signalling (e.g., in a transparent container) between WAB-node and BH-RAN-node.

Proposal 6: RAN3 to consider the candidate protocol stacks for Xn interface between WAB-node and Neighbour NG-RAN node in Figure 2 and Figure 3:

- Alt1: Xn-C/Xn-U data is transmitted through the connection between BH-RAN-node and Neighbour NG-RAN node rather than PDU session.

- Alt2: The XnAP message is transmitted via an RRC signalling (e.g., in a transparent container) over the Uu interface between WAB-node and BH-RAN-node

2.3 Aspects related to Mobility.

Furthermore, to support WAB, RAN3 should consider the necessary information exchanged between RAN nodes over Xn-AP, and the following information can be considered:

- WAB Node Indication which indicates that the handover is for a WAB-node, e.g., in XnAP: HANDOVER REQUEST message.

- BH PDU Session Indication may also be introduced to let the target RAN node know that a BH PDU Session should be setup for the WAB-node, e.g., in XnAP: HANDOVER REQUEST message.

- WAB Support Indication for a cell, e.g., in XN SETUP REQUEST and XN SETUP RESPONSE message. Based on it, the RAN node can determine whether to choose the corresponding cell as a candidate target cell for WAB-MT.

- The information of cells served by the WAB-node or the BH-RAN-node, e.g., in XN SETUP REQUEST and XN SETUP RESPONSE message. The RAN nodes may consider this information (e.g., cell-specific signal/channel configuration and multiplexing info) for scheduling, resource coordination, or resource multiplexing. Regarding the resource multiplexing for WAB, we think the solution for IAB can be used as a baseline for the WAB resource multiplexing.

Proposal 7: RAN3 to consider the following information exchanged between RAN nodes over Xn-AP to support the mobility of WAB-MT:

- WAB Node Indication and/or BH PDU Session Indication in XnAP: HANDOVER REQUEST message.

- WAB Support Indication and/or the information of cells served by the WAB-node or the BH-RAN-node in XN SETUP REQUEST and XN SETUP RESPONSE message.

WAB-node is typically worked as a mobile node to achieve vehicle-mounted relay. As the WAB-node moves, the configuration of WAB-gNB and the connected AMF may need to be updated, which shall further affect the mobility of on-board UEs. To support the mobility of a WAB-node together with its served UEs, two concurrent logical WAB-gNBs belonging to the same physical WAB-node can be introduced, which is similar to the solution for mobile IAB. During handover, the UEs connected to the WAB-gNB shall be handed over from the cell of the source logical WAB-gNB to the cell of the target logical WAB-gNB. In addition, the TP in the Annex to clause 4.3.4.2.2 of TR38.799 can be adopted for updating.

Proposal 8: Two concurrent logical WAB-gNBs belonging to the same physical WAB-node can be introduced to support the mobility of a WAB-node together with its served UEs.

The TP to TR38.799 for WAB, which captures the above analysis, is in the Annex.

Proposal 9: RAN3 to agree the TP to TR38.799 for WAB in Annex.

# Summary

This contribution provides our analysis of potential enhancements for WAB, and has the following proposals:

Proposal 1: RAN 3 to discuss whether a WAB-MT can access a legacy RAN node by considering functionalities limitation and acceptable performance.

Proposal 2: BH-RAN-node may broadcast a WAB-support indication via system information.

Proposal 3: WAB-MT to indicate a WAB-node indication to the network during random access procedure or in Msg5.

Proposal 4: Flow control for WAB-MT should be considered to avoid congestion on the Uu link between WAB-MT and BH-RAN-node.

Proposal 5: RAN3 to consider the candidate protocol stacks in Figure 1 for the Xn interface between WAB-node and BH-RAN-node, which uses the direct Xn connection instead of the PDU session.

Proposal 6: RAN3 to consider the candidate protocol stacks for Xn interface between WAB-node and Neighbour NG-RAN node in Figure 2 and Figure 3:

- Alt1: Xn-C/Xn-U data is transmitted through the connection between BH-RAN-node and Neighbour NG-RAN node rather than PDU session.

- Alt2: The XnAP message is transmitted via an RRC signalling (e.g., in a transparent container) over the Uu interface between WAB-node and BH-RAN-node

Proposal 7: RAN3 to consider the following information exchanged between RAN nodes over Xn-AP to support the mobility of WAB-MT:

- WAB Node Indication and/or BH PDU Session Indication in XnAP: HANDOVER REQUEST message.

- WAB Support Indication and/or the information of cells served by the WAB-node or the BH-RAN-node in XN SETUP REQUEST and XN SETUP RESPONSE message.

Proposal 8: Two concurrent logical WAB-gNBs belonging to the same physical WAB-node can be introduced to support the mobility of a WAB-node together with its served UEs.

Proposal 9: RAN3 to agree the TP to TR38.799 for WAB in Annex.

# References

[1] RAN3 #124 Meeting Report, Fukuoka, Japan, 20/05/2024 - 24/05/2024

[2] R3-243970, 3GPP TR 38.799, Study on additional topological enhancements for NR, V0.0.1

# Annex: TP to TR38.799

Start of Change

# X	Wireless Access and Backhaul (WAB)

## 4.2	WAB Architecture

Figure 4.2-4 shows protocol stack examples of Xn Control plane and User plane transport for WAB-node.

Figure 4.2-4: Protocol stack examples of Xn Control plane and User plane transport

Figure 4.2-x1 shows one alternative of protocol stack examples of Control plane and User plane transport for Xn interface between WAB-node and BH-RAN-node.

Figure 4.2-x1: Protocol stack examples of Control plane and User plane transport for Xn interface between WAB-node and BH-RAN-node.

Figure 4.2-x2 shows one alternative of protocol stack examples of Control plane and User plane transport for Xn interface between WAB-node and Neighbour NG-RAN node (Alt1).

Figure 4.2-x2: Protocol stack examples of Control plane and User plane transport for Xn interface between WAB-node and Neighbour NG-RAN node (Alt 1)

Figure 4.2-x3 shows another alternative of protocol stack example of Control plane transport for Xn interface between WAB-node and Neighbour NG-RAN node (Alt2).

Figure 4.2-x3: Protocol stack examples of Control plane transport for Xn interface between WAB-node and Neighbour NG-RAN node (Alt2)

##### 4.3.4.2.2 WAB-gNB mobility with change of UE’s AMF(s)

Due to WAB-node movement, the change of UE’s AMF(s) may be needed, based on, e.g., WAB-node’s current location and/or additional criteria. The NG connection handling and WAB-gNB configuration update may affect the served UEs.

- The WAB-gNB may obtain the configuration parameters needed to establish the connection to the UE’s new AMF(s).

- One or more new logical WAB-gNBs belonging to the same physical WAB-node shall be setup. And the new logical WAB-gNB(s) establishes NG connection(s) towards one or more new AMF(s).

- The new logical WAB-gNB(s) may activate one or more new cells, with new cell configuration parameters related to the WAB-gNB’s current location. The new cells may broadcast the radio parameters configured for the new AMF(s), e.g., TAC, etc. The old cell(s) remain(s) active.

- The UEs are handled as follows:

- A UE in RRC_CONNECTED state is handed over between an old cell served by the source logical WAB-gNB and a new cell served by the target logical WAB-gNB via NG-based handover with AMF relocation, as defined in TS 23.502.

- When all UEs in RRC_CONNECTED state have been handed over, the old cell(s) served by the source logical WAB-gNB are removed from service. A UE in RRC_IDLE or RRC_INACTIVE state camping on the old cell(s) served by the source logical WAB-gNB reselects a new cell served by the target logical WAB-gNB, and legacy procedure (e.g., Mobility Registration Update procedure as defined in TS 23.502) is performed.

- The NG connection(s) between the source logical WAB-gNB and the old AMF(s) are removed. The source logical WAB-gNB is removed by WAB-node.

### 4.3.x Flow control

A WAB-node may serve one or more cells and all the data scheduled by the cells shall be aggregated on WAB-MT, as a result, the Uu link between WAB-MT and BH-RAN-node is highly likely to be the bottleneck for data transmission, and congestion may happen if the data load is huge. Hence, flow control for WAB-MT should be considered to improve user experience. FFS how to perform flow control for WAB-MT.

End of Change