3GPP TSG-RAN WG3 #125	R3-244156

Maastricht, The Netherlands, 19th-23rd August 2024

Agenda item:		14.3

Title:		Discussion on NR NTN supporting regenerative NTN

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

In the previous two meetings, RAN3 got the following progresses on NR NTN regenerative payload enhancement:

There is no consensus to discuss new NTN architecture now; wait for an LS from SA2 on this particular issue. Continue to discuss the NG interface management solution based on requirement inputs from operators

RAN2 also have some progresses on it.

Agreements:

- Regarding potential issues with RRC re-establishment, RAN2 will wait for RAN3 input, if any

- Regarding potential issues with support of Inactive state, RAN2 will wait for RAN3 input, if any

- RAN2 aims at supporting RACH-less handover for regenerative architecture. Can further discuss whether any optimization is needed

- RAN2 assumes that network verified UE positioning is supported for regenerative architecture

- RAN2 assumes that the typical setting for common TA and kmac would be zero in case of regenerative mode with full gNB on board. FFS whether we capture in the spec that common TA and Kmac shall/should be set to zero.

There are some open issues in RAN3. And this paper will discuss on them.

# 2. Discussion

2.1 NG interface management

In the last meeting, RAN3 discussed how to handle when gNB leaves and revisit the serving area of the AMF. There are two camps: NG removal/setup and NG suspend/resume. Some companies are concerned about the NG suspend/resume methodology because the gNB and the AMF need to store the NG interface context, including the interface instance configuration and associated UE contexts.

However, in our understanding, the gNB and the AMF store interface configuration is enough to support the suspend/resume methodology. When the gNB revisits the serving area of AMF, the coverage of satellite gNB (and serving UEs) differs from the last geographical area (and corresponding UEs) the satellite gNB covers before it leaves. Therefore, it is not beneficial to store the UE context after the satellite gNB leaves the serving area of the AMF.

Proposal 1: The gNB and the AMF store the interface configuration is enough, and the UE context is unnecessary to support the suspend/resume methodology.

The gNB may store the following NG interface configurations to support NG interface suspend/resume:

- The AMF name. (This IE is used to identify the AMF uniquely.)

- The GUAMI (Globally Unique AMF Identifier) configured on that AMF Name.

- The set of TNL associations to be established between the NG-RAN node and the AMF.

- Relative AMF Capacity. (This IE indicates the relative processing capacity of an AMF with respect to the other AMFs in the AMF Set)

- The supported PLMN of this AMF.

The AMF may store the following NG interface configurations to support NG interface suspend/resume:

- The Global RAN Node ID

- RAN Node Name and Extended RAN Node Name

- Default Paging DRX

- The set of TNL associations to be established between the NG-RAN node and the AMF.

- The support TAI list of gNB before it leaves is not stored in the AMF.

Proposal 2: The NG interface configuration mentioned above should be considered to support the NG suspend/resume.

Since the CN knows the AMF's serving area and the satellite gNB's coverage, the NG interface may be resumed automatically when the TNL connection between gNB and AMF is established, and no extra NG signaling is needed. The TAC list may be updated after the NG-interface is resumed.

Proposal 3: The NG interface may be resumed automatically when the TNL connection between gNB and AMF is established, and no extra NG signaling is needed.

2.2 NG interface interruption handling for feeder link switch

In Rel-17/18, we defined hard feeder link switchover and soft feeder link switchover, where the hard feeder link switch requires the satellite to maintain only one feeder link at a time. This definition is also applicable to the regenerative payload in Rel-19. In the last meeting, RAN3 defined that "The regenerative payload terminates the Uu interface between the UE (via the service link) and the NG interface toward the 5GC (via the feeder link)" in BL CR[1].

The hard feeder link switchover between gNBs and NTN gateways may lead to an interruption of the NG interface (including NG-C and NG-U), and it may not have a direct impact on the Uu interface, which is different from the hard feeder link switchover in transparent payload.

Observation 1: The feeder link switchover in the regenerative payload will interrupt the NG interface, but the Uu interface may not be impacted.

Since the OAM can provide the time window (or the schedule) of the successive switchovers of the feeder link, the NG interface may be suspended during the hard feeder link switchover, and the Uu interface will not be impacted.

Proposal 4: The NG interface may be suspended during the hard feeder link switchover while the Uu interface may be unaware of the feeder link switchover interruption.

# 3. Conclusion

Observation 1: The feeder link switchover in the regenerative payload will interrupt the NG interface, but the Uu interface may not be impacted.

Proposal 1: The gNB and the AMF store the interface configuration is enough, and the UE context is unnecessary to support the suspend/resume methodology.

Proposal 2: The NG interface configuration mentioned above should be considered to support the NG suspend/resume.

Proposal 3: The NG interface may be resumed automatically when the TNL connection between gNB and AMF is established, and no extra NG signaling is needed.

Proposal 4: The NG interface may be suspended during the hard feeder link switchover while the Uu interface is unaware of the feeder link switchover interruption.

# References

[1] R3-243949 Support for Regenerative Payload in NR NTN