3GPP TSG-RAN WG3 #125	R3-244157

Maastricht, The Netherlands, 19th-23rd August 2024

Agenda item:		16.2

Title:		RAN architecture discussion on ambient IoT

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

In the last meeting, we agreed a CR R3-243972 [1] which includes a list of terminologies and a logical system architecture for AIoT topology 1. There are some open issues left, and this document will discuss on them.

# 2. Discussion

2.1 Logical System Architecture for AIoT topology 2

In the last meeting, RAN3 agreed on the logical system architecture for AIoT topology 1, as shown in the following figure:

Figure 6.4-1: Logical System Architecture for AIoT [1].

RAN3 defines the AIoT RAS (Radio Access System) instead of AIoT RAN because the “RAN” is always related to the NG-RAN node, while the RAS has a more general meaning beyond the NG-RAN node. Therefore, the definition of the command reader function is also applicable to UE acting as a reader. Also, RAN3 needs to consider the harmonized solution for both topology 1 and topology 2, so it is better to define unified logic function architecture for topology 1 and 2 to avoid any potential divergence caused by different architectures. Therefore, the logical system architecture for AIoT defined in Figure 6.4-1 of R3-243972 [1] is applicable to both topology 1 and topology 2. Moreover, the definition of common reader function should be updated to “a function that communicates with the AIoT device by means of AIoT radio, which can be included in NG-RAN node and UE”.

Proposal 1: The logical system architecture for AIoT defined in Figure 6.4-1 of R3-243962 is applicable to both topology 1 and topology 2.

Proposal 2: Updating the definition of common reader function: a function that communicates with the AIoT device by means of AIoT radio, which can be included in the NG-RAN node and UE.

2.2 Which logic function should support resources scheduling

Resource scheduling for R2D(DL) and D2R(UL) transmission of ambient IoT data and signalling should be performed in AIoT RAS. As defined in [1], the AIoT RAS is split into two parts:

Common reader function: a function that communicates with the AIoT device by means of AIoT radio. Editor’s Note 8: Further details on Common reader function is to be discussed by RAN1 and RAN2. AIoT RAN node function: a function residing in AIoT RAS. It contains e.g. the control of the AIoT radio resources used towards the AIoT device. Editor’s Note 9: further details are FFS. Note that “control of AIoT radio resources” does not necessarily imply dynamic configuration of resources but could also rely on static assignment of resources by means of OAM. Aspects concerning coordination of the Upper Layer functions (e.g. Inventory, Command) e.g. in case these functions have to be performed over a multitude of instances of the Common Reader Function are FFS.

The RAN2 defines the reader function includes a MAC layer (i.e., the common reader function is expected to have a MAC sublayer function). The radio resource scheduler is a function in the MAC sublayer. Therefore, the common reader function is possible to perform radio resource scheduling.

Then, in terms of the “control of AIoT radio resources” defined in the AIoT RAN node function, it has two interpretations:

- The AIoT RAN node function calculates the DL and UL radio resource scheduling, and the DL/UL scheduling configuration transmits from the AIoT RAN node function to the common reader function. Then, the common reader function uses the configured DL/UL radio resources to communicate with AIoT devices.

- The AIoT RAN node function assigns a static radio resource (e.g., a frequency range) to the common reader function. The common reader function will calculate the DL/UL resource scheduling in the given radio resource.

Therefore, whether the common reader function or AIoT RAN node function performs the AIoT resource scheduling should be discussed.

Proposal 3: RAN3 is suggested to discuss whether the common reader function or AIoT RAN node function performs the AIoT resource scheduling.

# 3. Conclusion

Proposal 1: The logical system architecture for AIoT defined in Figure 6.4-1 of R3-243962 is applicable to both topology 1 and topology 2.

Proposal 2: Updating the definition of common reader function: a function that communicates with the AIoT device by means of AIoT radio, which can be included in the NG-RAN node and UE.

Proposal 3: RAN3 is suggested to discuss whether the common reader function or AIoT RAN node function performs the AIoT resource scheduling.

# References

[1] R3-243972, (BL pCR to TR 38.769) Study on solutions for Ambient IoT in NR