3GPP TSG-RAN WG3 #125	R3-244158

Maastricht, The Netherlands, 19th-23rd August 2024

Agenda item:		16.3

Title:		RAN-CN interface impact on ambient IoT

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

For the previous meetings of the Rel-19, RAN3 got the following progress on ambient IoT RAN-CN interface enhancement:

Inventory over CN-RAN interface can be used for a single device, or a group of devices, or all devices. Command over CN-RAN interface can be used for a single device. FFS for command on a group of devices, or all devices. In the Inventory Request from the AIoT CN towards the AIoT RAS, it is FFS whether the following information may be included: Inventory Session ID (e.g., to allow multiple Inventory procedure instances to run in parallel): the purpose, scope and handling of this information in AIoT RAS are FFS. Estimated Number of expected responses from devices, or Minimum number of required responses from devices. FFS the usage of these information. Periodicity: to enable periodic inventory at AIoT RAS triggered by a single Inventory Request from the AIoT CN In Topology 2, it is FFS which entity (e.g., AIoT CN and/or AIoT enabled gNB) performs the UE reader selection. In topology1, it is FFS whether in response to a single Inventory Request, AIoT RAS provides only a single Inventory Report message or there can be also multiple Inventory Report messages provided.

There are some open issues left in RAN3. And this paper will discuss on them.

# 2. Discussion

2.1 Command on a group of devices or all devices

The last meeting discussed whether Command over AIoT CN-RAN interface can be used for a group of devices and/or all devices. In our understanding, a command used for a group of devices is sometimes demanded. For example, there is a group of AIoT sensors for data collection, and the CN want the AIoT sensors to report the stored data. Then, the CN can send a “Read” Command to the reader, and this “Read” Command can be sent to this group of AIoT sensors. However, we did not see any application scenario where the reader should send the Command to all devices. Since many AIoT devices work for different services and support different Commands, single Command for all devices may lead to unexpected failures or unwanted actions by the AIoT devices. Therefore, applying single Command to all AIoT devices of a reader is not reasonable.

Proposal 1: Command over the CN-RAN interface can be used for a group of devices but not all of devices.

In some cases, the AIoT-aware CN may want the single or group of AIoT devices to execute more than one Command in an inventory round. The AIoT-aware CN may send multiple Commands in sequence and each Command is in a CN-RAN message. Alternatively, multiple Commands may be included in a single CN-RAN message to reduce the signaling overhead.

Proposal 2: Single CN-RAN message may include multiple Commands to single or group of AIoT devices to reduce the signaling overhead.

2.2 The content of Inventory Request from the AIoT CN towards the AIoT RAS

The Inventory Session ID should be supported to uniquely identify the inventory round since there may be multiple inventory rounds in one CN-RAN interface instance.

The Estimated Number of Devices is beneficial to be included in the Inventory Request instead of the estimated number of expected responses from devices. For the estimated number of devices in the inventory request, the reader can generate the Query-like parameters based on the estimated number of devices. For example, if the estimated number is 100, the reader may generate a Query-like parameter value of 128 or 256.

If the inventory request does not include the estimated number of devices, the reader has to generate the Query-like value randomly. Device access conflicts may frequently occur if the query-like value is smaller than the actual number of devices. Moreover, if the Query-like value is much larger than the actual number of devices, the AIoT devices may need to wait significantly longer to access the reader. Therefore, the estimated number of devices can help the reader decide on an appropriate query-like value to optimize access conflict and time. Also, it can be used to inventory a group of devices and all of devices.

Regarding the Estimated Number of Expected Responses from devices, the number of devices' responses is highly related to the backscattering signal channel condition. The RAN2 does not exclude the device data retransmission and segmentation, so the data retransmission and segmentation has impact on the number of responses from devices. Therefore, the estimated number of expected responses from devices has a relatively lower correlation with the actual number of responses from devices. In addition, even if the CN can give a precise estimated number of expected responses from devices, we do not see obvious benefit from this number.

In addition, RAN3 has agreed to inventory a group of devices, so a Filter Criterion is necessary to filter the target group of devices. It could be a partial AIoT Device ID, and the AIoT Device whose Device ID matches the filter criteria will join this inventory round. Alternatively, a group of AIoT device IDs may be used instead of a filter criterion.

Moreover, the CN node ID and reader ID are necessary in the Inventory Request from the AIoT CN towards the AIoT RAS to uniquely identify the AIoT-aware CN node and the reader.

Proposal 3: The content of the Inventory Request from the AIoT CN towards the AIoT RAS is suggested to include the Inventory session ID, Estimated Number of Devices, Filter Criteria, CN node ID, and reader ID.

2.3 Which entity (e.g., AIoT CN and/or AIoT enabled gNB) performs the UE reader selection

In our understanding, both AIoT CN and AIoT-enabled gNB can perform the UE reader selection. The AIoT CN can send the AIoT reader configuration to a dedicated UE. Alternatively, the CN can send the AIoT reader configuration and the UE reader selection criteria to the gNB, and the gNB selects UE based on the UE reader selection criteria and then sends the AIoT reader configuration to the selected UE.

Proposal 4: AIoT CN and AIoT-enabled gNB can select UE readers.

Whether the AIoT CN and/or AIoT enabled gNB is supported to perform the UE reader selection, the AIoT CN should know if the AIoT service is provided by topology 2 because the UE reader selection is only applicable in topology 2. The CN may select the UE reader or send the UE reader selection criteria to the AIoT-enabled gNB when the CN is informed that topology 2 is desired.

Proposal 5: The AIoT CN should know the topology type to support AIoT CN and/or AIoT-enabled gNB UE reader selection.

2.4 Periodic inventory

The RAN2 has an agreement that "RAN2 assumes that the device will not support tracking/RAN area update procedure." For moving AIoT devices, the network cannot update the devices' location on time. Then, the CN may send the inventory or command message to readers not associated with AIoT devices. Suppose the CN triggers a periodic inventory of the readers in an area. In that case, the AIoT devices will respond to the network per each periodic interval so that the CN knows the association between readers and AIoT devices. So, periodic inventory is valuable to the device location.

Proposal 6: Support to enable periodic inventory at AIoT RAS triggered by a single Inventory Request from the AIoT CN.

# 3. Conclusion

Proposal 1: Command over the CN-RAN interface can be used for a group of devices but not all of devices.

Proposal 2: Single CN-RAN message may include multiple Commands to single or group of AIoT devices to reduce the signaling overhead.

Proposal 3: The content of the Inventory Request from the AIoT CN towards the AIoT RAS should include the Inventory session ID, Estimated Number of Devices, Filter Criteria, CN node ID, and reader ID.

Proposal 4: AIoT CN and AIoT-enabled gNB can select UE readers.

Proposal 5: The AIoT CN should know the topology type to support AIoT CN and/or AIoT-enabled gNB UE reader selection.

Proposal 6: Support to enable periodic inventory at AIoT RAS triggered by a single Inventory Request from the AIoT CN.

# References