3GPP TSG-RAN WG3 #125	R3-244159

Maastricht, The Netherlands, 19th-23rd August 2024

Agenda item:		16.4

Title:		Further discussion on Ambient IoT locating

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

For the previous meetings of the Rel-19, RAN3 got the following progress on ambient IoT locating enhancement:

AIoT device location information may be used for the following purposes: (a) improving the AIoT operation itself, e.g. by sending a Command to one or more readers (e.g., the last reader(s)) associated to the device rather than sending it blindly. (b) providing location information to the consumer of the AIoT service. Locating an Ambient IoT device at “reader ID granularity” is useful for both purposes. How to know the “reader” location is FFS. Whether and how to utilize more than one “readers” for location purposes is FFS depending on involved complexity. Signalling details on how to provide the location information to the A-IoT CN node.

There are some open issues left in RAN3. And this paper will discuss on them.

# 2. Discussion

2.1 More than one “readers” for location

RAN3 has agreed that locating an Ambient IoT device at "reader ID granularity," so the only scenario to utilize more than one reader for locating is that the AIoT device is in the overlapping service area of different readers. The AIoT device will respond to these readers' inventory, and the readers will send the AIoT device responses with the reader IDs to the AIoT CN. Then, the AIoT CN will know more than one reader associated with this AIoT device. Naturally, the AIoT CN can conclude the location of the AIoT device, which is expected to be the overlapping service area of these readers. The procedure described above has no impact on RAN3, and how the AIoT CN concludes the device location depends on the implementation of AIoT CN.

Observation 1: The only scenario to utilize more than one reader for locating is that the AIoT device is in the overlapping service area of different readers.

Proposal 1: Utilizing more than one "reader" for location purposes does not impact RAN3.

2.2 How to know the “reader” location

For topology 1, the common reader function in the AIoT enabled gNB, the reader location is expected to be the same as the location of AIoT enabled gNB’s TRP. Also, the CN (e.g., LMF) knows the location information of the TRPs.

If the CN assigns the reader IDs to the AIoT enabled gNB, the CN naturally knows the correlation between the AIoT enabled gNB’s TRPs and the reader IDs. Based on the above, the CN can conclude the reader's location. Therefore, for topology 1, if the CN assigns the reader IDs, the CN naturally knows the reader's location, and it does not impact RAN3.

If the CN does not assign the reader IDs (e.g., AIoT enabled gNB decides the reader IDs or the reader IDs are configured by the OAM), the readers need to report theirs reader IDs with corresponding TRPs information to CN. And then, the CN will know the mapping between TRPs location and reader location. Then, for topology 1, if the CN does not assign the reader IDs, readers need to report theirs reader IDs with corresponding TRPs information to CN as assistant information.

Observation 2: For topology 1, if the CN assigns the reader IDs, the CN knows the location of the TRPs and the mapping between the TRPs and the reader IDs. So, the CN has the location information of readers naturally.

Observation 3: For topology 1, if the CN does not assign the reader IDs, readers need to report theirs reader IDs with corresponding TRPs information to CN as assistant information. Then, the CN can conclude location information of readers.

Proposal 2: For topology 1, RAN3 need to discuss if the CN node or the AIoT enabled gNB assigns the reader IDs which decides how the CN knows the reader location.

For topology 2, the location of the reader should be the location of the UE reader. When a UE is configured as a reader, the positioning procedure needs to be triggered to acquire the location of this UE.

Proposal 3: For topology 2, the positioning procedure needs to be triggered to acquire the location of the UE when the UE is configured as a reader.

# 3. Conclusion

Observation 1: The only scenario to utilize more than one reader for locating is that the AIoT device is in the overlapping service area of different readers.

Observation 2: For topology 1, if the CN assigns the reader IDs, the CN knows the location of the TRPs and the mapping between the TRPs and the reader IDs. So, the CN has the location information of readers naturally.

Observation 3: For topology 1, if the CN does not assign the reader IDs, readers need to report theirs reader IDs with corresponding TRPs information to CN as assistant information. Then, the CN can conclude location information of readers.

Proposal 1: Utilizing more than one "reader" for location purposes does not impact RAN3.

Proposal 2: For topology 1, RAN3 need to discuss if the CN node or the NG-RAN assigns the reader IDs which decides how the CN knows the reader location.

Proposal 3: For topology 2, the positioning procedure needs to be triggered to acquire the location of the UE when the UE is configured as a reader.

# References