3GPP TSG-RAN WG3 #125	R3-244160

Maastricht, The Netherlands, Aug 19th – 23rd, 2024

Source:		NEC

Agenda item:		10.2. MRO Enhancements

Title:	MRO for LTM

Document for:		Discussion and decision

- Introduction

During previous RAN3 meetings, MRO enhancements for LTM were discussed, and the following agreements were made:

Work on scenarios of near failure LTM Work on scenarios for the differentiation of too early LTM, too late LTM and LTM to wrong cell RAN3 to prioritize MCG LTM over SCG LTM. RAN3 takes RAN2#125-bis agreement on MRO for LTM scenarios as baseline for further study. For failures due to wrong selection of candidate LTM cell, CU is in charge of root cause analysis and performs optimization. For failures due to inappropriate cell switch triggering (e.g. wrong cell selection at cell switch, wrong cell switch timing, …) source DU performs optimization. RAN3 to discuss enhancements related to the information of RACH-less access and RACH-based access from UE related to MRO for LTM.

In this contribution, we would like to provide our further consideration on MRO for LTM.

- Discussion

During last meeting, it was agreed that for failures due to inappropriate cell switch triggering (e.g., wrong cell selection at cell switch, wrong cell switch timing, …), source DU performs optimization. Currently, the gNB-CU can identify too late LTM cell switch, too early LTM cell switch or LTM cell switch to wrong cell, but the source-DU may not have the corresponding information. Therefore, F1AP enhancement is needed to provide the information of LTM cell switch failure to the source DU, i.e., one F1AP message can be transmitted from gNB-CU to the source gNB-DU to indicate the LTM failure information. For the content of the LTM failure information, the type of the LTM failure problem, the ID of the source cell, the target LTM cell, and re-established cell, and ID of the UE can be included. The F1AP message to indicate the LTM failure information can be ACCESS AND MOBILITY INDICATION.

Proposal 1: gNB-CU transmits, to the source gNB-DU, F1AP message including LTM failure information. RAN3 FFS on the content of the LTM failure information.

Additionally, for conventional mobility procedure, e.g., handover and PSCell change, the gNB-CU can perform MRO optimization and identifies ping-pong issue based on UE history information collected by the source gNB-CU or reported by the UE. However, for LTM cell switch, it is the gNB-DU makes decision on whether and when to trigger the cell switch. And if we only rely on existing mechanism, the gNB-DU will not have the UE history information, bad triggering time for LTM or ping-poing cell switch may happen.  Therefore, it is proposed that RAN3 considers F1 interface enhancements to transmit UE history information to the gNB-DU. And the possible F1AP message carrying UE history information may be CU-DU CELL SWITCH NOTIFICATION message.

Proposal 2: gNB-CU transmits, to the gNB-DU, F1AP message including UE history information.

For P1 and P2, the potential TP can be seen in Annex.

Proposal 3: RAN3 to consider the TP to TS 38.473 in Annex as a starting point.

To analysis near failure handover procedure, SHR and SPR were introduced with some triggering conditions, e.g., T310/T312/T304 triggers, to avoid storing and reporting information which are not necessary for the network. In legacy, the triggers are set by the gNB-CU. For LTM, one difference from the legacy handover procedure is that it is the source gNB-DU to triggers the execution of LTM cell switch, based on L1 measurement results from the UE. And should be the gNB-DU to perform the root cause analysis and optimization for the near failure of LTM cell switch. Therefore, which node determines the triggering condition and whether additional or new triggering condition are defined for optimization of near failure for LTM cell switch needs further discussion.

Proposal 4: RAN3 to discuss whether gNB-DU can determine the triggering condition of SHR or SPR for LTM cell switch.

# 3. Conclusion

In this paper, we provide our further consideration on MRO for LTM, and have the following proposals:

Proposal 1: gNB-CU transmits, to the source gNB-DU, F1AP message including LTM failure information. RAN3 FFS on the content of the LTM failure information.

Proposal 2: gNB-CU transmits, to the gNB-DU, F1AP message including UE history information.

Proposal 3: RAN3 to consider the TP to TS 38.473 in Annex as a starting point.

Proposal 4: RAN3 to discuss whether gNB-DU can determine the triggering condition of SHR or SPR for LTM cell switch.

# Annex: TP to TS 38.473

### 9.2.10	Self Optimisation Support Messages

#### 9.2.10.1	ACCESS AND MOBILITY INDICATION

This message is sent by gNB-CU to gNB-DU to provide access and mobility information to the gNB-DU.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| RA Report List |  | 0..1 |  |  | YES | ignore |
| >RA Report Item |  | 1 .. <maxnoofRAReports> |  |  | - |  |
| >>RA Report Container | M |  | OCTET STRING | Includes the RA-ReportList-r16 IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| RLF Report Information List |  | 0..1 |  |  | YES | ignore |
| >RLF Report Information Item |  | 1 .. <maxnoofRLFReports> |  |  | - |  |
| >>NR UE RLF Report Container | M |  | OCTET STRING | Includes the nr-RLF-Report-r16 IE contained in the UEInformationResponse message defined in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| Successful HO Report Information List |  | 0..1 |  |  | YES | ignore |
| >Successful HO Report Information Item |  | 1 .. <maxnoofSuccessfulHOReports> |  |  | - |  |
| >>Successful HO Report Container | M |  | OCTET STRING | Includes the SuccessHO-Report IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| Successful PSCell Change Report Information List |  | 0..1 |  |  | YES | ignore |
| >Successful PSCell Change Report information Item |  | 1..<maxnoofSuccessfulPSCellChangeReports> |  |  | - |  |
| >>Successful PSCell Change Report Container | M |  | OCTET STRING | Includes the SuccessPSCell-Report IE as defined in TS 38.331 [8]. | - |  |
| LTM Failure information | O |  | x.x.x.x | Includes information of a LTM cell switch failure event. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofRAReports | Maximum no. of RA Reports, the maximum value is 64. |
| maxnoofRLFReports | Maximum no. of RLF Reports, the maximum value is 64. |
| maxnoofSuccessfulHOReports | Maximum no. of Successful HO Reports, the maximum value is 64. |
| maxnoofSuccessfulPSCellChangeReports | Maximum no. of Successful PSCell Change Reports. Value is 64. |

#### 9.2.2.16	CU-DU CELL SWITCH NOTIFICATION

This message is sent by the gNB-CU to inform the gNB-DU about the initiation of the cell switch command to the UE.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Cell ID | M |  | NR CGI 9.3.1.12 |  | YES | reject |
| LTM Cell Switch Information |  | 0..1 |  |  | YES | ignore |
| >Joint or DL TCI State ID | M |  | OCTET STRING | Includes the TCI-StateId IE, as defined in TS 38.331 [8]. | - |  |
| >UL TCI State ID | O |  | OCTET STRING | Includes the TCI-UL-StateId IE, as defined in TS 38.331 [8]. | - |  |
| TA Information List |  | 0..1 |  |  | YES | ignore |
| >TA Information Item IEs |  | 1 .. <maxnoofTAList> |  |  | EACH | ignore |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>TA Value | M |  | INTEGER (0..4095) | Indicates the TA value as defined in TS 38.213 [31]. | - |  |
| UE History Information | O |  | y.y.y.y |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTAList | Maximum no. of TA values to be sent, the maximum value is 8. |

#### y.y.y.y	UE History Information

The UE History Information IE contains information about cells that a UE has been served by in active state prior to the target cell. The overall mechanism is described in TS 36.300 [12].

NOTE:	The definition of this IE is aligned with the definition of the UE History Information IE in TS 38.413 [5].

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Last Visited Cell List |  | 1..<maxnoofCellsinUEHistoryInfo> |  | Most recent information is added to the top of this list |
| >Last Visited Cell Information | M |  | z.z.z.z |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinUEHistoryInfo | Maximum number of last visited cell information records that can be reported in the IE. Value is 16. |