3GPP TSG-RAN WG3 Meeting #125	R3-244161

Maastricht, The Netherlands, 19th – 23rd August 2024

Agenda Item:		32

Source:		NEC

Title:			Input to 3GPP AI/ML Consistency Alignment SI

Document for:		Discussion and decision

# Introduction

A new Rel-19 SI on 3GPP AI/ML Consistency Alignment [1] was approved in SA#104 meeting. This study will investigate ongoing AI/ML work in TSG CT, TSG RAN and TSG SA Working Groups and identify instances of any potential misalignment and/or inconsistencies, with the following objectives included.

WT1: Identify AI/ML related activities of all working groups of Rel-18 features and Rel-19 studies and work items.

NOTE 1: The AI/ML related content captured in TR 21.918 ("Release 18 Description; Summary of Rel-18 Work Items") can be considered as a starting point.

WT2: Identify any potential inconsistencies on AI/ML related terminology (i.e. set of definitions, acronyms) across 3GPP, based on WT1.

WT3: Identify any potential misalignments and inconsistencies among existing AI/ML related features specified in 3GPP, including cross-domain (UE, RAN, core network, media, OAM, and application enablement) aspects. Examples of areas to be investigated are LCM for AI/ML, data collection/storage/exposure, model training/delivery/ (de)-activation/inference emulation, inference/storage/exposure, performance evaluation and accuracy monitoring.

NOTE 2:  Any RAN related aspects are subject to early coordination and feedback from TSG RAN.

WT4: Provide information on any potential outcome from WT1, WT2 and WT3 to the respective WGs (according to their Terms of Reference (ToR)) to resolve any issues with appropriate SA-level co-ordination as necessary.

NOTE 3: The study item does not impact ongoing studies and normative work for AI/ML across all SA/RAN/CT WGs for Rel-19.

In this contribution, we highlight the AI/ML related work accomplished by RAN3 e proposals for E2E AI/ML framework discussions and provide RAN3 input to the study.

# Discussion

## Background & motivation

The topic of E2E AI/ML framework has been a subject of some extended discussions at the 3GPP SA and RAN plenary TSG levels. The discussions were initially triggered by a discussion paper submitted to both RAN#102 [2] and SA#103 [3] by a group of operators. The discussion in SA#103 evolved towards a provisional agreement to start an SA level study aiming to track and investigate ongoing AI/ML related work in RAN and SA WGs and identify potential misalignments, overlap or inconsistency between solutions being developed by those working groups [4,5]. The discussion led to the approval of the new Rel-19 SI on 3GPP AI/ML Consistency Alignment [1]. The study aims at achieving consistent terminology and E2E framework, especially from the Life Cycle Management (LCM) perspective for AI/ML models and cross-domain training/inference data collection/storage.

## Input from RAN3 studies

The following two RAN3 studies are relevant to the study in the SA;

941010	Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN [6]

1020083	Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN [7]

This contribution primarily aims to highlight some key aspects and accomplishments by RAN3 on the specifications of AI/ML for NG-RAN for Rel-18 and also provide a summary of the RAN3 Rel-19 ongoing study on enhancements for AI/ML for NG-RAN, which are considered relevant to the ongoing SA discussions on the AI/ML Consistency Alignment.

## Rel-18 study

The Rel-18 study specifies the data collection enhancements and signalling support within existing NG-RAN interfaces and architecture (only limited to non-split architecture) for three use cases, including AI/ML-based Network Energy Saving, Load Balancing and Mobility Optimization.

The specifications TS38.300, TS38.401, TS38.420 and TS38.423 are impacted according to the study result in TR37.817.

The working assumption for RAN3 is that AI/ML algorithms and models are out of 3GPP scope, and the details of model performance feedback are also out of 3GPP scope [8]. Considering this statement is captured in TS38.300 and may affect other WGs, we need alignment with other WGs and decide whether to correct it.

Observation 1: The following working assumption in TS38.300 may affect other WGs.

“AI/ML algorithms and models are out of 3GPP scope, and the details of model performance feedback are also out of 3GPP scope”

The following terms and definitions have been agreed by RAN3 for the Rel-18 TS 38.300 [8], TS 38.401 [9], TS 38.423 [9].

AI	Artificial Intelligence

EC	Energy Cost

ML	Machine Learning

And the following AI/ML terminology references were added to TS 38.300 [10] and TS 38.401 [11] in last RAN3 meeting.

AI/ML Model Training follows the definition of “ML model training” as specified in clause 3.1 of TS 28.105.

AI/ML Model Inference follows the definition of “AI/ML inference” as specified in clause 3.1 of TS 28.105.

A relative LS has been sent to SA5, RAN2 (cc RAN1, RAN, SA) [12] to kindly ask SA5 to take the above into account and also pointed out the differences in terminology between ML Model Training and AI/ML Model Training used in SA5 and RAN3 respectively.

Observation 2: Different terminology are used in RAN3 and SA5.

RAN3: AI/ML Model Training; SA5: ML Model Training

RAN3: AI/ML Model Inference; SA5: AI/ML inference

A RAN intelligence functional framework to support AI/ML capability at NG-RAN was developed in the study phase.

Figure 4.2-1. Functional Framework for RAN Intelligence [13]

For the deployment of AI/ML in NG-RAN, the following scenarios are supported:

- AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the NG-RAN node.

- AI/ML Model Training and AI/ML Model Inference are both located in the NG-RAN node.

Observation 3: The RAN intelligence functional framework defined in RAN3 is generally inline with and can be regarded as part of the ML model lifecycle defined in SA5 [14].

Two signalling procedures used for the exchange of information to support AI/ML in NG-RAN are newly defined:

- Data Collection Reporting Initiation procedure to configure the collection and reporting of information.

- Data Collection Reporting procedure to perform the actual reporting.

For AI/ML-based NES, the metric of Energy Cost (EC), which can be used e.g. for the Network Energy Saving use case, is introduced as the AI/ML metric and can be exchanged within the neighbouring NG-RAN nodes upon request.

For AI/ML-based LB, the predicted resource status information and UE performance feedback can be configured to be collected and measured by an NG-RAN node.

For AI/ML-based MO, a cell-based UE trajectory prediction is transferred to the target NG-RAN node via the Handover Preparation procedure to provide information for subsequent mobility decisions. Cell-based UE trajectory prediction is limited to the first-hop target NG-RAN node. And the target NG-RAN node starts to collect the measured UE trajectory information as feedback information at successful Handover execution, and one-time reporting is supported.

Observation 4: The signalling procedures and the corresponding IEs defined to support AI/ML in NG-RAN should be RAN3 specific and not relative to the new SI.

## Rel-19 study

The Rel-19 study is to investigate two new AI/ML based use cases (Network Slicing and CCO) and identify enhancements to support AI/ML functionality, and further discussions on the Rel-18 leftovers (Mobility optimization for NR-DC, Split architecture support for Rel-18 use cases, Energy Saving enhancements and Continuous MDT collection).

The Rel-18 functional Framework and deployment scenarios are reused for Rel-19. Besides, RAN3 also investigated the deployment scenarios for split architecture and achieved the following agreement:

- AI/ML Model Training may be located in the OAM and AI/ML Model Inference may be located in the NG-RAN node (gNB-CU).

- AI/ML Model Training and AI/ML Model Inference may be both located in the NG-RAN node (gNB-CU).

For AI/ML-based network slicing, measured/predicted slice-level radio resource status and measured/predicted slice available capacity can be transferred between NG-RAN nodes to assist AI/ML network slicing. Besides, the granularity of UE performance feedback can be improved.

This study is still ongoing and focusing on discussing solutions and identifying specification impacts. The study result is captured in TR38.743. Until now, there is no potential misalignments and inconsistencies with other WGs identified.

Observation 5: No potential misalignments and inconsistencies with other WGs are identified from the ongoing Rel-19 study.

Since we believe that a summary/update on the work accomplished by RAN3 on AI/ML for NG-RAN, including AI/ML-related terms and procedures, would provide useful input to the newly started study on AI/ML Consistency Alignment, we propose RAN3 to agree the following proposal and provides input to SA.

Proposal 1: It is proposed that SI on 3GPP AI/ML Consistency Alignment takes into account the work done by RAN3 on the following aspects:

- AI/ML related terminologies;

- RAN intelligence functional framework to support AI/ML capability at NG-RAN and deployment scenarios;

- Signalling procedures used for the exchange of information to support AI/ML in NG-RAN and corresponding input/output/feedback.

Proposal 2: To agree the LS to SA in Annex.

# Proposals

In this contribution, we summarized the RAN1 progress on on-demand SSB and analysed the potential RAN3 impacts, and the following observations and proposals were drawn from the analysis.

Observation 1: The following working assumption in TS38.300 may affect other WGs.

“AI/ML algorithms and models are out of 3GPP scope, and the details of model performance feedback are also out of 3GPP scope”

Observation 2: Different terminology are used in RAN3 and SA5.

RAN3: AI/ML Model Training; AI/ML Model Inference

SA5: ML Model Training; AI/ML inference

Observation 3: The RAN intelligence functional framework defined in RAN3 is generally inline with and can be regarded as part of the ML model lifecycle defined in SA5 [12].

Observation 4: The signalling procedures and the corresponding IEs defined to support AI/ML in NG-RAN should be RAN3 specific and not relative to the new SI.

Observation 5: No potential misalignments and inconsistencies with other WGs are identified from the ongoing Rel-19 study.Proposal 1: RAN3 provides the following proposal to SA:

It is proposed that SI on 3GPP AI/ML Consistency Alignment takes into account the work done by RAN3 on the following aspects:

- AI/ML related terminologies;

- RAN intelligence functional framework to support AI/ML capability at NG-RAN and deployment scenarios;

- Signalling procedures used for the exchange of information to support AI/ML in NG-RAN and corresponding input/output/feedback.

Proposal 2: To agree the LS to SA in Annex.

# Reference

[1] SP-240970, Study on 3GPP AI/ML Consistency Alignment

[2] RP-233796, DP on Need for a consistent e2e AI/ML framework

[3] SP-240292, DP on Need for a consistent AI/ML E2E framework

[4] SP-240448, Way forward on AI/ML E2E framework

[5] SP-240575, Study on AI/ML E2E framework

[6] RP-233441, Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN

[7] RP-240323, New SID: Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN

[8] RP-233934, Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN, RAN2

[9] RP-234067, Support of AI/ML in NG-RAN, RAN3

[10] R3-243927, Correction on definition of AI/ML terminology

[11] R3-243928, Stage 2 corrections for AI/ML for NG-RAN

[12] R3-243969, LS on terminology definitions for AI-ML in NG-RAN, RAN3

[13] TR 37.817, Study on enhancement for Data Collection for NR and EN-DC

[14] TS 28.105, Artificial Intelligence / Machine Learning (AI/ML) management (Release 18)

# Annex LS to SA

3GPP TSG-RAN WG3 Meeting #125	R3-24xxxx

Maastricht, Netherlands, 19th – 23rd August 2024

Title:	[draft] LS on 3GPP AI/ML Consistency Alignment

Response to:

Release:	Rel-19

Work Item:	FS_AIML_CAL

Source:	NEC (to be RAN3)

To:	SA

Cc:

Contact person:	Hui Ma

hui.ma@emea.nec.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

In respect to the new Rel-19 SI on 3GPP AI/ML Consistency Alignment in SA, the following two RAN3 studies are involved.

941010	Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN

1020083	Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN

The R18 study (941010) specifies the data collection enhancements and signalling support within existing NG-RAN interfaces and architecture (only limited to non-split architecture) for three use cases, including AI/ML-based Network Energy Saving, Load Balancing and Mobility Optimization. The specifications TS38.300, TS38.401, TS38.420 and TS38.423 are impacted based on the conclusions captured in TR37.817.

The Rel-19 study (1020083) is to investigate two new AI/ML based use cases (Network Slicing and CCO) and identify enhancements to support AI/ML functionality, and further discussions on the Rel-18 leftovers (Mobility optimization for NR-DC, Split architecture support for Rel-18 use cases, Energy Saving enhancements and Continuous MDT collection). The study result is captured in TR38.743.

Based on the already completed normative specifications that have resulted from Rel-18 study and also considering the ongoing Rel-19 study, RAN3 would like to provide the followings key highlights as input to the 3GPP AI/ML Consistency Alignment in SA:

- The following working assumption in TS38.300 may be relevant to other WGs.

“AI/ML algorithms and models are out of 3GPP scope, and the details of model performance feedback are also out of 3GPP scope”

- RAN3 has been working closely with SA5 for the terminology development to avoid inconsistencies and avoid duplication. Nevertheless, in some instances RAN3 uses different terminologies, but nevertheless the actual definitions remain similar to those adopted by SA5. Typical examples are the following:

RAN3: AI/ML Model Training; SA5: ML Model Training

RAN3: AI/ML Model Inference; SA5: AI/ML inference

- The RAN intelligence functional framework defined in RAN3 is generally inline with and can be regarded as part of the ML model lifecycle defined in SA5.

- RAN3 had also investigated and agreed the deployment scenarios for split architecture and achieved the following agreement:

  - AI/ML Model Training may be located in the OAM and AI/ML Model Inference may be located in the NG-RAN node (gNB-CU).

  - AI/ML Model Training and AI/ML Model Inference may be both located in the NG-RAN node (gNB-CU).

Note: The Rel-18 functional Framework and deployment scenarios are also reused for the on-going Rel-19 work.

- The signalling procedures and the corresponding IEs defined to support AI/ML in NG-RAN should be RAN3 specific and not relative to the new SI.

- No potential misalignments and inconsistencies with other WGs are identified from the ongoing Rel-19 study.

RAN3 would therefore like to kindly ask the SI on 3GPP AI/ML Consistency Alignment takes into account the work done by RAN3 on the above aspects.

# 2	Actions

To SA:

ACTION: 	RAN3 kindly asks SA to take into account the work done by RAN3 on the following aspects and provide any follow up feedback if deemed necessary:

- AI/ML related terminologies;

- RAN intelligence functional framework to support AI/ML capability at NG-RAN and deployment scenarios;

- Signalling procedures used for the exchange of information to support AI/ML in NG-RAN and corresponding input/output/feedback.

# 3	Dates of next RAN3 meetings

Updated meeting schedule can be found at: https://portal.3gpp.org/?tbid=373&SubTB=381#/

RAN3#125bis		2024-10-14 - 2024-10-18		Hefei , CN

RAN3#126		2024-11-18 - 2024-11-22		Orlando, US