3GPP TSG-RAN WG3 Meeting #125	R3-244162

Maastricht, The Netherlands, 19th – 23rd August 2024

Agenda Item:		11.2

Source:		NEC

Title:			(TP to TR38.743) AI/ML based network slicing

Document for:		Discussion and decision

# Introduction

In this contribution, we continue discussing on AI/ML based network slicing based on the agreements and open issues from last RAN3 meeting [1] and provide the TP to TR38.743.

Agree to improve the granularity of UE performance feedback in Rel-19 for slicing.

FFS whether to include measured/predicted UE throughput per slice as input from local node.

FFS whether to introduce delivery of slice level UE throughput prediction/measurement during handover procedure.

FFS on SLA requirements fulfilment information exchange.

# Discussion

## 2.1	UE performance feedback

UE performance measurements metrics includes Average UE throughput DL/UL, Average packet delay DL/UL and Average packet loss DL, and the UE performance feedback can be collected at node level or cell level.

In order to perform optimal resource management and mobility decisions for network slicing, we already agreed that slice level radio resource status and slice available capacity are exchanged between nodes. As for the UE performance feedback collected from target node after UE handover, comparing to legacy UE performance feedback, slice level UE performance feedback has more advantages to evaluate and optimize the model performance.

Proposal 1: To support slice level UE performance feedback AI/ML-based network slicing.

## 2.2	Slice level UE throughput

As mentioned in 2.1, measured slice level UE throughput needs to be collected as UE performance feedback. In order to 1) monitor the UE performance by comparing the measured values with predicted values and 2) make the decision for slice level resource allocation action, measured and predicted slice level UE throughput also needs to be collected from local node as input data.

Proposal 2: To include measured/predicted UE throughput per slice as input from local node.

Considering that a UE-slice-MBR for an S-NSSAI is provide by AMF and gNB can use this information to decide whether a QoS Flow can be admitted or not during handover. And a UE-slice-MBR is more accurate then measured/predicted slice level UE throughput for mobility decision, so measured/predicted slice level UE throughput is not needed to be exchanged during handover.

Proposal 3: Not to support measured/predicted slice level UE throughput exchange during handover.

## 2.3	SLA requirements fulfilment

SLA is a contract between the Network Service Provider and the customer, that defines the performance standards that are to be met by the provider. To each network domain, SLA is break down to a set of attributes, e.g. PDCP throughput, PRB usage. SLA can be assured by dynamically controlling slice configurations based on e.g. slice level performance feedback.

OAM manages the life cycle of the end-to-end slice across the network domains and is in charge of slice configuration, slice resource allocation, slice performance management. SLA requirements fulfilment in RAN can be provided to OAM as input for AI/ML model training.

Proposal 4: SLA requirements fulfilment in RAN can be provided to OAM as input for AI/ML model training.

However, for each gNB, it can perform the prediction of e.g. slice-level radio resource status and then adjust the resource allocation accordingly by itself, therefore, it is no need to exchange SLA requirements fulfilment information between gNBs.

Proposal 5: Not to support SLA requirements fulfilment information exchange between gNBs.

The TP to TR38.743 for AI/ML based network slicing, which captures the above analysis, is in Annex.

Proposal 6: To agree the TP to TR38.743 for AI/ML based network slicing in Annex.

# Proposals

In this contribution, we provide our further analysis on the open issues of network slicing, and the following proposals were drawn from the analysis.

Proposal 1: To support slice level UE performance feedback AI/ML-based network slicing.

Proposal 2: To include measured/predicted UE throughput per slice as input from local node.

Proposal 3: Not to support measured/predicted slice level UE throughput exchange during handover.

Proposal 4: SLA requirements fulfilment in RAN can be provided to OAM as input for AI/ML model training.

Proposal 5: Not to support SLA requirements fulfilment information exchange between gNBs.

Proposal 6: To agree the TP to TR38.743 for AI/ML based network slicing in Annex.

# Reference

[1] RAN3_124_agenda_20240524_eom2

# Annex TP to TR38.743

<Start of change>

#### 4.1.2.4	Input data of AI/ML based Network Slicing:

To predict the optimized network slicing decisions, a gNB may need the following information as input data for AI/ML-based network slicing:

From local node:

-	Measured/Predicted radio resource status per slice

-	Measured/Predicted slice available capacity

-	Legacy predicted UE trajectory

-	Measured/Predicted slice level UE throughput

From neighbouring gNBs:

-	Measured/Predicted radio resource status per slice

-	Measured/Predicted slice available capacity

From the UE:

-	UE measurement report (e.g., UE RSRP, RSRQ, SINR measurement, etc), including cell level and beam level UE measurements

#### 4.1.2.5	Output data of AI/ML based Network Slicing:

AI/ML-based network slicing model in a gNB can generate the following information as output:

- Predicted radio resource status per slice

- Predicted slice available capacity

- Resource management decisions for resources within RRM policies (used by gNB internally)

- Slice aware mobility decisions (used by gNB internally)

#### 4.1.2.6	Feedback of AI/ML based Network Slicing:

To optimize the performance of AI/ML-based network slicing model, the following feedback can be considered to be collected from gNBs:

- Measured Radio resource status per slice

- Measured Slice available capacity

- Legacy and slice level UE performance feedback for those UEs handed over from the source gNB

#### 4.1.2.7	Potential standard impacts:

Following standard impacts is listed for subsequent Rel-19 normative work compared with what was specified during Rel-18.

Xn interface:

- Enhanced existing procedure to collect predicted information between gNBs:

  - Predicted radio resource status per slice

  - Predicted slice available capacity

<End of change>