3GPP TSG-RAN WG3 Meeting #125	R3-244163

Maastricht, The Netherlands, 19th – 23rd August 2024

Agenda Item:		11.3

Source:		NEC

Title:			(TP to TR38.743) AI/ML based CCO

Document for:		Discussion and decision

# Introduction

In previous RAN3 meetings, the following scenarios were agreed.

For AI/ML based CCO,

- AI/ML Model Training may be located in the OAM and AI/ML Model Inference may be located in the NG-RAN node (gNB-CU).

- AI/ML Model Training and AI/ML Model Inference may be both located in the NG-RAN node (gNB-CU).[1]

For split architecture, the standardization of AI/ML based future CCO state inference in the gNB-DU is not supported in R19, how to derive future CCO state is up to implementation in the gNB-DU, e.g., legacy or AI/ML. [2]

And we also discussed the possible solutions but only one general agreement can be achieved.

For non-split architecture, future CCO state can be derived via legacy mechanism or AI/ML techniques which is up to implementation.[2]

In this contribution, we continue discussing this issue based on the above agreements and provide our preferred solution and also the TP to TR38.743 accordingly.

# Discussion

Now there are two solution options under discussion. In order to realized CCO, to use AI/ML to predict a CCO issue with the consequent degradation of network (and UE) performance is one option (option 1). Furthermore, to use AI/ML to directly generate predicted coverage configuration based on potential CCO issues and to perform action to resolve the issue is another option (option 2).

Taking the agreed deployment scenarios into account, we analysed the detailed procedures for two options respectively.

Option 1: predicted CCO issue

For non-split architecture:

Step 0: gNB predicts the CCO issue.

Step 1: gNB generates the future coverage status, in legacy way, based on the predicted CCO issue and other information.

Step 2: gNB sends the future coverage status to neighbour gNBs.

For split-architecture

Step 0: gNB-CU predicts the CCO issue

Step 1: gNB-CU sends the predicted CCO issue to gNB-DU.

Step 2: gNB-DU generates the future coverage status based on the predicted CCO issue and other local information, whether only local information is used can be further discussed.

Step 3: gNB-DU sends the future coverage status to gNB-CU.

Step 4: gNB-CU sends the future coverage status to neighbour gNBs.

Option 2: predicted coverage configuration

For non-split architecture

Step 1: gNB predicts the coverage configuration based on the input information from local, UE and neighbouring nodes.

Step 2: gNB sends the predicted coverage configuration to neighbour gNBs.

For split-architecture

Step 1: gNB-CU predicts the coverage configuration based on the input information from local, UE, neighbouring nodes and gNB-DU.

Step 2: gNB-CU sends the predicted coverage configuration to neighbour gNBs.

Option 1 and option 2 have similar Xn impact, i.e. gNB sends the predicted/further coverage configuration to neighbour gNBs. The main difference between option 1 and option 2 is that whether predict CCO issue or predicts coverage configuration is an output from model inference.

Nowadays CCO is a part of SON function and the legacy CCO can be realized by collecting measurements from both network nodes and UEs in a rather long period of time, detecting the problem related to coverage and capacity, and then the network can automatically adjust the transmission parameters, e.g., Tx power, antenna tilt or azimuth, to overcome coverage holes. However, the existing mechanism uses the collected historical data to perform the adaptation which may not be applicable after network implementation. The pain point of legacy CCO is how to generate a optimal configuration parameters to avoid CCO issue.

Comparing to option 1, option 2 can take advantage of AI/ML tool to solve the problem completely. Therefore, we prefer to go with option 2.

Proposal 1: gNB (gNB-CU) predicts the coverage configuration based on input information.

Proposal 2: gNB-CU provides the predicted coverage configuration to gNB-DU to assist it for corrective action.

Regarding the time information related to the predicted coverage configuration, it is an optional information that can be send to gNB-DU together. However, since the predicted coverage configuration is just an assistance information for gNB-DU, we think it is also acceptable that gNB-DU decides itself when to implement it.

Observation: Time information is not an essential information for gNB-DU to implement the corrective action.

Below are the input, output and feedback of AI/ML model for CCO use case.

Input:

- From local node: Current/predicted cell load

- From the UE: UE measurement report (e.g., UE RS, RSRQ measurement, etc), RACH access information

- From neighbouring NG-RAN nodes: Current/predicted cell load, cell coverage change information

Output:

- Cell coverage adjustment strategy

- Predicted cell planning for future demands

Feedback:

- UE performance affected by the network slicing action, including bitrate, packet loss, latency.

- System KPIs (e.g., throughput, delay, RLF of current and neighbouring NG-RAN node)

The TP to TR38.743 for AI/ML based CCO, which captures the above analysis and the signalling procedures, is in Annex.

Proposal 3: To agree the TP to TR38.743 for AI/ML based CCO in Annex.

# Proposals

In this contribution, we provide our further analysis on the CCO solutions, and the following proposals were drawn from the analysis.

Proposal 1: gNB (gNB-CU) predicts the coverage configuration based on input information.

Proposal 2: gNB-CU provides the predicted coverage configuration to gNB-DU to assist it for corrective action.

Observation: Time information is not an essential information for gNB-DU to implement the corrective action.

Proposal 3: To agree the TP to TR38.743 for AI/ML based CCO in Annex.

# Reference

[1] RAN3_123bis_agenda_20240419_EOM

[2] RAN3_124_agenda_20240524_eom2

# Annex TP to TR38.743

<Start of change>

#### 4.2.2.2	Input of AI/ML based CCO

To predict the optimized CCO decisions, NG-RAN may need following information as input data for AI/ML based CCO:

From local node:

-	Current/Predicted cell load

From the UE:

-	UE measurement report (e.g., UE RS, RSRQ measurement, etc), RACH access information

From neighbouring NG-RAN nodes:

-	Current/Predicted cell load, cell coverage change information

#### 4.2.2.3	Output of AI/ML based CCO

AI/ML based CCO model can generate following information as output:

-	Cell coverage adjustment strategy

-	Predicted cell planning for future demands

#### 4.2.2.4	Feedback of AI/ML based CCO

To optimize the performance of AI/ML based CCO model, following feedback can be considered to be collected from NG-RAN nodes:

-	UE performance affected by the CCO action, including bitrate, packet loss, latency.

-	System KPIs (e.g., throughput, delay, RLF of current and neighbouring NG-RAN node)

#### 4.2.2.5	Standard Impact

Potential Xn interface impact:

-	Enhanced existing procedure to collect the input data information

-	CCO configuration predication between neighbouring NG-RAN nodes and source NG-RAN node

-	Enhanced existing procedure to retrieve feedback information

Potential F1 interface impact:

- Enhanced existing procedure to send the CCO configuration predication from gNB-CU to gNB-DU.

<End of change>