3GPP TSG-RAN WG3 Meeting #125	R3-244164

Maastricht, The Netherlands, 19th – 23rd August 2024

Agenda Item:		11.4

Source:		NEC

Title:			Energy Saving Enhancements

Document for:		Discussion and decision

# Introduction

In last RAN3 meeting, regarding Energy Saving Enhancements use case, the following agreements were achieved [1].

Measured Energy Cost can be transferred from DU to CU.

Only discuss on finer granularity of energy cost in Aug meeting and make the conclusion.

This contribution further discusses the listed two issues and the corresponding impacts on RAN3 specification.

# Discussion

## 2.1	Measured EC

We agreed in last meeting that measured EC can be transferred from DU to CU, so the follow-up question is whether to reuse the existing F1AP procedure or to define a new procedure for the measured EC transmission from DU to CU? Since only one parameter is newly added for this use case, it is simpler to reuse the existing F1AP procedure.

This existing Resource Status Reporting procedure is initiated by DU to report the result of measurements admitted by DU [2] and can be also used to transfer measured EC.

Proposal 1: To reuse F1 Resource Status Reporting procedure to transfer measured EC from DU to CU.

## 2.2	Finer granularity EC

EC is an index that is related to an actual energy consumption value. Since the energy consumption can be measured per node which shares the same hardware components, for example, antenna chains or digital units, it is agreed in Rel-18 that the Energy Cost is a node level parameter.

If energy saving needs to be achieved in cell level, the straightforward mechanism can be applied is to deactivation cells that are with low traffic load. In Rel-18 NES, cell DTX/DRX operation is introduced to facilitate reducing gNB transmission/reception activity time. The cell DTX/DRX pattern can be configured and activated separately or together. Considering to coordinate the cell activation/deactivation among a large number of gNBs is too complex to be accomplished by manual means, in order to address this complexity, AI/ML can help to effectively implement this feature at scale in order to meet operators’ demands on power consumption. For example, the cell DTX/DRX configuration from local and neighbor nodes can be collected as input to predict an optimal further configuration.

Proposal 2: To investigate how AI/ML help with cell DTX/DRX configuration if time allows.

# Proposals

In this contribution, we provide our further analysis on the open issues for ES use case, and the following proposals were drawn from the analysis.

Proposal 1: To reuse F1 Resource Status Reporting procedure to transfer measured EC from DU to CU.

Proposal 2: To investigate how AI/ML help with cell DTX/DRX configuration if time allows.

# Reference

[1] RAN3_124_agenda_20240524_eom2

[2] TS 38.473, F1 application protocol (F1AP)