3GPP TSG-RAN WG3 Meeting #125	R3-244165

Maastricht, The Netherlands, 19th – 23rd August 2024

Agenda Item:		11.4

Source:		NEC

Title:			Continuous MDT collection for AI/ML

Document for:		Discussion and decision

# Introduction

In last RAN3 meeting, two problems for continuous MDT collection, as below, were agreed to be captured in the TR [1].

- Problem A (measurement continuity): how to ensure that the same UE collecting MDT measurements during the same RRC state and across different RRC states.

- Problem B (trace correlation): how to ensure that the TCE which eventually receives the MDT reports can associate the received logged and immediate MDT measurements to a continuous data collection period from the same UE.

This contribution further discusses the above two items, the possible solutions and the corresponding impacts on RAN3 specification.

# Discussion

In Rel-18 discussion, RAN3 reached the following agreement on continuous MDT collection:

The existing MDT framework is used as baseline for data collection from the UE.

Continuous collection of MDT traces is beneficial only for AI/ML training in OAM. Continuous MDT collection is to enable the continuous collection of MDT data from the same UE across RRC state changes (RRC_Connected, RRC_Idle, RRC_Inactive).

According to TR37.817 [1] and also considering TR38.743 [2], the following data need to be collected from UE for AI/ML:

- UE location information (e.g., coordinates, serving cell ID, moving velocity) interpreted by gNB implementation when available

- UE measurement report (e.g., UE RSRP, RSRQ, SINR measurement, etc), including cell level and beam level UE measurements

- UE Mobility History Information

In order to limit the impact on UE power consumption and processing, continuous collection and logging of UE measurement should as much as possible rely on the measurements that are already supported in UE and so we only consider to continuous collect the above data.

## 2.1	Problem A (measurement continuity)

We already agreed that “Continuous collection of MDT traces is beneficial only for AI/ML training in OAM.”. Training data is the initial dataset used to train machine learning algorithms on how to make predictions. In order to increase the precision of a ML model, the process of training an ML model needs large volume of input data and a rather long training time. And the provision of training data is not time critical.

As in legacy, immediate MDT can be configured to UE for measurements in RRC Connected, while logged MDT in – RRC Idle and RRC Inactive. Regarding to the gap during RRC state transition, it is a extremely short period of time comparing to the long training time and the measurements during the transition can be negligible.

Besides, the collected raw data may still need to be processed and translated into usable information for training by data processing technology, which can also handle the sparse missing data issue, due to e.g. RRC state transition or other reasons.

Therefore, we think that, for AI/ML training, the measurement continuity during the same RRC state can be guaranteed in legacy way, and the measurement discontinuity during RRC state transition is negligible and is not needed to be considered.

Observation 1: Measurement continuity during the same RRC state can be guaranteed in legacy way.

Proposal 1: It is not need to consider measurement continuity during RRC state transition.

## 2.2	Problem B (trace correlation)

In the newly published SA5 specification TS28.558 [4], S-TMSI is included in all the UE level measurements to identify which UE sends the measurement report and this IE can be used in TCE to do the trace correlation.

However, RAN3 has the following concerns about using S-TMSI and sent a reply LS to SA5 for clarification [5].

- S-TMSI is not available in the gNB-DU and in the gNB-CU-UP.

- S-TMSI may not be always available in the gNB-CU-CP.

We suggested to wait for the reply from SA5 and then decide whether any enhancement or other solution is needed.

Proposal 2: It is proposed RAN3 to wait for the reply from SA5 on the trace correlation issue.

# Proposals

In this contribution, we provide our further analysis on continuous MDT collection for AI/ML and the following observation and proposals were drawn from the analysis.

Observation 1: Measurement continuity during the same RRC state can be guaranteed in legacy way.

Proposal 1: It is not need to consider measurement continuity during RRC state transition.

Proposal 2: It is proposed RAN3 to wait for the reply from SA5 on the trace correlation issue.

# Reference

[1] R3-243952, (TP for TR 38.743) Continuous Management-based MDT Data Collection, Nokia, Huawei, Qualcomm, NEC, Samsung, ZTE, CMCC, InterDigital, Ericsson

[2] TR37.817, Study on enhancement for Data Collection for NR and EN-DC

[3] TR38.743, Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN

[4] TS28.558, UE level measurements for 5G system

[5] R3-243941, Reply LS to SA5 on improved KPIs involving end-to-end data volume transfer time analytics, RAN3