3GPP TSG-RAN WG3 Meeting #125	R3-244166

Maastricht, The Netherlands, 19th – 23rd August 2024

Agenda Item:		11.4

Source:		NEC

Title:			Split architecture and NR-DC mobility optimization

Document for:		Discussion and decision

# Introduction

In last RAN3 meeting, regarding split architecture and mobility optimization use cases, the following agreements were achieved [1].

Split architecture: Transferring Measured UE performance from CU-UP to CU-CP.

NR-DC mobility optimization: Enhance the Dual Connectivity procedure (e.g., MN-initiated SN addition, MN-initiated SN change) to trigger the collection of measured UE performance.

This contribution further discusses the above two items and the corresponding impacts on RAN3 specification.

# Discussion

## 2.1	Split architecture

We agreed in last meeting to transfer measured UE performance from CU-UP to CU-CP, so the follow-up question is whether to reuse the existing E1AP procedure or to define a new procedure for the measured UE performance from CU-UP to CU-CP? Since only one parameter is newly added for this use case, it is simpler to reuse the existing E1AP procedure.

This existing Resource Status Reporting procedure is initiated by gNB-CU-UP to report the result of measurements admitted by gNB-CU-UP and can be also used to transfer measured UE performance.

Proposal 1: To reuse E1 Resource Status Reporting procedure to transfer measured UE performance from CU-UP to CU-CP.

## 2.2	NR-DC mobility optimization

Based on the agreement in last meeting, MN needs to trigger target SN to provide measured UE performance to it after SN addition or SN change. Since, as we have already agreed, inference locates at the MN, the target SN needs to send the measured UE performance to MN. To achieve this, the existing Data Collection Reporting Initiation procedure and Data Collection Reporting procedure can be reused between MN and SN.

Proposal 2: To support Data Collection Reporting Initiation procedure and Data Collection Reporting procedures between MN and SN.

In Rel-18, in HANDOVER REQUEST message that is sent by the source NG-RAN node to the target NG-RAN node, a Handover Data Collection ID is added to ask the target node to collect measured UE performance. Similarly, for both MN-initiated SN addition and MN-initiated SN change, a Handover Data Collection ID can be added to S-NODE ADDITION REQUEST message from MN to SN to ask the target SN to collect measured UE performance.

Proposal 3: To add a Handover Data Collection ID in the S-NODE ADDITION REQUEST message from MN to SN.

# Proposals

In this contribution, we provide our further analysis on split architecture and NR-DC mobility optimization use cases, and the following proposals were drawn from the analysis.

Proposal 1: To reuse E1 Resource Status Reporting procedure to transfer measured UE performance from CU-UP to CU-CP.

Proposal 2: To support Data Collection Reporting Initiation procedure and Data Collection Reporting procedure need to be supported between MN and SN.

Proposal 3: To add a Handover Data Collection ID in the S-NODE ADDITION REQUEST message from MN to SN.

# Reference

[1] RAN3_124_agenda_20240524_eom2