3GPP TSG-RAN WG3 Meeting #125	R3-244167

Maastricht, The Netherlands, 19th – 23rd August 2024

Agenda Item:		12.3

Source:		NEC

Title:			(Draft reply LS) Mobility between CAG cell and CSG cell

Document for:		Discussion and decision

# Introduction

In last RAN3 meetings, one LS on support of UE move between CAG cell of 5G Femto and CSG cell [1] from SA2 was received. In the LS, two solutions on how to enable interworking between CAG and CSG cells are captured and SA2 asks RAN3 (and RAN2) whether the two solutions mentioned above have any impact on the RAN (e.g., for RAN procedures).

RAN2 already sent the reply LS [2] to SA2 based on their analysis on UE impact, but RAN3 postponed the reply since no consensus on RAN impact.

In this contribution, we provide our analysis on the two solutions and a draft reply LS.

# Discussion

For Solution 1 (S2-2405814) in the SA2 LS, as captured in the RAN2’s reply LS,

The UE partitions CSG-CAG ID and constructs mapped CSG/CAG ID, and reports to the NG-RAN or E-UTRAN (depending on the considered mobility direction) as described in pCR (S2-2405814).

In last SA2 meeting, a set of new solutions was proposed and agreed to cover both idle mode and connected mode in S2-2407148 [3] and S2-2407149 [4].

For Solution 2 (S2-2405789), the correlation between CAG information and CSG information (including corresponding Home eNB ID) is configured locally in the AMF that the 5G NR Femto cell is connected to and it’s assumed that NG-RAN/E-UTRAN without enhancement can trigger the handover required for the UE towards the target CSG/CAG cell as normal cell. Since RAN recognizes the target CSG/CAG cell as an open cell during handover and fully relies on AMF to performs access control, if UE is not a member of the CSG cell, AMF will reject the handover and handover failure will happen. This kind of handover failure is possible to be happened frequently and repeatedly.

Besides, in this solution, UE treats a 4G Femto cell as a CSG cell in idle mode but as a macro cell in connected mode, and this will cause UE behaviour disorganized.

To address the above issues, RAN or/and UE impacts are foreseen.

Observation: Both Solution 1 (S2-2405814) and Solution 2 (S2-2405789) have RAN impacts.

In the Annex, we provide a reply LS based on the above analysis and propose the following:

Proposal: To rely SA2 based the draft LS in Annex.

# Proposals

In this contribution, we analysed the two solutions from SA2 and the following observation ad proposal were drawn from the analysis.

Observation: Both Solution 1 (S2-2405814) and Solution 2 (S2-2405789) have RAN impacts.

Proposal: To rely SA2 based the draft LS in Annex.

# Reference

[1] R3-243020, LS on Support of UE move between CAG cell of 5G Femto and CSG cell, SA2

[2] R2-2405942, Reply LS on Support of UE move between CAG cell of 5G Femto and CSG cell, RAN2

[3] S2-2407148, KI#1 New solution on idle mode mobility , Nokia, NEC

[4] S2-2407149, KI#1 New Solution for handover from/to CSG cell to/from CAG cell with configuration available at RAN node, Nokia, NEC

# Annex LS to SA

3GPP TSG-RAN WG3 Meeting #125	R3-24xxxx

Maastricht, Netherlands, 19th – 23rd August 2024

Title:	[draft] Reply LS on Support of UE move between CAG cell of 5G Femto and CSG cell

Response to:	S2-2405813 - R3-243020

Release:	Rel-19

Work Item:	FS_5G_Femto, FS_WAB_5GFemto_NR

Source:	NEC (to be RAN3)

To:	SA2

Cc:	RAN2

Contact person:	Hui Ma

hui.ma@emea.nec.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

RAN3 thanks SA2 for the LS on Support of UE move between CAG cell of 5G Femto and CSG cell.

Question 1:

RAN3 evaluated the two solutions and made the following observations:

Solution 1 (S2-2405814):

As replied by RAN2 in R2-2405942, the UE partitions CSG-CAG ID and constructs mapped CSG/CAG ID, and reports to the NG-RAN or E-UTRAN (depending on the considered mobility direction) as described in pCR (S2-2405814).

Solution 2 (S2-2405789):

Repeated handover failures may happen due to no access control at source RAN node. Since RAN recognizes the target CSG/CAG cell as an open cell during handover and fully relies on AMF to performs access control, if UE is not a member of the CSG cell, AMF will reject the handover and handover failure will happen. This kind of handover failure is possible to be happened frequently.

Besides, UE treats a 4G Femto cell as a CSG cell in idle mode but as a macro cell in connected mode, which causes UE behaviour disorganized.

To address the above issues, RAN or/and UE impacts are foreseen.

Question 2:

[To be update]

RAN3 would like to kindly ask SA2 to take the above into account takes into account.

# 2	Actions

To SA2:

ACTION: 	RAN3 kindly asks SA2 to take the above into account.

# 3	Dates of next RAN3 meetings

Updated meeting schedule can be found at: https://portal.3gpp.org/?tbid=373&SubTB=381#/

RAN3#125bis		2024-10-14 - 2024-10-18		China (TBC) , CN

RAN3#126		2024-11-18 - 2024-11-22		Orlando, US