3GPP TSG-RAN WG3 Meeting #125	R3-244168

Maastricht, The Netherlands, 19th – 23rd August 2024

Agenda Item:		12.3

Source:		NEC

Title:			NR femto

Document for:		Discussion and decision

# Introduction

In previous RAN3 meetings, the following four architecture options and two Xn options were proposed for NR femto [1, 2] were proposed.

1) NR Femto node directly connected to the 5GC;

2) The NR Femto GW (concentration for the CP application layer);

3) The SCTP concentrator (concentration for the transport layer);

4) The CU-DU split (femto as gNB-DU, GW as gNB-CU).

Option A: Xn support without Xn GW

Option B: NR Femto operating with Xn GW

In this contribution, we provide our consideration and preference on the architecture and also analyse other open issues that need to be addressed in this SI.

# Discussion

## 2.1 Overall RAN architecture

Same as HeNB in LTE, NR femto is a simple end user plug and play NR capable base station which is deployed at home or at enterprise premises and offers a cost-effective way to improve 5G indoor coverage, offload macro gNB network traffic, enable better voice quality, and better support for Enterprise mobility. Besides, in 5G, femto is also considered a powerful key to lead to efficient and effective usage of higher frequency spectrum, by extending coverage using higher frequency bands.

Considering the number of NR femto nodes in higher frequency bands can be quite large, it is beneficial to have a NR Femto GW between the RAN and 5GC to support a large number of NR Femto nodes in a scalable manner and to avoid the CP signalling storm to 5GC. Besides, although Security aspects are under SA3´s responsibility, but, in our opinion, SeGW is a must to separate operators’ network and Internet. If NR Femto GW is deployed, the SeGW can be integrated into NR Femto node GW. Since the functionality of a HeNB GW had been thoroughly discussed and already been specified in LTE, which we can use as a starting point for NR femto design, the standard efforts can be largely saved. Therefore, we prefer architecture option 2 The NR Femto node GW.

Regarding the Xn aspect, Xn GW is proposed with the aim to facilitate Xn signaling between the NR femto nodes. However, in real-world deployment, NR femto nodes are connected to each other via Internet, and thus the benefit of Xn GW cannot be achieved or can be replaced by NR Femto node GW. Therefore, we prefer Option A: Xn support without Xn GW.

But this architecture option selection should also depends on operators’ deployment requirements.

Proposal 1: To adopt architecture option 2 + option A for NR femto if this option can meet operators’ deployment requirements.

If architecture option 2 is agreed, below is the overall RAN architecture with NR Femto GW deployed.

Figure 1: Overall Architecture with deployed NR Femto node GW

After introducing NR Femto node and NR Femto node GW, The NG interface can be used as the interface:

- Between the NR Femto GW and the Core Network;

- Between the NR Femto node and the NR Femto node GW;

- Between the NR Femto node and the Core Network.

The Xn interface can be used as the interface:

- Between the NR Femto nodes;

- Between the NR Femto node and the gNB.

And the NR Femto node logical architecture is shown in figure 2, the NR Femto GW serves as a concentrator for the C-Plane, specifically the NG-C interface. The NG-U interface from the NR Femto node may be terminated at the NR Femto node GW, or a direct logical U-Plane connection between NR Femto node and UPF may be used.

Figure 2: NR NR Femto node Logical Architecture

The NR Femto GW appears to the AMF as an gNB. The NR Femto GW appears to the NR Femto node as an AMF. The NG interface between the NR Femto node and the 5GC is the same, regardless whether the NR Femto node is connected to the 5GC via a NR Femto GW or not.

Proposal 2: To agree the Overall Architecture with deployed NR Femto GW as shown in figure 1.

Proposal 3: To agree the NR Femto node Logical Architecture as shown in figure 2.

## 2.2 NR Femto node access mode

Same as in HeNB, we think it is still favourable that the NR Femto node can be configured to open, hybrid or closed access mode by operator. For closed mode NR Femto node, only UEs belong to the closed group that the NR Femto node is configured could be permitted to handover to the NR Femto node. If the NR Femto node is configured as Hybrid mode or Open mode, NR Femto node provides services to its associated both closed group members and non closed group members. But this issue is also subject to operators’ deployment requirement.

Proposal 3: To support three NR Femto node access mode: open mode, hybrid mode and closed mode. But this depends on operators’ deployment requirements.

## 2.3 In-bound mobility to NR Femto node

## Providing all the above mentioned three access modes (open mode, hybrid mode, closed mode) can be supported, the corresponding HO types involving different mode NR Femto nodes as shown in Table 1 need to be supported.

Table 1: NR Femto node HO types

| Source | Target |
| --- | --- |
| gNB or any NR Femto node | open access NR Femto node |
| gNB, or any NR Femto node | hybrid access NR Femto node |
| hybrid access NR Femto node or closed access NR Femto node | closed access NR Femto node |
| Any NR Femto node | gNB |

Proposal 3: To support the HO types as shown in table 1 for NR femto.

## As for inbound handover to a target NR Femto node, the network needs to perform access control based on the access mode, closed group ID and PLMN, etc. Since the NR Femto node GW, that connects to the 5GC, represents 5GC to the NR Femto nodes connected to it, NR Femto GW can perform access control.

## 2.4 local services access

To support local services access function, a NR Femto node Local GW or a local UPF can be collocated in NR Femto node, as shown in figure 3.

Figure 3: NR Femto node operating in local services access mode

In this case, there are several issues need to be considered:

- Additional functions: besides the same functions as supported by the gNB, there are additional functions need to be supported, for example, LGW IP address transfer to 5GC, packets sending and buffering, user plane path between the LGW and the NR Femto node, etc.

- Additional interfaces: the NR Femto GW needs to support an interface towards 5GC and an interface towards the IP network.

- Local PDN connection establishment

- Mobility of local PDN connection: whether to support?

Proposal 4: For local services access support for NR femto, the following issues need to be studied:

- Functions

- Interfaces

- Local PDN connection establishment,

- Mobility of local PDN connection.

# Proposals

In this contribution, we provide considerations the open issues for NR femto and the following proposals were drawn from the analysis.

Proposal 1: To adopt architecture option 2 + option A for NR femto if this option can meet operators’ deployment requirements.

Proposal 2: To support three NR Femto node access mode: open mode, hybrid mode and closed mode. But this depends on operators’ deployment requirements.

Proposal 3: To support the HO types as shown in table 1 for NR femto.

Proposal 4: For local services access support for NR femto, the following issues need to be studied:

- Functions

- Interfaces

- Local PDN connection establishment,

- Mobility of local PDN connection.

# Reference

[1] RAN3_123bis_agenda_20240419_EOM

[2] RAN3_124_agenda_20240524_eom2