3GPP TSG-RAN WG3 Meeting #125	R3-244169

Maastricht, The Netherlands, 19th – 23rd August 2024

Agenda Item:		17.2

Source:		NEC

Title:			Support on-demand SSB operation

Document for:		Discussion and decision

# Introduction

One objective on on-demand SSB is captured in WID [1] as follows:

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4]

  - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling)

Due to lake of agreements in other WGs, RAN3 had not started to discuss more details on this issue, but just one open issue on Xn interface impact? was brought up in last meeting.

In this contribution, we discuss this issues and other potential RAN3 impacts.

# Discussion

## RAN1 agreements

Below are the agreements made in RAN1 that may influence RAN3 discussion.

Agreement

Regarding the UE assumption on SSB transmission on a cell supporting on-demand SSB SCell operation, the following cases are identified for further study:

- Case #1: No always-on SSB on the cell

- Case #2: Always-on SSB is periodically transmitted on the cell

- FFS: Whether always-on SSB and on-demand SSB are not cell-defining SSB if transmitted.

FFS: Which scenario the above applies for

Agreement

- Support on-demand SSB SCell operation triggered by gNB.

- FFS Details of associated signaling/indication/configuration provided to UE

Agreement

For the identified scenarios and cases (as per RAN1#116 agreement), on-demand SSB can be triggered by gNB at least for the following scenarios/cases:

- Scenario #2 and Case #1

- Scenario #2 and Case #2

- Scenario #2A and Case #1

- Scenario #2A and Case #2

- FFS: Scenario #3A and Case #1

- FFS: Scenario #3A and Case #2

- FFS: Scenario #3B and Case #1

- FFS: Scenario #3B and Case #2

- For Case #1, once on-demand SSB is triggered, its transmission is in a periodic manner.

  - Note: This does not imply periodic on-demand SSB is transmitted indefinitely after triggered.

- Notes:

  - Scenario #2A refers to

    - “When UE receives SCell activation command (e.g., as defined in TS 38.321)”

  - Scenario #3A refers to

    - “After UE receives SCell activation command (e.g., as defined in TS 38.321) until SCell activation is completed”

  - Scenario #3B refers to

    - “When SCell activation is completed and SCell is activated” or

    - “After SCell activation is completed and SCell is activated”

  - For discussion purpose under AI 9.5.1, always-on SSB is SSB supported in Rel-18 specifications.

  - Timing for on-demand SSB transmission (e.g. when the triggered SSB starts and ends) will be separately discussed.

Agreement

- For a cell supporting on-demand SSB SCell operation,

  - L1 and/or L3 measurement based on on-demand SSB is supported for the cell.

    - FFS further details on L1 and/or L3 measurement

Agreement

For a cell supporting on-demand SSB SCell operation, further study the following options.

- Option 1: Separate signaling between legacy/existing signaling (e.g., RRC, MAC CE) providing SCell activation/deactivation and signaling providing On-demand SSB transmission indication.

- Option 2: A single signaling in which both SCell activation/deactivation and On-demand SSB transmission indication are provided.

  - FFS: Details of the signaling

- Other options are not precluded.

- FFS: Details on On-demand SSB transmission indication

## RAN3 impacts

Since RAN1 already agreed to Support on-demand SSB SCell operation triggered by gNB, we need to discuss which node, e.g. gNB-CU or gNB-DU or neighbor gNB, can determine and trigger on-demand SSB SCell operation.

Considering the scenario needs to be analyzed is for UEs in connected mode configured with CA, for both intra-/inter-band CA [1], in which a PCell is always associated to a CD-SSB located on the synchronization raster, we do not see the need to use a neighbour gNB to tiger on-demand SSB SCell operation and we think this should be an intra-gNB issue, therefore no Xn impact is foreseen.

Proposal 1: No Xn impact is foreseen for on-demand SSB SCell operation.

In legacy, after CA is set up, gNB uses MAC CE (SCell Activation/Deactivation) to activate or deactivate data transmission for SCell. Therefore, in CU/DU split, gNB-DU is the suitable node to trigger on-demand SSB SCell operation.

Proposal 2: gNB-DU triggers on-demand SSB SCell operation.

However, considering gNB-CU is more knowledgeable on the cell status (e.g. no always-on SSB, always-on SSB with a long periodicity or always-on SSB as in legacy) of the cells with all DUs connected to it and neighbour cells, gNB-CU can provide some guidance on on-demand SSB SCell operation, taking both energy saving and UE performance into account. For example, gNB-CU can suggest a SSB operation mode, SSB broadcast periodicity, or SSB broadcast duration, etc.

Proposal 3: gNB-CU can provide suggested SSB operation parameters to gNB-DU to assist the on-demand SSB SCell operation.

# Proposals

In this contribution, we summarized the RAN1 progress on on-demand SSB and analysed the potential RAN3 impacts, and the following proposals were drawn from the analysis.

Proposal 1: No Xn impact is foreseen for on-demand SSB SCell operation.

Proposal 2: gNB-DU triggers on-demand SSB SCell operation.

Proposal 3: gNB-CU can provide suggested SSB operation parameters to gNB-DU to assist the on-demand SSB SCell operation.

# Reference

[1] RP-234065, New WID: Enhancements of network energy savings for NR

[2] RAN3_124_agenda_20240524_eom2