3GPP TSG-RAN WG3 Meeting #125	R3-244170

Maastricht, The Netherlands, 19th – 23rd August 2024

Agenda Item:		17.3

Source:		NEC

Title:			Support on-demand SIB1

Document for:		Discussion and decision

# Introduction

As stated in the WID [1], SIB1 transmissions by a NES cell may be avoided or reduced when there are no UEs in RRC IDLE/INACTIVE mode being present. The NES cell can be deployed in a capacity cell or as a non-anchor cell, or it can be a target cell for cell re-selection. But in this case, how the idle/inactive mode UE obtains the SIB1 of a NES cell needs to be investigated. A uplink wake-up-signal (UL WUS) is proposed in [1] to be used by a idle/inactive UE to trigger the SIB1 transmission of a NES cell.

In last RAN3 meeting, it was agreed that “Focus on case2 in RAN3 first” [2] and a list of open issues were identified [3].

Case 2: UE transmits UL WUS to NES Cell, UE obtains the UL WUS configuration from Cell A, UE receives on-demand SIB1 from NES Cell

This contribution further discusses the listed open issues and the corresponding impacts on RAN3 specification.

# Discussion

## WUS provision

In case 2, a UE receives UL WUS configuration of a NES cell from Cell A, and then the UE sends an UL WUS to the NES cell to request it to transmit SIB1 for a specified duration. Once receiving the UL WUS, the NES cell can send SIB1 in the legacy way and for one or more legacy SIB1 periodicity.

Before the NES cell starts on demand SIB1 or when the UL WUS configuration changes, the NES cell needs to provide its UL WUS configuration to cell A. RAN3 has already reached the following agreement on inter-node exchange of UL WUS configuration from NES cell to Cell A.

UL WUS configuration from NES Cell to Cell A via Xn, e.g., before entering on-demand SIB1 mode in the NES Cell or when the UL WUS configuration is changed.

Regarding to the procedure for UL WUS config exchange over Xn interface, the existing NG-RAN node Configuration Update procedure, which is used to update application level configuration data needed for two NG-RAN nodes to interoperate correctly over the Xn-C interface [4], can be reused for UL WUS configuration exchange, as shown in figure 1. Cell A decides whether it can support UL WUS configuration provision for the NES cell after receiving the configuration and then responses in the Acknowledge message.

Figure 1: NG-RAN node Configuration Update procedure

In case of there are multiple Cell A for a single NES cell, which should be a rare case if under proper cell planning, the NES cell can choose one Cell A for UL WUS configuration provision. If the 1st chosen Cell A reject the request, then the NES cell can try another Cell A. The same procedure in figure 1 is still applicable in this case.

Proposal 1: To reuse NG-RAN node Configuration Update procedure for UL WUS configuration exchange over Xn.

Regarding the information included in UL WUS configuration that need to be added in the NG-RAN node Configuration Update message, we can discuss after RAN2 defining the UL WUS configuration parameters.

As RAN2 agreed that the UL WUS configuration shall include at least RACH configuration, and considering that in split architecture gNB-DU is in charge of RACH configuration, gNB-DU shall decide the UL WUS configuration of a NES cell.

Proposal 2: In split architecture, gNB-DU decides the UL WUS configuration of a NES cell.

And in this case, gNB-DU needs to send the UL WUS configuration to gNB-CU for inter-node exchange with cell A.

Proposal 3: In split architecture, gNB-DU sends the UL WUS configuration to gNB-CU over F1 for inter-node exchange with cell A.

## On demand SIB1 activation/deactivation

After the NES cell provide its UL WUS configuration to cell A, the NES cell can update Cell A if the current UL WUS configuration is changed or released. Meanwhile, the NES cell can activate or deactivate (switch to legacy way) on-demand SIB transmission at any time. If the NES cell is in SIB1 broadcast, the UE can receive SIB1 in legacy way; if the NES cell is in on-demand SIB1, the UE will request cell A for the UL WUS configuration first and then trigger the NES cell to send SIB1. Therefore, we think, if this activate/deactivate switch is dynamic and frequent, the NES cell may not need to informs cell A over Xn about this activate/deactivate status change.

However, considering there might be a case that is multiple NES cells under a single Cell A, it is beneficial to perform some coordination by Cell A on e.g. which NES cells under that Cell A can be activate on-demand SIB1 transmission, taking both energy saving and UE performance into account.

Proposal 4: Cell A can provide guidance to NES cell about On demand SIB1 activation/deactivation over Xn.

In split architecture, gNB-CU is more knowledgeable on the SIB1 transmission status and load status of all NES cells and neighbor cells, so we prefer gNB-CU to decide on-demand SIB1 activate/deactivate of a NES cell, or at least gNB-CU should provide guidance to gNB-DU on on-demand SIB1 activation/deactivation of a NES cell.

Proposal 5: gNB-CU decides or guides gNB-DU on the on-demand SIB1 activation or deactivation of a NES cell, and sends the request to gNB-DU.

## Other case

In last RAN3 meeting, we decided to focus on case 2 first, but the other cases should also be studied later. For example, in order to further reduce power consumption, there is one case that a NES cell never sends SIB1 and it depends on Cell A for SIB1 transmission (Case 3 in RAN1). In this case, UE transmits UL WUS to Cell A and UE obtains both UL WUS configuration and SIB1 from Cell A.

Since SIB1 of a NES cell is transmitted from Cell A, the SIB1 content of that NES cell needs to be transferred to Cell A in advance. Regarding the SIB1 content exchange over Xn interface, the existing NG-RAN node Configuration Update procedure can also be reused for SIB1 exchange, as shown in figure 2.

Figure 2: NG-RAN node Configuration Update procedure

As stated in WID [1], there is one checkpoint for this topic for normative work in RAN#105. Considering the above Case 3 is a possible scenario and also listed in RAN1 as one option, even though there is no discussion in previous RAN3 meetings due to lack of time, we think that this case should still be keep in scope for Rel-19.

    - Study procedures and signaling method(s) to support on-demand SIB1 for UEs in idle/inactive mode, including: [RAN1/2/3]

      - Triggering method by uplink wake-up-signal using an existing signal/channel.

      - Wake-up-signal configuration provisioning to UE

        - Note: No modification of SSB will be discussed under this objective

      - Information exchange between gNBs at least for the configuration of wake-up signal, if necessary.

      - Checkpoint for normative work in RAN#105

Proposal 6: Keep Case 3 (UE transmits UL WUS to Cell A and UE obtains both UL WUS configuration and SIB1 from Cell A) in scope of Rel-19 study.

# Proposals

In this contribution, we analysed four cases of on-demand SIB1 transmission of a NES cell and the following proposals were drawn from the analysis.

Proposal 1: To reuse NG-RAN node Configuration Update procedure for UL WUS configuration exchange over Xn.

Proposal 2: In split architecture, gNB-DU decides the UL WUS configuration of a NES cell.

Proposal 3: In split architecture, gNB-DU sends the UL WUS configuration to gNB-CU over F1 for inter-node exchange with cell A.

Proposal 4: Cell A can provide guidance to NES cell about On demand SIB1 activation/deactivation over Xn.

Proposal 5: gNB-CU decides or guides gNB-DU on the on-demand SIB1 activation or deactivation of a NES cell, and sends the request to gNB-DU.

Proposal 6: Keep Case 3 (UE transmits UL WUS to Cell A and UE obtains both UL WUS configuration and SIB1 from Cell A) in scope of Rel-19 study.

# Reference

[1] RP-234065, New WID: Enhancements of network energy savings for NR

[2] RAN3_124_agenda_20240524_eom2

[3] R3-243868, Summary of offline discussions: Support on-demand SIB1 for UEs, Ericsson (Moderator)

[4] TS38.423, NG-RAN; Xn application protocol (XnAP)