3GPP TSG-RAN WG3 Meeting #125	R3-244177

Maastricht, Netherland, 19 – 23 August 2024

Agenda Item:	9.2

Source:	Google

Title:	Discussion for Early TA acquisition and Preamble Resources for PDCCH order in LTM

Document for:	Discussions & Approval

# 1	Introduction and Background

The topic of CFRA resource configuration in Inter-gNB-DU LTM, covering both LTM Cell Switch Command and PDCCH Order, was discussed in CB#12_Mobilityenh [4]. While it gains some support, the aspect concerning PDCCH Order (specifically Early TA acquisition) requires further evaluation.

Additionally, there was debate on the necessity of RACH-ConfigDedicated for the source gNB-DU for generating the PDCCH Order. Some argued that EarlyUL-SyncConfig, along with the preamble index list, sufficiently equips the source gNB-DU to populate the PDCCH Order. Therefore, in this paper, we would like to provide some further analysis from the previous RAN2 discussions and highlight the current limitation in RAN3 specification.

## RAN2 discussions related to PDCCH Ordered RACH for LTM

RAN2#121bis-e made some agreements related to Early TA/PDCCH Order from the network perspective to the UE and an LS was sent to RAN3 in R3-232569 in RAN3#120 (Incheon).

R2-2304523 [Draft] Reply LS on PDCCH ordered RACH for LTM LS out Fujitsu, CATT - Samsung think that all options are feasible. And if we want to just list two options then we should use preferable. - Ericsson think R1 already agreed to not support option 3. Think the title should be changed if we want to include the La measurements. - Chair: a number of tohru comments that LS text is ok. Think that the last sentence explains why not all options are listed as feasible (even though they are – in principle .. ). Current Contents is agreeable as is. Include also agreements regarding L1 measurements for information (copy-past of agreements part), revise the title to be Reply LS on L1 measurement RS configuration and PDCCH ordered RACH for LTM The revised LS out is approved unseen in R2-2304553

Early TA and RACH-less R2-2302750 Discussion on the early TA acquisition Intel Corporation discussion Rel-18 NR_Mob_enh2-Core DISCUSSION P1 - MTK support - Samsung think separate config is not needed - Intel think that the configuration of candidate cell and configuration for early TA may be received at different times. - Ericsson think it will be still in the LTM structure but not in the specific candidate cell config. Ericsson clarifies that this should still be cell specific. - vivo has concerns. Chair point out that there is a significant majority support. P2 - Intel explains that regarding the RACH preamble resource, source DU can have a small pool from target DU(s) that source DU can use among multiple UEs, and this is already in PDCCH order. Intention is to allow “real time” interaction to not have to allocate a very large pool of CFRA resource. - MTK think this is reasonable but R1 can decide. - Nokia wonder if PDCCH order refers to a RRC config. Chair think it is clear that PDCCH refers to something. - Leonovo want a solution where UE does RACH autonomously towards candidate neighbour cells to acquire TA early. P3 - ZTE think that preamble without RAR is a new thing and the O2 should be the baseline. Intel think that this is RACH towards a different cell, so this is new in any case. - Ericsson indicate that R1 already agreed that O1 and O2 shall be supported. - FW think that when DC is enabled then RACH will not have interruption on serving cell and think that the latency could be less. - Chair: FW Lenovo and Xiaomi support O3 in order to optimize for the LTM operation for cell switching in DC, a big majority of companies think O3 is not needed. - Chair: O1 O2, seems difficult to decide. From RAN2 perspective, to enable shared preamble resource among multiple UEs, it is beneficial that the information that identifies the allocated CFRA resource (i.e., SS/PBCH index, RACH occasion, and Random Access Preamble index) can be indicated in the PDCCH order (as legacy intra-cell PDCCH order). RRC RACH configuration for early TA acquisition (e.g., including whether RAR needs to be received) is specific per target cell and is signalled separately (separate IEs) from the candidate cell configuration (the part that need to be applied at cell switch). R2 assumes that Early TA RACH option 3 (with RAR from candidate cell) is not needed in Rel-18.

R2-2302750 Discussion on the early TA acquisition [3]

## 2.2 How to configure CFRA resource of candidate cells The configuration of early TA acquisition is provided to UE from source cell prior to the transmission of PDCCH order. As explained in Table 1, the PDCCH order itself contains some configuration of preamble, such as preamble index, SSB index and the RACH occasion index. And in early TA acquisition case, the PDCCH order also contains the indication of candidate cell, e.g., a candidate configuration index of a candidate cell. But PDCCH order alone is not sufficient, other necessary PRACH configurations have to be provided by RRC message, e.g., cell specific RACH parameters (IE RACH-ConfigCommon). Table 1. DCI format 1_0 used for PDCCH order DCI fields; Meaning. Identifier for DCI formats; The value of this bit field is always set to 1, indicating a DL DCI format. Frequency domain resource assignment; If the CRC of the DCI format 1_0 is scrambled by C-RNTI and the "Frequency domain resource assignment" field are of all ones, the DCI format 1_0 is for random access procedure initiated by a PDCCH order. Random Access Preamble index; This field contains 6 bits. It indicates which Random access preamble to use in case of Contention Free Random Access(CFRA) or the value 000000 in the case of Contention based Random Access(CBRA) procedure. UL/SUL indicator; ‘0’ for the non-supplementary uplink, and ‘1’ for the supplementary uplink. SS/PBCH index; If the value of the "Random Access Preamble index" is not all zeros, this field indicates the SS/PBCH that shall be used to determine the RACH occasion for the PRACH transmission; otherwise, this field is reserved.. PRACH Mask index; If the value of the "Random Access Preamble index" is not all zeros, this field indicates the RACH occasion associated with the SS/PBCH indicated by "SS/PBCH index" for the PRACH transmission; otherwise, this field is reserved. Reserved bits; 10 bits. There are two options to provide RACH parameters of a candidate cell for early TA acquisition. Option 1: RACH configuration is included in the candidate cell RRC model. In current RRC model design, a RRCReconfiguration Message or an IE CellGroupConfig can be used for a candidate cell. Based on current ASN.1 structure, the common RACH configuration parameters are already available within IE ReconfigurationWithSync. Furthermore, there is a field rach-ConfigDedicated within IE ReconfigurationWithSync to provide CFRA configuration. If we follow this way, i.e., CFRA resource is configured in RRC message for a candidate cell, PDCCH order only needs to indicate which candidate cell is triggered for early TA acquisition. Or if CFRA resource is not configured in RRC message, which means the common RACH configuration is provided by RRC, and dedicated preamble configuration for CFRA (and also the candidate cell ID for early TA acquisition) is provided by PDCCH order only. Option 2: separate configuration, i.e., not a part of LTM candidate cell RRC model. Although option 1 can work, we still see two drawbacks of it. The first one is UE has to decode the candidate cell’s configuration before it’s selected as the target cell, which may lead to some unnecessary decoding operation at UE side. The second one is option 1 includes the RACH configuration for early TA as part of the LTM candidate cell configuration, thus the reconfiguration of RACH parameters can’t be done without touching the LTM candidate cell configuration. To avoid unnecessary decoding (e.g. the UE may partially decode just the RACH configuration for early TA acquisition) and make this procedure more flexible (e.g., the preamble configuration can be generated/updated later separately), the RACH configuration for early TA acquisition of a candidate cell can be a separate configuration as illustrated in Figure 3. Similar to option 1, this separate configuration can only include common RACH configuration parameters, then PDCCH order provides the remaining preamble configuration, or, this separate configuration can include all CFRA configuration parameters, then the PDCCH order only contains the indication of candidate cell ID (candidate configuration index). Figure 3: separate RACH configuration for early TA acquisition According to the latest RAN1 agreement as below, additional RRC configuration is needed for RAR reception. Since this indication is for the early TA before reception of Cell Switch Command, it makes more sense to provide this configuration out of the candidate cell RRC model. Agreement Whether RAR needs to be received is configured by RRC.. Proposal 1: RRC RACH configuration for early TA acquisition (e.g., including whether RAR needs to be received) is signalled separately to the candidate cell configuration. ## 2.3 How to handle the resource allocation of preamble for a candidate cell The preamble resource is configured by the combination of SSB index, PRACH mask index and preamble index as illustrated in Figure 4. Since one SSB index could map to multiple PRACH occasions, e.g., ssb-perRACH-Occasion is oneFourth and PRACH Mask Index is 0, this means all 4 PRACH occasions can be used for preamble transmission. And in each PRACH occasion, the preamble with the configured preamble index can be transmitted by UE. In total, there are 4 preamble resources that can be used by UE. Figure 4: the configuration of Preamble resources Preamble resource for early TA acquisition towards a candidate cell is allocated by this candidate cell. And there are two major options of preamble resource allocation. Option 1: preamble resource is UE specific. If CFRA resource (including SS/PBCH index, RACH occasion, and Random Access Preamble index) is configured in RRC, this resource is UE specific and can’t be used by other UEs. Option 2: preamble resource can be shared by multiple UEs. If only cell specific RACH configuration is provided in RRC, and PDCCH order indicates the other (UE specific) CFRA configuration, it can enable shared preamble resource by multiple UEs. E.g., the UE measures SSB beams of candidate cells and reports L1/L3 measurement results to source cell, then based on measurement report source cell selects a candidate cell, a PRACH occasion and a preamble according to a SSB index (e.g., this SSB is with the highest RSRP of this candidate cell). Then source cell indicates the preamble resource to UE by PDCCH order. From UE point of view, when the source cell triggers the PRACH via PDCCH order, the UE will use it for preamble transmission. Then the UE will release it, i.e., the UE doesn’t occupy this preamble resource all the time, and this preamble resource can be used by other UEs later. From network perspective, a candidate cell needs to allocate several preamble resources (e.g., preamble pool) to all UEs in a source cell, and source cell makes sure to trigger ONE UE at a time for early TA acquisition and maintain the mapping between preamble and UE, i.e., only one UE uses one specific preamble resource at a time. When a candidate cell receives the preamble, it estimates the TA value according to this preamble, and sends back the combination of TA value and preamble index to source cell (in case TA is sent to UE in source cell). An example procedure is illustrated in Figure 5. In this way, it can still save preamble resources by reusing preambles among multiple UEs for each candidate cell. Figure 5: the concept of preamble pool Proposal 2: To enable shared preamble resource among multiple UEs, CFRA configuration (i.e., SS/PBCH index, RACH occasion, and Random Access Preamble index) is indicated in the PDCCH order, i.e. not configured in the RRC RACH configuration.

Observation 0: RAN2 suggests sharing “preamble resources”, instead of only “preamble pool”, among multiple UEs and the PDCCH Order indicates the allocated CFRA resource (SSB index, PRACH mask index, and Preamble index).

Later in RAN2#123-bis (Xiamen) RAN2 also prepare an LS to RAN3 (treated as R3-237140 in Chicago).

R2-2311576 LS on CSI resource configuration and on early RACH for LTM Huawei LS out is approved in R2-2311332 --- For LTM CSI resource configuration, RAN2 agreed that: - the LTM CSI resource configuration is generated by the CU; - the list of LTM CSI resource configuration is common for all the LTM candidate cells (as in current RRC running CR). For early RACH for LTM, RAN2 agreed that: - For each candidate target cell towards which early RACH is supported, the UE is provided with a RACH configuration (per source per candidate), which can be the same for multiple source cells; - RAN2 understands that the source DU needs to know the early RACH configuration for each candidate cell, so that the source cell can know how to set the PDCCH order information for early RACH; - The candidate DU provides the TA value and its associated information to the source DU via the CU, e.g. preamble index, RO information (i.e. RA-RNTI) and candidate cell identity, so that the source DU can identify the UE. RAN3 can design the necessary network signalling.

RAN2 has implemented the agreed-upon changes in TS 38.300 below. The RRC RACH configuration for early TA acquisition, defined as EarlyUL-SyncConfig in TS 38.331, is adopted by RAN3 as part of the Early UL Sync Configuration IE in TS 38.473. The PDCCH Order, similar to legacy intra-cell PDCCH orders, indicates SS/PBCH index, RACH occasion, and Random Access Preamble index. Notably, unlike inter-DU early TA acquisition, legacy PDCCH orders are used intra-cell, primarily when the network has downlink data for a briefly desynchronized UE.

Observation 1: The UE is provided with both the PDCCH Order (i.e., SS/PBCH index, RACH occasion, and Random Access Preamble index) and the configured cell specific RACH parameters (in EarlyUL-Sync) for it to do CFRA to the candidate cell for early TA acquisition.

TS 38.300  9.2.3.5.2	C-Plane Handling

4b. The UE may perform UL synchronization with LTM candidate cell(s) before receiving the cell switch command, by using UE-based TA measurement, if configured, and/or by transmitting a preamble towards the candidate cell, as triggered by the gNB. When UE-based TA measurement is configured, UE acquires the TA value(s) of the candidate cell(s) by measurement. UE performs early TA acquisition with the candidate cell(s) as requested by the network before receiving the cell switch command as specified in clause 9.2.6. This is done via CFRA triggered by a PDCCH order from the source cell, following which the UE sends preamble towards the indicated candidate cell. In order to minimize the data interruption of the source cell due to CFRA towards the candidate cell(s), the UE does not receive random access response from the network for the purpose of TA value acquisition and the TA value of the candidate cell is indicated in the cell switch command. The UE does not maintain the TA timer for the candidate cell and relies on network implementation to guarantee the TA validity.

9.2.6	Random Access Procedure

For the random access procedure towards an LTM candidate cell for early UL TA acquisition, CFRA triggered by a PDCCH order is used. The UE sends MSG1 towards the cell without monitoring for a response from it as shown in Figure 9.2.6-1 (e). To support UE power ramping, the UE may perform MSG1 retransmission as indicated by the network. … (e) CFRA without network response with 4-step RA type Figure 9.2.6-1: Random Access Procedures

For easier access of the relevant reference for respective information such as table and IEs, they are attached in the appendix of this paper.

# 2 Discussion

## Early TA Acquisition

According to TS 38.300, the UE may perform UL synchronization with LTM candidate cell(s) before receiving the cell switch command by transmitting a preamble towards the candidate cell, as triggered by the gNB. This is done via CFRA triggered by a PDCCH order from the source cell, following which the UE sends preamble towards the indicated candidate cell. The PDCCH Order, similar to the CFRA parameters used in LTM Cell Switch Command MAC CE, also has the dynamic CFRA configuration including SSB index for random access, preamble index, rach occasion (RO) mask index, SUL/UL carrier for RA (can be found in the TS 38.212, Sec 7.3.1.2.1 as in the appendix). Therefore, the random access resources for CFRA is indicated in the PDCCH Order instead of the RRC reconfiguration to the UE.

In inter-DU scenario, the Early UL Sync Configuration IE in the current F1AP contains the cell specific RACH parameters EarlyUL-SyncConfig IE. The candidate gNB-DU derives a timing advance value and transmits the TA along with the TA information including the cell ID, preamble index, source gNB-DU ID, and a RA-RNTI in the DU-CU TA Information Transfer message to the gNB-CU and gNB-CU transmits the CU-DU TA Information Transfer message to the source gNB-DU. An illustration of inter-DU scenario per current specification is given below.

Figure 1: Illustration of Ealry TA acquisition for inter-DU LTM as per current specification

From the UE perspective, it is able to derive the same mapping between the PRACH occasions and the SSBs as the candidate gNB-DU with the parameters (e.g., by the ssb-PerRACH-Occasion-r18 and msg1-FDM) obtained in the ltm-EarlyUL-SyncConfig-r18 or ltm-EarlyUL-SyncConfigSUL-r18 and the ltm-SSB-Config (e.g., ssb-PositionsInBurst-r18). An example illustration how the SSB to PRACH occasion mapping is done is provided below with some parameter assumed.

An example of TS 38.211 Table 6.3.3.2-3: Random access configurations for FR1 and unpaired spectrum

| PRACH Configuration Index | Preamble format | nf mod x=y |  | Subframe number | Starting symbol | Number of PRACH slots within a subframe | , number of time-domain PRACH occasions within a PRACH slot | , PRACH duration |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 109 | A2 | 1 | 0 | 0,1,2,3,4,5,6,7,8,9 | 9 | 1 | 1 | 4 |

Figure 2: Example #1 illustration for the PRACH occasion and SSB Index mapping

However, for the inter-gNB-DU scenario, from the source gNB-DU perspective, there are additional issues and restrictions for the current specification.

#### Issue 1: RACH Occasion information association upon TA Information Transfer

First, a RA-RNTI is carried in the DU-CU TA Information Transfer message for indicating the exact PRACH occasion the UE has transmitted the RA preamble. At the source gNB-DU side, to calculate a RA-RNTI, it first needs to construct a mapping as shown in Fig.2. To have the exact mapping between the SSB indexes and the PRACH occasions, the source gNB-DU needs to have the ssb-PositionsInBurst; however, it is absent at the source gNB-DU side from the current specifications.

Without the knowledge of ssb-PositionsInBurst, the source gNB-DU does not know the exact number of SSB’s as well as the exact SSB Indexes transmitted by the candidate gNB-DU. Therefore, it cannot construct a mapping as shown in the Figure 1 and would fail to calculate the RA-RNTI. In case that the source gNB-DU needs to differentiate two UEs with the same candidate cell ID and preamble index but different RA-RNTIs (e.g., the PDCCH orders for the two UEs have different SSB Index and/or PRACH Mask Index value so that the PRACH occasions are different), the source gNB-DU would fail to differentiate and utilize the TA values for the Early UL Sync for the two UEs.

RA-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id [6]

Observation 2: In current specification, in the inter-DU scenarios, the source gNB-DU does not know the ssb-PositionsInBurst for the candidate cell and cannot create the PRACH Occasion to SSB index mapping. It is unable to calculate the RA-RNTI value.

One may argue that the source gNB-DU might be able to deduce the exact SSB Indexes transmitted by the candidate gNB-DU by the LTM-CSI-ResourceConfig. However, the gNB-CU, which generates the LTM-CSI-ResourceConfig, can selectively include SSB resource sets. For example, the gNB-DU1 indicates to the gNB-CU {SSB0, SSB2, SSB3, SSB5, SSB6, SSB7} in the ssb-PositionsInBurst of candidate ID1 and the gNB-DU2 indicates {SSB0, SSB1, SSB2} in the ssb-PositionsInBurst of candidate ID2. The gNB-CU based on these can include ({SSB0, ID1}, {SSB2, ID1}, {SSB5, ID1}, {SSB1, ID2}) in the SSB resource set within LTM-CSI-ResourceConfig (i.e., SSB3 is not selected by the gNB-CU). It can be seen that that the source gNB-DU might not be able to infer the exact SSB Indexes transmitted by the candidate gNB-DU for candidate ID1. In other words, it cannot differentiate if the ssb-PositionsInBurst should be interpreted as {SSB0, SSB1, SSB2, SSB3, SSB4, SSB5} or {SSB0, SSB2, SSB5}, for example, which can result in different actual SSB to PRACH occasion mappings.

Figure 3: Example #2 illustration for the PRACH occasion and SSB Index mapping

This discrepancy in understanding RACH Occasion to SSB index mapping can lead to errors in calculating RA-RNTI, which depends on the position of the PRACH occasion that the preamble index is sent. If the source gNB-DU infers the PRACH occasion location differently from the candidate gNB-DU and UE, it may derive an incorrect RA-RNTI, causing issues in associating the TA Information Transfer message. The mapping logic of SSB and PRACH occasion(s) is described in TS 38.213 Sec 8.1.

For the Example #2 shown in Fig. 3, the source gNB-DU would think the PRACH occasions associated with the SSB5 for candidate ID1 locates at Slot #4 (if interpreting the sequence as SSB 0, 2, 5) while the candidate gNB-DU and the UE thinks Slot #6 (the actual sequence is SSB 0, 2, 3, 5). This then leads to different RA-RNTI values calculated at the source gNB-DU and the candidate gNB-DU for the same UE. Therefore, unless there is a clear guidance that the gNB-CU always creates a SSB resource set containing a complete list of SSB Indexes associated with a candidate cell, the LTM-CSI-ResourceConfig cannot serve as a replacement for the ssb-PositionsInBurst for such purpose.

Proposal 1: The ssb-PositionsInBurst for the candidate cell at the candidate gNB-DU is provided to the source gNB-DU for obtaining PRACH occasion to SSB Index mapping and calculating the RA-RNTI in the Early TA Acquisition.

#### Issue 2: CFRA preamble resources utilization in PDCCH Order

Second, since only the “preamble index list” is given to the source gNB-DU, it must determine how to fill in the SSB Index and the PRACH Mask Index in the PDCCH Order. Since the candidate gNB-DU doesn't specify these to the source gNB-DU, it needs to assume the UE could use any RACH Occasion associated with any SSB.

Considering the candidate gNB-DU might have assigned the same cell-specific parameters to UEs from another gNB-DU, it can only assign a unique preamble index list to UEs from this source gNB-DU to prevent collision. This could lead to the candidate gNB-DU quickly running out of assignable preamble indexes for this cell-specific parameter. This is an additional limitation not present in intra-gNB-DU scenarios where all preamble resource parameters are controlled.

Under the current specification, while assigning one preamble index per gNB-DU might alleviate the issue, the source gNB-DU must trigger ONE UE at a time for early TA acquisition for this preamble “index”, making it less efficient than when the source gNB-DU has a set of preamble "resources" allocated by the candidate gNB-DU and only needs to ensure one UE uses one specific preamble resource (a combination of SSB index, PRACH mask index, and preamble index) at a time.

For the case that one RACH Occasions can only associated with one SSB (i.e., ssb-PerRACH-Occasion-r18 is 1/8, ¼, ½, or 1), especially that the msg1-FDM has a value more than one (e.g., 2, 4, 8), the candidate gNB-DU can have more ways to differentiate the UE from a certain source gNB-DU like a combination of SSB Index, PRACH Mask Index, and Preamble Index. Therefore, the same preamble index value can be assigned to different gNB-DUs and the candidate gNB-DU can still differentiate the UE by the exact RACH Occasion the preamble index was received (with different SSB or different position in time or frequency domain, for example).The most straightforward way to signal such preamble resource would be using the RACH-ConfigDedicated IE to signal the necessary PDCCH Order Information to the source gNB-DU.

Observation 3: In the intra-DU scenario, the preamble resource the gNB-DU can utilize is the combination of SSB index, PRACH mask index and preamble index for the early TA acquisition. The gNB-DU can trigger multiple UEs for early TA acquisition as long as the preamble resources are different.

Observation 4: In inter-DU scenarios, the candidate gNB-DU's limited signalling (only preamble index) forces it to assume all PRACH occasions are usable, assigning unique preamble indexes per source gNB-DU to avoid collisions, unlike the optimized resource use in intra-DU scenarios.

Proposal 2: The candidate gNB-DU provides the RACH Dedicated Configurations with the allowable preamble resources to the source gNB-DU for the CFRA resources for Early Sync.

#### Options for signalling the PDCCH Order Information for issue 2

In the previous meeting, for the PDCCH Order Information, two options were brought up for sending RACH-ConfigDedicated from the candidate gNB-DU to the source gNB-DU via the gNB-CU. The first option involves a single RACH-ConfigDedicated for the source gNB-DU and potential gNB-DUs for subsequent LTM, while the second option provides separate RACH-ConfigDedicated to each. The first option is compatible with the current per-gNB-DU preamble index list, provided the single RACH-ConfigDedicated contains comprehensive information on all preamble indexes and associated SSB indexes for reference by the source gNB-DU.

#### Option 1 – Add one PDCCH Order Information #### 9.3.1.328 Early UL Sync Configuration This IE indicates the early UL sync configurations for the UE. IE/Group Name; Presence; Range; IE type and reference; Semantics description. RACH Configuration; M; OCTET STRING; Includes the EarlyUL-SyncConfig IE, as defined in TS 38.331 [8].. PDCCH Order Information; O; OCTET STRING; Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. This IE contains the CFRA resource configuration for the PDCCH order.. LTM gNB-DUs List; 0..1; This IE contains the IDs of the source gNB-DU and candidate gNB-DU(s).. >LTM gNB-DUs Item IEs; 1..< maxnoofLTMgNBDUs>. >>LTM gNB-DU ID; M; gNB-DU ID 9.3.1.9. >>Preamble Index List; O; 9.3.1.329. Range bound; Explanation. maxnoofLTMgNBDUs; Maximum no. of gNB-DUs allowed to be configured with LTM towards one UE, the maximum value is 8.. #### 9.3.1.329 Preamble Index List This IE indicates the list of preamble indexes to be used for the UE. IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Preamble Index Item IEs; 1..< maxnoofLTMCells>; -. >Preamble Index; M; INTEGER (0..63); -. Range bound; Explanation. maxnoofLTMCells; Maximum no. of Cells configured LTM allowed towards one UE, the maximum value is 8..

Option 2 offers a greater flexibility in setting the RO mask index per gNB-DU, as only one ra-ssb-OccasionMaskIndex can be configured in a single RACH-ConfigDedicated IE. With per-gNB-DU PDCCH Order Information, the configurable preamble resources for early TA can be the combination of (SSB Index, Preamble Index, and RO Mask), whereas option 1 might limit the combination to (SSB Index and Preamble Index) with a shared RO Mask. This additional flexibility in option 2, however, eliminates the need for the preamble index list as it has been covered by the RACH-ConfigDedicated IE.

#### Option 2 – Add per-(source)gNB-DU PDCCH Order Information #### 9.3.1.328 Early UL Sync Configuration This IE indicates the early UL sync configurations for the UE. IE/Group Name; Presence; Range; IE type and reference; Semantics description. RACH Configuration; M; OCTET STRING; Includes the EarlyUL-SyncConfig IE, as defined in TS 38.331 [8].. LTM gNB-DUs List; 0..1; This IE contains the IDs of the source gNB-DU and candidate gNB-DU(s).. >LTM gNB-DUs Item IEs; 1..< maxnoofLTMgNBDUs>. >>LTM gNB-DU ID; M; gNB-DU ID 9.3.1.9. >>PDCCH Order Information; O; OCTET STRING; Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. This IE contains the CFRA resource configuration for the PDCCH order.. Range bound; Explanation. maxnoofLTMgNBDUs; Maximum no. of gNB-DUs allowed to be configured with LTM towards one UE, the maximum value is 8.. #### 9.3.1.329 Void

Given that both options introduce non-backward-compatible changes to the stage 3 specification, we propose adopting option 2 to address the PDCCH Order Information issue, as it provides greater implementation flexibility.

Proposal 3: RAN3 to agree the CR in R3-24XXXX to signal the SSB Positions In Burst and RACH Dedicated Configurations for the CFRA resources for Early Sync.

# 3	Conclusion

Observation 0: RAN2 suggests sharing “preamble resources”, instead of only “preamble pool”, among multiple UEs and the PDCCH Order indicates the allocated CFRA resource (SSB index, PRACH mask index, and Preamble index).

Observation 1: The UE is provided with both the PDCCH Order (i.e., SS/PBCH index, RACH occasion, and Random Access Preamble index) and the configured cell specific RACH parameters (in EarlyUL-Sync) for it to do CFRA to the candidate cell for early TA acquisition.

Observation 2: In current specification, in the inter-DU scenarios, the source gNB-DU does not know the ssb-PositionsInBurst for the candidate cell and cannot create the PRACH Occasion to SSB index mapping. It is unable to calculate the RA-RNTI value.

Proposal 1: The ssb-PositionsInBurst for the candidate cell at the candidate gNB-DU is provided to the source gNB-DU for obtaining PRACH occasion to SSB Index mapping and calculating the RA-RNTI in the Early TA Acquisition.

Observation 3: In the intra-DU scenario, the preamble resource the gNB-DU can utilize is the combination of SSB index, PRACH mask index and preamble index for the early TA acquisition. The gNB-DU can trigger multiple UEs for early TA acquisition as long as the preamble resources are different.

Observation 4: In inter-DU scenarios, the candidate gNB-DU's limited signalling (only preamble index) forces it to assume all PRACH occasions are usable, assigning unique preamble indexes per source gNB-DU to avoid collisions, unlike the optimized resource use in intra-DU scenarios.

Proposal 2: The candidate gNB-DU provides the RACH Dedicated Configurations with the allowable preamble resources to the source gNB-DU for the CFRA resources for Early Sync.

Proposal 3: RAN3 to agree the CR in R3-24XXXX to signal the SSB Positions In Burst and RACH Dedicated Configurations for the CFRA resources for Early Sync.

# 4	Reference

[1] R2_121bis-e Chair Notes EOM

[2] R2-2304553 (R3-232575), Reply LS on L1 measurement RS configuration and PDCCH ordered RACH for LTM, Fujitsu, CATT

[3] R2-2302750, Discussion on the early TA acquisition, Intel Corporation

[4] R3-243791, Summary of offline for Mobility Enhancements, Ericsson

[5] TS 38.212v18.2.0

[6] TS 38.321v18.2.0

[7] TS 38.331v18.2.0

[8] TS 38.300v18.2.0

[9] R3-243162, Corrections for CFRA resource provision in LTM, Google

# Appendix

TS 38.213, Section 8.1 – the SSB index to PRACH occasion mapping logic

SS/PBCH block indexes provided by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon are mapped to valid PRACH occasions in the following order where the parameters are described in [4, TS 38.211]. - First, in increasing order of preamble indexes within a single PRACH occasion - Second, in increasing order of frequency resource indexes for frequency multiplexed PRACH occasions - Third, in increasing order of time resource indexes for time multiplexed PRACH occasions within a PRACH slot - Fourth, in increasing order of indexes for PRACH slots An association period, starting from frame 0, for mapping SS/PBCH block indexes to PRACH occasions is the smallest integer number in the set determined by the PRACH configuration period according Table 8.1-1 such that NTxSSB SS/PBCH block indexes are mapped at least once to the PRACH occasions within the association period, where a UE obtains NTxSSB from the value of ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon. If after an integer number of SS/PBCH block indexes to PRACH occasions mapping cycles within the association period there is a set of PRACH occasions or PRACH preambles that are not mapped to NTxSSB SS/PBCH block indexes, no SS/PBCH block indexes are mapped to the set of PRACH occasions or PRACH preambles. An association pattern period includes one or more association periods and is determined so that a pattern between PRACH occasions and SS/PBCH block indexes repeats at most every 160 msec. PRACH occasions not associated with SS/PBCH block indexes after an integer number of association periods, if any, are not used for PRACH transmissions. … Table 8.1-1: Mapping between PRACH configuration period and SS/PBCH block to PRACH occasion association period PRACH configuration period (msec); Association period (number of PRACH configuration periods). 10; {1, 2, 4, 8, 16}. 20; {1, 2, 4, 8}. 40; {1, 2, 4}. 80; {1, 2}. 160; {1}.

TS 38.212, Sec 7.3.1.2.1 – the content of PDCCH order

DCI format 1_0 is used for the scheduling of PDSCH in one DL cell. The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI: - Identifier for DCI formats - 1 bits - The value of this bit field is always set to 1, indicating a DL DCI format - Frequency domain resource assignment - bits where is given by Clause 7.3.1.0 If the CRC of the DCI format 1_0 is scrambled by C-RNTI and the "Frequency domain resource assignment" field are of all ones, the DCI format 1_0 is for random access procedure initiated by a PDCCH order, with all remaining fields set as follows: - Random Access Preamble index - 6 bits according to ra-PreambleIndex in Clause 5.1.2 of [8, TS38.321] - UL/SUL indicator - 1 bit. If the value of the "Random Access Preamble index" is not all zeros and if the UE is configured with supplementaryUplink in ServingCellConfig in the cell, this field indicates which UL carrier in the cell to transmit the PRACH according to Table 7.3.1.1.1-1; otherwise, this field is reserved - SS/PBCH index - 6 bits. If the value of the "Random Access Preamble index" is not all zeros, this field indicates the SS/PBCH that shall be used to determine the RACH occasion for the PRACH transmission; otherwise, this field is reserved. - PRACH Mask index - 4 bits. If the value of the "Random Access Preamble index" is not all zeros, this field indicates the RACH occasion associated with the SS/PBCH indicated by "SS/PBCH index" for the PRACH transmission, according to Clause 5.1.1 of [8, TS38.321]; otherwise, this field is reserved - Cell indicator - log2C+1 bits indicating the cell for the corresponding PRACH transmission if the UE is configured with higher layer parameter EarlyUlSyncConfig, where C is the number of candidate cells configured with higher layer parameter EarlyUlSyncConfig; 0 bit otherwise. The bit field index 0 of the cell indicator field is mapped to the serving cell, and other bit field indexes are mapped to the candidate cells configured with higher layer parameter EarlyUlSyncConfig according to an ascending order of a candidate identity configured by ltm-CandidateId, with the bit field index 1 mapped to the candidate cell with the smallest candidate identity. …

TS 38.321 – RA-RNTI calculation and PRACH Mask Index/ ra-ssb-OccasionMaskIndex values

### 5.1.3 Random Access Preamble transmission … The RA-RNTI associated with the PRACH occasion in which the Random Access Preamble is transmitted or the RA-RNTI associated with the last valid PRACH occasion in the set of PRACH occasions (as specified in TS 38.213 [6]) for Msg1 repetition, is computed as: RA-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id where s_id is the index of the first OFDM symbol of the PRACH occasion (0 ≤ s_id < 14), t_id is the index of the first slot of the PRACH occasion in a system frame (0 ≤ t_id < 80), where the subcarrier spacing to determine t_id is based on the value of μ specified in clause 5.3.2 in TS 38.211 [8] for μ = {0, 1, 2, 3}, and for μ = {5, 6}, t_id is the index of the 120 kHz slot in a system frame that contains the PRACH occasion (0 ≤ t_id < 80), f_id is the index of the PRACH occasion in the frequency domain (0 ≤ f_id < 8), and ul_carrier_id is the UL carrier used for Random Access Preamble transmission (0 for NUL carrier, and 1 for SUL carrier). ## 7.4 PRACH Mask Index values Table 7.4-1: PRACH Mask Index values PRACH Mask Index/ msgA-SSB-SharedRO-MaskIndex/ ssb-SharedRO-MaskIndex; Allowed PRACH occasion(s) of SSB. 0; All. 1; PRACH occasion index 1. 2; PRACH occasion index 2. 3; PRACH occasion index 3. 4; PRACH occasion index 4. 5; PRACH occasion index 5. 6; PRACH occasion index 6. 7; PRACH occasion index 7. 8; PRACH occasion index 8. 9; Every even PRACH occasion. 10; Every odd PRACH occasion. 11; Reserved. 12; Reserved. 13; Reserved. 14; Reserved. 15; Reserved.

TS 38.473

#### 9.2.1.24 DU-CU TA INFORMATION TRANSFER This message is sent by the gNB-DU to inform the gNB-CU about TA information. Direction: gNB-DU gNB-CU IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Message Type; M; 9.3.1.1; YES; ignore. Transaction ID; M; 9.3.1.23; YES; reject. DU to CU TA Information List; 1; YES; ignore. >DU to CU TA Information Item IEs; 1 .. <maxnoofTAList>; EACH; ignore. >>Candidate Cell ID; M; NR CGI 9.3.1.12; -. >>TA Value; M; INTEGER (0..4095); Indicates the TA value as defined in TS 38.213 [31].; -. >>Preamble Index; M; INTEGER (0..63); -. >>RA-RNTI; M; INTEGER (0..65535, ...); RA-RNTI as defined in TS 38.321 [16].; -. >>Source gNB-DU ID; M; gNB-DU ID 9.3.1.9; -. >>Tag ID Pointer; O; OCTET STRING; Includes the tag-Id-ptr contained in the TCI-UL-State IE or the TCI-State IE, as defined in TS 38.331 [8].; -. #### … #### 9.3.1.328 Early UL Sync Configuration This IE indicates the early UL sync configurations for the UE. IE/Group Name; Presence; Range; IE type and reference; Semantics description. RACH Configuration; M; OCTET STRING; Includes the EarlyUL-SyncConfig IE, as defined in TS 38.331 [8].. LTM gNB-DUs List; 0..1; This IE contains the IDs of the source gNB-DU and candidate gNB-DU(s).. >LTM gNB-DUs Item IEs; 1..< maxnoofLTMgNBDUs>. >>LTM gNB-DU ID; M; gNB-DU ID 9.3.1.9. >>Preamble Index List; O; 9.3.1.329. Range bound; Explanation. maxnoofLTMgNBDUs; Maximum no. of gNB-DUs allowed to be configured with LTM towards one UE, the maximum value is 8.. #### 9.3.1.329 Preamble Index List This IE indicates the list of preamble indexes to be used for the UE. IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Preamble Index Item IEs; 1..< maxnoofLTMCells>; -. >Preamble Index; M; INTEGER (0..63); -. Range bound; Explanation. maxnoofLTMCells; Maximum no. of Cells configured LTM allowed towards one UE, the maximum value is 8..

#### EarlyUL-SyncConfig

The IE EarlyUL-SyncConfig is used to configure random access resources for the early UL synchronization procedure.

EarlyUL-SyncConfig information element

-- ASN1START

-- TAG-EARLYUL-SYNCCONFIG-START

EarlyUL-SyncConfig-r18 ::=         SEQUENCE {

frequencyInfoUL-r18                FrequencyInfoUL,

rach-ConfigGeneric-r18             RACH-ConfigGeneric,

bwp-GenericParameters-r18          BWP,

ssb-PerRACH-Occasion-r18           ENUMERATED {oneEighth, oneFourth, oneHalf, one, two, four, eight, sixteen} OPTIONAL, -- Need R

prach-RootSequenceIndex-r18        CHOICE {

l839                               INTEGER (0..837),

l139                               INTEGER (0..137)

}                                                                                                             OPTIONAL, -- Need R

ltm-PRACH-SubcarrierSpacing-r18    SubcarrierSpacing                                                          OPTIONAL, -- Need L139

n-TimingAdvanceOffset-r18          ENUMERATED { n0, n25600, n39936, spare1 }                                  OPTIONAL, -- Need R

...

}

-- TAG-EARLYUL-SYNCCONFIG-STOP

-- ASN1STOP

EarlyUL-SyncConfig field descriptions

frequencyInfoUL This field provides basic parameters of an uplink carrier for PRACH transmission on a candidate cell.

ltm-PRACH-SubcarrierSpacing Indicates subcarrier spacing of PRACH for LTM (see TS 38.211 [16], clause 5.3.2). Only the following values are applicable depending on the used frequency: FR1: 15 or 30 kHz FR2-1: 60 or 120 kHz FR2-2: 120, 480, or 960 kHz If absent, the UE applies the SCS as derived from the prach-ConfigurationIndex in RACH-ConfigGeneric (see tables Table 6.3.3.1-1, Table 6.3.3.1-2, Table 6.3.3.2-2 and Table 6.3.3.2-3, TS 38.211 [16]).

n-TimingAdvanceOffset The N_TA-Offset to be applied for all uplink transmissions on a candidate cell.

rach-ConfigGeneric RACH parameters for performing a random access procedure on a candidate cell.

ssb-PerRACH-Occasion This field indicated the number of SSBs for RACH occasion.

| Conditional Presence | Explanation |
| --- | --- |
| L139 | The field is mandatory present if prach-RootSequenceIndex L=139, otherwise the field is absent, Need S. |

#### LTM-Candidate

The IE LTM-Candidate concerns a LTM candidate configuration to add or modify.

LTM-Candidate information element

-- ASN1START

-- TAG-LTM-CANDIDATE-START

LTM-Candidate-r18 ::=     SEQUENCE {

ltm-CandidateId-r18                            LTM-CandidateId-r18,

ltm-CandidatePCI-r18                           PhysCellId                                            OPTIONAL,    -- Need M

ltm-SSB-Config-r18                             LTM-SSB-Config-r18                                    OPTIONAL,    -- Need M

ltm-CandidateConfig-r18                        OCTET STRING (CONTAINING RRCReconfiguration)          OPTIONAL,    -- Need M

ltm-ConfigComplete-r18                         ENUMERATED {true}                                     OPTIONAL,    -- Need R

ltm-EarlyUL-SyncConfig-r18                     OCTET STRING (CONTAINING EarlyUL-SyncConfig-r18)      OPTIONAL,    -- Need R

ltm-EarlyUL-SyncConfigSUL-r18                  OCTET STRING (CONTAINING EarlyUL-SyncConfig-r18)      OPTIONAL,    -- Need R

ltm-TCI-Info-r18                               LTM-TCI-Info-r18                                      OPTIONAL,    -- Need M

ltm-NoResetID-r18                              INTEGER (1..maxNrofLTM-Configs-plus1-r18)             OPTIONAL,    -- Need N

ltm-UE-MeasuredTA-ID-r18                       INTEGER (1..maxNrofLTM-Configs-plus1-r18)             OPTIONAL,    -- Need N

...

}

LTM-SSB-Config-r18 ::= SEQUENCE {

ssb-Frequency-r18                              ARFCN-ValueNR,

subcarrierSpacing-r18                          SubcarrierSpacing,

ssb-Periodicity-r18                            ENUMERATED {ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1} OPTIONAL,   -- Need R

ssb-PositionsInBurst-r18                       CHOICE {

shortBitmap                                    BIT STRING (SIZE (4)),

mediumBitmap                                   BIT STRING (SIZE (8)),

longBitmap                                     BIT STRING (SIZE (64))

}                                                                                                    OPTIONAL,   -- Need R

ss-PBCH-BlockPower-r18                         INTEGER (-60..50)                                     OPTIONAL,   -- Need R

...

}

-- TAG-LTM-CANDIDATE-STOP

-- ASN1STOP

LTM-Candidate field descriptions

ltm-CandidateConfig This field includes an RRCReconfiguration message used to configure an LTM candidate configuration.

ltm-CandidateId This field indicates an LTM candidate configuration.

ltm-CandidatePCI This field identifies the PCI of the SpCell of the configuration contained in ltm-CandidateConfig.

ltm-ConfigComplete This field indicates whether the LTM candidate configuration within ltm-CandidateConfig is a complete configuration.

ltm-EarlyUL-SyncConfig, ltm-EarlyUL-SyncConfigSUL A configuration used to perform the early UL synchronization procedure over an UL or SUL carrier.

#### LTM-Config

The IE LTM-Config is used to provide LTM candidate configurations.

LTM-Config information element

-- ASN1START

-- TAG-LTM-CONFIG-START

LTM-Config-r18 ::=   SEQUENCE {

ltm-ReferenceConfiguration-r18        SetupRelease {ReferenceConfiguration-r18}                             OPTIONAL,   -- Need M

ltm-CandidateToReleaseList-r18        SEQUENCE (SIZE (1..maxNrofLTM-Configs-r18)) OF LTM-CandidateId-r18    OPTIONAL,   -- Need N

ltm-CandidateToAddModList-r18         SEQUENCE (SIZE (1..maxNrofLTM-Configs-r18)) OF LTM-Candidate-r18      OPTIONAL,   -- Need N

ltm-ServingCellNoResetID-r18          INTEGER (1..maxNrofLTM-Configs-plus1-r18)                             OPTIONAL,   -- Need N

ltm-CSI-ResourceConfigToAddModList-r18  SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourceConfigurations-r18)) OF LTM-CSI-ResourceConfig-r18

OPTIONAL,   -- Need N

ltm-CSI-ResourceConfigToReleaseList-r18 SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourceConfigurations-r18)) OF LTM-CSI-ResourceConfigId-r18

OPTIONAL,   -- Need N

attemptLTM-Switch-r18                 ENUMERATED {true}                                                    OPTIONAL, -- Cond LTM-MCG

ltm-ServingCellUE-MeasuredTA-ID-r18   INTEGER (1..maxNrofLTM-Configs-plus1-r18)                             OPTIONAL,   -- Need N

...

}

-- TAG-LTM-CONFIG-STOP

-- ASN1STOP

LTM-Config field descriptions

attemptLTM-Switch If present, the UE shall execute an LTM cell switch if selected cell is a LTM candidate cell as described in clause 5.3.7.3.

ltm-ReferenceConfiguration This field includes an RRCReconfiguration message used to configure a reference configuration for LTM.

| Conditional Presence | Explanation |
| --- | --- |
| LTM-MCG | This field is optional present for the MCG, Need R, if the UE is configured with at least an LTM candidate configuration associated to the MCG. Otherwise, the field absent. |

#### RACH-ConfigDedicated

The IE RACH-ConfigDedicated is used to specify the dedicated random access parameters.

RACH-ConfigDedicated information element

-- ASN1START

-- TAG-RACH-CONFIGDEDICATED-START

RACH-ConfigDedicated ::=        SEQUENCE {

cfra                            CFRA                                                                    OPTIONAL, -- Need S

ra-Prioritization               RA-Prioritization                                                       OPTIONAL, -- Need N

...,

[[

ra-PrioritizationTwoStep-r16    RA-Prioritization                                                       OPTIONAL, -- Need N

cfra-TwoStep-r16                CFRA-TwoStep-r16                                                        OPTIONAL  -- Need S

]]

}

CFRA ::=                    SEQUENCE {

occasions                       SEQUENCE {

rach-ConfigGeneric              RACH-ConfigGeneric,

ssb-perRACH-Occasion            ENUMERATED {oneEighth, oneFourth, oneHalf, one, two, four, eight, sixteen}

OPTIONAL  -- Cond Mandatory

}                                                                                                       OPTIONAL, -- Need S

resources                       CHOICE {

ssb                             SEQUENCE {

ssb-ResourceList                SEQUENCE (SIZE(1..maxRA-SSB-Resources)) OF CFRA-SSB-Resource,

ra-ssb-OccasionMaskIndex        INTEGER (0..15)

},

csirs                           SEQUENCE {

csirs-ResourceList              SEQUENCE (SIZE(1..maxRA-CSIRS-Resources)) OF CFRA-CSIRS-Resource,

rsrp-ThresholdCSI-RS            RSRP-Range

}

},

...,

[[

totalNumberOfRA-Preambles INTEGER (1..63)                                                             OPTIONAL -- Cond Occasions

]],

[[

msg1-RepetitionNum-r18          ENUMERATED {n2, n4, n8, spare1}                                               OPTIONAL -- Cond 4StepCFRArep

]]

}

CFRA-TwoStep-r16 ::=                    SEQUENCE {

occasionsTwoStepRA-r16                  SEQUENCE {

rach-ConfigGenericTwoStepRA-r16         RACH-ConfigGenericTwoStepRA-r16,

ssb-PerRACH-OccasionTwoStepRA-r16       ENUMERATED {oneEighth, oneFourth, oneHalf, one,

two, four, eight, sixteen}

}                                                                                                     OPTIONAL, -- Need S

msgA-CFRA-PUSCH-r16                     MsgA-PUSCH-Resource-r16,

msgA-TransMax-r16                       ENUMERATED {n1, n2, n4, n6, n8, n10, n20, n50, n100, n200}    OPTIONAL, -- Need S

resourcesTwoStep-r16                    SEQUENCE {

ssb-ResourceList                        SEQUENCE (SIZE(1..maxRA-SSB-Resources)) OF CFRA-SSB-Resource,

ra-ssb-OccasionMaskIndex                INTEGER (0..15)

},

...

}

CFRA-SSB-Resource ::=           SEQUENCE {

ssb                             SSB-Index,

ra-PreambleIndex                INTEGER (0..63),

...,

[[

msgA-PUSCH-Resource-Index-r16   INTEGER (0..3071)     OPTIONAL  -- Cond 2StepCFRA

]]

}

CFRA-CSIRS-Resource ::=         SEQUENCE {

csi-RS                          CSI-RS-Index,

ra-OccasionList                 SEQUENCE (SIZE(1..maxRA-OccasionsPerCSIRS)) OF INTEGER (0..maxRA-Occasions-1),

ra-PreambleIndex                INTEGER (0..63),

...

}

-- TAG-RACH-CONFIGDEDICATED-STOP

-- ASN1STOP

CFRA-CSIRS-Resource field descriptions

csi-RS The ID of a CSI-RS resource defined in the measurement object associated with this serving cell.

ra-OccasionList RA occasions that the UE shall use when performing CF-RA upon selecting the candidate beam identified by this CSI-RS. The network ensures that the RA occasion indexes provided herein are also configured by prach-ConfigurationIndex and msg1-FDM. Each RACH occasion is sequentially numbered, first, in increasing order of frequency resource indexes for frequency multiplexed PRACH occasions; second, in increasing order of time resource indexes for time multiplexed PRACH occasions within a PRACH slot and Third, in increasing order of indexes for PRACH slots.

ra-PreambleIndex The RA preamble index to use in the RA occasions associated with this CSI-RS.

CFRA field descriptions

msg1-RepetitionNum Indicates the MSG1 repetition number used for contention free 4-step random access type in TS 38.321 [3]. If this field is absent, the UE performs contention free 4-step random access without MSG1-Repetitions.

occasions RA occasions for contention free random access. If the field is absent, the UE uses the RA occasions configured in RACH-ConfigCommon in the first active UL BWP.

ra-ssb-OccasionMaskIndex Explicitly signalled PRACH Mask Index for RA Resource selection in TS 38.321 [3]. The mask is valid for all SSB resources signalled in ssb-ResourceList.

rach-ConfigGeneric Configuration of contention free random access occasions for CFRA. The UE shall ignore preambleReceivedTargetPower, preambleTransMax, powerRampingStep, ra-ResponseWindow signaled within this field and use the corresponding values provided in RACH-ConfigCommon.

ssb-perRACH-Occasion Number of SSBs per RACH occasion.

totalNumberOfRA-Preambles Total number of preambles used for contention free random access in the RACH resources defined in CFRA, excluding preambles used for other purposes (e.g. for SI request). If the field is absent but the field occasions is present, the UE may assume all the 64 preambles are for RA. The setting should be consistent with the setting of ssb-perRACH-Occasion, if present, i.e. it should be a multiple of the number of SSBs per RACH occasion.

CFRA-SSB-Resource field descriptions

msgA-PUSCH-Resource-Index Identifies the index of the PUSCH resource used for MSGA CFRA. The PUSCH resource index indicates a valid PUSCH occasion (as specified in TS 38.213 [13], clause 8.1A) and the associated DMRS resources corresponding to a PRACH slot. The PUSCH resource indexes are sequentially numbered and are mapped to valid PUSCH occasions corresponding to a PRACH slot which are ordered, first, in increasing order of frequency resource indexes for frequency multiplexed PUSCH occasions; second, in increasing order of DMRS resource indexes within a PUSCH occasion, where a DMRSid resource index is determined first in an ascending order of a DMRS port index and then in an ascending order of a DMRS sequence index, third in increasing order of time resource indexes for time multiplexed PUSCH occasions within a PUSCH slot and fourth, in increasing order of indexes for PUSCH slots. For the case of contention free 2-step random access type, if this field is absent, the UE shall use the value 0.

ra-PreambleIndex The preamble index that the UE shall use when performing CF-RA upon selecting the candidate beams identified by this SSB.

ssb The ID of an SSB transmitted by this serving cell.

CFRA-TwoStep field descriptions

msgA-CFRA-PUSCH PUSCH resource configuration(s) for msgA CFRA.

msgA-TransMax Max number of MsgA preamble transmissions performed before switching to 4-step type random access (see TS 38.321 [3], clauses 5.1.1). This field is only applicable when 2-step and 4-step RA type are configured and switching to 4-step type RA is supported. If the field is absent in cfra-TwoStep, switching from 2-step RA type to 4-step RA type is not allowed.

occasionsTwoStepRA RA occasions for contention free random access. If the field is absent, the UE uses the RA occasions configured in RACH-ConfigCommonTwoStepRA in the first active UL BWP.

ra-SSB-OccasionMaskIndex Explicitly signalled PRACH Mask Index for RA Resource selection in TS 38.321 [3]. The mask is valid for all SSB resources signalled in ssb-ResourceList.

rach-ConfigGenericTwoStepRA Configuration of contention free random access occasions for CFRA 2-step random access type.

ssb-PerRACH-OccasionTwoStep Number of SSBs per RACH occasion for 2-step random access type.

RACH-ConfigDedicated field descriptions

cfra Parameters for contention free random access to a given target cell. If this field and cfra-TwoStep are absent, the UE performs contention based random access.

cfra-TwoStep Parameters for contention free 2-step random access type to a given target cell. Network ensures that cfra and cfra-TwoStep are not configured at the same time. If this field and cfra are absent, the UE performs contention based random access. This field may only be present if msgA-ConfigCommon is configured on the BWP.

ra-prioritization Parameters which apply for prioritized random access procedure to a given target cell (see TS 38.321 [3], clause 5.1.1).

ra-PrioritizationTwoStep Parameters which apply for prioritized 2-step random access type procedure to a given target cell (see TS 38.321 [3], clause 5.1.1).

| Conditional Presence | Explanation |
| --- | --- |
| Mandatory | The field is mandatory present. |
| Occasions | The field is optionally present, Need S, if the field occasions is present, otherwise it is absent. |
| 2StepCFRA | The field is optionally present for the case of 2-step RA type contention free random access, Need S, otherwise it is absent. |
| 4StepCFRArep | For non-(e)RedCap UEs, the field is optionally present, Need S, if resources is set to ssb and there is one FeatureCombinationPreambles entry indicating only msg1-Repetitions which is associated with the same Msg1 repetition number. For RedCap UEs or if RedCap is considered to be applicable for this Random Access procedure for eRedCap UEs, the field is optionally present, Need S, if resources is set to ssb and there is one FeatureCombinationPreambles entry indicating only redCap and msg1-Repetitions which is associated with the same Msg1 repetition number. For eRedCap UEs, if eRedCap is considered to be applicable for this Random Access procedure, the field is optional present, Need S, if resource is set to ssb and there is one FeatureCombinationPreambles entry indicating only eRedCap and msg1-Repetitions which is associated with the same Msg1 repetition number. Otherwise, it is absent. |

#### RACH-ConfigGeneric

The IE RACH-ConfigGeneric is used to specify the random-access parameters both for regular random access as well as for beam failure recovery.

RACH-ConfigGeneric information element

-- ASN1START

-- TAG-RACH-CONFIGGENERIC-START

RACH-ConfigGeneric ::=              SEQUENCE {

prach-ConfigurationIndex            INTEGER (0..255),

msg1-FDM                            ENUMERATED {one, two, four, eight},

msg1-FrequencyStart                 INTEGER (0..maxNrofPhysicalResourceBlocks-1),

zeroCorrelationZoneConfig           INTEGER(0..15),

preambleReceivedTargetPower         INTEGER (-202..-60),

preambleTransMax                    ENUMERATED {n3, n4, n5, n6, n7, n8, n10, n20, n50, n100, n200},

powerRampingStep                    ENUMERATED {dB0, dB2, dB4, dB6},

ra-ResponseWindow                   ENUMERATED {sl1, sl2, sl4, sl8, sl10, sl20, sl40, sl80},

...,

[[

prach-ConfigurationPeriodScaling-IAB-r16    ENUMERATED {scf1,scf2,scf4,scf8,scf16,scf32,scf64}              OPTIONAL,   -- Need R

prach-ConfigurationFrameOffset-IAB-r16      INTEGER (0..63)                                                 OPTIONAL,   -- Need R

prach-ConfigurationSOffset-IAB-r16          INTEGER (0..39)                                                 OPTIONAL,   -- Need R

ra-ResponseWindow-v1610                     ENUMERATED { sl60, sl160}                                       OPTIONAL,   -- Need R

prach-ConfigurationIndex-v1610              INTEGER (256..262)                                              OPTIONAL    -- Need R

]],

[[

ra-ResponseWindow-v1700                     ENUMERATED {sl240, sl320, sl640, sl960, sl1280, sl1920, sl2560} OPTIONAL    -- Need R

]]

}

-- TAG-RACH-CONFIGGENERIC-STOP

-- ASN1STOP

RACH-ConfigGeneric field descriptions

msg1-FDM The number of PRACH transmission occasions FDMed in one time instance. (see TS 38.211 [16], clause 6.3.3.2).

msg1-FrequencyStart Offset of lowest PRACH transmission occasion in frequency domain with respective to PRB 0. The value is configured so that the corresponding RACH resource is entirely within the bandwidth of the UL BWP. (see TS 38.211 [16], clause 6.3.3.2).

powerRampingStep Power ramping steps for PRACH (see TS 38.321 [3],5.1.3). This field is set to the same value for different repetition numbers associated with a specific FeatureCombination.

prach-ConfigurationFrameOffset-IAB Frame offset for ROs defined in the baseline configuration indicated by prach-ConfigurationIndex and is used only by the IAB-MT. (see TS 38.211 [16], clause 6.3.3.2).

prach-ConfigurationIndex PRACH configuration index. For prach-ConfigurationIndex configured under beamFailureRecoveryConfig, the prach-ConfigurationIndex can only correspond to the short preamble format, (see TS 38.211 [16], clause 6.3.3.2). If the field prach-ConfigurationIndex-v1610 is present, the UE shall ignore the value provided in prach-ConfigurationIndex (without suffix).

prach-ConfigurationPeriodScaling-IAB Scaling factor to extend the periodicity of the baseline configuration indicated by prach-ConfigurationIndex and is used only by the IAB-MT. Value scf1 corresponds to scaling factor of 1 and so on. (see TS 38.211 [16], clause 6.3.3.2).

prach-ConfigurationSOffset-IAB Subframe/Slot offset for ROs defined in the baseline configuration indicated by prach-ConfigurationIndex and is used only by the IAB-MT. (see TS 38.211 [16], clause 6.3.3.2).

preambleReceivedTargetPower The target power level at the network receiver side (see TS 38.213 [13], clause 7.4, TS 38.321 [3], clauses 5.1.2, 5.1.3). Only multiples of 2 dBm may be chosen (e.g. -202, -200, -198, ...). This field is set to the same value for different repetition numbers associated with a specific FeatureCombination.

preambleTransMax Max number of RA preamble transmission performed before declaring a failure (see TS 38.321 [3], clauses 5.1.4, 5.1.5). The UE shall ignore this field in case rach-ConfigGeneric is included within an EarlyUL-SyncConfig IE.

ra-ResponseWindow Msg2 (RAR) window length in number of slots. The network configures a value lower than or equal to 10 ms when Msg2 is transmitted in licensed spectrum and a value lower than or equal to 40 ms when Msg2 is transmitted with shared spectrum channel access (see TS 38.321 [3], clause 5.1.4). UE ignores the field if included in SCellConfig. If ra-ResponseWindow-v1610 or ra-ResponseWindow-v1700 is signalled, UE shall ignore the ra-ResponseWindow (without suffix). The field ra-ResponseWindow-v1700 is applicable to SCS 480 kHz and SCS 960 kHz. The UE shall ignore this field in case rach-ConfigGeneric is included within an EarlyUL-SyncConfig IE.

zeroCorrelationZoneConfig N-CS configuration, see Table 6.3.3.1-5 in TS 38.211 [16].