3GPP TSG-RAN WG3 Meeting #125	R3-244180

Maastricht, Netherland, 19 – 23 August 2024

Agenda Item:	13.2

Source:	Google

Title:	Discussion on Early Sync and CFRA resource configurations in inter-CU LTM

Document for:	Discussions & Approval

# 1	Introduction

In Rel-18 correction in RAN3#123bis, regarding the issue of CFRA resource configuration in Inter-gNB-DU LTM, based on the context of CB# 6_LTM and issue 1 in R3-241994, it has been agreed as below.

The rach-ConfigDedicated needs to be transferred from candidate gNB-DU to the source gNB-DU via F1. FFS it is the one inside the RRCRecofiguration or a new one.

Although it was mainly for the CFRA resource configuration used in LTM Cell Switch Command MAC CE, the rach-ConfigDedicated is also needed for the CFRA triggered by the network using the PDCCH order.

In RAN2#125-bis, the following agreement for Early Sync phase were made.

Agreements on early sync phase:

- Early DL and UL sync is also supported for inter-CU LTM.  Inform RAN3 of this. Early DL sync using CSI-RS should be considered, pending RAN1 approval.

- PDCCH ordered early RACH is supported for inter-CU LTM.

- For early TA acquisition, Rel-18 option is baseline. FFS for RAR based option.

RAN2 made further agreement on Early Sync in RAN2#126:

Early sync:

RAR based option

- RAR-based TA acquisition is not supported for inter-CU LTM for non-conditional LTM. FFS on conditional LTM.

Therefore, Rel-19 LTM will continue to support intra-CU inter-DU LTM and the early sync phase for the inter-CU LTM. The UE will not wait for RAR after performing the CFRA triggered by the PDCCH order (i.e., not receiving RAR from the candidate DU) as in Rel-18. We can thus make some preliminary agreements on this topic.

Proposal 1: RAN3 to support CFRA resource configuration signalling for early sync in inter-CU LTM. Take Rel-18 decisions and functions as the baseline.

RAN3 has not yet decided whether to support signalling of two sets of CFRA resource configuration for early sync and LTM Cell Switch Command for Rel-18 (under the Rel-18 correction discussion). As for now the PDCCH order in early sync phase was agreed in RAN2, RAN3 can confirm that at least one CFRA resource configuration is needed at the source gNB-DU side in the inter-CU LTM.

Proposal 2: At least one CFRA resource configuration (e.g., PDCCH order information and/or CFRA parameters in the LTM Cell Switch Command) is signalled from the candidate gNB-DU to the source gNB-DU.

RAN3 had agreed in RAN3#123bis to re-use the Handover Request message and Handover Request Acknowledge messages for inter-CU LTM. As in the intra-CU LTM the CFRA resource configuration signalling can be part of the LTM preparation phase, the Handover Request Acknowledge message can be used to signal the CFRA resource configuration and the SSB configuration for early sync with the candidate cell.

Prioritize to support inter-CU LTM over Xn interface, and RAN3 specify the inter-CU LTM solutions for standalone scenario first. Reuse existing Xn Handover Request and Handover Request ACK for Inter-CU LTM initial preparation.

Proposal 3: The CFRA resource configuration and SSB configuration for early sync the candidate cell are signalled in the Handover Request Acknowledge message between the source gNB-CU and the target gNB-CU.

In the Rel-18 RAN3 had defined the DU-CU TA Information Transfer and CU-DU TA Information Transfer message and RAN2 had decided to support early UL for inter-CU LTM as well. The candidate gNB-DU would have to identify the Source gNB-DU ID based on the preamble index it had assigned in the CFRA resource configuration during the LTM preparation phase. In the inter-CU LTM scenario, an inter-CU TA Information Transfer message is required and the Source gNB ID information might be necessary to be enhanced to the existing F1AP DU-CU TA Information Transfer message so that the candidate gNB-CU can identify which source gNB/gNB-CU to forward the TA information. The SSB configuration obtained by the Source gNB during the Handover preparation can be used by the Source gNB/gNB-DU to associate the RO information along with the TA value with the UE in the inter-CU TA Informatio transfer.

Proposal 4: To support early TA acquisition for inter-CU LTM, introduce an inter-CU TA Information transfer message.

# 3	Conclusion

Proposal 1: RAN3 to support CFRA resource configuration signalling for early sync in inter-CU LTM. Take Rel-18 decisions and functions as the baseline.

Proposal 2: At least one CFRA resource configuration (e.g., PDCCH order information and/or CFRA parameters in the LTM Cell Switch Command) is signalled from the candidate gNB-DU to the source gNB-DU.

Proposal 3: The CFRA resource configuration and SSB configuration for early sync with the candidate cell are signalled in the Handover Request Acknowledge message between the source gNB-CU and the target gNB-CU.

Proposal 4: To support early TA acquisition in inter-CU LTM, introduce an inter-CU TA Information transfer message.