3GPP TSG-RAN WG3 Meeting #125                               R3-244181

Maastricht, NL, Aug 19th – 23rd, 2024

Source:	CATT

Title:	(TP to NTN BL CRs) Support of MBS broadcast service for NR NTN

Agenda Item:	14.2

Document for:	Discussion and Decision

Introduction

For MBS broadcast service topic, the NR NTN Phase 3 WID [1] content is as follows:

Specify signaling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3] - Specify SIB signaling to indicate the intended service area in case the satellite footprint covers a larger area. [RAN2] - Specify the necessary signaling between CN and NG-RAN. [RAN3]

In this contribution, we will discuss the potential enhancement to MBS broadcast service topic. Base on the discussion, we will provide corresponding observations and proposals.

Discussion

Base on the objective of the NR NTN WID [1], RAN3 needs to specify signalling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN. In the current NGAP, the MBS Service Area Information is defined by a list of NR-CGI or TAI. TAI list used to define the MBS broadcast area could also be applied for NR NTN system, e.g. the operator expects to broadcast some MBS service for a city, a province. Similarly, for earth fixed cell or quasi earth fixed cell deployment, the cell ID list could also be used to describe the MBS service area.

Proposal 1: Existing MBS Service Area Information defined by a list of NR-CGI or TAI could also be applied for NTN.

Then let’s further consider the case where the intended broadcast area is smaller than a NTN cell, or not equal to the coverage of the NTN cell(s). The last RAN3 meeting has agreed the work assumption that the mapped cell ID can be used to define MBS broadcast service in NTN.

We understand it’s possible to use Mapped cell ID(s)to describe the intended MBS service area over NG. Then NG-RAN node translate the geographical fixed areas of the mapped cell ID(s) and broadcast the info to the UE in the corresponding System Information. The design of Uu is pending to RAN2. To make usage of Mapped Cell ID for intended service area over NG, some stage 2 update is needed.

Proposal 2: Turn WA into agreement: Mapped Cell ID(s) could be used to describe the intended MBS service area for NTN.

If the mapped cell OAM configured/pre-planned could not be used to describe the intended MBS service area, we also need to consider the other way, e.g. using GNSS description, it could accurately and precisely define the service area of MBS broadcast. And its detailed format can have the same format as the area broadcast in the Uu link, which is pending to RAN2.

Proposal 3: GNSS description could be considered for intended MBS service area, the detailed format can be aligned with the area broadcast in the Uu link, which is pending to RAN2.

As above, all of the solutions could work, which means MBS broadcast area for NR NTN can be defined by TAI list or NR CGI list(as legacy), Mapped cell ID list, or GNSS description. It is proposed to discuss and agree the TP for BLCR on TS 38.300 and TS 38.413. And besides that, we should also send LS to SA2, to indicate that mapped cell ID(s) and GNSS description could be used to describe the intended MBS service area for NTN. The draft LS is provided in appendix 5.

Proposal 4: It is proposed to send LS to SA2 on MBS broadcast service for NR NTN system and the draft LS is provided in appendix 5

Proposal 5: It is proposed to discuss and agree the TP for BLCR to TS 38.300 and TS 38.413 on support of Mapped Cell ID.

Conclusion

According to the analysis in section 2, the following observations and proposals are provided:

Proposal 1: Existing MBS Service Area Information defined by a list of NR-CGI or TAI could also be applied for NTN.

Proposal 2: Turn WA into agreement: Mapped Cell ID(s) could be used to describe the intended MBS service area for NTN.

Proposal 3: GNSS description could be considered for intended MBS service area, the detailed format can be aligned with the area broadcast in the Uu link, which is pending to RAN2.

Proposal 4: It is proposed to send LS to SA2 on MBS broadcast service for NR NTN system and the draft LS is provided in appendix 5.

Proposal 5: It is proposed to discuss and agree the TP for BLCR to TS 38.300 and TS 38.413 on support of Mapped Cell ID.

Reference

- RP-240775 Rel-19 Moderator WID NR NTN phase 3

- RAN3_124_agenda_20240524_eom2

Draft LS to SA2

Title:	[Draft] LS on MBS broadcast service for NR NTN system

Release:	Rel-19

Work Item:	NR_NTN_Ph3-Core

Source:	CATT (to be RAN3)

To:	SA2

Cc:	RAN2

Contact Person:

#### Name:	Jiancheng Sun

#### E-mail Address:	sunjiancheng@catt.cn

Attachments:	n/a

1. Overall Description:

RAN3 has discussed the MBS broadcast service for NR NTN system. RAN3 assumes that the MBS Service Area Information defined by a list of NR-CGI or a list of TAI could also be applied for NTN system.

Besides that, both mapped cell ID(s) and GNSS description could be used to describe the intended MBS service area for NTN in the NGAP, especially for the case the intended service area is small than a cell. How the GNSS description is encoded could align with the NR-Uu design to be decided by RAN2.

2. Actions:

To SA2:

ACTION: RAN3 respectfully requests SA2 to take the above information into consideration in their future work.

3. Date of Next TSG-RAN WG3 Meetings:

RAN3#125-bis	       Oct. 14th – Oct. 18st  	2024		Hefei, CN

RAN3#126	       Nov 18th – Nov 22th	2024		Orlando, US

TP to BLCR for TS 38.300, support of NTN MBS broadcasting

START OF CHANGES

16.14.5	NG-RAN signalling

The Cell Identity, as defined in TS 38.413 [26] and TS 38.423 [50], used in following cases corresponds to a Mapped Cell ID, irrespective of the orbit of the NTN payload or the types of service links supported:

-	The Cell Identity indicated by the gNB to the Core Network as part of the User Location Information;

-	The Cell Identity used for Paging Optimization in NG interface;

-	The Cell Identity used for Area of Interest;

-	The Cell Identity used for PWS.

The Cell Identity included within the target identification of the handover messages allows identifying the correct target cell. The cell identity used in the NG and Xn handover messages, Xn Setup and Xn NG-RAN Node Configuration Update procedures is expected to be Uu Cell ID.

The Cell Identities used in the RAN Paging Area during Xn RAN paging allow the identification of the correct target cells for RAN paging.

The Cell Identities used to describe the MBS broadcast area may be the Mapped Cell ID.

NOTE 1:	The Cell Identity used for RAN Paging is assumed to typically represent a Uu Cell ID.

The mapping between Mapped Cell IDs and geographical areas is configured in the RAN and Core Network.

NOTE 2:	A specific geographical location may be mapped to multiple Mapped Cell ID(s), and such Mapped Cell IDs may be configured to indicate differerent geographical areas (e.g. overlapping and/or with different dimensions).

The gNB is responsible for constructing the Mapped Cell ID based on the UE location information received from the UE, if available. The mapping may be pre-configured (e.g., up to operator's policy) or up to implementation.

NOTE 3:	As described in TS 23.501 [3], the User Location Information may enable the AMF to determine whether the UE is allowed to operate at its present location. Special Mapped Cell IDs or TACs may be used to indicate areas outside the serving PLMN's country.

The gNB reports the broadcasted TAC(s) of the selected PLMN to the AMF as part of ULI. In case the gNB knows the UE's location information, the gNB may determine the TAI the UE is currently located in and provide that TAI to the AMF as part of ULI.

END OF CHANGES

TP to BLCR for TS 38.413, support of NTN MBS broadcasting

START OF CHANGES

9.3.1.209	MBS Service Area Information

This IE contains MBS service area information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MBS Service Area Cell List |  | 0..<maxnoofCellsforMBS> |  |  | - | - |
| >NR CGI | M |  | 9.3.1.7 |  | - | - |
| MBS Service Area TAI List |  | 0..<maxnoofTAIforMBS> |  |  | - | - |
| >TAI | M |  | 9.3.3.11 |  | - | - |
| GNSS Description | O |  | FFS |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsforMBS | Maximum no. of cells allowed within one MBS Service Area. Value is 8192. |
| maxnoofTAIforMBS | Maximum no. of TAs allowed within one MBS Service Area. Value is 1024. |

Editor’s Note: The details of the GNSS Description is FFS which is pending to RAN2 decision..

NEXT CHANGE

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

<<<<<<<<<<unchanged part is skipped>>>>>>>>>>>

id-TimeSinceFailure,

id-ClockQualityReportingControlInfo,

id-RANfeedbacktype,

id-QoSFlowTSCList,

id-TSCTrafficCharacteristicsFeedback,

id-ANPacketDelayBudgetUL,

id-MBSCommServiceType,

id-MobileIAB-MTUserLocationInformation,

id-PDUsetQoSParameters,

id-PDUSetbasedHandlingIndicator,

id-N6JitterInformation,

id-ECNMarkingorCongestionInformationReportingRequest,

id-ECNMarkingorCongestionInformationReportingStatus,

id-XrDeviceWith2Rx,

id-GNSS Description,

<<<<<<<<<<<<<<<<<<<< Next Change>>>>>>>>>>>>>>>>>>>>

MBS-ServiceAreaInformation ::= SEQUENCE {

mBS-ServiceAreaCellList		MBS-ServiceAreaCellList																OPTIONAL,

mBS-ServiceAreaTAIList		MBS-ServiceAreaTAIList																OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {MBS-ServiceAreaInformation-ExtIEs} }	OPTIONAL,

...

}

MBS-ServiceAreaInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ID id-GNSS Description		CRITICALITY ignore	TYPE (FFS)	PRESENCE optional }

...

}

<<<<<<<<<<<<<<<<<<<< Next Change>>>>>>>>>>>>>>>>>>>>

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

NGAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

<<<<<<<<<<unchanged part is skipped>>>>>>>>>>>

id-ERedCapIndication									ProtocolIE-ID ::= 427

id-XrDeviceWith2Rx										ProtocolIE-ID ::= 428

id-UserPlaneErrorIndicator								ProtocolIE-ID ::= 429

id-SLPositioningRangingServiceInfo						ProtocolIE-ID ::= 430

id-GNSS Description										ProtocolIE-ID ::= xxx

END OF CHANGES