3GPP TSG-RAN WG3 #125                                                                     	R3-244183

Maastricht, NL, 19 – 23 August, 2024

Source:	CATT

Title:	(TP to TR 38.769) A-IoT architecture aspects

Agenda Item:	16.2

Document for:	Discussion and Decision

- Introduction

In the previous RAN3 meetings, we discussed the architecture aspects for A-IoT [1], corresponding agreements and open issues are as follows:

RAN3 considers both Topologies at the same time looking whether commonalities are applicable.

[Topo1] AIoT RAN node: 
Corresponds to the basestation in Figure 4.2.1.1-1 in TR 38.848; 
A RAN node providing AIoT radio, and connecting with an AIoT-aware CN node via the XX interface. Details of the AIoT-aware CN node is subject to SA2.

[Topo2] UE Reader: 
A UE (corresponds to the intermediate node in Figure 4.2.1.2-1 in TR 38.848);
Providing AIoT radio, and connecting with a gNB (may be an AIoT enhanced gNB, corresponds to the basestation in Figure 4.2.1.2-1 in TR 38.848) via NR Uu interface.

XX interface: 
FFS whether it is the NG interface or a new interface, details to be discussed by RAN3 with SA2 progress of AIoT-aware CN node.

XXAP (XX Application Protocol): 
The radio network layer signalling protocol for the XX interface. It is FFS whether it is NGAP or a new XXAP.

It is FFS whether control plane transport or user plane transport is used for XXAP.

Common Reader Function: 
A function providing AIoT radio. It may reside within the AIoT RAN node in Topology 1 and reside within the UE Reader in Topology 2.  It is FFS whether there is a need to define the Common Reader Function.

  - It is FFS whether and in which way the Common Reader Function, if defined, communicates with the AIoT-aware CN node.

For Topology 1, RAN3 starts with AIoT RAN node being aggregated.

In this contribution, we will further discuss the architecture aspects on support of A-IoT, and provide the observations and proposals accordingly.

- Discussion

Based on the previous discussion, the overall architecture agreed was captured in the BL pCR [2].

It consists of the following architectural elements:

AIoT device: equipment with characteristics outlined e.g. in TS 22.369 [x] and TR 38.848 [2].

Editor’s Note 3: Further details FFS, if any.

AIoT RAS (AIoT Radio Access System): hosts certain functions for AIoT as part of the functional split between RAN and CN.

Editor’s Note 4: Further details regarding AIoT functions hosted in the AIoT RAS and the respective functional split to be decided by RAN2, RAN3 and SA2.

AIoT radio: radio interface between AIoT device and Common reader function.

Editor’s Note 5: Further details on AIoT radio to be discussed by RAN1 and RAN2.

AIoT CN: hosts certain functions for AIoT as of the functional split between RAN and CN

Editor’s Note 6: Further details regarding AIoT functions hosted in the AIoT CN and the respective functional split to be decided by RAN2, RAN3 and SA2.

XX interface: interface between the AIoT RAS and the AIoT CN on which certain AIoT specific functions are performed.

Editor’s Note 7: The functions represented by the XX interfaces are FFS. It is also FFS whether this interface represents a new logical interface or is equal to NG. E.g. for topology 1 it may only represent a single interface instance, e.g. a new interface between AIoT RAS and AIoT CN, for topology 2 it might represent either 2 interface instances, one instance for NG and one instance “XX” for a new interface between AIoT CN and AIoT RAS, or one instance for NG alone.

Common reader function: a function that communicates with the AIoT device by means of AIoT radio.

Editor’s Note 8: Further details on Common reader function is to be discussed by RAN1 and RAN2.

AIoT RAN node function: a function residing in AIoT RAS. It contains e.g. the control of the AIoT radio resources used towards the AIoT device.

Editor’s Note 9: further details are FFS. Note that “control of AIoT radio resources” does not necessarily imply dynamic configuration of resources but could also rely on static assignment of resources by means of OAM. Aspects concerning coordination of the Upper Layer functions (e.g. Inventory, Command) e.g. in case these functions have to be performed over a multitude of instances of the Common Reader Function are FFS.

Figure 6.3-1: Logical System Architecture for AIoT.

Editor’s Note 9: whether the logical system architecture above is applied to topology 2 to be discussed in next meeting.

In the last RAN3 meeting, the architecture of Topology 2 was discussed in the offline discussion, as shown in the figure below:

However, the above architecture for Topology 2 was not agreed, a few companies wondered whether RAN2 should be involved in the architecture discussion, and whether the “Common reader function” of the intermediate UE belongs to a part of RAN or UE. It’s clarified in the RAN#104, the architecture discussion is in the scope of RAN3.

To our understanding, it’s no doubt that there’re two parts of the intermediate UE in the Topology 2 from functionality point of view, reader part (Common Reader Function) and the UE part. We assume no need to discuss and decide whether common Reader function is part of UE or part of RAN, we can simply call it AIoT-enabled UE.  The AIoT-enabled UE includes two parts, common reader function and UE. AIoT-enabled UE is connected to AIoT-enabled gNB via NR-Uu interface, then connect to AIoT CN via NG interface.

Proposal 1: For Topology 2, we can use the terminology “AIoT-enabled UE”, which includes two parts, common reader function and UE. AIoT-enabled UE is connected to AIoT-enabled gNB via NR-Uu interface, then connect to AIoT CN via NG interface.

To make better understanding of the Topology 2, we prefer to have separate architecture figures for Topology 1 and Topology 2, the proposed architectures are shown in the figure below:

Figure 6.3.x Logical System Architecture for AIoT, Topology 1

Figure 6.3.y Logical System Architecture for AIoT, Topology 2

The figure for Topology 2 as above just shows the very basic architecture for topology 2, which allows different solutions to connect between AIoT-enabled UE and AIoT CN.

Proposal 2: It’s beneficial to have separate figures to show the system architectures for Topology 1 and Topology 2.

As above, we understand no need the terminology “RAS”, we can keep using the “AIoT RAN” for the Topology 1, and using “AIoT-enabled gNB” for Topology 2.

Proposal 3: No need to define “AIoT RAS”, just use “AIoT RAN” for Topology 1.

For Top 1, the XX interface between AIoT RAN and AIoT CN could be:

- Option 1: Reuse NGAP with necessary enhancements, e.g. supporting Inventory, Command procedures.

- Option 2: Define a new interface xx, between AIoT RAN and AIoT CN, direct connection.

- Option 3: Define a new interface xx, between AIoT RAN and AIoT CN, NRPPa like, routing via N2.

For the Option 1, no need to define the new interface, AIoT related signalings and IEs are introduced to the NGAP, not too much impact is foreseen.

For the option 2, AIoT RAN could directly connect to the “AIoT Function” in the CN (e.g. AIoTF), AMF is not involved, NGAP may also not impact.

The option 3 is similar to the option 2, the difference is the XX interface is routing via AMF, via the NGAP, just like the NRPPa, or SMF container.

As above, all of the 3 options are feasible. If the AIoT function is part of AMF, or tightly connect to AMF, option 1 seems better. If AIoT Function and AMF are deployed separately, option 2 or 3 could be considered. As the relationship between the AIoT Function (AIoTF) and AMF to be discussed and decided by SA2, which option to be taken need to be checked with SA2, whether and how the AMF is involved to support AIoT functions.

Proposal 4: Send LS to SA2 to check whether and how AMF is involved for A-IoT.  (Draft LS is in the Annex)

For the Topology 2, the intermediate node is a UE, NGAP is used to connect to the AMF for the UE, and some enhancement to the NGAP is needed, e.g. authorization of the UE reader.

If a direct interface between the UE reader and the AIoT CN is needed, the interface could be NAS like, or LPP like. This is pending to RAN2 and SA2.

Considering the commonalities of Topology 1 and Topology 2, it’s easier to use NGAP to connect between A-IoT RAN/AIoT enabled gNB and A-IoT CN.

Proposal 5: Considering the commonalities of Topology 1 and 2, it’s slightly preferred to use NGAP between A-IoT RAN node/AIoT-enabled gNB and AIoT CN, with necessary enhancement.

To allow the flexibility, the AIoT RAN node in Topology 1, or AIoT-enabled gNB in Topology 2 may also serve the legacy NR UEs. However, there’s no extra specification impact to support the legacy NR UEs over NR-Uu, NGAP.

Proposal 6: AIoT RAN node or AIoT-enabled gNB may also support the legacy NR UEs, without any extra impact to NR-Uu and NGAP.

Base on the discussion above, we provide the TP to TR 38.769 in section 5. It’s proposed to discuss and agree the TP to TR 38.769 on system architecture aspects to support A-IoT.

Proposal 7: Discuss and agree the TP to TR 38.769 (in section 5) on AIoT system architecture aspects.

- Conclusion

Based on the discussion in section 2, the following proposals are provided:

Proposal 1: For Topology 2, we can use the terminology “AIoT-enabled UE”, which includes two parts, common reader function and UE. AIoT-enabled UE is connected to AIoT-enabled gNB via NR-Uu interface, then connect to AIoT CN via NG interface.

Proposal 2: It’s beneficial to have separate figures to show the system architectures for Topology 1 and Topology 2.

Proposal 3: No need to define “AIoT RAS”, just use “AIoT RAN” for Topology 1.

Proposal 4: Send LS to SA2 to check whether and how AMF is involved for A-IoT.  (Draft LS is in the Annex)

Proposal 5: Considering the commonalities of Topology 1 and 2, it’s slightly preferred to use NGAP between A-IoT RAN node/AIoT-enabled gNB and AIoT CN, with necessary enhancement.

Proposal 6: AIoT RAN node or AIoT-enabled gNB may also support the legacy NR UEs, without any extra impact to NR-Uu and NGAP.

Proposal 7: Discuss and agree the TP to TR 38.769 (in section 5) on AIoT system architecture aspects.

- Reference

- RAN3#124bis Chair Notes

- R3-243972 (BL pCR to TR 38.769) Study on solutions for Ambient IoT in NR

- TP to TR 38.769 AIoT architecture aspects

<<<<<<<<<<<<<<<<<< Begin of the changes >>>>>>>>>>>>>>>>>>>>>>>

## 6.3	RAN architecture aspects

Editor’s note 1: Corresponds to the second RAN3 objective in the SID, to identify RAN architecture aspects, including whether support for split architecture is necessary.

This chapter attempts to identify and describe architectural elements necessary to define a RAN architecture for support of Ambient IoT embedded in the overall 5G system architecture in support of topology 1 and topology 2 (as defined in TR 38.848 [2]).

Editor’s Note 2: What functionalities are hosted by the 5GS for AIoT is TBD.

This chapter also attempts to identify a functional split between RAN and CN.

Figure 6.3-1 depicts the logical system architecture for AIoT.

It consists of the following architectural elements:

AIoT device: equipment with characteristics outlined e.g. in TS 22.369 [x] and TR 38.848 [2].

Editor’s Note 3: Further details FFS, if any.

AIoT RAN (AIoT Radio Access Network): hosts certain functions for AIoT as part of the functional split between RAN and CN.

Editor’s Note 4: Further details regarding AIoT functions hosted in the AIoT RAN and the respective functional split to be decided by RAN2, RAN3 and SA2.

AIoT radio: radio interface between AIoT device and Common reader function.

Editor’s Note 5: Further details on AIoT radio to be discussed by RAN1 and RAN2.

AIoT CN: hosts certain functions for AIoT as of the functional split between RAN and CN

Editor’s Note 6: Further details regarding AIoT functions hosted in the AIoT CN and the respective functional split to be decided by RAN2, RAN3 and SA2.

XX interface: interface between the AIoT RAN and the AIoT CN on which certain AIoT specific functions are performed. In this release, we only consider XX is equal to NG.

Editor’s Note 7: The functions represented by the XX interfaces are FFS.

Common reader function: a function that communicates with the AIoT device by means of AIoT radio.

Editor’s Note 8: Further details on Common reader function is to be discussed by RAN1 and RAN2.

AIoT RAN node function: a function residing in AIoT RAN. It contains e.g. the control of the AIoT radio resources used towards the AIoT device.

Editor’s Note 9: further details are FFS. Note that “control of AIoT radio resources” does not necessarily imply dynamic configuration of resources but could also rely on static assignment of resources by means of OAM. Aspects concerning coordination of the Upper Layer functions (e.g. Inventory, Command) e.g. in case these functions have to be performed over a multitude of instances of the Common Reader Function are FFS.

AIoT-enabled UE: a UE which connects to AIoT-enabled gNB via NR-Uu, and it holds common reader function to communicate with AIoT device by means of AIoT radio.

AIoT-enabled gNB: a gNB with necessary enhancement to support AIoT, e.g. authorization and resource management for the AIoT-enabled UE. AIoT-enabled gNB is also able to support the legacy NR UE.

Figure 6.3.x Logical System Architecture for AIoT, Topology 1

Figure 6.3.y Logical System Architecture for AIoT, Topology 2

<<<<<<<<<<<<<<<<<< End of the changes >>>>>>>>>>>>>>>>>>>>>>>

- Draft LS to SA2

Title:	[Draft] LS on Clarification of architecture aspects for Ambient IoT

Response to:	-

Release:	Rel-19

Work Item:	FS_AmbientIoT

Source:	CATT (to be RAN3)

To:	SA2

Cc:	RAN1, RAN2

Contact Person:	Jiancheng Sun

E-mail Address:	sunjiancheng (at) catt (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1	Overall description

While progressing the study on Ambient IoT, RAN3 discussed the system architectures on support of Ambient IoT. The system architecture for Topology 1 agreed by RAN3 is illustrated with the figure below:

On how to connect between AIoT RAN and AIoT CN, three potential options are assumed, as follows:

- Option 1: Reuse NGAP with necessary enhancements, e.g. supporting Inventory, Command procedures.

- Option 2: Define a new interface XX, between AIoT RAN and “AIoT Function” in the Core Network, direct connection, AMF is not involved.

- Option 3: Define a new interface XX, between AIoT RAN and “AIoT Function” in the Core Network, NRPPa like, routing via AMF, XXAP is carried by NGAP.

We see SA2 is also discussing the A-IoT overall architectures, RAN3 would like to ask SA2 the following questions:

Question 1: Whether and how AMF is involved to support A-IoT functions?

Question 2: Which option (as above) is preferred to support the “XX” interface between AIoT RAN node and AIoT CN?

2	Actions

To SA2:

ACTION: RAN3 kindly asks SA2 to provide the feedback to the above questions on Ambient IoT architecture aspects.

3	Date of next RAN3 meetings

RAN3#125	2024-08-19 – 2024-08-23 Maastricht, NL

RAN3#125bis	2024-10-14 – 2024-10-18 Hefei, CN