3GPP TSG-RAN WG3 #125                                             R3-244184

Maastricht, NL, 19th – 23rd Aug, 2024

Source:	CATT

Title:	(TP to TR 38.769) Network signalling aspects for Topology 1

Agenda Item:	16.3

Document for:	Decision

Introduction

The following agreements and FFS were captured in last RAN3 meeting:

Inventory over CN-RAN interface can be used for a single device, or a group of devices, or all devices.

Command over CN-RAN interface can be used for a single device. FFS for command on a group of devices, or all devices.

In the Inventory Request from the AIoT CN towards the AIoT RAS, it is FFS whether the following information may be included:

Inventory Session ID (e.g., to allow multiple Inventory procedure instances to run in parallel): the purpose, scope and handling of this information in AIoT RAS are FFS.

Estimated Number of expected responses from devices, or Minimum number of required responses from devices.

FFS the usage of these information.

Periodicity: to enable periodic inventory at AIoT RAS triggered by a single Inventory Request from the AIoT CN

In Topology 2, it is FFS which entity (e.g., AIoT CN and/or AIoT enabled gNB) performs the UE reader selection.

In topology1, it is FFS whether in response to a single Inventory Request, AIoT RAS provides only a single Inventory Report message or there can be also multiple Inventory Report messages provided.

According to the WI [1], Topologies 1 & 2(as shown in the figures below) should be considered in R19.

Topology 1: BS ↔ Ambient IoT device  	Topology 2: BS ↔ intermediate node ↔ Ambient IoT device

In this contribution, we will further analyze the potential RAN3 impact for topology1 to support inventory and command service for A-IoT, and base on the discussion, we will provide our observations and proposals accordingly.

Discussion

As discussed in [2], considering the commonalities of Topology 1 and 2, NGAP is preferred to be reused between A-IoT RAN node/AIoT-enabled gNB and AIoT CN, the work flow supporting inventory and command in topology 1 is illustrated in following figure according to RAN2/3 understanding:

Figure1 work flow of inventory/command

Step 1: CN sends a XXAP/NGAP message to indicate RAN to perform inventory or command operation.

Step 2: RAN sends Initial Trigger Message over the AIoT radio interface for the device.

Step 3: The device performs random access to A-IoT RAN.

Step 4: Data communication may be triggered after random access.

Step 5: A-IoT RAN sends device ID to CN. CN verify device ID and then send XXAP/NGAP message to A-IoT RAN to confirm the validity of device ID.

Authentication and authorization procedure may be needed which is up to the conclusion of SA3.

Step 6: Inventory or command complete message is sent between CN and A-IoT RAN.

For service type in step1, inventory or command service type may be needed for agnostic of the type of request from the AIoT CN. Another solution is introducing two procedures (inventory request procedure and command request procedure) to indicate service type.

In last RAN2 meeting, inventory only, inventory and command and Command only cases were defined as below:

Agreements 1 As baseline, the “inventory only” case is supported by the procedure: - Step A: A-IoT paging; - Step B: Device ID transmission (via Random Access or without using RA). Details are FFS 2 As baseline, the “inventory and command” case is supported by the procedure: - Step A: A-IoT paging; - Step B: Device ID transmission (via Random Access or without using RA). Details are FFS - Step C: reader to device data transmission (e.g. the R2D command), and - Step D: corresponding device to reader data transmission (e.g. the feedback). FFS whether this is optional, pending other WG discussions. Clarify in TR that inventory and command doesn’t mean that AIoT paging includes both Inventory and Command in the same message. This doesn’t mean that inventory and command are received by the reader at the same time from upper layer. 3 From RAN2 point of view we will study “Command only” use case. FFS the options on how to support it : Initial trigger message from the reader contains the command. Final feasibility depends on SA2 and SA3 work/conclusions. Use baseline procedure for “inventory and command”(i.e. first triggers inventory procedure and then sends command)

For inventory only and command only case defined in RAN2, in XXAP/NG, single procedure may be defined with the service types “inventory” or “command”. Of course, it’s also feasible to define separate XXAP/NGAP procedures, e.g. inventory procedure and command procedure.

Observation 1: To support RAN2 defined inventory only and command only case, either solution is feasible as below:

- Solution 1: A single XXAP/NGAP procedure with service types, e.g. inventory, command;

- Solution 2: separate XXAP/NGAP procedures for inventory and command.

To support inventory and command case defined in RAN2, if solution 1 is adopted, the XXAP/NGAP procedures could be:

- A single XXAP/NGAP message indicates inventory service type at first and then a subsequent XXAP/NGAP message indicates command service type.

- A single XXAP/NGAP message includes both inventory and command service type.

- A single XXAP/NGAP message indicates command service type, AIoT RAN performs inventory first before send the command to the device.

If solution 2 is adopted, the XXAP/NGAP procedures could be:

- One inventory request procedure triggered at first and then a subsequent command request procedure.

- Define another new procedure for inventory + command.

- Only use the command procedure in the XXAP/NGAP, AIoT RAN performs inventory first before send the command to the device.

Either solutions above may be possible to trigger inventory and command case defined in RAN2. It may be finally decided by SA2/RAN2.

Observation 2: to support RAN2 defined inventory and command case, either solution above is possible, but it is finally up to RAN2/SA2:

Proposal 1: It is proposed for RAN3 to define a single XXAP/NGAP procedure with different service types, or define separate XXAP/NGAP procedures for inventory, command, even inventory + command.

In the last meeting, AIoT Device Identification was agreed to be used to trigger inventory request to a single device, a group of devices, or all devices.

Since there may be security issue when transferring Device info directly in step2 message in AIoT radio interface, device info may be included or partially included in AIoT NAS PDU because AMF/AIoTF may apply security protection to NAS. But it is finally up to conclusion of SA2/SA3.

Proposal 2: How to include AIoT Device Identification in inventory/command request procedure is finally up to conclusion of SA2/SA3. The following way may be possible:

- including AIoT Device Identification explicitly in inventory/command request procedure;

- including AIoT Device Identification in AIoT NAS PDU;

- partially including AIoT Device Identification in AIoT NAS PDU;

- Both 1) and 2) at the same message, or both 1) and 3) at the same message.

For the information in step 1 message when triggering inventory procedure, the open issue captured in last RAN3 meeting is listed below:

In the Inventory Request from the AIoT CN towards the AIoT RAS, it is FFS whether the following information may be included:

Inventory Session ID (e.g., to allow multiple Inventory procedure instances to run in parallel): the purpose, scope and handling of this information in AIoT RAS are FFS.

Estimated Number of expected responses from devices, or Minimum number of required responses from devices.

FFS the usage of these information.

Periodicity: to enable periodic inventory at AIoT RAS triggered by a single Inventory Request from the AIoT CN

For Inventory Session ID or Task ID, it is used to support multiple inventory procedures in parallel. We are not sure whether this feature can be supported in AIoT radio interface, e.g. multiple inventory procedures are performed simultaneously in the AIoT radio interface. If not, only supporting parallel inventory in XXAP/NGAP interface would increase the complexity in A-IoT RAN, e.g. AIoT RAN needs to hold the 2nd, 3rd inventory procedures, and perform the procedures one by one after the ongoing inventory procedure is completed. Moreover, it is not an elementary function, i.e. enhanced function may be not supported in R19.

Proposal 3: whether multiple inventory procedures could be performed simultaneously in the AIoT radio interface to be confirmed by RAN1/RAN2.

Proposal 4: No clear benefits to support multiple inventory procedures in XX/NG in parallel. AIoT CN should not initiate multiple inventory procedures in case ongoing inventory procedure is not completed.

For the cause/purpose of inventory, it may indicate the cause of triggering inventory procedure, for example, inventory for Command, inventory for locating, inventory for requesting device ID, we think A-IoT RAN just trigger inventory procedure without considering the purpose.

Proposal 5: No need to indicate the cause/purpose of the inventory procedure.

For the inventory scope/area, A-IoT RAN may use this information to select suitable reader. As we discussed in [2], in Topology 1, signalling based solution and OAM based solution are both feasible to provide the reader information (e.g. GNSS location, service area) to the AIoT CN. So, AIoT CN is capable to select reader, i.e. it is not needed for A-IoT RAN to select reader based on scope. Therefore, if reader ID list is provided from AIoT CN to AIoT RAN, AIoT RAN triggers the inventory procedure in the selected readers. Or else, AIoT RAN triggers inventory in all the Reader(s) it holds.

Proposal 6: For Topology 1, AIoT CN may provide Reader ID list to the AIoT RAN as the inventory area. If Reader ID(s) is not provided, AIoT RAN triggers the inventory procedure in all the reader(s) it holds.

For Estimated Number of devices, if AIoT CN could provide estimated number of device for all devices inventory, A-IoT RAN may allocate random access resource refer to the estimated number of device. But it is finally up to the conclusion of RAN2.

Proposal 7: Estimated Number of devices seems beneficial for AIoT radio resource management in AIoT RAN, details are pending to RAN2.

For Periodicity, it is used for A-IoT RAN to trigger periodic inventory procedure by a single request from CN. For example, operator start a task to inventory warehouse once a day, it may be better for CN than for NG-RAN to trigger inventory procedure every day because CN is more reliable. So, it is not needed to send periodicity to A-IoT RAN.

Proposal 8: Periodically inventory, if needed, could be triggered by the AIoT server or AIoT CN. No need to introduce the Periodicity into in the XXAP/NGAP.

For the step 1 message when triggering command procedure, the following agreement is captured in last RAN3 meeting.

Command over CN-RAN interface can be used for a single device. FFS for command on a group of devices, or all devices.

We have agreed command service is performed to a single device. For the issue whether command service could be triggered towards a group of device, we think it is possible. For example, triggering read/write operation to a group of device. As for command operation to all devices, we are not sure whether there is the scenario to read/write all devices.

So, device info in step 1 message may include a device or a group of device info.

Proposal 9: The scenario of triggering command to a group of devices may be valid, so it is proposed to use AIoT Device Identification to indicate a device or a group of device in XXAP/NGAP message for command request.

In case of writing device, the action “write” and related “data” is encapsulated as NAS PDU and NG-RAN do not decode NAS. In case of reading device, the action “read” is also encapsulated as NAS PDU. So, NAS info is necessary in XXAP/NGAP message for command request.

Proposal 10: It is proposed to include NAS information in XXAP/NGAP message for command request.

In step 6, an inventory complete message may be needed for inventory service.

If NG-RAN or CN decide that inventory is complete, an inventory complete message is necessary to inform the other side. First it is reasonable for CN to complete the inventory procedure, for example, based on predefined time duration, or operator decides to complete the ongoing the inventory procedure. As for A-IoT RAN, whether it could check the completion of inventory is up to RAN2. RAN3 may wait for the conclusion of RAN2.

Proposal 11: It is proposed to introduce an inventory complete message in XXAP/NGAP interface. CN may indicate A-IoT RAN the completion of inventory procedure. Whether A-IoT RAN could check the completion of inventory is up to RAN2.

In step 6, a command response message may be sent to CN in XXAP/NGAP interface for command service.

A-IoT RAN may need to inform CN the result of command operation, for example, for read operation, the read data should be provided from A-IoT RAN to CN as a result. For write operation, result of write operation, i.e. success or failure, may be sent to CN.

The result of command operation may be included in NAS PDU in order to keep A-IoT information transparent to A-IoT RAN.

Proposal 12: It is proposed to introduce a command response message which may include the result of command operation in NAS PDU from A-IoT RAN to CN.

Whether there is a need to setup device context in NG-RAN still needs consideration.

For the inventory only request or command only request procedure, there is no subsequent procedure. So, maybe there is no needed to keep device context.

As discussed in P1, for inventory and command case, device context may be used for following procedure. For example, Radio resource allocated for the first inventory procedure could also be used for the following command procedure.

Proposal 13: For inventory only request or command only request procedure, there is no needed to setup device context. For inventory and command case, device context may be setup for the following procedure.

If device context is needed, the following information may be included in device context for topology 1:

- Device NGAP ID.

- Device ID. How the Device ID is defined is up to SA2.

- Security information, if AS security is required. It is up to SA3.

- Radio resource. It is up to RAN2 to decide the requirement.

Device NGAP ID may be allocated by RAN when performing the first inventory procedure. Device NGAP ID is used to indicate device to support per device NAGP message. So, device NGAP ID should be kept in device context. Device context may be helpful for the following command or data transport procedure. Based on device NGAP ID from NGAP message, related device context, for example, radio resource, could be found to assist NG-RAN in sending following command or data to device.

CN performs device ID verifying within the first inventory procedure. If Device ID can be kept in NG-RAN, NG-RAN can also take the responsible to verify the device ID afterwards. The benefit is decreasing the time duration of the following inventory procedure.

Security information may be included in device context, if AS security is required. It is up to SA3.

Proposal 14: The device context may include:

-   Device NGAP ID.

-	Device ID.

-	Security information. (If AS security is required)

-	Radio resource. (It is up to RAN2 to decide the requirement)

Conclusions

In this contribution, we mainly discussed the network signalling for topology 1. Based on the discussion above, the following proposals are provided:

Observation 1: To support RAN2 defined inventory only and command only case, either solution is feasible as below:

- Solution 1: A single XXAP/NGAP procedure with service types, e.g. inventory, command;

- Solution 2: separate XXAP/NGAP procedures for inventory and command.

Observation 2: to support RAN2 defined inventory and command case, either solution above is possible, but it is finally up to RAN2/SA2:

Proposal 1: It is proposed for RAN3 to define a single XXAP/NGAP procedure with different service types, or define separate XXAP/NGAP procedures for inventory, command, even inventory + command.

Proposal 2: How to include AIoT Device Identification in inventory/command request procedure is finally up to conclusion of SA2/SA3. The following way may be possible:

- Including AIoT Device Identification explicitly in inventory/command request procedure;

- including AIoT Device Identification in AIoT NAS PDU;

- partially including AIoT Device Identification in AIoT NAS PDU;

- Both 1) and 2) at the same message, or both 1) and 3) at the same message.

Proposal 3: whether multiple inventory procedures could be performed simultaneously in the AIoT radio interface to be confirmed by RAN1/RAN2.

Proposal 4: No clear benefits to support multiple inventory procedures in XX/NG in parallel. AIoT CN should not initiate multiple inventory procedures in case ongoing inventory procedure is not completed.

Proposal 5: No need to indicate the cause/purpose of the inventory procedure.

Proposal 6: For Topology 1, AIoT CN may provide Reader ID list to the AIoT RAN as the inventory area. If Reader ID(s) is not provided, AIoT RAN triggers the inventory procedure in all the reader(s) it holds.

Proposal 7: Estimated Number of devices seems beneficial for AIoT radio resource management in AIoT RAN, details are pending to RAN2.

Proposal 8: Periodically inventory, if needed, could be triggered by the AIoT server or AIoT CN. No need to introduce the Periodicity into in the XXAP/NGAP.

Proposal 9: The scenario of triggering command to a group of devices may be valid, so it is proposed to use AIoT Device Identification to indicate a device or a group of device in XXAP/NGAP message for command request.

Proposal 10: It is proposed to include NAS information in XXAP/NGAP message for command request.

Proposal 11: It is proposed to introduce an inventory complete message in XXAP/NGAP interface. CN may indicate A-IoT RAN the completion of inventory procedure. Whether A-IoT RAN could check the completion of inventory is up to RAN2.

Proposal 12: It is proposed to introduce a command response message which may include the result of command operation in NAS PDU from A-IoT RAN to CN.

Proposal 13: For inventory only request or command only request procedure, there is no needed to setup device context. For inventory and command case, device context may be setup for the following procedure.

Proposal 14: The device context may include:

-   Device NGAP ID.

-	Device ID.

-	Security information. (If AS security is required)

-	Radio resource. (It is up to RAN2 to decide the requirement)

Reference

[1] RP-240826 Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR

[2] R3-244183 (TP for TR 38.769) A-IoT architecture aspects, CATT

TP for TR 38.769

<<<<<<<<<<<<<<<<<< Begin of the changes >>>>>>>>>>>>>>>>>>>>>>>

6.3	Impacts on CN-RAN interface

Editor’s note: Corresponds to the first RAN3 objective in the SID.

6.3	Impacts on CN-RAN interface

Editor’s note: Corresponds to the first RAN3 objective in the SID, to identify necessary impacts on signaling and procedures for CN-RAN interface.

- 6.3.1	Information exchanged between AIoT CN and AIoT RAS

- 6.3.1.1	Inventory

Inventory can be sent by the AIoT CN for a single device, or a group of devices, or all devices.

The Inventory Request from the AIoT CN to the AIoT RAS, may include the following:

(1) AIoT Device Identification (to find a single device, a group of devices, or all devices)

Note 1:	The definition of this identification is out of RAN3 scope.

Editor’s Note: It is FFS whether AIoT RAS needs to interpret/store/process it.

(2)	NAS information (device info may be included in NAS PDU)

Editor’s Note: It is up to SA3/SA2 conclusion on security issue.

(3)	Estimated Number of devices (for all device inventory)

Editor’s Note: Further check with RAN2 on AIoT radio resource allocation.

(4)   Service type (indicate inventory or command request for agnostic of the type of request)

(5)   Reader ID list

Multiple individual AIoT Device IDs (one ID per device) can be provided to the AIoT CN via a single Inventory Report.

Editor’s Note: It is up to SA2 whether device ID is sent transparent or not.

- 6.3.1.2	Command

Command can be sent by the AIoT CN for a single device or a group of devices.

Editor’s Note: it is FFS for command on all devices.

Editor note: it is FFS whether AIoT RAS can remain agnostic of the type of request from the AIoT CN (need to differentiate command and inventory)

The Command Request from the AIoT CN to the AIoT RAS, may include the following:

- AIoT Device Identification (to find a single device or a group of devices)

Note 1:	The definition of this identification is out of RAN3 scope.

- NAS information

- Service type (indicate inventory or command request for agnostic of the type of request)

- Reader ID list

----------------End of the Changes----------------