3GPP TSG-RAN WG3 #125                                            R3-244185

Maastricht, NL, 19th – 23rd Aug, 2024

Source:	CATT

Title:	(TP to TR 38.769) Network signalling aspects for Topology 2

Agenda Item:	16.3

Document for:	Decision

Introduction

The following agreements and FFS were captured in last RAN3 meeting:

Inventory over CN-RAN interface can be used for a single device, or a group of devices, or all devices.

Command over CN-RAN interface can be used for a single device. FFS for command on a group of devices, or all devices.

In the Inventory Request from the AIoT CN towards the AIoT RAS, it is FFS whether the following information may be included:

Inventory Session ID (e.g., to allow multiple Inventory procedure instances to run in parallel): the purpose, scope and handling of this information in AIoT RAS are FFS.

Estimated Number of expected responses from devices, or Minimum number of required responses from devices.

FFS the usage of these information.

Periodicity: to enable periodic inventory at AIoT RAS triggered by a single Inventory Request from the AIoT CN

In Topology 2, it is FFS which entity (e.g., AIoT CN and/or AIoT enabled gNB) performs the UE reader selection.

In topology1, it is FFS whether in response to a single Inventory Request, AIoT RAS provides only a single Inventory Report message or there can be also multiple Inventory Report messages provided.

According to the WI [1], Topologies 1 & 2(as shown in the figures below) should be considered in R19.

Topology 1: BS ↔ Ambient IoT device  	Topology 2: BS ↔ intermediate node ↔ Ambient IoT device

In this contribution, we will provide some analysis on the potential RAN3 impact for topology 2 to support inventory and command service for A-IoT, and base on the discussion, we will provide our observations and proposals accordingly.

Discussion

For topology 2 scenario, some devices can only communicate with certain intermediate UE reader. To paging these devices, NG-RAN should first send message to intermediate UE reader in Uu interface and then intermediate UE reader is responsible for paging these devices.

On how to transfer A-IoT information between immediate UE reader and CN, as discussed in SA2, there are mainly three solutions:

- Solution 1: RRC based solution

- Solution 2: NAS based solution

- Solution 3: user plane solution

RRC based solution and NAS based solution is also called control plane solution because A-IoT information is included in NGAP message. For NAS based solution, A-IoT information would be included in NAS PDU in NGAP NAS TRANSPORT message. For RRC based solution, A-IoT information would be directly sent in NGAP message. For user plane solution, A-IoT information is sent to UE as data transferred via PDU session.

NAS based solution and user plane solution may have no impact on NGAP interface because NG-RAN does not decode and handle NAS PDU and data in user plane. For these solutions, there may be not need to enhance NGAP interface message and A-IoT information is transparent for NG-RAN.

For RRC based solution, A-IoT information is explicitly included in NGAP message which may reuse the message defined for topology 1. So, the benefit for RRC base solution may be a common NGAP message used for both topology 1 and topology 2.

Which solution would be captured in WI is finally up to SA2/RAN2 and RAN3 may wait for the conclusion of SA2/RAN2 and then begin NGAP design.

Proposal 1: It is proposed for RAN3 to discuss above solutions on transferring A-IoT information in NGAP interface for topology 2 which is up to the conclusion of SA2/RAN2.

The following FFS is captured in last RAN3 meeting:

- In Topology 2, it is FFS which entity (e.g., AIoT CN and/or AIoT enabled gNB) performs the UE reader selection.

For NAS based solution and user plane solution, CN should include A-IoT information in NAS PDU or user plane data which is related to certain UE, i.e. AIoT CN has selected UE reader when transferring A-IoT information. For RRC based solution, either CN or NG-RAN could take the responsibility to perform the UE reader selection. It’s preferred that UE reader selection is performed by the AIoT CN.

Proposal 2: For Topology 2, UE reader selection should be performed by AIoT CN.

UE reader is selected for the following requirements in topology 2:

- Sending inventory/command request to certain devices.

- Sending inventory/command request to certain area.

To sending inventory/command request to certain devices, the relationship between UE reader and the related devices is necessary which could be collected by previous inventory procedure and then be saved in AIoT CN or in AIoT-enabled gNB, or both.

To sending inventory/command request to certain area, AIoT CN could obtain the location of the UE reader via OAM or via LCS and select UE reader(s).

Which entity should save the relationship between UE reader and the related devices depends on the conclusion of P2. If AIoT CN to perform UE reader selection, it may be better for AIoT CN to save it, whether RAN needs to keep the mapping need to be further considered.

Proposal 3: For Topology 2, the mapping/relationship between UE readers and AIoT devices should be kept in AIoT CN. Whether RAN needs to use it is FFS.

For RRC based solution, A-IoT information is explicitly included in NGAP message which may be the same as topology 1, i.e. the following information should also be included in NGAP message for topology 2:

- AIoT Device Identification

- NAS information

- Estimated Number of devices

- Service type (indicate inventory or command request for agnostic of the type of request)

- Reader ID list

Then the AIoT-enabled gNB provides the associated information to UE reader over enhanced NR Uu signalling to trigger inventory/command procedure.

Proposal 4: For RRC based solution, A-IoT information in NGAP message for topology 2 may be the same as the A-IoT information in XXAP/NGAP message for topology 1.

Conclusions

In this contribution, we mainly discussed the RAN-CN Interface Impact on topology 2. Based on the discussion above, the following proposals are provided:

Proposal 1: It is proposed for RAN3 to discuss above solutions on transferring A-IoT information in NGAP interface for topology 2 which is up to the conclusion of SA2/RAN2.

Proposal 2: For Topology 2, UE reader selection should be performed by AIoT CN.

Proposal 3: For Topology 2, the mapping/relationship between UE readers and AIoT devices should be kept in AIoT CN. Whether RAN needs to use it is FFS.

Proposal 4: For RRC based solution, A-IoT information in NGAP message for topology 2 may be the same as the A-IoT information in XXAP/NGAP message for topology 1.

Reference

[1] RP-240826 Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR

TP for TR 38.769

<<<<<<<<<<<<<<<<<< Begin of the changes >>>>>>>>>>>>>>>>>>>>>>>

6.3	Impacts on CN-RAN interface

Editor’s note: Corresponds to the first RAN3 objective in the SID, to identify necessary impacts on signaling and procedures for CN-RAN interface.

- 6.3.1	Information exchanged between AIoT CN and AIoT RAS

- 6.3.2	Topology 2

On how to transfer A-IoT information between immediate UE reader and AIoT CN, as discussed in SA2, there are mainly three solutions:

Solution 1: RRC based solution

For RRC based solution, A-IoT information would be explicitly exchanged between AIoT-enabled gNB and AIoT CN over NGAP, and be exchanged between the AIoT-enabled gNB and immediate UE reader over RRC signalling.

Solution 2: NAS based solution

For NAS based solution, A-IoT information would be included in the NAS PDU of the intermediate UE, which will be transferred between AIoT RAN and AIoT CN in the NGAP NAS TRANSPORT messages.

Solution 3: User plane based solution

For user plane solution, A-IoT information is exchanged between immediate UE reader and AIoT CN via UE’s PDU session.

Editor’s Note: It is up to RAN2/SA2 to decide the final solution(s) for topology 2.

AIoT CN should select proper UE reader(s) for inventory or command procedures, and save the mapping/relationship between UE readers and AIoT devices.

The following A-IoT information may be included in NGAP message for topology 2 as for topology 1:

(1) AIoT Device Identification (to find a single device, a group of devices, or all devices)

(2) NAS information (device info may be included in NAS PDU)

Editor’s Note: It is up to SA3/SA2 conclusion on security issue.

(3) Estimated Number of devices (for all device inventory)

Editor’s Note: Further check with RAN2 on AIoT radio resource allocation.

(4) Service type (indicate inventory or command request for agnostic of the type of request)

(5) UE reader ID(s)

Editor’s Note: What kind of UE reader ID(s) to be used is FFS.

----------------End of the Changes----------------