3GPP TSG-RAN WG3 #125                                                                     	R3-244186

Maastricht, NL, 19 - 23 August, 2024

Source:	CATT

Title:	(TP for TR 38.879) Locating of A-IoT device

Agenda Item:	16.4

Document for:	Discussion and Decision

- Introduction

In the previous RAN3 meeting, we discussed the support of “positioning” or “locating” of the A-IoT device, some agreements or common understandings are achieved as below:

AIoT device location information may be used for the following purposes:

(a) improving the AIoT operation itself, e.g. by AIoT CN sending a Command to one or more readers (e.g., the last reader(s)) associated to the device rather than sending it blindly.

(b) providing location information to the consumer of the AIoT service.

Locating an Ambient IoT device at “reader ID granularity” is useful for both purposes.

Editor’s Note 1: How to know the “reader” location is FFS. Whether to use more than one “readers” within one AIoT RAS for location purposes is FFS.

Editor’s Note 2: Signalling details on how to provide the location information of the AIoT device to the AIoT CN needs further study.

FFS for other granularities.

Some agreements have been achieved, e.g. “reader ID granularity” are helpful, however, the details on how to support “reader ID granularity” is not decided.  In this contribution, we will further discuss the support of locating aspects for A-IoT, and provide the observations and proposals accordingly.

- Discussion

Issue 1: How to know the “reader” location is FFS.

To our understanding, in topology 1, a “reader” looks like a TRP, an antenna, a beam, or a cell of a gNB, which is used to serve the AIoT devices in a specific area. To make the deployment more flexible, maybe the reader is co-located with the gNB, maybe not, and it’s reasonable to support multiple “readers” per A-IoT RAN node, and it’s assumed the “readers” are stationary. How the readers are connected to the gNB is out of RAN3 scope, as shown in the figure below:

Figure 1: one A-IoT RAN node holds one or multiple readers

Observation 1: In Topology 1, one A-IoT RAN node may hold one or multiple readers.

Observation 2: In Topology 1, it’s assumed the BS reader(s) are stationary.

Observation 3: In Topology 1, how the “reader(s)” are connected to the A-IoT RAN node is up to implementation, out of 3GPP.

For Topology 1, it’s assumed the A-IoT RAN node should know the GNSS location of each reader, and the GNSS area served by each reader. Signalling based solution and OAM based solution are both feasible to provide the reader information to the AIoT CN. Considering the “readers” are deployed stable, the details of the reader(s), e.g. GNSS location of the readers, the service area of the readers are fixed, it’s easier to configure the reader information to the AIoT capable CN compared with the signalling based way.

Proposal 1: In Topology 1, signalling based solution and OAM based solution are both feasible to provide the reader information (e.g. GNSS location, service area) to the AIoT CN. OAM based solution is preferred considering the readers are assumed to be stationary.

Issue 2: Whether to use more than one “readers” within one AIoT RAS for location purposes is FFS.

Base on the discussion above, more than one “readers” could be deployed within one A-IoT RAN. However, whether to consider using the multiple “readers” to improve the accuracy of locating to be further discussed.

As shown by the figure below, if multiple “readers” are deployed to cover the same area, the devices within the overlapped area may be served by different readers, it’s possible to use it to improve the accuracy of locating of the device.

Figure 2: Using multiple readers to improve the accuracy of device locating

However, in this scenario, how to do the inventory and command for the devices should be studied first, e.g. whether and how to send the inventory in the readers, how an A-IoT device response if it receives the inventory request from different readers?

For now, without any clear requirement on A-IoT locating, we do not need to consider using more than one “readers” within one AIoT RAS for location purposes.

Proposal 2: In this study item, we do not need to consider using more than one “readers” within one AIoT RAS for location purposes.

Issue 3: Signalling details on how to provide the location information of the AIoT device to the AIoT CN needs further study.

For Topology 1, if more than one readers are associated to an A-IoT RAN node, the reader ID could be included in the inventory response or command response, then, the CN could deduce the reader granularity device location via the reader ID.

Proposal 3: If more than one readers are associated to an A-IoT RAN node, the reader ID could be included in the inventory response or command response. With that, CN could deduce the reader granularity location of each device.

How to define the reader ID could be further studied, e.g. including the gNB ID as the first part of the “reader ID”, following a few bits to identify the readers.

Proposal 4: Further discussion is needed on the definition of the reader ID.

For Topology 2, it’s assumed that one intermediate UE holds only one Reader, it also knows the UE is a reader during UE registration procedure (e.g. according to the UE capability). The inventory/command response is reported to the CN via the UE specific signalling, e.g. the NAS of the UE. The signalling details to be further discussed in RAN2 and RAN3.

Observation 4: In Topology 2, intermediate UE is registered to the network as UE Reader, and one intermediate UE holds only one reader.

As above, the CN is able to know the mapping between the AIoT devices and the UE reader(s), it knows the reader granularity location of an AIoT device without extra information.

Figure 3: Example of Top2, mapping between UE readers and AIoT devices.

Proposal 5: In Topology 2,  UE specific signalling will be used to carry the inventory/command signallings. AIoT CN knows the reader granularity location of an AIoT device without any additional information.

For Topology 2, the UE reader may be stationary, maybe moveable, how the CN could get the location of the UE reader need to be discussed.

Observation 5: In Topology 2, UE reader may be stationary, maybe moveable.

If UE reader stationary, the UE location could be pre-configured/OAM configured to the AIoT CN. If the UE reader is moveable, OAM based solution is not feasible. No matter whether UE is stationary or moveable, AIoT CN could obtain the location of the UE reader via LCS.

Proposal 6: In Topology 2, AIoT CN could obtain the location of the UE reader via OAM configuration or via LCS.

- Conclusion

Based on the discussion in section 2, the following observations and proposals are provided:

Observation 1: In Topology 1, one A-IoT RAN node may hold one or multiple readers.

Observation 2: In Topology 1, it’s assumed the BS reader(s) are stationary.

Observation 3: In Topology 1, how the “reader(s)” are connected to the A-IoT RAN node is up to implementation, out of 3GPP.

Observation 4: In Topology 2, intermediate UE is registered to the network as UE Reader, and one intermediate UE holds only one reader.

Observation 5: In Topology 2, UE reader may be stationary, maybe moveable.

Proposal 1: In Topology 1, signalling based solution and OAM based solution are both feasible to provide the reader information (e.g. GNSS location, service area) to the AIoT CN. OAM based solution is preferred considering the readers are assumed to be stationary.

Proposal 2: In this study item, we do not need to consider using more than one “readers” within one AIoT RAS for location purposes.

Proposal 3: If more than one readers are associated to an A-IoT RAN node, the reader ID could be included in the inventory response or command response. With that, CN could deduce the reader granularity location of each device.

Proposal 4: Further discussion is needed on the definition of the reader ID.

Proposal 5: In Topology 2,  UE specific signalling will be used to carry the inventory/command signallings. AIoT CN knows the reader granularity location of an AIoT device without any additional information.

Proposal 6: In Topology 2, AIoT CN could obtain the location of the UE reader via OAM configuration or via LCS.

- Reference

- RAN3#123bis Chair Notes

- RP-234058 New SID: Study on solutions for Ambient IoT (Internet of Things) in NR

- TR 22.840 Study on Ambient power-enabled Internet of Things

- TR 38.848 Study on Ambient IoT (Internet of Things) in RAN

- TP for TR 38.879 AIoT Locating

<<<<<<<<<<<<<<<<<<<<<< Begin of the Changes >>>>>>>>>>>>>>>>>>>>>>>>

## 6.8	Locating ambient IoT devices

Editor’s note 1: Proximity determination may be in a 6.8.x sub-clause, or another arrangement, depending on how the study proceeds.

Editor’s note 2: Corresponds to the third RAN3 objective in the SID, to identify potential solutions for locating an Ambient IoT device with no specification impact, e.g., reusing existing user location report, or minimal specification impact to convey location information to core network.

### 6.8.1	General

AIoT device location information may be used for the following purposes:

(a) improving the AIoT operation itself, e.g. by AIoT CN sending a Command to one or more readers (e.g., the last reader(s)) associated to the device rather than sending it blindly.

(b) providing location information to the consumer of the AIoT service.

Locating an Ambient IoT device at “reader ID granularity” is useful for both purposes.

In Topology 1, one A-IoT RAN may hold one or multiple readers. The reader(s) are assumed to be stationary. The Reader Information (e.g. Location of the Reader, service area of the reader) could be configured to AIoT RAN and AIoT CN via OAM.

Figure 6.8-x: one A-IoT RAN node holds one or multiple readers

In Topology 1, the reader ID could be provided to the AIoT CN in the response message of the inventory or command procedure. With that, AIoT CN could deduce the reader granularity location of each AIoT device.

Editor’s Note x: How to define the reader ID is FFS.

In Topology 2, intermediate UE is registered to the network as UE Reader, and one intermediate UE holds only one UE reader. In Topology 2, UE reader may be stationary, maybe moveable. AIoT CN could obtain the location of the UE reader via OAM or via LCS.

Figure 6.8.y: Mapping between UE readers and AIoT devices in Top2

In Topology 2,  UE specific signalling will be used to carry the inventory/command signallings. AIoT CN knows the reader granularity location of an AIoT device without any additional information.

Note x: In this study item, using more than one “readers” for location purposes is not considered.

Editor’s Note 2: Signalling details on how to provide the location information of the AIoT device to the AIoT CN needs further study.

<<<<<<<<<<<<<<<<<<<<<< End of the Changes >>>>>>>>>>>>>>>>>>>>>>>>