3GPP TSG-RAN WG3 Meeting #125	R3-244187

# Maastricht, NL, 19th – 23rd August, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.305 | CR | draft | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of pre-configured SRS activation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, Huawei, ZTE, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-07-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Pre-configured SRS configuration is supported for LPHAP in Rel-18 Positioning WI. It’s agreed in RAN2 that UE could request for activation of pre-configured SRS configuration by a specific cause value in RRCResumeRequest message. However, how to activate the pre-configured SRS for the UE in the network side is not clear. This CR provides the overall procedure in Stage 2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add preconfigured SRS Configuration Activation Procedure, with or without anchor relocation in the stage 2. Impact Analysis: Impact assessment towards the previous version of the specification (same release): The CR has a protocol and function impact. The impact is limited to the positioning SRS preconfigured. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing Stage 2 support for preconfigured SRS Configuration Activation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.14.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 38.423 CR1333 TS 38.455 CR0158 |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<<<<<<<< Begin of the Changes >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

### 7.14.x	Pre-configured SRS Configuration Activation Procedure

Figure 7.14.x-1 shows the Pre-configured SRS Configuration Activation procedure, with full context relocation.

Figure 7.14.x-1: Pre-configured SRS Configuration Activation, with full context relocation

0.	The UE in RRC_INACTIVE state is pre-configured with one or more area-specific SRS for positioning configurations as specified in clause 7.14.2 and receives a request from upper layers to activate a pre-configured SRS for positioning.

1.	The UE sends a RRCResumeRequest message with the resume cause "srs-PosConfigOrActivationReq" to request activation of the pre-configured SRS.

2.	The receiving gNB triggers the Retrieve UE Context procedure towards the last serving gNB and indicates the specific cause value in the Retrieve UE Context Request message.

3.	The last serving gNB sends NRPPa Positioning Information Update message to the LMF including the Cell ID of the receiving gNB where the UE resumes from, and indicates pre-configured SRS is activated for the UE.

4.	The last serving gNB relocates the full UE context to the receiving gNB which includes the pre-configured SRS configuration, the Routing ID and the NRPPa Transaction ID in the Retrieve UE Context Response message.

5.	The receiving gNB triggers the Path Switch Request procedure towards the AMF.

6.	The AMF responds with the Path Switch Request Acknowledge.

7.	The receiving gNB activates the pre-configured SRS for the UE and sends the RRCRelease message to the UE.

8.	The receiving gNB indicates the last serving gNB to release the UE context.

Figure 7.14.x-2 shows the Pre-configured SRS Configuration Activation procedure, without context relocation.

Figure 7.14.x-2: Pre-configured SRS Configuration Activation, without context relocation

0.	The UE in RRC_INACTIVE state is pre-configured with one or more area-specific SRS for positioning configurations as specified in clause 7.14.2 and receives a request from upper layers to activate a pre-configured SRS for positioning.

1.	The UE sends RRCResumeRequest message with the resume cause "srs-PosConfigOrActivationReq" to request activation of the pre-configured SRS.

2.	The receiving gNB triggers the Retrieve UE Context procedure towards the last serving gNB and indicates the specific cause value in the Retrieve UE Context Request message.

3.	The last serving gNB sends NRPPa Positioning Information Update message to the LMF including the Cell ID of the receiving gNB where the UE resumes from, and notifies the LMF that the pre-configured SRS is activated for the UE.

4.	The last serving gNB decides not to relocate the UE context, it responses with Retrieve UE Context Failure, where the pre-configured SRS configuration is included in the RRCRelease message.

5. The receiving gNB activates the pre-configured SRS for the UE and sends the RRCRelease message to the UE.

<<<<<<<<<<<<<<<<<<<<<<<<<< End of the Changes >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>