3GPP TSG-RAN WG3 Meeting #125	R3-244188

# Maastricht, NL, 19th – 23rd August, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.305 | CR | draft | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of UE specific SRS Reservation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, Huawei, ZTE, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-07-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In Rel-18, SRS Information Reservation Notification procedure was introduced in the NRPPa to reserve the area-specific SRS configuration(s). However, the target gNBs in the validity area(s) could not identify the reserved SRS configuration(s) for the UE, it still need to retrieve the area-specific SRS configuration(s) from the last serving gNB. To make use of reserved SRS configuration(s) more efficiently, it’s better to link the reserved SRS configuration(s) with the I-RNTI. When UE requests for activation of the pre-configured SRS for positioning in RRCResumeRequest, the receiving gNB identifies the “reserved” pre-configrued SRS configuration for the UE, it notifies the last serving gNB pre-configured SRS configuration(s) is activated in the Retrieve UE Context Request. The last serving gNB indicates LMF the pre-configured SRS is activated, and sends the Retrieve UE Context Failure towards the receiving gNB. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The changes include: - Add a NOTE in Area-specific SRS (Pre-)configuration Allocation Procedure to allow using I-RNTI to associate with the Area-specific SRS (Pre-)configuration for the UE. - Introduce Pre-configured SRS Configuration Activation procedure, with pre-configured SRS reserved in the Receiving gNB. Impact Analysis: Impact assessment towards the previous version of the specification (same release): The CR has a protocol and function impact. The impact is limited to the positioning SRS preconfigured. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE specific SRS reservation and activation are not well supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.14.2, 7.14.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 38.423 CR1334 TS 38.455 CR0159 TS 38.473 CR1457 |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< Begin of the change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

### 7.14.2	Area-specific SRS (Pre-)configuration Allocation Procedure

Figure 7.14.2-1 shows the Area-specific SRS (Pre-)configuration Allocation procedure.

Figure 7.14.2-1: Area-specific SRS (Pre-)configuration Allocation Procedure

1.	The LMF sends the NRPPa Positioning Information Request message to the serving gNB of the UE for Area-specific SRS (Pre-)configuration allocation. In case of Area-specific SRS configuration allocation, the LMF includes the Requested SRS Transmission Characteristics including an associated Positioning Validity Area Cell List. In case of Area-specific SRS pre-configuration allocation, the LMF includes a list of Requested SRS Transmission Characteristics, each with the associated Positioning Validity Area Cell List.

2.	The serving gNB allocates the area-specific SRS resources, and moves the UE to RRC_INACTIVE state by sending the RRC Release message, which includes the area-specific SRS (pre-)configuration(s).

3.	The serving gNB responds with the NRPPa Positioning Information Response message to the LMF, including one or more SRS configuration(s), each with the associated Positioning Validity Area Cell List.

4.	The LMF notifies the gNBs within the positioning validity area(s) to reserve the SRS resources.

NOTE 1:	Step 4 may occur prior to any of steps 1 through 3.

NOTE 2: I-RNTI may be used to identify the SRS configuration(s) for the UE in the step 3 and 4, the receiving gNB uses it to identify the reserved SRS (pre-)configuration(s) when receives pre-configured SRS activation request in the RRCResumeRequest.

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< Next change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

### 7.14.x	Pre-configured SRS Configuration Activation Procedure

Figure 7.14.x-1 shows the Pre-configured SRS Configuration Activation procedure, with pre-configured SRS reserved in the Receiving gNB.

Figure 7.14.x-1: Pre-configured SRS Configuration Activation, with pre-configured SRS reserved

0.	The UE in RRC_INACTIVE state is pre-configured with one or more area-specific SRS for positioning configurations as specified in clause 7.14.2 and receives a request from upper layers to activate a pre-configured SRS for positioning. The pre-configured SRS configuration(s) is reserved in the Receiving gNB.

1.	The UE sends RRCResumeRequest message with the resume cause "srs-PosConfigOrActivationReq" to request activation of the pre-configured SRS.

2.	The receiving gNB identifies the reserved pre-configured SRS for the UE, triggers the Retrieve UE Context procedure towards the last serving gNB and indicates pre-configured SRS is activated.

NOTE: When the receiving gNB successfully identifies the reserved pre-configured SRS for the UE by I-RNTI, it activates the pre-configured SRS for the UE immediately without waiting for the pre-configured SRS configuration in the Retrieve UE Context Response/Failure. Or else, it activates the pre-configured SRS for the UE after receives the pre-configured SRS configuration from the last serving gNB in the Retrieve UE Context Response/Failure.

3.	The last serving gNB sends NRPPa Positioning Information Update message to the LMF including the Cell ID of the receiving gNB where the UE resumes from, and notifies LMF that the pre-configured SRS is activated for the UE.

4.	The last serving gNB sends Retrieve UE Context Failure to the Receiving gNB, which includes the RRCRelease message.

5. The receiving gNB forwards the RRCRelease message to the UE, the UE moves to the RRC_INACTIVE state.

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< End of the change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>