3GPP TSG-RAN WG3 Meeting #125	R3-244189

Maastricht, NL, 19th – 23rd August, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.455 | CR | 0157 | rev | - | Current version: | 18.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to Positioning SRS BW Aggregation and Tx Hopping |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, Ericsson, ZTE, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-07-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 1. SRS for Positioning with Tx Frequency Hopping for RedCap UEs, RAN1 agreed that the number of hops: ENUMERATED {2,3,4,5,6} (new IE). However, in the NRPPa, the value range of Number of Hops is defined as INTEGER(1..6), value 1 should not be used. 2. For Positioning SRS BW Aggregation, RAN1/RAN2 agreed that the number of “aggregated combinations” is up to 32, and for each combination, only one SRS resource set from each of the 2 or 3 carriers are aggregated, e.g. CC#1 SRS resource set 1 + CC#2 SRS resource set 2 + CC#3 SRS resource set 3. In NRPPa, the Aggregated Positioning SRS Resource Set List IE includes at most 48 SRS resource sets to be aggregated in one combination, which equal to 3CCs * 16 SRS Resource sets per CC. It’s not well aligned with the design of TS 38.331, which may cause erroneous during SRS configuration. To make alignment with NR-Uu, and make the IE encoding more clear, it’s proposed to move the Aggregated Positioning SRS Resource Set List IE to the top level of the SRS Configuration IE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The changes include: - Clarify that value 1 is not used for the number of hops. - Redefine the IE “Aggregated Positioning SRS Resource Set List” to support up to 32 combinations, and move the IE to the top level of the SRS Configuration IE. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release) because it affects the Positioning SRS BW Aggregation only. The CR has impact from functional and protocol point of view. The CR is non-backwards compatible from protocol point of view. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | IE encoding for SRS BW aggregation, Redcap Positioning are not clear which may cause confusion. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.6.2, 9.2.28, 9.2.32, 9.2.100, 9.3.5, 9.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 38.473 CR1466 |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< Begin of the change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

### 9.2.28	SRS Configuration

This information element contains the SRS configuration configured by the NG-RAN node for the UE.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| SRS Carrier List |  | 1 |  |  | - |  |
| >SRS Carrier List Item |  | 1..<maxnoSRS-Carriers> |  |  | - |  |
| >>Point A | M |  | INTEGER (0..3279165) | NR ARFCN | - |  |
| >>Uplink Channel BW-PerSCS-List |  | 1 |  | Corresponds to sCS-SpecificCarrierList in TS 38.331 [13] | - |  |
| >>>SCS Specific Carrier |  | 1. .<maxnoSCSs> |  |  | - |  |
| >>>>Offset To Carrier | M |  | INTEGER(0..2199,…) | First usable RB to Point A in the number of PRBs | - |  |
| >>>>Subcarrier Spacing | M |  | ENUMERATED(kHz15, kHz30, kHz60, kHz120,…, kHz480, kHz960) |  | - |  |
| >>>>Carrier Bandwidth | M |  | INTEGER(1..275,…) |  | - |  |
| >>Active UL BWP |  | 1 |  | Only the configuration in the active UL BWP is needed. | - |  |
| >>>Location And Bandwidth | M |  | INTEGER(0..37949,…) | Corresponds to information provided in locationAndBandwidth contained in BWP IE as defined in TS 38.331 [13] | - |  |
| >>>Subcarrier Spacing | M |  | ENUMERATED(kHz15, kHz30, kHz60, kHz120,…, kHz480, kHz960) |  | - |  |
| >>>Cyclic Prefix | M |  | ENUMERATED(Normal, Extended) |  | - |  |
| >>>Tx Direct Current Location | M |  | INTEGER(0..3301,…) |  | - |  |
| >>>Shift7dot5kHz | O |  | ENUMERATED(true,…) |  | - |  |
| >>>SRS Config |  | 1 |  | Corresponds to information provided in SRS-Config IE as defined in TS 38.331 [13] | - |  |
| >>>>SRS Resource List |  | 0..<maxnoSRS-Resources> |  |  | - |  |
| >>>>>SRS Resource | M |  | 9.2.29 | Corresponds to information provided in SRS-Resource contained in SRS-Config IE as defined in TS 38.331 [13] | - |  |
| >>>>Positioning SRS Resource List |  | 0..< maxnoSRS-PosResources> |  |  | - |  |
| >>>>>Positioning SRS Resource | M |  | 9.2.30 | Corresponds to information provided in SRS-PosResource contained in SRS-Config IE as defined in TS 38.331 [13] | - |  |
| >>>>SRS Resource Set List |  | 0..<maxnoSRS-ResourceSets> |  |  | - |  |
| >>>>>SRS Resource Set | M |  | 9.2.31 | Corresponds to information provided in SRS-ResourceSet contained in SRS-Config IE as defined in TS 38.331 [13] | - |  |
| >>>>Positioning SRS Resource Set List |  | 0..<maxnoSRS-PosResourceSets> |  |  | - |  |
| >>>>>Positioning SRS Resource Set | M |  | 9.2.32 | Corresponds to information provided in SRS-PosResourceSet contained in SRS-Config IE as defined in TS 38.331 [13] | - |  |
| >>NR PCI | O |  | INTEGER (0..1007) | Physical Cell ID of the cell that contains the SRS carrier | - |  |
| Aggregated Positioning SRS Resource Set List | O |  | 9.2.94 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoSRS-Carriers | Maximum no of carriers for SRS. Value is 32. |
| maxnoSCSs | Maximum no of SCS spacings for a carrier. Value is 5. |
| maxnoSRS-Resources | Maximum no of SRS resources per UL BWP. Value is 64. |
| maxnoSRS-PosResources | Maximum no of positioning SRS resources per UL BWP. Value is 64. |
| maxnoSRS-ResourceSets | Maximum no of SRS resource sets per UL BWP. Value is 16. |
| maxnoSRS-PosResourceSets | Maximum no of positioning SRS resource sets per UL BWP. Value is 16. |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< Next change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

### 9.2.32	Positioning SRS Resource Set

This information element indicates a positioning SRS resource set in the UE for UL SRS transmission.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Positioning SRS Resource Set ID | M |  | INTEGER(0..15) |  | - |  |
| Positioning SRS Resource ID List |  | 1..<maxnoSRS-PosResourcePerSet> |  |  | - |  |
| >Positioning SRS Resource ID | M |  | INTEGER(0..63) |  | - |  |
| CHOICE Resource Type | M |  |  |  | - |  |
| >periodic |  |  |  |  |  |  |
| >>PosperiodicSet | M |  | ENUMERATED(true,…) |  | - |  |
| >semi-persistent |  |  |  |  |  |  |
| >>Possemi-persistentSet | M |  | ENUMERATED(true,…) |  | - |  |
| >aperiodic |  |  |  |  |  |  |
| >>SRS Resource Trigger | M |  | INTEGER(1..3) |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoSRS-PosResourcePerSet | Maximum no of positioning SRS resources per positioning SRS resource set. Value is 16. |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< Next change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

### 9.2.94	Aggregated Positioning SRS Resource Set List

This information element is used to indicate the aggregated Positioning SRS Resource Set List.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Aggregated SRS Positioning Resource Set List |  | 1 |  |  |
| >Aggregated SRS Positioning Resource Set Item |  | 1..< maxnoAggregatedPosSRSCombinations> |  |  |
| >>Combined Positioning SRS Resource Set List |  | 1 |  |  |
| >>>Combined Positioning SRS Resource Set Item |  | 2..< maxnoaggregatedPosSRS-ResourceSets> |  |  |
| >>>>Point A | M |  | INTEGER (0..3279165) | NR ARFCN |
| >>>>NR PCI | O |  | INTEGER(0..1007) |  |
| >>>>Positioning SRS Resource Set ID | M |  | INTEGER(0..15) |  |
| >>>>SCS Specific Carrier |  | 1 |  |  |
| >>>>>Offset To Carrier | M |  | INTEGER (0..2199, …) |  |
| >>>>>Subcarrier Spacing | M |  | ENUMERATED(kHz15, kHz30, kHz60, kHz120, … , kHz480, kHz960) |  |
| >>>>>Carrier Bandwidth | M |  | INTERGER (1..275, …) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoaggregatedPosSRS-ResourceSets | Maximum no of aggregated SRS Positioning Resource Sets among 2~3 Carriers. Value is 3. |
| maxnoAggregatedPosSRSCombinations | Maximum no of Aggregated SRS Positioning Resource Set Combinations. Value is 32. |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< Next change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

### 9.2.100	Tx Hopping Configuration

This information element indicates the SRS Tx frequency hopping configuration.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Overlap Value | M |  | ENUMERATED(rb0, rb1, rb2, rb4) |  |
| Number of Hops | M |  | INTEGER(1..6) | In this release, value 1 is not applied. |
| Slot Offset for Remaining Hops List |  | 1 |  |  |
| >Slot Offset for Remaining Hops Item |  | 1..<maxnoofHopsMinusOne> |  |  |
| >>CHOICE slot offset remaining hops | M |  |  |  |
| >>>aperiodic |  |  |  |  |
| >>>>Slot Offset | O |  | INTEGER(1..32) |  |
| >>>>Start Position | O |  | INTEGER(0..13) | In symbols |
| >>>semi-persistent |  |  |  |  |
| >>>>SRS Periodicity | M |  | 9.2.99 |  |
| >>>>Offset | M |  | INTEGER(0..81919, …) | In slots |
| >>>>Start Position | O |  | INTEGER(0..13) | In symbols |
| >>>periodic |  |  |  |  |
| >>>>SRS Periodicity | M |  | 9.2.99 |  |
| >>>>Offset | M |  | INTEGER(0..81919, …) |  |
| >>>>Start Position | O |  | INTEGER(0..13) | In symbols |

| Range bound | Explanation |
| --- | --- |
| maxnoofHopsMinusOne | Maximum no of hops that can be configured for positioning SRS transmission minus one. Value is 5. |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< Next change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NRPPA-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) nrppa (4) version1 (1) nrppa-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-MeasurementQuantities-Item,

id-CGI-NR,

id-SFNInitialisationTime-NR,

id-GeographicalCoordinates,

id-ResultSS-RSRP,

id-ResultSS-RSRQ,

id-ResultCSI-RSRP,

id-ResultCSI-RSRQ,

id-AngleOfArrivalNR,

id-ResultNR,

id-ResultEUTRA,

maxCellinRANnode,

maxCellReport,

maxNrOfErrors,

maxNoMeas,

maxnoOTDOAtypes,

maxServCell,

id-OtherRATMeasurementQuantities-Item,

id-WLANMeasurementQuantities-Item,

maxGERANMeas,

maxUTRANMeas,

maxWLANchannels,

maxnoFreqHoppingBandsMinusOne,

id-TDD-Config-EUTRA-Item,

maxNrOfPosSImessage,

maxnoAssistInfoFailureListItems,

maxNrOfSegments,

maxNrOfPosSIBs,

maxnoPosMeas,

maxnoTRPs,

maxnoTRPInfoTypes,

maxNoOfMeasTRPs,

maxNoPath,

maxnoofAngleInfo,

maxnolcs-gcs-translation,

maxnoBcastCell,

maxnoSRSTriggerStates,

maxnoSpatialRelations,

maxNRMeas,

maxEUTRAMeas,

maxIndexesReport,

maxCellReportNR,

maxnoSRS-Carriers,

maxnoSCSs,

maxnoSRS-Resources,

maxnoSRS-PosResources,

maxnoSRS-ResourceSets,

maxnoSRS-ResourcePerSet,

maxnoSRS-PosResourceSets,

maxnoSRS-PosResourcePerSet,

maxPRS-ResourceSets,

maxPRS-ResourcesPerSet,

maxNoSSBs,

maxnoofPRSresourceSet,

maxnoofPRSresource,

maxnoofULAoAs,

maxNoPathExtended,

maxnoARPs,

maxnoTRPTEGs,

maxnoUETEGs,

maxFreqLayers,

maxnoPRSTRPs,

maxNumResourcesPerAngle,

maxnoAzimuthAngles,

maxnoElevationAngles,

maxnoVACell,

maxnoaggregatedPosSRS-Resources,

maxnoaggregatedPosSRS-ResourceSets,

maxnoAggPosPRSResourceSets,

maxnoofTimeWindowSRS,

maxnoofTimeWindowMeas,

maxnoPreconfiguredSRS,

maxnoofHopsMinusOne,

maxnoAggCombinations,

maxnoAggregatedPosSRSCombinations,

<<<<<<<<<<<<<< unchanged texts omitted >>>>>>>>>>>>>>>

#### -- P

<<<<<<<<<<<<<< unchanged texts omitted >>>>>>>>>>>>>>>

PosSRSResourceSet-Item ::= SEQUENCE {

possrsResourceSetID				INTEGER(0..15),

possRSResourceIDPerSet-List		PosSRSResourceIDPerSet-List,

posresourceSetType				PosResourceSetType,

iE-Extensions		ProtocolExtensionContainer { { PosSRSResourceSet-Item-ExtIEs} }	OPTIONAL,

...

}

PosSRSResourceSet-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

PosSRSResourceSet-Aggregation-List ::= SEQUENCE (SIZE (1..maxnoAggregatedPosSRSCombinations)) OF PosSRSResourceSet-Aggregation-Item

PosSRSResourceSet-Aggregation-Item ::= SEQUENCE {

combined-posSRSResourceSet-List			Combined-PosSRSResourceSet-List,

pointA				INTEGER (0..3279165),

pCI-NR				INTEGER(0..1007)		OPTIONAL,

possrsResourceSetID	INTEGER(0..15),

iE-Extensions		ProtocolExtensionContainer { { PosSRSResourceSet-Aggregation-Item-ExtIEs} }	OPTIONAL,

...

}

PosSRSResourceSet-Aggregation-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

Combined-PosSRSResourceSet-List ::= SEQUENCE (SIZE (2..maxnoaggregatedPosSRS-ResourceSets)) OF Combined-PosSRSResourceSet-Item

Combined-PosSRSResourceSet-Item::= SEQUENCE {

pointA				INTEGER (0..3279165),

pCI-NR				INTEGER(0..1007)		OPTIONAL,

possrsResourceSetID	INTEGER(0..15),

scs-specificCarrier	SCS-SpecificCarrier,

iE-Extensions						ProtocolExtensionContainer { { Combined-PosSRSResourceSet-Item-ExtIEs} } OPTIONAL,

...

}

Combined-PosSRSResourceSet-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

#### -- S

<<<<<<<<<<<<<< unchanged texts omitted >>>>>>>>>>>>>>>

SRSConfiguration ::= SEQUENCE {

sRSCarrier-List		SRSCarrier-List,

iE-Extensions			ProtocolExtensionContainer { { SRSConfiguration-ExtIEs } } OPTIONAL,

...

}

SRSConfiguration-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

{ ID id-PosSRSResourceSet-Aggregation-List	CRITICALITY ignore EXTENSION PosSRSResourceSet-Aggregation-List		PRESENCE optional },

...

}

#### -- T

<<<<<<<<<<<<<< unchanged texts omitted >>>>>>>>>>>>>>>

TxHoppingConfiguration ::= SEQUENCE {

overlapValue						ENUMERATED {rb0, rb1, rb2, rb4},

numberOfHops						INTEGER (1..6),

-- The value 1 is not supported in this version of the specification.

slotOffsetForRemainingHopsList		SlotOffsetForRemainingHopsList,

iE-extensions		ProtocolExtensionContainer { { TxHoppingConfiguration-ExtIEs } }	OPTIONAL,

...

}

TxHoppingConfiguration-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< Next change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

NRPPA-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) nrppa (4) version1 (1) nrppa-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM NRPPA-CommonDataTypes;

-- **************************************************************

--

-- Elementary Procedures

--

-- **************************************************************

id-errorIndication												ProcedureCode ::= 0

id-privateMessage												ProcedureCode ::= 1

id-e-CIDMeasurementInitiation									ProcedureCode ::= 2

id-e-CIDMeasurementFailureIndication							ProcedureCode ::= 3

id-e-CIDMeasurementReport										ProcedureCode ::= 4

id-e-CIDMeasurementTermination									ProcedureCode ::= 5

id-oTDOAInformationExchange										ProcedureCode ::= 6

id-assistanceInformationControl									ProcedureCode ::= 7

id-assistanceInformationFeedback								ProcedureCode ::= 8

id-positioningInformationExchange								ProcedureCode ::= 9

id-positioningInformationUpdate									ProcedureCode ::= 10

id-Measurement													ProcedureCode ::= 11

id-MeasurementReport											ProcedureCode ::= 12

id-MeasurementUpdate											ProcedureCode ::= 13

id-MeasurementAbort												ProcedureCode ::= 14

id-MeasurementFailureIndication									ProcedureCode ::= 15

id-tRPInformationExchange										ProcedureCode ::= 16

id-positioningActivation										ProcedureCode ::= 17

id-positioningDeactivation										ProcedureCode ::= 18

id-pRSConfigurationExchange										ProcedureCode ::= 19

id-measurementPreconfiguration									ProcedureCode ::= 20

id-measurementActivation										ProcedureCode ::= 21

id-sRSInformationReservationNotification						ProcedureCode ::= 22

-- **************************************************************

--

-- Lists

--

-- **************************************************************

maxNrOfErrors								INTEGER ::= 256

maxCellinRANnode							INTEGER ::= 3840

maxIndexesReport							INTEGER ::= 64

maxNoMeas									INTEGER ::= 64

maxCellReport								INTEGER ::= 9

maxCellReportNR								INTEGER ::= 9

maxnoOTDOAtypes								INTEGER ::= 63

maxServCell									INTEGER ::= 5

maxEUTRAMeas								INTEGER ::= 8

maxGERANMeas								INTEGER ::= 8

maxNRMeas									INTEGER ::= 8

maxUTRANMeas								INTEGER ::= 8

maxWLANchannels								INTEGER ::= 16

maxnoFreqHoppingBandsMinusOne				INTEGER ::= 7

maxNoPath									INTEGER ::= 2

maxNrOfPosSImessage							INTEGER ::= 32

maxnoAssistInfoFailureListItems				INTEGER ::= 32

maxNrOfSegments								INTEGER ::= 64

maxNrOfPosSIBs								INTEGER ::= 32

maxNoOfMeasTRPs								INTEGER ::= 64

maxnoTRPs									INTEGER ::= 65535

maxnoTRPInfoTypes							INTEGER ::= 64

maxnoofAngleInfo							INTEGER ::= 65535

maxnolcs-gcs-translation					INTEGER ::= 3

maxnoBcastCell								INTEGER ::= 16384

maxnoSRSTriggerStates						INTEGER ::= 3

maxnoSpatialRelations						INTEGER ::= 64

maxnoPosMeas								INTEGER ::= 16384

maxnoSRS-Carriers							INTEGER ::= 32

maxnoSCSs									INTEGER ::= 5

maxnoSRS-Resources							INTEGER ::= 64

maxnoSRS-PosResources						INTEGER ::= 64

maxnoSRS-ResourceSets						INTEGER ::= 16

maxnoSRS-ResourcePerSet						INTEGER ::= 16

maxnoSRS-PosResourceSets					INTEGER ::= 16

maxnoSRS-PosResourcePerSet					INTEGER ::= 16

maxPRS-ResourceSets							INTEGER ::= 2

maxPRS-ResourcesPerSet						INTEGER ::= 64

maxNoSSBs									INTEGER ::= 255

maxnoofPRSresourceSet						INTEGER ::= 8

maxnoofPRSresource							INTEGER ::= 64

maxnoofULAoAs								INTEGER ::= 8

maxNoPathExtended							INTEGER ::= 8

maxnoARPs									INTEGER ::=	16

maxnoUETEGs									INTEGER ::= 256

maxnoTRPTEGs								INTEGER ::= 8

maxFreqLayers								INTEGER ::= 4

maxNumResourcesPerAngle						INTEGER ::= 24

maxnoAzimuthAngles							INTEGER ::= 3600

maxnoElevationAngles						INTEGER ::= 1801

maxnoPRSTRPs								INTEGER ::= 256

maxnoVACell									INTEGER ::= 32

maxnoaggregatedPosSRS-Resources				INTEGER ::= 3

maxnoaggregatedPosSRS-ResourceSets			INTEGER ::= 3

maxnoAggPosPRSResourceSets					INTEGER ::= 3

maxnoofTimeWindowSRS						INTEGER ::= 16

maxnoofTimeWindowMeas						INTEGER ::= 16

maxnoPreconfiguredSRS						INTEGER ::= 16

maxnoofHopsMinusOne							INTEGER ::= 5

maxnoAggCombinations						INTEGER ::= 2

maxnoAggregatedPosSRSCombinations			INTEGER ::= 32

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< End of the change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>