3GPP TSG-RAN WG3 Meeting #125	R3-244192

# Maastricht, Netherlands, 19 – 23 August 2024

Agenda item:	16.3

Source:	Xiaomi

Title:	(TP for TR 38.769) AIoT interface impacts between RAN and CN

Document for:	Other

1	Introduction

RAN3#124:

Inventory over CN-RAN interface can be used for a single device, or a group of devices, or all devices.

Command over CN-RAN interface can be used for a single device. FFS for command on a group of devices, or all devices.

What’s the device context？

The content in Inventory and Command?

In this contribution, we’d like to discuss the interface impacts according to the discussion in RAN3 123bis meeting.

With regard the RAN and CN interface impacts, following topics will be discussed in this Agenda item:

For topology 1

- Information Inventory paging

- Device info transfer and device context

- Command operation

For topology 2

- UE selection and AIoT resource allocation

2	Discussion

## Stage2 procedure for topology 1

For Topology 1, we think the stage2 procedure of inventory and command can be as follows

Figure 1 stage 2 procedure for AIoT operation in T1

In step 1, the AIoT CN sends the AIoT inventory paging to the AIoT gNB, along with a specific task ID. The task ID uniquely identifies an AIoT inventory operation.

In step 2, as the inventory may cost a long time, the AIoT RAN node can send the acknowledgement to AIoT CN to indicate that the inventory operation is performed.

In step 3, The AIoT gNB, acting as the reader, initiates the inventory procedure with one or more device(s). Handshake details are based on discussions in RAN1/2. After or during the handshake, the AIoT device(s) may transmit its UL AIoT device info to the AIoT gNB.

In step 4, the AIoT RAN node transfers the one or more UL AIoT device info along with the task ID to the AIoT AMF. If it’s inventory only operation, AIoT RAN node does not need to have the device context, if it’s inventory + command operation, the AIoT RAN node should generate a RAN specific device ID for the device and include it in any device-associated messages.

In step 5, if further command operation is needed for the specific device, the AIoT CN sends the AIoT command request to the AIoT gNB, along with the RAN specific device ID, AIoT command and DL AIoT device info (if any).

In step 6, the AIoT RAN node performs the AIoT command operation, if any, it may transmit the DL AIoT device to the AIoT device, and/or receive the UL AIoT device info from the AIoT device. Considering that the command operation is aimed for a specific device, the response message can be sent after the AIoT command operation over Uu.

In step7, the AIoT RAN node send the AIoT command response to the AIoT AMF, along with the RAN specific device ID, AIoT command and UL AIoT device info (if any).

The following are detail discussion based on the above stage 2 procedure.

Considering the inventory paging over Uu may last a long time e.g. wait for the device response or collect multiple devices info, before the inventory paging is finished over Uu, the AIoT RAN node can send an acknowledgement to the AIoT CN to confirm that the operation is on-going.

Proposal 1, Introduce a new class 1 message (e.g., AIoT Inventory paging) for AIoT inventory paging and execution acknowledgement between the AIoT CN to the AIoT gNB.

## Information Inventory paging

| Information in inventory paging request message | Analysis | Proposal |
| --- | --- | --- |
| AIoT Device Identification (to find a single device, a group of devices, or all devices) | This identification can be MASK info, which will be further transfer to device, so that device can decide whether to response the inventory paging. | Needed, can be MASK info |
| Scope of inventory request (e.g., a certain area in which the inventory is to be triggered) | We think this area information can be used for gNB to decide whether a reader needs to perform this inventory paging. This area information can be list of TRPs, a list of cells, etc. | Needed, can be list of TRPs or cells. |
| Inventory Session/Task ID | gNB may receive multiple inventory requests from AIoT CN(s), similar to positioning, there’s transaction ID to identify a specific procedure. | Needed |
| Estimated Number of devices | This may help reader to allocate radio resources, to perform re-triggering or to set Q value, but the above can also be done by reader’s implementation. E.g. the reader can know from the identification info whether a single or multiple device are expected, the reader can set Q and re-trigger inventory by knowing the Collision over AIoT radio. This info may help to reduce the processing complexity of the reader. | Beneficial, but not mandatory |
| Periodicity | It is possible that the inventory process is periodic | Needed |

Proposal 2, Inventory Session/Task ID and Periodicity can be included in the inventory paging message.

Proposal 3, the AIoT Device Identification in inventory paging message can be MASK info that will finally be sent to device.

Proposal 4, Scope of inventory request in the inventory paging message can be list of TRPs or cells.

## Device info transfer and device context

In step 4, there may be multiple UL AIoT device(s)’ information to be transferred to AIoT CN, and this AIoT device information can be conveyed as container, the gNB may not be aware of it. To support it, it’s proposed to introduce a class 2 message “e.g. inventory report” from AIoT RAN node to AIoT CN.

Proposal 5, introduce a new class 2 message “e.g. inventory report” from AIoT RAN node to AIoT CN to transfer the UL AIoT device info container.

In step 5, In cases of inventory + command operations, the AIoT RAN node needs to have device context. To support it, the AIoT RAN node can generate a RAN specific device ID for a specific device and include it in any device associated messages, the RAN specific device ID (similar to NGAP UE ID) is used to identify a specific device within the RAN that communicates with the AIoT gNB.

As stated in step 4, in case of it’s an inventory-only operation, the device info transmission is one-short, there’s no need to maintain device context, i.e. in step 5, there’s no need to introduce RAN specific device ID.

If the AIoT RAN node needs to maintain the device context, how and when to release the context needs to be further discussed.

Proposal 6, for inventory + command operation, AIoT RAN node generates the RAN device ID for a specific device and includes it in the message that transfer the device specific info.

Proposal 7, for inventory only operation, the device info transmission is one-shot transmission, the AIoT RAN node does not need to have device context.

## Command operation

The step 5 to step 7 are performed if the command operation is needed. It’s proposed to introduce a class 1 message for command related operation, and the class 1 message is device-associated, i.e. the RAN specific device ID can be introduced in the class 1 message to identify specific device.

Proposal 8, a new class 1 message can be introduced between AIoT CN and AIoT RAN node to support command operation, and this class 1 message is device associated.

## UE reader selection and resource allocation in topology 2

For UE AIoT resource allocation, the serving gNB must verify if the UE is authorized as a UE reader to allocate AIoT radio resources. Similar to other authorizations (e.g., SL, SL positioning), the AMF sends the UE’s authorization information via UE context-related messages to the serving gNB, which stores this information as part of the UE context. This authorization information is transferred during UE handover and UE context retrieval procedures.

Proposal 9, RAN3 agrees that the AIoT authorization information for the UE reader should be sent from the serving AMF to the serving gNB in UE context-related messages and stored as part of the UE context in the serving gNB.

Regarding UE reader selection, there’re two options to perform UE reader selection

- option 1, AIoT CN performs the UE reader selection

- option 2, gNB performs the UE reader selection

For option 1, if RRC-based solution is chosen, the AIoT CN can sends the AIoT operation info to the serving gNB of the selected UE reader, then the serving gNB sends the AIoT operation info to UE reader via RRC signalling, the serving gNB is aware of the AIoT operation for the UE reader, it can allocate AIoT radio resources directly.

Figure 2 UE selection in AMF, RRC-based solution in T2

For option 1, if NAS/UP-based solution is chosen, the AIoT CN can sends the AIoT operation info to UE via NAS/UP, the serving gNB is not aware of the AIoT operation for the UE reader, in this case, UE reader can send AIoT resource request to the serving gNB or the AIoT CN indicate the AIoT operation to the serving gNB, so that the serving gNB can allocate AIoT radio resource for the UE reader.

Figure 3 UE selection in AMF, NAS/UP-based solution in T2

For option 2, the AIoT CN can sends the AIoT operation request to gNB, gNB can select UE reader and then allocate AIoT radio resources to the selected UE reader.

Figure 4 UE selection in gNB in T2

Proposal 10, RAN3 discuss the RAN-CN impacts for the following cases:

- option 1, AIoT CN performs the UE reader selection, including RRC-based and NAS/UP-based solutions

- option 2, gNB performs the UE reader selection

In the Annex, we provided a TP for TR 38.769 to reflect the potential RAN-CN interface impacts for topology 1 and topology 2.

Proposal 11, RAN3 agrees the TP for TR 38.769 in the Annex.

3	Conclusion

In this contribution, we discussed the RAN-CN interface impacts to support AIoT in topology 1 and topology 2, and have the following observations and proposals.

## Inventory paging

Proposal 1, Introduce a new class 1 message (e.g., AIoT Inventory paging) for AIoT inventory paging and execution acknowledgement between the AIoT CN to the AIoT gNB.

Proposal 2, Inventory Session/Task ID and Periodicity can be included in the inventory paging message.

Proposal 3, the AIoT Device Identification in inventory paging message can be MASK info that will finally be sent to device(s).

Proposal 4, Scope of inventory request in the inventory paging message can be list of TRPs or cells.

Proposal 5, introduce a new class 2 message “e.g. inventory report” from AIoT RAN node to AIoT CN to transfer the UL AIoT device info container.

## Device info transfer and device context

Proposal 6, for inventory + command operation, AIoT RAN node generates the RAN device ID for a specific device and includes it in the message that transfer the device specific info.

Proposal 7, for inventory only operation, the device info transmission is one-shot transmission, the AIoT RAN node does not need to have device context.

## Command operation

Proposal 8, a new class 1 message can be introduced between AIoT CN and AIoT RAN node to support command operation, and this class 1 message is device associated.

## UE reader selection and resource allocation in topology 2

Proposal 9, RAN3 agrees that the AIoT authorization information for the UE reader should be sent from the serving AMF to the serving gNB in UE context-related messages and stored as part of the UE context in the serving gNB.

Proposal 10, RAN3 discuss the RAN-CN impacts for the following cases:

- option 1, AIoT CN performs the UE reader selection, including RRC-based and NAS/UP-based solutions

- option 2, gNB performs the UE reader selection

Proposal 11, RAN3 agrees the TP for TR 38.769 in the Annex.

4	Reference

[1]	R3-243872, Inventory and Command between AIoT CN amd AIoT RAS

5	Annex (TP to R3-243872 for TR 38.769)

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

6.3	Impacts on CN-RAN interface

Editor’s note: Corresponds to the first RAN3 objective in the SID, to identify necessary impacts on signaling and procedures for CN-RAN interface.

6.3.1	Information exchanged between AIoT CN and AIoT RAN

6.3.1.1	Inventory

Inventory can be sent by the AIoT CN for a single device, or a group of devices, or all devices.

The Inventory Request from the AIoT CN to the AIoT RAN, may include the following:

(1) AIoT Device Identification (to find a single device, a group of devices, or all devices)

Note 1:	The definition of this identification is out of RAN3 scope.

Editor’s Note: It is FFS whether AIoT RAS needs to interpret/store/process it.

(2)	Scope of inventory request (e.g., a certain area in which the inventory is to be triggered)

Multiple individual AIoT Device IDs (one ID per device) can be provided to the AIoT CN via a single Inventory Report.

Editor’s Note: It is up to SA2 whether device ID is sent transparent or not.

6.3.1.2	Command

Command can be sent by the AIoT CN for a single device.

Editor’s Note: it is FFS for command on a group of devices, or all devices.

Editor note: it is FFS whether AIoT RAS can remain agnostic of the type of request from the AIoT CN (need to differentiate command and inventory)

6.3.1.3	UE reader authorization

The serving gNB of UE receives the UE reader’s AIoT authorized information from the serving AMF of UE in initial UE context setup procedure, and it may also receive the updated authorized information from the AMF in UE context modification procedure, path switch procedure. The AIoT authorized information can be used by serving gNB for UE reader selection (FFS) and AIoT radio resource allocation.

6.3.1.4	UE reader selection and AIoT resource allocation

In case of UE selection in AMF and RRC-based signalling is used, the AIoT CN notifies the serving gNB of UE reader the AIoT operation information for the UE reader, the serving gNB can allocate AIoT radio resources accordingly and configured to UE reader.

Figure 6.3.1.4-1: UE selection in AMF, RRC-based solution for T2.

In case of UE selection in AMF and NAS/UP-based signalling is used, either AIoT CN or UE reader sends AIoT radio resource request to the serving gNB, the serving gNB can allocate AIoT radio resources accordingly and configured to UE reader.

Figure 6.3.1.4-2: UE selection in AMF, NAS/UP-based solution for T2.

In case of UE selection in gNB, AIoT CN sends AIoT operation request to gNB, and gNB performs the UE selection and allocate AIoT radio resources accordingly and configured to UE reader.

Figure 6.3.1.4-3: UE selection in gNB for T2.

<<<<<<<<<<<<<<<<<<<< Change Ends >>>>>>>>>>>>>>>>>>>>