3GPP TSG-RAN WG3 Meeting #125	R3-244193

# Maastricht, Netherlands, 19 – 23 August 2024

Agenda item:	16.4

Source:	Xiaomi

Title:	(TP for TR 38.769) Locating AIoT device

Document for:	Other

1	Introduction

In previous meetings, we discussed how to locate AIoT device in different granularities, however, the granularities other than “reader ID level” are hard to progress if there’s no RAN1 inputs. In this contribution, we’d like to focus on “reader ID” level locating issues based on the following progress. For other granularities, we can wait for other WG’s conclusion.

AIoT device location information may be used for the following purposes:

(a) improving the AIoT operation itself, e.g. by sending a Command to one or more readers (e.g., the last reader(s)) associated to the device rather than sending it blindly.

(b) providing location information to the consumer of the AIoT service.

Locating an Ambient IoT device at “reader ID granularity” is useful for both purposes.

How to know the “reader” location is FFS.

Whether and how to utilize more than one “readers” for location purposes is FFS depending on involved complexity.

Signalling details on how to provide the location information to the A-IoT CN node.

2	Discussion

## Improving AIoT operation

improving the AIoT operation itself, e.g. by sending a Command to one or more readers (e.g., the last reader(s)) associated to the device rather than sending it blindly.

The target device(s) may be surrounded by multiple readers, which can either be AIoT RAN readers or UE readers. The AIoT CN may need to identify appropriate reader(s) that can contact with the target AIoT device for further operations.

The following figure shows an example for how the AIoT CN knowns which readers can connect with the target AIoT device.

a) The AIoT CN can send inventory paging to the readers in a specific area. Subsequently, the readers independently initiate Inventory procedures to establish communication with the target devices, as depicted in Figure 1(a).

b) Upon receiving a response from the target device. The inventory report message can include the reader ID and optionally the RSSI of the backscatter signal received from the target device as shown in Figure 3(b). This enables AIoT CN find to one or more readers (e.g., the last reader(s)) associated to the device rather than sending it blindly.

Figure 1 solution 2 to find an appropriate “reader” close to the A-IoT device

Observation 1, In scenarios where a target device is surrounded by multiple readers, the inclusion of reader IDs along with the respective measurements of the received backscatter signals can improve the further AIoT operations.

Proposal 1, the reader ID and the corresponding receiving signal strength from the target device can be included in the AIoT operation response message, e.g. inventory report, command response to improve the AIoT operation.

## Clarification on Reader ID

While the concept of reader ID has been discussed since the first meeting, there remains ambiguity regarding its definition. We believe further discussion is necessary to establish a clear understanding. We propose existing identification methods can be re-used as follows:

For Topology 1: The reader can be uniquely identified using the gNB ID and TRP ID. As one TRP can be seen as a reader that using different antennas in different locations to transmit AIoT signal.

For Topology 2: Depending on the signaling solutions, different types of IDs can be used for reader identification:

- RRC-based Signaling Solution: The XXAP UE ID should be utilized to identify the UE reader, providing a reliable means of distinguishing between different readers.

- NAS-based Signaling Solution: Either the GUTI (Globally Unique Temporary Identifier) or the SUPI (Subscription Permanent Identifier) can be used to identify the UE reader.

- UP-based Signaling Solution: The UE IP address can serve as a unique identifier for the UE reader

Proposal 2, RAN3 discuss the definition of the reader ID.

Proposal 3, for Topology 1, the reader ID can be gNB ID and TRP ID; for topology 2, the reader ID can be XXAP UE ID, GUTI, SUPI or IP address depending on the signalling solutions.

Providing location information

providing location information to the consumer of the AIoT service.

For topology 1, providing location information can be interpreted as providing the reader’s location information. There are two options:

- Option 1: Similar to TRP information exchange, the gNB provides TRP information to AIoT CN in advance, including geo-coordinates for each TRP, using non-AIoT operation-associated signalling.

- Option 2: The TRP’s geo-coordinates can be included in the inventory/command response message.

For option 1, the possible signaling flow can be as follows:

In steps 1-2, the AIoT RAN can provide a list of TRP IDs that support AIoT operations, along with the corresponding geo-coordinates for each TRP. This information can help the AIoT CN decide which TRP to use for a specific AIoT operation. For example, the AIoT CN can determine that the target device is near to a specific TRP based on the service request and the TRP’s geo-coordinates.

In steps 3-5, the AIoT RAN performs AIoT operations, such as inventory, as requested by the AIoT CN. It can include the TRP ID, representing the reader ID, in the response or report message. The CN can then determine the reader’s location based on the TRP ID and its corresponding geo-coordinates.

For option 2, the possible signaling flow can be as follows:

In steps 1-3, the AIoT RAN performs AIoT operations, such as inventory, as requested by the AIoT CN. It can include the TRP ID, representing the reader ID, and the corresponding geo-coordinates of the TRP in the response or report message. This allows the CN to directly determine the reader’s location.

Both options are viable. Option 1 may reduce signaling overhead, as the AIoT RAN does not need to report the TRP’s geo-coordinates every time. Additionally, obtaining the TRP information in advance can be helpful for reader selection.

For topology 2, the information may vary depending on the signaling solution used.

RRC-based signaling solution: The serving gNB of the UE reader is aware of the AIoT operation but typically does not have right to obtain the UE’s GNSS location information. From the serving gNB’s perspective, it can only provide the user location information of the UE reader, including cell and TA information, to the CN.

In steps 1-5, the UE reader performs the AIoT operation as requested. In step 5, the serving gNB includes the XXAP UE ID and user location information, such as CGI, TAI, and Age of Location (existing user location information can be reused).

In step 6, if the CN requires more precise UE location information, it may initiate location services to obtain the geo-coordinates of the UE reader.

For NAS/UP-based solution, either the UE reader or the CN may initiate location service to obtain more precise UE reader location information.

In steps 1, 2, and 4, the UE reader performs the AIoT operation as requested. In step 4, the UE reader may include the UE ID and user location information, such as serving cell, TAC information, and geo-coordinates if step 3 is performed.

In step 6, the CN can initiate location services to obtain the geo-coordinates of the UE reader. The location service can be performed using existing mechanisms.

Proposal 4, for topology 1, the TRP ID can be included in inventory/command response/report message.

Proposal 5, RAN3 discuss the following options for AIoT CN to obtain the TRP’s geo-coordinates.

- Option 1: the gNB provide TRP’s geo-coordinates in advance using non-AIoT operation-associated signalling

- Option 2: the gNB provide TRP’s geo-coordinates in the inventory/command response/report message.

Proposal 6, for topology 2, if RRC-based solution is used, the gNB provides the user location information of the UE reader in the inventory/command response message to CN.

Proposal 7, for topology 2, if NAS/UP solution is used, UE reader can provide its location information including cell, TAC, geo-coordinates in the NAS/UP inventory/command response message to CN.

Proposal 8, RAN3 agrees the TP for TR 38.386 in the Annex.

3	Conclusion

In this contribution, we discussed how to locating an AIoT device, and have the following observations and proposals.

## Improving AIoT operation

Observation 1, In scenarios where a target device is surrounded by multiple readers, the inclusion of reader IDs along with the respective measurements of the received backscatter signals can improve the further AIoT operations.

Proposal 1, the reader ID and the corresponding receiving signal strength from the target device can be included in the AIoT operation response message, e.g. inventory report, command response to improve the AIoT operation.

## Clarification on Reader ID

Proposal 2, RAN3 discuss the definition of the reader ID.

Proposal 3, for Topology 1, the reader ID can be gNB ID and TRP ID; for topology 2, the reader ID can be XXAP UE ID, GUTI, SUPI or IP address depending on the signalling solutions.

Providing location information

Proposal 4, for topology 1, the TRP ID can be included in inventory/command response/report message.

Proposal 5, RAN3 discuss the following options for AIoT CN to obtain the TRP’s geo-coordinates.

- Option 1: the gNB provide TRP’s geo-coordinates in advance using non-AIoT operation-associated signalling

- Option 2: the gNB provide TRP’s geo-coordinates in the inventory/command response/report message.

Proposal 6, for topology 2, if RRC-based solution is used, the gNB provides the user location information of the UE reader in the inventory/command response message to CN.

Proposal 7, for topology 2, if NAS/UP solution is used, UE reader can provide its location information including cell, TAC, geo-coordinates in the NAS/UP inventory/command response message to CN.

Proposal 8, RAN3 agrees the TP for TR 38.386 in the Annex.

4	Reference

[1] R3-243964 (TP for TR 38.769) Locating Ambient-IoT device

5	Annex (TP to R3-243964 for TR 38.769)

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

## 6.8	Locating Ambient IoT devices

Editor’s note: Proximity determination may be in a 6.8.x sub-clause, or another arrangement, depending on how the study proceeds.

### 6.8.1	General

AIoT device location information may be used for the following purposes:

(a) improving the AIoT operation itself, e.g. by sending a Command to one or more readers (e.g., the last reader(s)) associated to the device rather than sending it blindly.

(b) providing location information to the consumer of the AIoT service. Locating an Ambient IoT device at “reader ID granularity” is useful for both purposes.

Editor’s Note 1: How to know the “reader” location is FFS. Whether to use more than one “readers” within one AIoT RAS for location purposes is FFS.

Editor’s Note 2: Signalling details on how to provide the location information of the AIoT device to the AIoT CN needs further study.

### 6.8.2	Providing Location Information

For topology 1, providing location information can be interpreted as providing the reader’s location information.

Option 1, the AIoT RAN node provide the TRP information to AIoT CN including the geo-coordinates of each TRP, using non-AIoT operation-associated signalling in step 2. If there’s AIoT operation, the AIoT RAN node include the TRP ID in the AIoT operation response or report in step 5.

Figure 6.8.2-1: Providing reader TRP location information for T1 (option 1).

Option 2, If there’s AIoT operation, the AIoT RAN node include the TRP ID and the corresponding geo-coordinates of the TRP in the AIoT operation response or report in step 3.

Figure 6.8.2-2: Providing reader TRP location information for T1 (option 2).

For topology 2, the information may vary depending on the signaling solution used.

RRC-based signaling solution: The serving gNB of the UE reader provide the user location information of the UE reader, including cell and TA information, to the CN.

Figure 6.8.2-3: Providing reader location information for T2 (RRC-based solution).

For NAS/UP-based solution, UE reader may include the UE ID and user location information, such as serving cell, TAC information, and geo-coordinates if step 3 is performed, either the UE reader or the CN may initiate location service to obtain more precise UE reader location information.

Figure 6.8.2-4: Providing reader location information for T2 (NAS/UP-based solution).

<<<<<<<<<<<<<<<<<<<< Change Ends>>>>>>>>>>>>>>>>>>>>