3GPP TSG-RAN WG3 Meeting #125	R3-244194

# Maastricht, Netherlands, 19 – 23 August 2024

Agenda item:	14.2

Source:	Xiaomi

Title:	(TP to TS 38.413) Support of broadcast service

Document for:	Other

1	Introduction

RAN3#124:

WA: The mapped cell ID can be used to define MBS broadcast service in NTN.

- Other options can be further checked.

In this contribution, we’d like to further discuss the intended service area of a broadcast service according to the progresses in RAN3 and RAN2.

RAN2 progress in RAN2 125bis meeting

Agreements:

- For MBS broadcast service we don’t restrict the work to any satellite constellation type

- We prioritize working on a solution for MBS broadcast but we don’t preclude other broadcast services, namely ETWS

- We will cover at least the case where the indicated intended service area covers a portion of a NTN cell

- The intended service area can cover the area of more than one NTN cells (or portions thereof)

- Can discuss next time whether the broadcast transmission can be limited to the intended service area only (i.e. no transmission happens outside of the intended serive area)

- At least the following geographical area formats to model service area can be further considered (the signalling of other information than the geographical information can be considered):

- Circles (like for TN coverage)

- Geographical area information, e.g. via polygons, to better approximate the intended shape of service area

RAN2 progress in RAN2 126 meeting

Agreements:

- For MBS broadcast service, both EFC and EMC are supported.

- RAN2 will not define means for the NW to prevent the reception of the content of the service outside of the intended service area.

- MBS broadcast intended service area is provided via system information

- For MBS broadcast RAN2 considers the following possibilities for including the service area information: SIB20/ SIB21/ MBSBroadcastConfiguration. FFS for ETWS

- When intended service area (e.g., geographical area/TN coverage) is provided for MBS broadcast service, it needs to be associated with MBS session (FFS on the details)

2	Discussion

In last RAN3 meeting, there’s a working assumption “WA: The mapped cell ID can be used to define MBS broadcast service in NTN.”. As per the RAN2 agreements, the intended area for MBS broadcast can be defined by a geographical area or TN coverage. There is a low likelihood of broadcasting the mapped cell ID directly to the UE, since the UE may not be aware of how the mapped cell ID correlates to a specific geographical area. Therefore, the gNB is required to convert the mapped cell ID into a geographical area format suitable for broadcasting over the Uu interface.

If geographical area information or civic address information was provided by the AF as MBS service area, NEF/MBSF translates the MBS service area to Cell ID list or TAI list.

According to TS 23.247, the NEF/MBSF is capable of translating the geographical area information or civic address information to Cell ID list or TAI list, and the reverse process is also feasible, the gNB can also translates Cell ID list or TAI list to geographical area information.

Observation 1, the network entities “NEF/MBSF” can translates the geographical area information or civic address information to Cell ID list or TAI list, which proves that gNB as network entity can also translates Cell ID list or TAI list back to geographical area information.

Proposal 1, RAN3 agree the WA and rewording it correctly, mapped cell ID and TAI can be used to define the intended area for broadcast service in NTN.

In addition, according to the procedural text for MBS service areas outlined in TS 38.413 and TS 23.247, the NG-RAN node uses MBS service area information to determine the broadcasting of MBS service data. However, with the introduction of new requirements (i.e. broadcast the intended area), the existing procedural text does not meet these new requirements.

TS 38.413:

If the MBS Service Area IE is included in the BROADCAST SESSION SETUP REQUEST message, the NG-RAN node shall take it into account as specified in TS 23.247 [44].

3.	The AMF transfers the MBS Session Resource Setup Request message, which contains the N2 SM information in the received Namf_MBSBroadcast_ContextCreate Request to all NG-RANs which support MBS in the MBS service area. The AMF includes the MBS service area.

4.	NG-RAN creates a Broadcast MBS Session Context and stores the TMGI and the QoS Profile in the MBS Session Context. The LL SSM are optional parameters and only provided by MB-SMF to NG-RAN if N3mb multicast transport is configured to be used in the 5GC. If MBS FSA ID(s) were received, the NG-RAN may use those MBS FSA ID(s)s to determine cells/frequencies within the MBS service area to broadcast MBS session data based on OAM configuration about the MBS FSA IDs and related frequencies.

TS 23.247:

If the MBS Service Area IE is included in the BROADCAST SESSION SETUP REQUEST message, the NG-RAN node shall take it into account as specified in TS 23.247 [44]. In case of NTN, the NG-RAN node shall translate the Cell ID list or TAI list in MBS Service Area IE to the area information broadcast to UE as specified in TS 38.331 [18].

If mapped cell ID is used. New behaviour text needs to be added either in TS 38.413 or in TS 23.247. The following is an example for TS 38.413, if TS 23.247 needs to be updated, RAN3 may needs to send an LS to SA2.

Observation 2, if the existing MBS service area IE is used to indicate the intended area for MBS broadcast service in NTN, the existing procedural texts does not meet these new requirements (i.e. broadcast the intended area)

Proposal 2, RAN3 agree to either add new behaviour texts in TS 38.413 or send LS to SA2 and ask SA2 to update the stage2 description in TS 23.247.

In the Annex, a TP to TS 38.413 is to reflect the proposals in the contribution.

Proposal 3, RAN3 agree the TP to TS 38.413 in the Annex.

3	Conclusion

In this contribution, we discussed how to support the broadcast service in NTN, and have the following observations and proposals:

Observation 1, the network entities “NEF/MBSF” can translates the geographical area information or civic address information to Cell ID list or TAI list, which proves that gNB as network entity can also translates Cell ID list or TAI list back to geographical area information.

Proposal 1, RAN3 agree the WA and rewording it correctly, mapped cell ID and TAI can be used to define the intended area for broadcast service in NTN.

Observation 2, if the existing MBS service area IE is used to indicate the intended area for MBS broadcast service in NTN, the existing procedural texts does not meet these new requirements (i.e. broadcast the intended area)

Proposal 2, RAN3 agree to either add new behaviour texts in TS 38.413 or send LS to SA2 and ask SA2 to update the stage2 description in TS 23.247.

Proposal 3, RAN3 agree the TP to TS 38.413 in the Annex.

4	Reference

[1]	RP-240775, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Thales, CATT

5	Annex (TP to TS 38.413)

<<<<<<<<<<<<<<<<<<<< Change Start>>>>>>>>>>>>>>>>>>>>

## 8.17	Broadcast Session Management Procedures

### 8.17.1	Broadcast Session Setup

#### 8.17.1.1	General

The purpose of the Broadcast Session Setup procedure is to request the NG-RAN node to setup MBS session resources for a broadcast MBS session. The procedure uses non-UE associated signalling.

#### 8.17.1.2	Successful Operation

Figure 8.17.1.2-1: Broadcast Session Setup, successful operation.

The AMF initiates the procedure by sending a BROADCAST SESSION SETUP REQUEST message to the NG-RAN node. If the NG-RAN node accepts all the MBS QoS flows in the MBS session at least in one of its cells, the NG-RAN node responds with the BROADCAST SESSION SETUP RESPONSE message.

If the MBS Service Area IE is included in the BROADCAST SESSION SETUP REQUEST message, the NG-RAN node shall take it into account as specified in TS 23.247 [44]. In case of NTN, the NG-RAN node shall translate the Cell ID list or TAI list in MBS Service Area IE to the area information broadcast to UE as specified in TS 38.331 [18].

If the MBS Session FSA ID List IE is included in the BROADCAST SESSION SETUP REQUEST message, the NG-RAN node shall take it into account to determine cells/frequencies within the MBS service area to broadcast MBS session data as specified in TS 23.247 [44].

If the Supported UE Type List IE is included in the BROADCAST SESSION SETUP REQUEST message, the NG-RAN node shall, if supported, take it into account for configuring MBS session resources.

If the Associated Session ID IE is included in the BROADCAST SESSION SETUP REQUEST message the NG-RAN node shall, if supported, take this information into account to determine whether MBS session resource sharing is possible, as specified in TS 38.300 [8]. If the NG-RAN node decides to not establish shared NG-U resources towards the 5GC it shall include the Shared NG-U Not Established IE set to "not established" in the MBS Session Setup or Modification Response Transfer IE.

<<<<<<<<<<<<<<<<<<<< Next Change>>>>>>>>>>>>>>>>>>>>

8.17.2	Broadcast Session Modification

8.17.2.1	General

The purpose of the Broadcast Session Modification procedure is to request the NG-RAN node to update the MBS session resources or the area related to a previously established broadcast MBS session. The procedure uses non-UE associated signalling.

8.17.2.2	Successful Operation

Figure 8.17.2.2-1: Broadcast Session Modification, successful operation.

The AMF initiates the procedure by sending a BROADCAST SESSION MODIFICATION REQUEST message to the NG-RAN node.

If the MBS Service Area IE is included in the BROADCAST SESSION MODIFICATION REQUEST message, the NG-RAN node shall update the MBS service area and send the BROADCAST SESSION MODIFICATION RESPONSE message. In case of NTN, the NG-RAN node shall translate the Cell ID list or TAI list in MBS Service Area IE to the area information broadcast to UE as specified in TS 38.331 [18].

If the MBS Session Modification Request Transfer IE is included in the BROADCAST SESSION MODIFICATION REQUEST message, the NG-RAN node shall replace the previously provided information by the newly received one and update the MBS session resources and area as requested and send the BROADCAST SESSION MODIFICATION RESPONSE message.

If the Supported UE Type List IE is included in the BROADCAST SESSION MODIFICATION REQUEST message, the NG-RAN node shall, if supported, take it into account for configuring MBS session resources.

<<<<<<<<<<<<<<<<<<<< Change ends>>>>>>>>>>>>>>>>>>>>