3GPP TSG-RAN WG3 Meeting #125	R3-244195

# Maastricht, Netherlands, 19 – 23 August 2024

Agenda item:	14.3

Source:	Xiaomi

Title:	(TPs to TS 38.300) Support of Regenerative Payload

Document for:	Other

1	Introduction

There is no consensus to discuss new NTN architecture now; wait for an LS from SA2 on this particular issue.

Continue to discuss the NG interface management solution based on requirement inputs from operators

In this contribution, we’d like to further discuss the Support of Regenerative payload based on RAN3’s below common understanding and SA2’s study conclusion in [1].

The following topics are discussed in this paper.

- NG/Xn connection handling.

- Inactive UE support

- Mapped cell ID

2	Discussion

### NG/Xn connection handling

The conclusion for supporting gNB/eNB on-board the satellite is based on the following principles:

-	When the eNB/gNB leaves the service area of an AMF/MME (e.g. when setting over the horizon) any new procedures or updates to existing procedures to handle the N2/S1 connection will be defined by RAN WG3. SA WG2 will align its specifications once this has been determined by RAN WG3.

-	gNB /eNB IP address change due to feeder link switch can be supported using existing N2 and S1 procedures. There are no normative impacts to SA WG2 specifications due to this. SA WG2 will align its specifications if new solution is developed by RAN WG3.

-	RAN transitioning between different NTN GWs, TAIs and potentially between MMEs/AMFs can be supported by using existing procedures, e.g. the eNB/gNB releasing UEs to (E)CM_IDLE before changing NTN gateway, or by using existing load balancing, trigger handover when it realizes it is going to leave a tracking area due to its own movement, etc.

-	AMF/MME can treat the Mapped Cell IDs as for Rel-17.

-	Knowledge of when regenerative payload type could be used to assist with CN PDB determination and policy decisions. Introducing new RAT types is not required and it is assumed deployments can determine the usage of regenerative payload type if needed.

According to SA2’s below conclusion, SA2 levees the NG connection handling to RAN3.

Regarding how to handle the NG or Xn connection between the gNB onboard and AMF/gNB on ground due to feeder link switch or feeder link failure, we can first discuss NG connection, as it’s related UE experience, and also related to the first question in SA2’s LS and SA2’s TR as above, there are four options had been discussed in the contributions in last RAN3 meeting.

- option 1, NG connection setup and removal

- option 2, NG suspends and resumes

- option 3, NG connection setup and release by implementation

- option 4, NG Reset

In case of a feeder link switch, we recommend the following approach: before releasing or suspending the NG connection, ensure that UEs associated with this NG connection are migrated to other gNBs or AMF. This migration can be achieved through UE handover procedures.

Both the gNB onboard the satellite and the AMF on the ground can predict feeder link switches based on preconfigured information about the satellite and gateway (GW). If the gNB onboard detects an impending feeder link switch, it should initiate UE handover procedures for RRC_CONNECTED UEs. Once all UEs are successfully migrated, the NG connection can be released or suspended. In this case, an indication signaling from the gNB to the AMF is necessary (option 3 can be excluded).

If all UEs are already migrated, option 4 (NG reset) may not be needed. NG reset is typically used to release all UE associations, but if there are no active UE associations, there’s no need for release (option 4 can be excluded).

Regardless of whether option 1 or option 2 is chosen, the main purpose is to inform the AMF that the NG connection will not be used due to the feeder link switch. The difference lies in whether the AMF or gNB needs to store connection information for future reconnection. Storing connection information is beneficial, especially if a store-and-forward feature will be introduced later. While some companies express concerns about limited storage space on satellites, we believe that the cost of storing connection information is minimal, and some satellites may have ample storage capacity. Therefore, we think both option 1 and option 2 can be supported, allowing the gNB on the satellite to make informed decisions. For example, suspending connections it deems important and frequently used while releasing less frequently used connections

In case of feeder link failure, direct communication between the gNB and AMF may not be possible, it seems that option 3 can be considered in this case. If gNB identified the feeder link is failure, it can release all the UE context and NG connections related to the AMFs that communicated via this feeder link.

Observation 1, in case of feeder link switch, our preference is for the onboard gNB to migrate all relevant connected UEs to other gNBs or AMFs. This ensures seamless UE experiences. After all UEs are successfully migrated (i.e. handover), the NG connection can be either suspended or released.

Observation 2, if all the UEs are migrated before feeder link switch, NG reset procedure seems not needed for NG connection handling, as it is used to release the UE associations over NG connection.

Proposal 1, the onboard gNB should inform the corresponding AMF(s) that the connection can be released or suspended due to the feeder link switch once all relevant UEs have moved to other gNBs or AMFs.

Observation 3, it’s beneficial and flexible for gNBs on satellites with ample storage capacity to decide whether to store connection information for future use.

Proposal 2, support both NG connection release and NG connection suspending/resuming, allowing the gNB on the satellite to make informed decisions on whether to release or suspend a NG connection.

Proposal 3, in case of feeder link failure, where the gNB and AMF rely on this feeder link, they can release UE contexts or connection information by implementation.

We believe that it is feasible for the gNB onboard a satellite to establish an Xn connection with the gNB on the ground through the feeder link. This aspect of the Xn connection via the feeder link appears to be omitted in the figure presented in the BL CR. To provide a more comprehensive representation of the scenario, we propose to add the Xn connection via the feeder link to the figure in the BL CR.

Xn connection via feeder link has the same issue as NG, it may be changed due to the movement of the satellite. In existing mechanism, Xn Setup, Xn Removal procedure are specified, as Xn interface is not mandatory for UE connection and experience, there’s no need to suspend the connection, the existing Xn setup and removal procedure can be re-used for Xn connection handling.

Proposal 4, add Xn connection via feeder link between onboard gNB and on-ground gNB in the figure in BL CR for TS 38.300.

Proposal 5, the Xn Setup and Removal procedure can be re-used to setup/release Xn connection in case of regenerative payload.

### Inactive UE support

According to previous discussion and existing specifications, RRC_INACTIVE state is supported in transparent payload, and the legacy principles are applied.

16.14.3.1	Mobility in RRC_IDLE and RRC_INACTIVE

The same principles as described in 9.2.1 apply to mobility in RRC_IDLE for NTN and the same principles as described in 9.2.2 apply to mobility in RRC_INACTIVE for NTN unless hereunder specified.

TS 38.300:

RRC Inactive State Considerations for Regenerative Payloads:

- GEO Satellites: For gNBs located on GEO satellites, the existing principles can be directly applied.

- Non-GEO Satellites: In the case of non-GEO satellites, further discussion is required to determine how to define the RNA for regenerative payloads. This is due to the continuous movement of the cell served by the satellite.

Definition of RNA in NTN

RAN-NotificationAreaInfo ::=        CHOICE {

cellList                            PLMN-RAN-AreaCellList,

ran-AreaConfigList                  PLMN-RAN-AreaConfigList,

...

}

PLMN-RAN-AreaCellList ::=           SEQUENCE (SIZE (1.. maxPLMNIdentities)) OF PLMN-RAN-AreaCell

PLMN-RAN-AreaCell ::=               SEQUENCE {

plmn-Identity                       PLMN-Identity                                                       OPTIONAL,   -- Need S

ran-AreaCells                       SEQUENCE (SIZE (1..32)) OF  CellIdentity

}

PLMN-RAN-AreaConfigList ::=         SEQUENCE (SIZE (1..maxPLMNIdentities)) OF PLMN-RAN-AreaConfig

PLMN-RAN-AreaConfig ::=             SEQUENCE {

plmn-Identity                       PLMN-Identity                                                       OPTIONAL,   -- Need S

ran-Area                            SEQUENCE (SIZE (1..16)) OF  RAN-AreaConfig

}

RAN-AreaConfig ::=                  SEQUENCE {

trackingAreaCode                    TrackingAreaCode,

ran-AreaCodeList                    SEQUENCE (SIZE (1..32)) OF  RAN-AreaCode                            OPTIONAL    -- Need R

}

Similar as TAC in NTN, we propose that the RNA in NTN also can be defined as earth-fixed to minimize the frequency of RNA updates. Below is the IE content in RNA, which can be either list of cells or list of RAN areas, and the RAN areas contains a list of TACs. Given that the cell coverage in NTN may be larger the area of a single TAC, and considering the dynamic nature of cell over the Uu interface for non-GEO satellites, it is recommended that the RNA in NTN be defined as a list of TACs. This approach ensures that the RNA remains consistent and earth-fixed, despite the movement of the satellite.

Proposal 6, RAN3 agrees to support inactive UE in NTN regenerative payload.

Proposal 7, RAN3 agrees that RNA in NTN are earth fixed.

Proposal 8, RAN3 agrees the onboard gNB broadcast RNA only including the ran-AreaConfigList (i.e. list of TACs)

Inactive UE mobility

When a UE transitions to the RRC_INACTIVE state and the anchor gNB is situated on a non-GEO satellite, if UE resumes, the existing retrieve UE context procedure can be re-used, it is crucial to note that the existing retrieval procedure is contingent upon the availability of an inter-satellite link between the anchor gNB and the receiving gNB. As shown the below figure, at T1, orange satellite sends UE to RRC Inactive state and keeps UE context, when UE resumes, the orange satellite that have UE context may move to other side of the earth, the blue satellite may receive UE’s resume request but it may not able to communicate with the orange satellite.  In light of the potential absence of an inter-satellite link, it is recommended that the RAN3 to discuss how to support the inactive UE mobility issue in case of non-GEO coverage.

Proposal 9, in case of non-GEO, the inter-satellite link may not be available, RAN3 needs to discuss how to support inactive UE mobility in case of non-GEO coverage.

### Issue 3, mapped cell ID

In Non-GEO Satellites, due to their orbital movement, their satellite beams cover different portions of that area. Two scenarios for associating logical network identifiers with physical satellite beams had been studied in TR38.821 [2]:

Stationary Identifiers on Ground: Continuously reconfigure the association between physical satellite beams and logical cells so that the same gNB ID, cell ID, and TAC are always linked to the same geographical area.

Moving Identifiers on Ground: Fix the association between physical satellite beams and logical cells, allowing the gNB ID, cell ID, and TAC to follow the satellite beam(s) and “sweep” across the coverage area.

In R17, for transparent payload, mapped cell ID is introduced to solve the issue. According to TS 38.300, the mapping between Mapped Cell IDs and geographical areas is configured in the RAN and Core Network, and the mapped cell ID is used in the following cases:

-  The Cell Identity indicated by the gNB to the Core Network as part of the User Location Information;

- The Cell Identity used for Paging Optimization in NG interface;

- The Cell Identity used for Area of Interest;

- The Cell Identity used for PWS.

In the last RAN3 meeting, most companies proposed reusing the mapped cell ID for regenerative payload. Our concern regarding reusing the mapped cell ID is that there is no clear statement in the specification about whether the mapped cell ID follows the same planning as the legacy CGI. For instance, in the case of the gNB on the non-GEO satellite, the gNB ID represents a moving gNB. This implies that the cell ID, which includes this gNB ID, is also moving. To address this concern, we recommend that RAN3 clarify in stage 2 that the mapped cell ID is independent of the gNB ID of the satellite

Observation 4: It is not clear whether the mapped cell ID follows the same planning as the legacy CGI, which includes the gNB ID of a gNB that can be moving if it’s deployed on the satellite.

Proposal 10: RAN3 should add stage 2 clarification about the mapped cell ID to emphasize that the mapped cell ID is independent of the gNB ID on the satellite.

3	Conclusion

Based on the discussion, we have the following observations and proposals.

### Issue 1, NG/Xn connection handling

Observation 1, in case of feeder link switch, our preference is for the onboard gNB to migrate all relevant connected UEs to other gNBs or AMFs. This ensures seamless UE experiences. After all UEs are successfully migrated (i.e. handover), the NG connection can be either suspended or released.

Observation 2, if all the UEs are migrated before feeder link switch, NG reset procedure seems not needed for NG connection handling, as it is used to release the UE associations over NG connection.

Proposal 1, the onboard gNB should inform the corresponding AMF(s) that the connection can be released or suspended due to the feeder link switch once all relevant UEs have moved to other gNBs or AMFs.

Observation 3, it’s beneficial and flexible for gNBs on satellites with ample storage capacity to decide whether to store connection information for future use.

Proposal 2, support both NG connection release and NG connection suspending/resuming, allowing the gNB on the satellite to make informed decisions on whether to release or suspend a NG connection.

Proposal 3, in case of feeder link failure, where the gNB and AMF rely on this feeder link, they can release UE contexts or connection information by implementation.

Proposal 4, the Xn Setup and Removal procedure can be re-used to setup/release Xn connection in case of regenerative payload.

### Issue 2, inactive UE support

Proposal 6, RAN3 agrees to support inactive UE in NTN regenerative payload.

Proposal 7, RAN3 agrees that RNA in NTN are earth fixed.

Proposal 8, RAN3 agrees the onboard gNB broadcast RNA only including the ran-AreaConfigList (i.e. list of TACs)

Proposal 9, in case of non-GEO, the inter-satellite link may not be available, RAN3 needs to discuss how to support inactive UE mobility in case of non-GEO coverage.

### Issue 3, mapped cell ID (Q3 in SA2’s LS)

Observation 4: It is not clear whether the mapped cell ID follows the same planning as the legacy CGI, which includes the gNB ID of a gNB that can be moving if it’s deployed on the satellite.

Proposal 10: RAN3 should add stage 2 clarification about the mapped cell ID to emphasize that the mapped cell ID is independent of the gNB ID on the satellite.

4	Reference

[1] 3GPP TR 23.700-29 V19.0.0, Study on integration of satellite components in the 5G architecture

[2] 3GPP TR 38.821 V16.1.0, Solutions for NR to support non-terrestrial networks (NTN)

5	Annex A (TP to BL CR TS 38.300)

----------------------------------------------------Start of change----------------------------------------------------

16.14	Non-Terrestrial Networks

16.14.1	Overview

Figures 16.14.1-1 and 16.14.1-X below illustrate examples of a Non-Terrestrial Network (NTN) providing non-terrestrial NR access to the UE by means of an NTN payload and an NTN Gateway, depicting a service link between the NTN payload and a UE, and a feeder link between the NTN Gateway and the NTN payload. In Figure 16.14.1-1 the payload is transparent; in Figure 16.14.1-X the payload is regenerative and hosts a gNB.

Figure 16.14.1-1: Overall illustration of an NTN with transparent payload

Figure 16.14.1-X: Overall illustration of an NTN with regenerative payload hosting a gNB

NOTE 1:	Figures 16.14.1-1 and 16.14.1-X illustrate an NTN; RAN4 aspects are out of scope.

The NTN transparent payload transparently forwards the radio protocol received from the UE (via the service link) to the NTN Gateway (via the feeder link) and vice-versa. The regenerative payload terminates the Uu interface between the UE (via the service link), the NG interface toward the 5GC (via the feeder link), and the Xn interface with gNB (via the feeder link). The following connectivity is supported by the NTN payload:

Editor’s Note: The above section needs further checking.

-	An NTN gateway may serve multiple transparent or regenerative NTN payloads;

-	A transparent or regenerative NTN payload may be served by multiple NTN gateways;

-	A regenerative NTN payload may terminate one or more inter-satellite links toward other regenerative payloads;

-	The feeder link is a transport link; it may transport the NG interface between the 5GC and the gNB hosted by the regenerative payload;

-	The inter-satellite link is a transport link; it may transport the Xn interface between the gNBs hosted by the two regenerative payloads.

NOTE 2:	In this release, the transparent NTN-payload may change the carrier frequency, before re-transmitting it on the service link, and vice versa (respectively on the feeder link).

For NTN, the following applies in addition to Network Identities as described in clause 8.2:

-	A Tracking Area corresponds to a fixed geographical area. Any respective mapping is configured in the RAN;

-	A Mapped Cell ID as specified in clause 16.14.5.

Three types of service links are supported:

-	Earth-fixed: provisioned by beam(s) continuously covering the same geographical areas all the time (e.g., the case of GSO satellites);

-	Quasi-Earth-fixed: provisioned by beam(s) covering one geographic area for a limited period and a different geographic area during another period (e.g., the case of NGSO satellites generating steerable beams);

-	Earth-moving: provisioned by beam(s) whose coverage area slides over the Earth surface (e.g., the case of NGSO satellites generating fixed or non-steerable beams).

With NGSO satellites, the gNB can provide either quasi-Earth-fixed service link or Earth-moving service link, while gNB operating with GSO satellite can provide Earth fixed service link or quasi-Earth-fixed service link.

In this release, the UE supporting NTN is GNSS-capable.

In NTN, the distance refers to Euclidean distance.

----------------------------------------------------Next change----------------------------------------------------

16.14.3	Mobility and State transition

16.14.3.1	Mobility in RRC_IDLE and RRC_INACTIVE

The same principles as described in 9.2.1 apply to mobility in RRC_IDLE for NTN and the same principles as described in 9.2.2 apply to mobility in RRC_INACTIVE for NTN unless hereunder specified.

The network may broadcast multiple Tracking Area Codes (TACs) per PLMN in an NR NTN cell. A TAC change in the System Information is under network control, i.e. it may not be exactly synchronised with real-time illumination of beams on ground.

The network may broadcast a list of RAN area codes or RA code(s) as RAN area which includes a list of TAC(s).

For the NTN-TN mobility, the network may broadcast cell information on NR TN and EUTRA TN coverage areas in SIB25. This is supported for Earth-Fixed, Quasi-Earth-fixed and Earth-Moving cells. The coverage information consists in a list of geographical TN areas, with associated frequency information also indicated. UE can skip TN measurement based on the broadcast TN coverage information.

The UE can determine the network type (terrestrial or non-terrestrial) implicitly by the existence of cellBarredNTN in SIB1.

The NTN ephemeris is provided in SIB19. In an NTN cell, it includes serving cell's NTN payload ephemeris and optionally neighbouring cell's NTN payload ephemeris.

----------------------------------------------------Next change----------------------------------------------------

16.14.5	NG-RAN signalling

The Cell Identity, as defined in TS 38.413 [26] and TS 38.423 [50], used in following cases corresponds to a Mapped Cell ID, irrespective of the orbit of the NTN payload or the types of service links supported:

-	The Cell Identity indicated by the gNB to the Core Network as part of the User Location Information;

-	The Cell Identity used for Paging Optimization in NG interface;

-	The Cell Identity used for Area of Interest;

-	The Cell Identity used for PWS.

When the Mapped Cell ID is used, the Cell Identity is independent of the gNB Identity of the gNB on the satellite.

The Cell Identity included within the target identification of the handover messages allows identifying the correct target cell. The cell identity used in the NG and Xn handover messages, Xn Setup and Xn NG-RAN Node Configuration Update procedures is expected to be Uu Cell ID.

The Cell Identities used in the RAN Paging Area during Xn RAN paging allow the identification of the correct target cells for RAN paging.

NOTE 1:	The Cell Identity used for RAN Paging is assumed to typically represent a Uu Cell ID.

The mapping between Mapped Cell IDs and geographical areas is configured in the RAN and Core Network.

NOTE 2:	A specific geographical location may be mapped to multiple Mapped Cell ID(s), and such Mapped Cell IDs may be configured to indicate differerent geographical areas (e.g. overlapping and/or with different dimensions).

The gNB is responsible for constructing the Mapped Cell ID based on the UE location information received from the UE, if available. The mapping may be pre-configured (e.g., up to operator's policy) or up to implementation.

NOTE 3:	As described in TS 23.501 [3], the User Location Information may enable the AMF to determine whether the UE is allowed to operate at its present location. Special Mapped Cell IDs or TACs may be used to indicate areas outside the serving PLMN's country.

The gNB reports the broadcasted TAC(s) of the selected PLMN to the AMF as part of ULI. In case the gNB knows the UE's location information, the gNB may determine the TAI the UE is currently located in and provide that TAI to the AMF as part of ULI.

----------------------------------------------------End of change----------------------------------------------------