3GPP TSG-RAN WG3 Meeting #125	R3-244196

# Maastricht, Netherlands, 19 – 23 August 2024

Agenda item:	15.2

Source:	Xiaomi

Title:	Support of Store and Forward in IoT NTN

Document for:	Discussion

1	Introduction

Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore:

- Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2, RAN4]

-At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3]

Note: Strive to minimise UE impact.

Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required.

In this contribution, we’d like to discuss the Support of Store & Forward (S&F) satellite operation with full eNB as regenerative payload based on the following WID [1] and conclusion from SA2 [2].

The following topics are discussed in this paper.

- RAN3 discussion scope (architecture)

- eNB only onboard architecture

- onboard eNB and onboard CN in different satellites

- storage admission control

2	Discussion

### RAN3 discussion scope (architecture)

According to SA2’s study in [1], the following main architecture examples discussed in SA2 are shown in Figure 1, Figure 2 and Figure 3.

Figure 1 example arch for split-MME on board

Figure 2 example arch for full CN onboard

Figure 3 example arch for only eNB onboard

According to the SA2’s conclusion in 8.2 in [2], the architectures including a full CN onboard the satellite shown in figure 2 and Split MME onboard architecture shown in figure 1 are going to be supported in normative phase. Although eNB onboard solution shown in figure 3 has been discussed, there’s no conclusion in SA2 whether to support it or not, as we think it’s mainly to be decided in RAN3. Compare to the CN onboard architecture, eNB only onboard requires less payload on the satellite, offering more flexibility for operator deployments.

For the eNB-only onboard architecture, it may have big impacts on supporting attach procedures. If this architecture is supported, the attach procedure may not needs to be considered, meaning it can be assumed that the UE has already successfully attached by other means, such as via TN or eNB+CN onboard NTN.

Observation 1, SA2 concludes that full CN onboard the satellite and Split MME onboard architecture are supported in R19.

Observation 2, SA2 discussed eNB only onboard architecture and solution but has no conclusions.

Observation 3, eNB only onboard architecture has main impacts on RAN3 and should be decided by RAN3.

Proposal 1, RAN3 needs to discuss whether to support eNB only onboard architecture for store and forward operation in R19.

Proposal 2, if RAN3 agree to support eNB only onboard architecture, to reduce the complexity and spec impacts, it can be assumed that the UE has already successfully attached, e.g. via TN, or eNB+CN onboard NTN, i.e. how to support attach procedure is not in scope of this architecture.

According to SA2’s conclusion, to support S&F in split MME or full CN onboard architecture, the main RAN impact is that the UE must be aware that a satellite supports S&F mode. For example, the eNB can broadcast support for S&F operation as part of System Information, which falls mainly within the scope of RAN2.

In split MME or full CN onboard architecture, one possible deployment option is that the onboard eNB and CN parts are deployed on different satellites, communicating via inter-satellite links as shown in Figure 4.

Figure 4 example arch for eNB and CN in different satellites

Proposal 3, RAN3 discuss whether to support the architecture that the onboard eNB and onboard CN are deployed in different satellites that is align with SA2’s conclusion.

### eNB only onboard architecture

According to TS 36.300 and TS 23.401, for MO/MT data transmission, the following procedures can be used:

- CIoT optimisations

- EDT

Using MO-EDT for CP CIoT as an example, in step 1, the onboard eNB can store the NAS PDU from the UE if there is no feeder link available, and then forward it in step 2 once the feeder link becomes available. If there is DL data in step 5, the eNB may not serve the UE as it moves away. To simplify the solution, for store and forward operations, if there is subsequent DL data after MO-data transmission, it can be treated as a MT data. In other words, for store and forward operations, one-shot MO data or MT data transmission can be supported. Any subsequent DL or UL data can be treated as new data.

Figure 5 MO-EDT for CP CIoT

With this principle, for the store and forward of one-shot MO-data and MT-data, the signalling flow can be as follows：

Figure 6 one-shot MO/MT data store and forward

The assumption is that the UE is already attached to the network, with its context stored in the on ground MME.

Steps 0-2: If the UE wants to send MO data and the network broadcasts support for store and forward, it reconnects with eNB1 on satellite 1, sets up an RRC connection, and includes the UL data (e.g., NAS) in the RRC message.

Step 3: If the feeder link is unavailable, eNB1 stores the received UL data and sends an RRC release to the UE, knowing it is unlikely to serve the UE until it can connect with the on-ground MME when the feeder link becomes available.

Step 4: When the feeder link is available, eNB1 forwards the stored data using the initial UE message.

Step 5: The MME recognizes this as store and forward data and releases the S1 connection.

Steps 6-7: If there is DL data, the MME treated it as MT data, and sends paging and DL data to eNB2, which may connect with the UE later. eNB2 stores the paging and DL data if the service link with the UE is unavailable.

Steps 8-10: When the service link becomes available, eNB2 performs paging and sends the DL data to the UE. Afterward, the RRC connection can be released.

For a MT data delivery, the step 6-10 can be performed.

Proposal 4, if eNB only onboard architecture is supported, one-shot MO and one-shot MT data transmission by using store and forward can be supported.

### onboard eNB and onboard CN in different satellites

if the onboard eNB and onboard CN deployed in different satellites, we believe that the SA2’s solution can be used and there will be no additional SA2 impacts. One consideration is that the eNB needs to be aware of whether the MME on the other satellite has an available feeder link and whether store and forward (S&F) operation is supported, so that it can decide whether to serve UE or whether to broadcast the store and forward operation to UE.

The following scenarios may exist (assuming the eNB and CN can communicate via inter-satellite link):

Case 1: The eNB has no feeder link, while the MME has a feeder link. The eNB can provide normal service to the UE.

Case 2: Both the eNB and CN have no feeder link, and both support S&F. The eNB can broadcast an S&F indication to the UE.

Case 3: The eNB has a feeder link and can connect to the ground CN, allowing the eNB to provide normal service to the UE.

It is evident that the eNB’s behaviour may vary depending on the feeder link status of both the eNB and the MME. Therefore, it would be beneficial for the eNB to be aware of the feeder link status of the CN on other satellites.

Proposal 5, if the scenario that onboard eNB and onboard CN are deployed in different satellites is supported, eNB needs to be aware of the feeder link status of CN on other satellites.

### Storage admission control

No matter which architectures are used, storage admission control should be considered due to the limited storage capacity of the satellite. For both MO and MT data, the eNB should perform storage admission control. If storage space is insufficient, the eNB can reject storing the requested MT or MO data.

Proposal 6, eNB needs to perform storage admission control, RAN3 can further discuss how to support storage admission control.

3	Conclusion

Based on the discussion, we have the following observations and proposals.

### RAN3 discussion scope (architecture)

Observation 1, SA2 concludes that full CN onboard the satellite and Split MME onboard architecture are supported in R19.

Observation 2, SA2 discussed eNB only onboard architecture and solution but has no conclusions.

Observation 3, eNB only onboard architecture has main impacts on RAN3 and should be decided by RAN3.

Proposal 1, RAN3 needs to discuss whether to support eNB only onboard architecture for store and forward operation in R19.

Proposal 2, if RAN3 agree to support eNB only onboard architecture, to reduce the complexity and spec impacts, it can be assumed that the UE has already successfully attached, e.g. via TN, or eNB+CN onboard NTN, i.e. how to support attach procedure is not in scope of this architecture.

Proposal 3, RAN3 discuss whether to support the architecture that the onboard eNB and onboard CN are deployed in different satellites that is align with SA2’s conclusion.

### eNB only onboard architecture

Proposal 4, if eNB only onboard architecture is supported, one-shot MO and one-shot MT data transmission by using store and forward can be supported.

### onboard eNB and onboard CN in different satellites

Proposal 5, if the scenario that onboard eNB and onboard CN are deployed in different satellites is supported, eNB needs to be aware of the feeder link status of CN on other satellites.

### Storage admission control

Proposal 6, eNB needs to perform storage admission control, RAN3 can further discuss how to support storage admission control.

4	Reference

[1] RP-241624, New WID: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3

[2] TR 23.700-29-J00, Study on integration of satellite components in the 5G architecture.