3GPP TSG-RAN WG3 #125	R3-244198

Maastricht, Netherlands, 19 – 23 Aug. 2024

Agenda Item:	13.2. Support for inter-CU LTM

Source:	NTT DOCOMO INC.

Title:	Discussion on inter-CU LTM non-DC case

Document for:	Discussion, agreement

Introduction

At the previous meeting, RAN3 reached the following agreements on LTM preparation [1].

Source gNB-CU initiates the handover preparation procedure for inter-gNB-CU LTM.

Early sync configuration (TCI state and RACH configuration) can be obtained during the LTM preparation phase through the handover request and handover request acknowledge messages.

FFS on the LTM modification procedures.

And RAN2 reached the following agreements [2].

Stage-2 signaling flows and procedures

- The preparation of inter-CU LTM configuration is initiated by the source gNB-CU.

- For each candidate cell, the preparation of lower layer configuration is initiated by the candidate gNB-CU, based on the LTM request from the source gNB-CU. RAN2 assumes the interaction between the candidate gNB-CU and candidate gNB-DU follows the same signaling procedure for intra-CU LTM.

- The source gNB-CU is responsible to collect the configurations and information of candidate cells from multiple candidate gNB-CUs and generates the common CSI resource configuration for L1 measurement on candidate cells.

- In order to support subsequent LTM, the source gNB-CU needs to inform the candidate gNB-CU(s) about the common CSI resource configuration and the collected information of candidate cells from multiple candidate gNB-CUs. The candidate gNB-CU(s) responds with the candidate configuration to the source gNB-CU accordingly (if needed).

Preparation:

RRC Configuration/structure

- The RRC signalling structure and modelling for Rel-18 LTM is taken as the baseline for inter-CU LTM.

LTM Candidate ID

- For inter-CU LTM, LTM candidate ID is unique across all the participating gNB-CUs.

Max number of LTM candidate IDs

- The maximum number of LTM candidate cell configuration is 8, regardless of whether these are intra-CU or inter-CU LTM candidate configurations.

And also, inter-gNB LTM without PDCP anchor change proposed by Qualcomm [3][4] was discussed in RAN2 and RAN3, but no agreement was reached.

Based on the above, this paper discusses the preparation phase of Inter-CU LTM and without PDCP anchor change.

Discussion

Inter-CU LTM without PDCP anchor change

QC proposed solution for LTM that avoids security key change and PDCP reestablishment in inter-CU LTM by switching the UE to the DU under the target gNB without changing the PDCP anchor in the C/U-plane. This method could maximize the benefit of low interruption of LTM in the following scenarios, for example.

- When a UE moves in an area near a single gNB and repeatedly performs intra-CU LTM, and sometimes ping-pong occurs between inter-gNB cells when crossing the boundary with neighbor gNB.

- When a single gNB covers a large area and a UE that moves out of the area to an neighbor gNB is expected to return to the original gNB within a certain period of time.

- When a UE is expected to move between the coverage boundaries of two gNBs

On the other hand, if intra-CU cell switch occurs frequently after moving to a neighboring gNB, Xn signaling may increase compared to normal inter-CU LTM. Therefore, both method of inter-CU LTM with and without PDCP anchor change should be supported in order to allow gNB to select either one based on situations.

Observation 1:	Inter-CU LTM without PDCP anchor change has significant advantages in terms of low interruption in certain scenarios.

Proposal 1:	As well as normal inter-CU LTM with security key change, inter-CU LTM without PDCP anchor change should also be supported.

LTM preparation

According to the RAN2 agreement of the previous meeting, in order to support the Xn signaling for the preparation phase similar as Rel18 intra-CU LTM F1AP, at least the following information needs to be sent from the source CU to the target CU in the Handover Request.

- Request for inter-CU LTM preparation

- Target candidate cell

In legacy L3 handover, the source CU determines the target cell and includes it in the Handover Request message. In inter-CU LTM source CU should also determine the target/candidate cell and include it in the Handover Request message. The source CU should also decide which beam of each target/candidate cell to include as a candidate. If the candidate CU decides candidate beams, since the candidate beams are included in the common CSI resource configuration, this method increases the round trip of Xn signaling in the preparation phase.

Proposal 2:	Target candidate cell ID should be included in Handover Request message.

- LTM Configuration ID of target candidate cell

The LTM configuration ID assigned to the target candidate cell can be determined by the source CU or the candidate CU. However, if the candidate CU assigns the LTM configuration ID, same as the case of target candidate cell decision, since it increases the round trip of Xn signaling as well as the target candidate cell decision.

Proposal 3:	Allocation of LTM configuration ID for the target candidate cell should be done by the source CU before sending Handover Request message to the target CU, and it should be included in Handover Request message.

- LTM CSI Resource Configuration

The source CU can generate the LTM CSI Resource Configuration based on the cell/beam information associated with each gNB obtained in the Xn Setup procedure or the NG-RAN node configuration update procedure. The candidate CU generates the lower layer configuration based on this information.

Observation 2:	Source CU can generate LTM CSI Resource Configuration based on the information received in Xn Setup procedure or NG-RAN node Configuration Update procedure.

Proposal 4:	LTM CSI Resource Configuration could be generated by source CU before sending Handover Request message to the target CU, and it should be included in Handover Request message.

- Request for RACH configuration for early sync

We have already agreed in a previous meeting to include this request in the Handover Request message. On the other hand, whether this request is per CU or per DU (or per cell) needs further discussion.

Proposal 5:	RAN3 to discuss whether the request for RACH configuration for early sync is per candidate CU or per candidate DU (or candidate cell).

- Request for UE based TA measurement

Since RAN2 is currently discussing whether UE based TA measurement can be used in inter-CU LTM, RAN3 should wait for RAN2's conclusion.

Proposal 6:	RAN3 should wait for RAN2 decision on a use of UE based TA measurement in inter-CU LTM.

- LTM Configuration ID Mapping List

As well as Rel18 intra-CU LTM, LTM Configuration ID Mapping List should be sent to all candidate CUs in order to support subsequent LTM.

Proposal 7:	LTM Configuration ID Mapping List should be sent to the all candidate CU(s) via Handover Request message.

- Candidate gNB-DU ID List

Similar to Rel18 intra-CU LTM, when the candidate DUs allocate RACH resources, the CU should send a Candidate gNB-DU ID List in order to allocate the same RACH resource to the same source DU to save resources. However, this point needs further discussion since the current specification does not guarantee that gNB-DU IDs are unique among neighboring gNBs.

Proposal 8:	Candidate gNB-DU ID List should be sent to the all candidate CU(s) via Handover Request message. How to ensure uniqueness of gNB-DU IDs between different gNBs can be further discussed.

- Request for generation of reference configuration

In RAN2, there are two options to configure reference configuration: 1) only one reference configuration in one LTM Config, and 2) one reference configuration for each candidate CU. RAN3 impact depends on which option is supported, however, since RAN2 is currently under discussion, RAN3 should wait for the RAN2 conclusion.

Proposal 9:	How to generate and configure reference configuration should be discussed in RAN3 after RAN2 conclusion.

If some candidate cells/beams are rejected by the candidate CU/DU, the LTM configuration ID or the LTM CSI Resource Configuration may be re-generated by the source CU, and it requires another Xn signaling round trip. Besides, for a subsequent LTM, all candidate CUs need to know about the information related to the LTM configuration generated by all other candidate CUs, and it requires another Xn signaling round trip after the initial Handover Request procedure. If the source CU modifies the LTM configuration, candidate CUs need to retrieve the updated information (e.g. lower layer configuration) from the candidate CU, so the class 1 procedure should be used. Notification of configuration information for subsequent LTM can work with either the class 1 or class 2 procedure. To reduce complexity, we think we should support the class 1 procedure for updating the LTM configuration after the Initial Handover request for both update and notification of LTM configuration.

Proposal 10:	LTM modification procedure should be class 1 procedure. Whether reuse legacy or introduce new one can be further discussed.

Security key handling

RAN2 sent an LS [5] to SA3 asking about the feasibility of the solutions for security issue in the inter-CU LTM non-DC case. In this LS, RAN2 proposed Option1~4 solutions, however, it is still under discussion in SA3 [6].

The RAN3 impact analysis would include the following perspectives: Xn signaling and NG signaling for security related information update after LTM execution is basically the same as for legacy HO. However, if Option 1 (MAC CE based solution) is to be supported, F1 enhancement is required to transfer security related information handled by the CU to be sent via MAC CE from the DU to the UE.

Observation 3:	Security key handling in Inter-CU LTM is still under discussion in other WGs.

Observation 4:	If MAC CE based solution is supported by RAN2, there will be F1 impacts since the DU need to obtain the security related information handled by the CU.

Conclusions and proposals

Our proposals are summarized below.

Observation 1:	Inter-CU LTM without PDCP anchor change has significant advantages in terms of low interruption in certain scenarios.

Proposal 1:	As well as normal inter-CU LTM with security key change, inter-CU LTM without PDCP anchor change should also be supported.

Proposal 2:	Target candidate cell ID should be included in Handover Request message.

Proposal 3:	Allocation of LTM configuration ID for the target candidate cell should be done by the source CU before sending Handover Request message to the target CU, and it should be included in Handover Request message.

Observation 2:	Source CU can generate LTM CSI Resource Configuration based on the information received in Xn Setup procedure or NG-RAN node Configuration Update procedure.

Proposal 4:	LTM CSI Resource Configuration could be generated by source CU before sending Handover Request message to the target CU, and it should be included in Handover Request message.

Proposal 5:	RAN3 to discuss whether the request for RACH configuration for early sync is per candidate CU or per candidate DU (or candidate cell).

Proposal 6:	RAN3 should wait for RAN2 decision on a use of UE based TA measurement in inter-CU LTM.

Proposal 7:	LTM Configuration ID Mapping List should be sent to the all candidate CU(s) via Handover Request message.

Proposal 8:	Candidate gNB-DU ID List should be sent to the all candidate CU(s) via Handover Request message. How to ensure uniqueness of gNB-DU IDs between different gNBs can be further discussed.

Proposal 9:	How to generate and configure reference configuration should be discussed in RAN3 after RAN2 conclusion.

Proposal 10:	LTM modification procedure should be class 1 procedure. Whether reuse legacy or introduce new one can be further discussed.

Observation 3:	Security key handling in Inter-CU LTM is still under discussion in other WGs.

Observation 4:	If MAC CE based solution is supported by RAN2, there will be F1 impacts since the DU need to obtain the security related information handled by the CU.

References

- RAN3#124 Chair Notes

- RAN2#126 Chair Notes

- R2-2405469, “Inter-gNB LTM and moving PDCP anchor”

- R3-243522, “Inter-gNB LTM and keep PDCP anchor”

- R3-243114, “LS on security handling for inter-CU LTM in non-DC cases”

- R3-244022, “Reply LS to RAN2 on security handling for inter-CU LTM in non-DC cases”