3GPP TSG-RAN WG3 #125	R3-244200

Maastricht, Netherlands, 19 – 23 Aug. 2024

Agenda Item:	14.3. Support of Regenerative payload

Source:	NTT DOCOMO INC.

Title:	Discussion on Xn impact for NR NTN regenerative payload

Document for:	Discussion, agreement

Introduction

At the previous meeting, a stage2 CR was agreed upon and captured the following [1].

- An NTN gateway may serve multiple transparent or regenerative NTN payloads;

- A transparent or regenerative NTN payload may be served by multiple NTN gateways;

- A regenerative NTN payload may terminate one or more inter-satellite links toward other regenerative payloads;

- The feeder link is a transport link; it may transport the NG interface between the 5GC and the gNB hosted by the regenerative payload;

- The inter-satellite link is a transport link; it may transport the Xn interface between the gNBs hosted by the two regenerative payloads.

On the other hand, if only transport of Xn IF over ISL is supported, frequent switch over of Xn IF may occur due to satellite movement, which may cause legacy procedure failures and signaling overhead of Xn IF establishment. This paper discusses solutions to this problem.

Discussion

In a past RAN2 meeting, contributions [2][3] were submitted pointing out that regenerative payload satellite causes ISL switch over, which may result in failure of UE context acquisition for RRC reestablishment. For example, if an ISL switch over occurs during the Xn HO procedure, however, this case could be handled by HO failure procedure and triggering NG HO procedure or by appropriate HO timing control based on implementation. Although, frequent Xn IF presence/absence changes may lead to gNB implementation complexity and signaling overhead due to procedure failure and frequent Xn Setup procedure execution.

Observation:	Due to Xn/ISL switch over, Xn handover procedure and RRC Reestablishment procedure would be fail, or, frequent Xn (re-)setup would be signaling burden.

It is difficult to avoid ISL/feeder link switch over itself because this problem is caused by the satellite orbit. On the other hand, by ensuring that Xn IFs are always established, it is possible to continue to support Xn procedures such as the Retrieve UE Context procedure for RRC reestablishment. One solution is to transport Xn IF over the feeder link and the NTN GW. As shown in the figure, Xn IF can be established regardless of the presence of ISL by transporting Xn IF between satellite#1 - NTN GW#1 - NTN GW#2 - satellite#2. If an ISL switchover occurs as the satellite moves, the Xn procedure can be performed via the feeder link and NTN GW transportation. Even when a feeder link switchover occurs and the Xn IF is interrupted, the feeder link is always reconnected, so the Xn IF can always be reconnected as well.

Fig. Xn IF transport over feeder links and GWs.

On the other hand, there are two possible problems with supporting Xn IF over feeder link: 1) frequent Xn setups due to feeder link switch over, and 2) long round trips due to two passes over the feeder link. The problem 1) is that in case that the frequency of ISL switch over is higher than that of feeder link switch over, when a group of satellites is on the same orbit, Xn IF over feeder link rather increases signaling overhead. In this case, this problem can be avoided by using only Xn IF transport with ISL, which we already support. The issue 2) needs to be discussed because long round trips may affect the legacy mechanism in Xn-C and Xn-U, respectively.

Proposal 1:	Discuss how to avoid the impact on legacy Xn procedures due to frequent Xn switch over.

Proposal 2:	Support Xn IF transportation over feeder link.

Proposal 3:	RAN3 to discuss the details of support of Xn IF transportation over feeder link (e.g. long round trip).

Conclusions and proposals

Our proposals are summarized below.

Observation:	Due to Xn/ISL switch over, Xn handover procedure and RRC Reestablishment procedure would be fail, or, frequent Xn (re-) setup would be signaling burden.

Proposal 1:	Discuss how to avoid the impact on legacy Xn procedures due to frequent Xn switch over.

Proposal 2:	Support Xn IF transportation over feeder link.

Proposal 3:	RAN3 to discuss the details of support of Xn IF transportation over feeder link (e.g. long round trip).

References

- R3-243949, “Support for Regenerative Payload in NR NTN”

- R2-2404799, “Considerations on regenerative payload in NTN”

- R2-2405156, “Regenerative payload for NR NTN”