3GPP TSG-RAN WG3 Meeting #125	R3-244202

Maastricht, Netherlands, 19th Aug – 23rd Aug 2024

Agenda Item:		12.3

Source:		Baicells

Title:		(TP to TR 38.799) Proposals on NR Femto Architecture

Document for:		Discussion and decision

- Introduction

The latest version of TR 38.799 is 1.10 in [1]. It had defined 4 Architecture Options, and Option 1 and Option 2 thereof are with the following scope:

5.2.1.1 Option 1 As shown in Figure 5.2.1.1-1, in this option the NR Femto node connects to the 5GC directly as a gNB by means of the NG interface. Figure 5.2.1.1-1: Option 1 for NR Femto Architecture NOTE: The SeGW is out of RAN3 scope. 5.2.1.2 Option 2 Figure 5.2.1.2-1 shows a logical architecture for the NR Femto that has a set of NG interfaces to connect the NR Femto node to the 5GC. Figure 5.2.1.2-1: Option 2 for NR Femto Architecture NOTE: The SeGW and the NR Femto Management System are out of RAN3 scope. The NG-RAN architecture may deploy an NR Femto Gateway (NR Femto GW) to allow the NG interface between the NR Femto node and the 5GC to support a large number of NR Femto nodes in a scalable manner. The NR Femto GW serves as a concentrator for the C-Plane, specifically the NG-C interface. The NG interface is defined as the interface: - Between the NR Femto GW and the 5GC; - Between the NR Femto node and the NR Femto GW; - Between the NR Femto node and the 5GC; The NR Femto GW appears to the AMF as a gNB. The NR Femto GW appears to the NR Femto node as an AMF. The NG interface between the NR Femto node and the 5GC is the same regardless of whether the NR Femto node is connected to the 5GC via an NR Femto GW or not.

As mentioned above, we can conclude that Option 1 is used for NR Femto Node connecting to 5GC directly, while Option 2 Architecture is for NR Femto Node connecting to 5GC with a NR Femto GW.

In our opinion, the difference between Option1 and Option 2 is whether deploys a NR Femto GW or not. In order to make this intention more clearly, we have the following proposals:

Proposal 1: Change the description for Option 2 to deploy an NR Femto GW as a mandatory choice.

---------------------------Change1 starts ----------------------------------

5.2.1.2	Option 2

Figure 5.2.1.2-1 shows a logical architecture for the NR Femto that has a set of NG interfaces to connect the NR Femto node to the 5GC.

Figure 5.2.1.2-1: Option 2 for NR Femto Architecture

NOTE:	The SeGW and the NR Femto Management System are out of RAN3 scope.

The NG-RAN architecture deploys an NR Femto Gateway (NR Femto GW) to allow the NG interface between the NR Femto node and the 5GC to support a large number of NR Femto nodes in a scalable manner. The NR Femto GW serves as a concentrator for the C-Plane, specifically the NG-C interface.

The NG interface is defined as the interface:

-	Between the NR Femto GW and the 5GC;

-	Between the NR Femto node and the NR Femto GW;

The NR Femto GW appears to the AMF as a gNB. The NR Femto GW appears to the NR Femto node as an AMF.

---------------------------Change1 ends ----------------------------------

Proposal 2: Merge Option1 into Option 2 for the NR Femto GW as an optional deployment choice.

---------------------------Change2 starts ----------------------------------

5.2.1.1	Option X

Figure 5.2.1.2-1 shows a logical architecture for the NR Femto that has a set of NG interfaces to connect the NR Femto node to the 5GC.

Figure 5.2.1.2-1: Option X for NR Femto Architecture

NOTE:	The SeGW and the NR Femto Management System are out of RAN3 scope.

The NG-RAN architecture may deploy an NR Femto Gateway (NR Femto GW) to allow the NG interface between the NR Femto node and the 5GC to support a large number of NR Femto nodes in a scalable manner. The NR Femto GW serves as a concentrator for the C-Plane, specifically the NG-C interface.

The NG interface is defined as the interface:

-	Between the NR Femto GW and the 5GC;

-	Between the NR Femto node and the NR Femto GW;

-	Between the NR Femto node and the 5GC;

The NR Femto GW appears to the AMF as a gNB. The NR Femto GW appears to the NR Femto node as an AMF. The NG interface between the NR Femto node and the 5GC is the same regardless of whether the NR Femto node is connected to the 5GC via an NR Femto GW or not.

---------------------------Change2 ends ----------------------------------

Proposal 3:  Adopt Proposal 1 or Proposal 2 into TR 38.799.

- Conclusion

Base on the discussion above, we make the following proposals.

Proposal 1: Change the description for Option 2 to deploy an NR Femto GW as a mandatory choice.

Proposal 2: Merge Option 1 into Option 2 to deploy the NR Femto GW as an optional choice.

Proposal 3: Adopt Proposal 1 or Proposal 2 into TR 38.799.

- Reference

- TR 38.799v1.10