3GPP TSG-RAN WG3 Meeting #R3-125		R3-244203 Maastricht, Netherlands, Aug 19-23, 2024

Title: 	Resource management for Inter-CU LTM

Source: 	Rakuten

Agenda item:	13.2

Document Type:	Discussion and agreement

# 1. Introduction

In this contribution, we discuss the resource management issues associated with configuration and maintenance of multiple candidate cells, the impacts on gNB-DU and the importance of gNB-DU initiated reconfiguration of LTM target cell configuration.

# 2. Discussion

## 2.1 Resource management for multiple LTM target cells

The lead time (duration between LTM HO preparation and execution) in an LTM HO could be unpredictably long. Secondly, not all prepared target cell configurations end up in LTM cell switches. Radio resources are scarce and considering that a UE can be configured with multiple LTM candidate cells, resource reservation (including PRACH preambles) is always going to be an issue. Reservation of radio resources in all the configured LTM candidate cells for a UE leads to sub-optimal performance and overhead on the network. Especially considering that this is applicable on a large scale, for all UEs configured with LTM in the network.

RAN1 has agreed solutions like UE performing UL sync to acquire timing advance from the candidate/target cell before the serving cell switch with the objective of reducing HO latency and reducing user service interruption.

Based on the UE’s L1 measurements, the serving gNB-DU may ask the UE to perform UL sync with the candidate/target cell to acquire UE’s candidate/cell timing advance. Hence, UE performing UL sync with the candidate/target cell can be considered to indicate a likelihood of UE undergoing LTM HO to the given target cell.

To address the resource reservation issues, the following could be considered - at the time of preparing an LTM target cell, the gNB-CU could indicate to the candidate/target gNB-DU whether or not the UE is configured to either perform UL sync (RACH procedure). Based on this, the target gNB-DU shall not reserve all radio resources immediately at the time of LTM candidate/target cell configuration preparation. At this time, the target gNB-DU may also defer reserving other radio resources whose availability is limited.

Based on the UE performing a UL sync operation, the candidate/target gNB-DU can anticipate an incoming LTM cell switch and initiate resource reservation at the candidate/target cell including update of a RACH resource configuration corresponding to the UE’s best beam/beam-group at that point of time.

This introduces multiple benefits

- This ensures that the resource reservation at the target gNB-DU is performed very close to the LTM serving cell switch and hence optimized, as the likelihood of an LTM cell switch is strong. This also prevents unnecessary blocking of resources for a longer duration in case of long lead times.

- This also ensures that the right RACH resources corresponding to the best beam/beam-group for the UE is reserved at the target gNB-DU.

- The UE performing UL sync can be a good indicator of the likelihood of a LTM serving cell change. This enables the target gNB-DU to issue a target cell re-configuration (if required) based on the prevalent resource conditions at that time.

The newly reserved radio resources including the new or modified PRACH resources, resulting in a modified candidate/target cell configuration at the candidate/target gNB-DU is delivered to the UE via the gNB-CU using a gNB-DU initiated F1: UE Context Modification Required procedure.

Observation 1: gNB-DU initiated target cell re-configuration can be used to enable optimized resource reservation.

However, in Rel18, gNB-DU initiated LTM candidate cell re-configuration is not permitted by RAN3. This is a big limitation considering the fact that there could be changes in resource status and availability between LTM candidate cell preparation and execution. As an example, a simple RACH preamble modification based on beam or beam-group change isn’t possible after the candidate cell configuration has been prepared. Once the candidate cell configuration is prepared, it is either executed or cancelled. This, in our view, is too inflexible and limiting genuine cases requiring re-configuration.

Proposal 1: gNB-DU initiated candidate/target cell re-configuration is reconsidered in Release 19.

## 2.3 Assistance information for target cell preparation

L1/L2 triggered mobility could be associated with multiple flavours like Inter Cell Beam Management (ICBM), ICBM with dynamic switching, LTM Serving cell switch, LTM serving cell change with dynamic switching etc. These variants are being discussed in RAN1 and RAN2. Each of these variants need different kind of resource reservation at the candidate/target cell.

For example, LTM serving cell switch could be considered as the primary form where the UE undergoes serving cell change by adopting the target cell configuration.

In Inter Cell Beam Management (ICBM), CSI measurement configuration and TCI states for PDCCH and PDSCH are modified to include target cell (intra DU or inter DU) aspects.

Configurations are extended to include resources of multiple cells.

- The reporting configuration for the UE is maintained as common across all cells associated with ICBM, the target cells may reserve resources at the same location in PUSCH/PUCCH for reporting.

- Modification of beam management related configuration to enable efficient beam switching operation without impacting cell specific beam refinement operation within same cell is needed.

- Paging and System Information Broadcast are performed from the serving cell.

Dynamic switching between two intra gNB-DU or inter gNB-DU cells is when ping-pong is expected between them, and the involved cells have to be prepared for that without causing any additional damage to the user-plane interruption time.

Since LTM is applicable to both intra- DU and intra-CU inter-DU scenarios, and gNB-CU is responsible for preparation of LTM target cells, we think two aspects are very important here.

- Awareness at gNB-CU regarding the technical feasibility of the desired target cell configuration at the target gNB-DU.

- Resource situation at the target gNB-DU to accede to the request made by gNB-CU.

For a), the gNB-CU may need to be aware of the UE’s current configuration at serving gNB-DU. On top of this, the gNB-CU needs to be able to decide if the same or a different configuration is feasible at the target gNB-DU. Some of this may need serving gNB-DU specific configuration information at the gNB-CU.

For b), each gNB-DU which could be a potential target gNB-DU, the gNB-CU needs awareness of resource availability of each variant before making a request, failing which there could be a lot of rejections.

Observation 2: Awareness of gNB-DU specific information at gNB-CU to determine technical feasibility of the desired LTM target cell configuration at target gNB-DU and awareness of resource situation at each target gNB-DU is required at gNB-CU before initiating target cell preparation.

Proposal 2: RAN3 discusses and agrees solutions to select the appropriate variant of LTM for each UE.

## 2.3 Resource Reservation for UE-based TA measurement

UE-based TA measurement is a way of TA acquisition where the UE derives TA based on Rx timing difference between current serving cell and candidate cell as well as TA value for the current serving cell.

The UE-based TA acquisition is an alternative way of acquiring the candidate cell TA, but this has the following characteristics

    - This is based on UE capability and is configured by the gNB-CU using RRC signaling.

    - The TA computation is sometimes accurate and some other times may not be accurate.

    - This doesn’t require UE to perform RACH towards candidate cell to acquire TA and hence there may be no need for RACH resources reserved at the target gNB-DU.

Since the accuracy of UE-based TA acquisition is dependent on many factors and may vary between any two pairs of serving and candidate cells, the gNB-CU may configure the UE to perform UE-based TA measurement judiciously.

However, the RACH resources reserved at the LTM candidate cell are limited and hence valuable. So, we think that whenever the gNB-CU configures the UE with UE-based TA measurement, this could be indicated over F1 (for intra-CU LTM) and Xn (for inter-CU LTM), so that the candidate gNB-DU may skip allocating RACH resources for that UE.

Proposal 3: Whenever Serving gNB-CU configures a UE with UE-based TA measurement, it explicitly indicates to the candidate gNB-DU over F1 and Xn interface whether the RACH resources should be reserved for the UE.

# 3. Conclusion

Based on the discussion in this paper, we have the following observations and proposals.

Observation 1: gNB-DU initiated target cell re-configuration can be used to enable optimized resource reservation.

Observation 2: Awareness of gNB-DU specific information at gNB-CU to determine technical feasibility of the desired LTM target cell configuration at target gNB-DU and awareness of resource situation at each target gNB-DU is required at gNB-CU before initiating target cell preparation.

Proposal 1: gNB-DU initiated candidate/target cell re-configuration is reconsidered in Release 19.

Proposal 2: RAN3 discusses and agrees solutions to select the appropriate variant of LTM for each UE.

Proposal 3: Whenever Serving gNB-CU configures a UE with UE-based TA measurement, it explicitly indicates to the candidate gNB-DU over F1 and Xn interface whether the RACH resources should be reserved for the UE.

# 4. Reference

RP-240299, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom, 3GPP TSG RAN #103, Maastricht, Netherlands, March 18-21, 2024