3GPP TSG-RAN WG3 Meeting #125	R3-244220

Maastricht, NL, 19 - 23 Aug, 2024

Title:	Discussions on UE's awareness of MBS Communication Service Type for QMC

Agenda Item:	8.3

Source:	Huawei

Document for:	Discussion and decision

1	Introduction

During last meeting, the issue on UE's awareness of MBS communication service type was discussed [1], but there is no consensus, this paper tries to have further analysis on this issue and give some observations and suggestion.

2	Discussion

The outcome of this issue from last meeting could refer to the chairnotes as follows:

The UE should be aware whether to conduct QMC for MBS in broadcast or in multicast mode.

There is no consensus on whether any enhancements are needed on the top of current mechanism in this meeting.

FFS whether the UE is informed via RRC or via the QMC configuration container or existing solutions.

In order to answer this question, we could try to decompose this question into two sub-questions as follows:

- If any entity from top to down in the network, OAM/CN, gNB or UE, doesn’t the MBS service type of ongoing MBS service type;

- If the UE needs to be explicitly informed that for which MBS service type the QoE measurement is.

For the first question, we could try to check the current specs one by one:

- If OAM/CN knows the MBS service type;

- If the gNB knows the MBS service type;

- If the UE MBS service type.

For the OAM/CN, according the current OAM specs, it seems that OAM already knows the service type, check SA5 spec 28.622 4.3.54.2, there is an AMCJob attribute called mBSCommunicationServiceType, see below:

For the gNB, the answer is clearly yes, since the procedure for broadcast and multicast are different, gNB knows if there is MBS service ongoing or not, and which service type, broadcast, multicast or unicast this ongoing service is.

The same logic also applies to the UE. The UE also knows the MBS service type when it is receiving the MBS service, since the bearer and corresponding configuration procedure is different. There might be a small point we need to be aware is that, the UE here mainly refers to AS layer of the UE, then a further question is, whether the UE upper layer is aware of the MBS service type or not.

If we check SA4 spec 26.502 4.5.3	MBS User Service parameters, there is also an filed which specifies the MBS service type, see below:

Table 4.5.3-1: Baseline parameters of MBS User Service entity Parameter; Cardinality; Assigner; Description. User Service Identifier; 1..1; MBSF; A unique identifier for this MBS User Service in the MBSF.. External service identifiers; 1..*; MBS Application Provider; A unique identifier for this MBS User Service that is also present in the MBS User Service Announcement. If assigned in a globally unique manner, this identifier may be useful in correlating this MBS User Service with the same service delivered by a different system.. Service type; 1..1; Indicates whether this MBS User Service is distributed via Multicast MBS Session(s) or Broadcast MBS Session(s). Service class; 1..1; The class of this MBS User Service, expressed as a fully-qualified term identifier from a controlled vocabulary such as the OMNA BCAST Service Class [16], e.g. urn:oma:bcast:oma_bsc:st:1.0.. Service announcement modes; 1..*; Determines whether the MBS User Service Announcement compiled by the MBSF is advertised to the MBSF Client at reference point MBS-5 and/or advertised to the MBSF Client via the MBS Distribution Session and/or passed back to the MBS Application Provider via reference point Nmb10.. Service names; 1..*; A set of distinguishing names for this MBS User Service, one per language.. Service descriptions; 1..*; A set of descriptions of this MBS User Service, one per language.. Main service language; 0..1; The main language of this MBS User Service..

In our understanding, this IE “Service type” obviously is used in UE upper layer.

With the understandings above, clearly, we could have the following observations:

Observation 1: OAM knows the MBS service type of the ongoing MSB service, so do the gNB and the UE.

Observation 2: Both the UE AS layer and upper layer knows the the MBS service type of the ongoing MSB service

Maybe we could try to take a further analysis, since both the OAM and the gNB know the MBS service type of the ongoing MBS service, technically the OAM should be clear in advance what it wants to do, and the gNB then just follows the direction; furthermore, the gNB could even respond with failure message without configuring QoE measurement towards the UE, if the indicated MBS service type in the QoE measurement request doesn’t match the real one at the gNB side.

In addition, in SA5 spec 28.405 section 4.5 (Management based activation in NR) and 4.6 (Signalling based activation in NR) for the E2E procedure, see below for signalling based as example, which clearly illustrated that mBSCommunicationServiceType is not explicitly transmitted over Uu interface, either it should be part of container, or it is not needed over Uu interface.

Obersvation 3: The OAM should be clear in advance what it wants to do, and the gNB then just follows the direction;

Observation 3bis: Technically the gNB could even respond with failure message without configuring QoE measurement towards the UE, if the indicated MBS service type in the QoE measurement request from OAM/CN doesn’t match the real one at the gNB side.

With the observations above, it is obvious that there is no need for the UE to be explicitly informed of the MBS service type, since the QMCJob containing service type indication will be sent to the UE upper layer, and the UE upper layer knows the service type of the ongoing MBS service.

Observation 4: There is no need for the UE to be explicitly informed of the MBS service type, since the QMCJob containing service type indication will be sent to the UE upper layer, and the UE upper layer knows the service type of the ongoing MBS service.

With all the analysis above, we would like RAN3 to discuss all the observations and reach the conclusion that there is no need to introduce addional mechanism on this issue.

Proposal: It is proposed RAN3 discuss the observations and, agree that there is no need to introduce addional mechanism on this issue based on the observations.

3	Conclusion

This document discussed the issue on UE's awareness of MBS communication service type, and have the following observations and one proposal:

Observation 1: OAM knows the MBS service type of the ongoing MSB service, so do the gNB and the UE.

Observation 2: Both the UE AS layer and upper layer knows the the MBS service type of the ongoing MSB service

Obersvation 3: The OAM should be clear in advance what it wants to do, and the gNB then just follows the direction;

Observation 3bis: Technically the gNB could even respond with failure message without configuring QoE measurement towards the UE, if the indicated MBS service type in the QoE measurement request from OAM/CN doesn’t match the real one at the gNB side.

Observation 4: There is no need for the UE to be explicitly informed of the MBS service type, since the QMCJob containing service type indication will be sent to the UE upper layer, and the UE upper layer knows the service type of the ongoing MBS service.

Proposal: It is proposed RAN3 discuss the observations and, agree that there is no need to introduce addional mechanism on this issue based on the observations.

4	Reference

[1]	R3-243367, MBS Communication Service Type for QMC (Ericsson)