3GPP TSG-RAN WG3 #125                                             	    R3-244228

Maastricht, Netherlands, August 19th –23th, 2024

Agenda item:	   13.2

Source: 		Qualcomm Incorporated, NTT DOCOMO, Vodafone, Bharti Airtel (India), Sony

Title: 	Inter-gNB LTM with no change of RRC/PDCP anchor

Document for:	Discussion

# 1	Introduction

RAN#102 agreed the following WID objective [1]:

- Specify support for inter-CU Layer 2 Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - … - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support

RAN3#123bis agreements [2] :

Prioritize to support inter-CU LTM over Xn interface, and RAN3 specify the inter-CU LTM solutions for standalone scenario first.

Reuse existing Xn Handover Request and Handover Request ACK for Inter-CU LTM initial preparation.

Confirm the case that inter-CU LTM is not configured in both MCG and SCG at the same time.

Early data forwarding can be supported for inter-CU LTM. When to trigger can be further discussed.

Cell Switch Notification from source DU to target DU (in different gNB from source) for LTM execution.

Discuss on the inter-CU LTM solutions for standalone scenario…

RAN3#124 agreements [4]:

Source gNB-CU initiates the handover preparation procedure for inter-gNB-CU LTM.

Introduce a new procedure on Xn to transfer the TA information.

A new XnAP class 2 procedure, namely LTM Cell Switch Notification is introduced on Xn to forward the target Cell ID and target TCI state ID(s) from the source gNB-CU to the target gNB-CU.

Reuse Handover Success procedure over Xn for Rel-19 Inter-CU LTM, to tell the source CU that the UE has accessed to the target Cell.

The Handover Cancel message is reused by the source gNB to release the reserved resource for LTM candidate cells in the candidate gNBs.

Early data forwarding can be triggered before the Source gNB triggers a MAC CE Command to the UE to change cells, timing is left to implementation.

Early sync configuration (TCI state and RACH configuration) can be obtained during the LTM preparation phase through the handover request and handover request acknowledge messages.

A candidate gNB can initiate cancellation of configured LTM candidate cell(s) of its own. Details are FFS.

FFS on the LTM modification procedures.

FFS on whether to reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exist in the source gNB.

This paper discusses various aspects of inter-gNB LTM with fixed RRC and PDCP anchor that utilizes R18 Intra-CU LTM design principles.

# 2	Use cases for fixed RRC and PDCP anchor

## 2.1	Reduce the frequency of costly handovers

Figure 1 – Fixing the anchor reduces the cost of most LTM cell switches. Costly cell switches with anchor change are only performed when UE traffic requirements can no longer be met due to backhaul constraints, e.g., backhaul transport latency or due to network loading or topology constraints.

Figure 1 shows an example of a UE trajectory comprising a sequence of six cell switches:

- For the first three cell switches, routing UE traffic towards the subsequent target cells via the red RRC/PDCP anchor incurs low backhaul transport latency and does not violate UE traffic characteristics requirements. Thus, these cell switches do not incur a RRC/PDCP anchor change, which reduces the complexity of the LTM handover procedure for both the UE and the NW, e.g., no RRC context transfer, no security change, no PDCP reestablishment, and no path switch towards the CN. Note that Xn delay is negligible since fibre optic cables transfer data at speed of light.

- For the fourth cell switch, triangular routing of UE traffic towards the target cell via the red RRC/PDCP anchor incurs high backhaul transport latency and maynot meet UE traffic characteristics requirements. In this case, based on gNB implementation choice, the RRC/PDCP anchor is relocated to the green gNB. The change of anchor requires reconfiguration of security, RRC context transfer, PDCP reestablishment, and a path switch towards the CN.

- For the remainder of the cell switches, routing UE traffic towards the subsequent target cells via the green RRC/PDCP anchor incurs low backhaul transport latency and does not have impact on UE traffic characteristics requirements. Again, no change of RRC/PDCP anchor is required for these cell switches.

Observation 1: A handover procedure with RRC/PDCP anchor change is more signalling intensive to the UE and the NW compared to a handover procedure without RRC/PDCP anchor change.

Observation 2: Pre-LTM, intra-gNB handovers are not signalling intensive and inter-gNB handovers involve lot of signalling (i.e, costly) since the latter always incur a change of the RRC/PDCP anchor.

Observation 3: Any delay due to Inter-gNB Xn links for both signalling and user plane traffic is negligible since fiber optic cables carry data at speed of light. (typical inter gNB distance of 250meters to 15km is negligible transmission delay for fiber optic transmission).

Observation 4: Extending the fixed-RRC/PDCP-anchor mode to the inter-gNB cell-switch scenario where permissible reduces the frequency of costly handovers for both the UE and the NW.

## 2.2	Reduce the overhead of the ping-pong LTM HO scenario

Figure 2 – Comparison of the ping-pong scenario b/w gNB1 and gNB2 for the cases of variable vs. fixed RRC/PDCP anchors.

Figure 2 illustrates the scenario where a UE ping-pongs b/w cells of two neighboring gNBs. Changing the RRC/PDCP anchor to the target gNB per cell switch implies the need for:

- security update, RRC context transfer, PDCP reestablishment and switching the UP datapath to gNB2 for the forward handovers, and

- security update, RRC context transfer, PDCP reestablishment and switching the UP datapath back to gNB1 for the reverse handovers,

which is wasteful relative to the short duration that the UE spends on each cell. This overhead can be tremendously cut down by supporting inter-gNB cell switches with no change of RRC/PDCP anchor.

Observation 5: For the ping-pong scenario, changing the RRC/PDCP anchor per cell switch incurs too much overhead relative to the short duration that the UE spends on each cell. Performing the cell switches with no change of RRC/PDCP anchor is much more efficient.

Based on the merits above, inter-gNB LTM with no change of RRC/PDCP anchor should be supported.

Proposal 1: Inter-gNB LTM with no switch of RRC/PDCP anchor is supported as baseline.

# 3	Architecture and protocol stacks

| a) Integrated gNB | b) Split gNB architecture | c) Split gNB-architecture with separate CP/UP |
| --- | --- | --- |
| Figure 3 – A UE establishes RRC/DRB towards gNB1 using an RLC connection to gNB2 |  |  |

Figure 3 depicts an LTM scenario where the UE switches from a gNB1 cell to a gNB2 cell but retains it RRC connection and DRBs with gNB1. In this scenario:

- PDCP and RRC context resides in gNB1 while RLC resides in gNB2.

- In case of split architecture, RRC/PDCP resides in gNB1-CU, whereas RLC resides in gNB2-DU. The latter only establishes a single F1-C connection to gNB2-CU. gNB2-CU simply acts as RRC signalling routing function in transparent manner.

- In case of CP/UP separation, UE’s RRC terminates at gNB1-CU-CP and UE’s DRB(s) terminate(s) at gNB1-CU-UP. The latter only establishes a single E1 connection to gNB1-CU-CP.

- All the above are transparent to the UE

| a) Integrated gNB | b) Split gNB architecture [w/(o) CP/UP separation] |
| --- | --- |
| Figure 4 – Control plane protocol stacks for separate PDCP/RLC-anchor scenario |  |
| a) Integrated gNB | b) Split gNB architecture [w/(o) CP/UP separation] |
| Figure 5 – User plane protocol stacks for separate PDCP/RLC-anchor scenario |  |

The corresponding control plane and user plane protocol stacks are illustrated respectively in Figures 4 and 5. Main points to emphasize:

- From UE perspective, native Uu stack applies as in legacy.

- From NW perspective, having the PDCP anchor and RLC anchor at separate gNBs is agnostic to the internal architecture of the two gNBs. Furthermore, as in the legacy 5G architecture, no cross-gNB F1-C or E1 interfaces are needed.

- For data forwarding b/w gNB1 and gNB2, multiple options hold for the gNB2-endpoint of the UP data forwarding tunnel. These options depend on gNB2’s implementation and are transparent to gNB1.

Observation 6: Having PDCP and RLC anchors in separate gNBs is agnostic to the internal architecture of the two gNBs.

Observation 7: Having PDCP and RLC anchors in separate gNBs does not require a cross-gNB F1-C or E1 interface.

Observation 8: Having PDCP and RLC anchors in separate gNBs assumes native (Rel-15) Uu stack for the UE.

Proposal 2: Inter-gNB LTM with no switch of RRC/PDCP anchor is supported for the integrated gNB architecture, the split gNB architecture without CP/UP separation, and the split gNB architecture with CP/UP separation. Inter-gNB F1-C or E1 interfaces are not supported in Rel-19.

# 4	Backhaul transport

In case PDCP and RLC reside in separate gNBs, forwarding of CP and UP traffic is required between the two gNBs.

- For CP traffic, this can reuse the Xn RRC Transfer procedure between the PDCP-terminating gNB and the RRC-terminating gNB.

- For UP traffic, this can be carried over a GTP-U tunnel between the PDCP-terminating gNB and the RLC-terminating gNB. The two gNBs exchange the tunnel endpoints during LTM preparation. For the case of split-architecture, the RLC-terminating gNB may select the tunnel endpoint as that of its gNB-CU or gNB-DU, which is up to implementation (see Figure 5b).

Proposal 3: Reuse the Xn RRC transfer procedure for the transport of the UE’s RRC messages b/w the PDCP-terminating gNB and the RLC-terminating gNB.

Proposal 4: A GTP-U tunnel transports the PDUs of a UE’s DRB b/w the PDCP-terminating gNB and the RLC-terminating gNB. The two gNBs exchange the tunnel endpoints during LTM preparation. In case of split architecture, the RLC-terminating gNB selects its tunnel-endpoint based on implementation.

# 5	Preparation of the LTM candidate configuration

In case PDCP and RLC reside in separate gNBs, the RLC-terminating gNB generates the CellGroupConfig for its own LTM candidate cells, whereas the PDCP-terminating gNB generates the corresponding radioBearerConfig for all the LTM candidate cells. The RLC-terminating gNB provides the CellGroupConfig of an LTM candidate configuration to the PDCP-terminating gNB during LTM preparation. In case of split-architecture, the gNB-DU of the RLC-terminating gNB provides the CellGroupConfig to the gNB-CU of the RLC-terminating gNB, which then forwards the CellGroupConfig to the PDCP-terminating gNB.

Proposal 5: The RLC-terminating gNB generates the CellGroupConfig for its own LTM candidate cell and shares it with the PDCP-terminating gNB. The PDCP-terminating gNB generates the corresponding radioBearerConfig and forwards the combined LTM candidate configuration to the UE via RRC.

# 6	Interaction between the PDCP anchor and the CN

## 6.1	On the need for Path Switch Request procedure

According to TS 38.413, a Path Switch Request procedure is performed towards the AMF to request the switch of the DL termination point of the NG-U transport bearer. In the case of inter-gNB LTM cell switch with fixed PDCP anchor, the DL NG-U termination point does not change. This implies that no Path Switch Request procedure is needed towards the CN.

Observation 9: During inter-gNB LTM cell switch with fixed PDCP anchor, the DL NG-U termination point does not change.

Proposal 6: No Path Switch Request procedure is required following an inter-gNB LTM cell switch with no PDCP anchor change.

## 6.2	User location information reporting

Without a path switch, user location reporting will have to be performed by the PDCP-terminating gNB. This requires the PDCP-terminating gNB to know the cell ID and tracking area code of the LTM candidate cell served by the RLC-terminating gNB. This information is available based on exchange of served cell information during Xn Setup.

Observation 10: The PDCP-terminating gNB and the RLC-terminating gNB exchange served cell information during Xn setup, which includes cell IDs and tracking area codes.

Proposal 7: User location information reporting is performed by the PDCP-terminating gNB based on served cell info exchange with the RLC-terminating gNB during Xn setup.

## 6.3	Recommended cells and RAN nodes for paging

In legacy, the serving NG-RAN may include list of recommended cells and RAN nodes for paging in the UE CONTEXT RELEASE COMPLETE message to the AMF.

In case the PDCP-terminating gNB decides to release the UE context, it can always reuse legacy signaling and include the RAN Node ID of the RLC-terminating gNB and its associated LTM candidate cell ID(s) among the list of recommended cells and RAN nodes for paging. This is left to implementation.

Proposal 8: Legacy signaling can be reused by the PDCP-terminating gNB to include the RAN Node ID of the RLC-terminating gNB and the associated LTM candidate cell ID(s) among the list of recommended cells and RAN nodes for paging sent to the AMF. Inclusion of these IDs is up to implementation.

Based on the above, the CN has all the information it needs to support tracking-area-level-based and cell-level-based paging.

Observation 11: All legacy information (ULI, recommended cells/TAs/RAN nodes) required to support tracking-area-level-based and cell-level-based paging are available at the CN for the case of separate PDCP-terminating and RLC-terminating gNBs.

Proposal 9: From RAN3 perspective, inter-gNB LTM cell switch with fixed PDCP anchor has no impact to CN-based paging.

# 7	Releasing UE to RRC_INACTIVE

In case the PDCP-terminating gNB releases the UE to RRC_INACTIVE while served on a cell of a separate RLC-terminating gNB, the PDCP-terminating gNB provides the UE with an I-RNTI. Upon connection resume, the UE provides the I-RNTI to the new gNB. As per legacy, the new gNB identifies the old PDCP-terminating gNB based on the I-RNTI and sends a Retrieve UE Context Request message to the PDCP-terminating gNB including the I-RNTI.

Observation 12: In case the UE with separate PDCP/RLC anchors is released to RRC_INACTIVE, the UE includes the I-RNTI into the RRC resume message as in legacy. This allows the new serving gNB to identify the old PDCP-terminating gNB and attempt UE context retrieval as in legacy.

To verify the UE context, the UE includes the resumeMAC-I into the RRC resume message. As per legacy, the UE computes this token using the PCI of the last serving cell and the C-RNTI at the last serving cell. In this case, the last serving cell belongs to the old RLC-terminating gNB, whereas the resumeMAC-I is forwarded by the new serving gNB to the old PDCP-terminating gNB.

Observation 13: In case of separate PDCP/RLC anchors, the last PCI and C-RNTI used for computing the resumeMAC-I by the UE correspond to the old RLC-terminating gNB, whereas the new serving gNB forwards the resumeMAC-I to the old PDCP-terminating gNB.

This implies the old PDCP-terminating gNB must have the PCI and C-RNTI of the last serving cell of the old RLC-terminating gNB prior to releasing the UE to RRC_INACTIVE. Otherwise, the old PDCP-terminating gNB won’t be able to recompute the resumeMAC-I and verify the UE. However, this information is already carried inside the configuration of the LTM candidate cell that the RLC-terminating gNB provides to the PDCP-terminating gNB during LTM preparation. This means that the old PDCP-terminating gNB has the necessary information to recompute the resumeMAC-I and validate the UE.

Observation 14: During LTM preparation, the RLC-terminating gNB provides the PDCP-terminating gNB with a configuration of the LTM candidate cell which includes the PCI and the UE’s C-RNTI. During RRC resume, the old PDCP-terminating gNB can use this info to compute the resumeMAC-I and validate the UE.

Thus, release to RRC_INACTIVE can be supported for a UE with separate PDCP/RLC-terminating gNBs as per legacy.

Proposal 10: Release to RRC_INACTIVE and resume procedures apply for a UE with separate PDCP/RLC-terminating gNBs as in legacy.

# 8	Support of subsequent LTM

Figure 6 – Subsequent inter-gNB LTM with fixed PDCP-terminating LTM candidate gNB and changing RLC-terminating LTM candidate gNB

Figure 6 illustrates the inter-gNB LTM scenario where a UE with separate PDCP/RLC gNB-termination points performs a subsequent LTM execution. In such a case, the UE sends an RRCReconfigurationComplete message to the PDCP-terminating LTM candidate gNB via the new serving RLC-terminating LTM candidate gNB. Furthermore, backhaul transport is redirected to the UE’s new serving location. No RRC signaling is needed.

Proposal 11: Subsequent inter-gNB LTM with fixed PDCP-terminating gNB and moving RLC-terminating gNB is supported.

# Conclusion

This contribution discussed various aspects of inter-gNB LTM with fixed RRC/PDCP anchor. The following observations and proposals have been made:

How to support efficient R19 Inter-CU LTM HO without security update:

Observation 1: A handover procedure with RRC/PDCP anchor change is more signalling intensive to the UE and the NW compared to a handover procedure without RRC/PDCP anchor change.

Observation 2: Pre-LTM, intra-gNB handovers are not signalling intensive and inter-gNB handovers involve lot of signalling (i.e, costly) since the latter always incur a change of the RRC/PDCP anchor.

Observation 3: Any delay due to Inter-gNB Xn links for both signalling and user plane traffic is negligible since fiber optic cables carry data at speed of light. (typical inter gNB distance of 250meters to 15km is negligible transmission delay for fiber optic transmission).

Observation 4: Extending the fixed-RRC/PDCP-anchor mode to the inter-gNB cell-switch scenario where permissible reduces the frequency of costly handovers for both the UE and the NW.

Observation 5: For the ping-pong scenario, changing the RRC/PDCP anchor per cell switch incurs too much overhead relative to the short duration that the UE spends on each cell. Performing the cell switches with no change of RRC/PDCP anchor is much more efficient.

Proposal 1: Inter-gNB LTM with no switch of RRC/PDCP anchor is supported as baseline.

Architecture and Protocol stacks :

Observation 6: Having PDCP and RLC anchors in separate gNBs is agnostic to the internal architecture of the two gNBs.

Observation 7: Having PDCP and RLC anchors in separate gNBs does not require a cross-gNB F1-C or E1 interface.

Observation 8: Having PDCP and RLC anchors in separate gNBs assumes native (Rel-15) Uu stack for the UE.

Proposal 2: Inter-gNB LTM with no switch of RRC/PDCP anchor is supported for the integrated gNB architecture, the split gNB architecture without CP/UP separation, and the split gNB architecture with CP/UP separation. Inter-gNB F1-C or E1 interfaces are not supported in Rel-19.

Backhaul Transport:

Proposal 3: Reuse the Xn RRC transfer procedure for the transport of the UE’s RRC messages b/w the PDCP-terminating gNB and the RLC-terminating gNB.

Proposal 4: A GTP-U tunnel transports the PDUs of a UE’s DRB b/w the PDCP-terminating gNB and the RLC-terminating gNB. The two gNBs exchange the tunnel endpoints during LTM preparation. In case of split architecture, the RLC-terminating gNB selects its tunnel-endpoint based on implementation.

Preparation of LTM Candidate Configuration:

Proposal 5: The RLC-terminating gNB generates the CellGroupConfig for its own LTM candidate cell and shares it with the PDCP-terminating gNB. The PDCP-terminating gNB generates the corresponding radioBearerConfig and forwards the combined LTM candidate configuration to the UE via RRC

On the need for Path Switch Request procedure:

Observation 9: During inter-gNB LTM cell switch with fixed PDCP anchor, the DL NG-U termination point does not change.

Proposal 6: No Path Switch Request procedure is required following an inter-gNB LTM cell switch with no PDCP anchor change.

User location information reporting:

Observation 10: The PDCP-terminating gNB and the RLC-terminating gNB exchange served cell information during Xn setup, which includes cell IDs and tracking area codes.

Proposal 7: User location information reporting is performed by the PDCP-terminating gNB based on served cell info exchange with the RLC-terminating gNB during Xn setup.

Recommended cells and RAN nodes for paging:

Proposal 8: Legacy signaling can be reused by the PDCP-terminating gNB to include the RAN Node ID of the RLC-terminating gNB and the associated LTM candidate cell ID(s) among the list of recommended cells and RAN nodes for paging sent to the AMF. Inclusion of these IDs is up to implementation.

Observation 11: All legacy information (ULI, recommended cells/TAs/RAN nodes) required to support tracking-area-level-based and cell-level-based paging are available at the CN for the case of separate PDCP-terminating and RLC-terminating gNBs.

Proposal 9: From RAN3 perspective, inter-gNB LTM cell switch with fixed PDCP anchor has no impact to CN-based paging.

Releasing UE to RRC_INACTIVE:

Observation 12: In case the UE with separate PDCP/RLC anchors is released to RRC_INACTIVE, the UE includes the I-RNTI into the RRC resume message as in legacy. This allows the new serving gNB to identify the old PDCP-terminating gNB and attempt UE context retrieval as in legacy.

Observation 13: In case of separate PDCP/RLC anchors, the last PCI and C-RNTI used for computing the resumeMAC-I by the UE correspond to the old RLC-terminating gNB, whereas the new serving gNB forwards the resumeMAC-I to the old PDCP-terminating gNB.

Observation 14: During LTM preparation, the RLC-terminating gNB provides the PDCP-terminating gNB with a configuration of the LTM candidate cell which includes the PCI and the UE’s C-RNTI. During RRC resume, the old PDCP-terminating gNB can use this info to compute the resumeMAC-I and validate the UE.

Proposal 10: Release to RRC_INACTIVE and resume procedures apply for a UE with separate PDCP/RLC-terminating gNBs as in legacy.

Support of subsequent LTM:

Proposal 11: Subsequent inter-gNB LTM with fixed PDCP-terminating gNB and moving RLC-terminating gNB is supported.

# References

- RP-234006, New WID: NR mobility enhancements Phase 4, 3GPP TSG RAN Meeting #102, Edinburgh, Scotland, December 11-15, 2023

- Chairman Notes; TSG RAN WG3 Meeting #123bis

- R2-2403929, LS on security handling for inter-CU LTM in non-DC cases, 3GPP TSG RAN WG2#125bis, Changsha, China, 15th - 19th April 2024

- Chairman Notes; TSG RAN WG3 Meeting #124

- R3-243522 : Inter-gNB LTM and keep PDCP anchor (Qualcomm Incorporated, NTT DOCOMO)

# Annex

## A.1	Example procedure for NAS transport

Figure 7 – Example procedure for transport of NAS messages within RRC containers carried via Xn

As illustrated in Fig.7, exchange of NAS messages b/w the CN and the UE is transparent to the signaling path b/w the RRC-terminating gNB and the UE.

## A.2	Example procedure for RRC reconfiguration initiated by RLC-terminating gNB (if supported)

Figure 8 – Example procedure for RRC reconfiguration initiated by RLC-terminating gNB

RAN2/3 first have to agree to support the scenario where candidate gNBs can modify the LTM candidate configurations of their served cells. In this case, the RLC-terminating gNB shares the new CellGroupConfig to the RRC-terminating gNB. The RRC-terminating gNB prepares an RRC message including the updated configuration and sends it to the UE via the RLC-terminating gNB. The RRC-terminating gNB acknowledges the configuration request from the RLC-terminating gNB.