3GPP TSG-RAN WG3 Meeting #125	R3-244229

Maastricht, Netherlands, Aug 19th - Aug 23rd, 2024

Agenda item:	13.2

Source:	Qualcomm Incorporated

Title:	Signalling enhancements for inter-CU LTM HO

WID/SID:	NR_Mob_Ph4-Core – Release 19

Document for:	Discussion and decision

# Introduction

In the last RAN3 meeting (RAN3 #124 [1]), reasonable progress was achieved, and some agreements were reached on inter-CU LTM assuming anchor CU switch-based solution.

RAN3 #124 agreements:

Source gNB-CU initiates the handover preparation procedure for inter-gNB-CU LTM.

Introduce a new procedure on Xn to transfer the TA information.

A new XnAP class 2 procedure, namely LTM Cell Switch Notification is introduced on Xn to forward the target Cell ID and target TCI state ID(s) from the source gNB-CU to the target gNB-CU.

Reuse Handover Success procedure over Xn for Rel-19 Inter-CU LTM, to tell the source CU that the UE has accessed to the target Cell.

The Handover Cancel message is reused by the source gNB to release the reserved resource for LTM candidate cells in the candidate gNBs.

Early data forwarding can be triggered before the Source gNB triggers a MAC CE Command to the UE to change cells, timing is left to implementation.

Early sync configuration (TCI state and RACH configuration) can be obtained during the LTM preparation phase through the handover request and handover request acknowledge messages.

A candidate gNB can initiate cancellation of configured LTM candidate cell(s) of its own. Details are FFS.

FFS on the LTM modification procedures.

FFS on whether to reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exist in the source gNB.

In this contribution, we discuss the following aspects of inter-CU LTM for our proposed solution in which the source gNB serves as an anchor [2]; please see our companion contribution [2] for an overview discussion covering the motivation and the main technical aspects.

- LTM candidate cell configuration,

- Early TA RACH configuration,

- TA Information Transfer,

- LTM CSI configuration,

- LTM Cell Switch Notification.

# 2 LTM candidate cell configuration

We first make some remarks regarding the LTM preparation procedure in our proposed solution in which the source gNB serves as an anchor.

Remarks regarding LTM preparation in our proposed solution in which the source gNB serves as an anchor

During LTM preparation, the UE is connected to the source DU and the source CU and is served by the source cell. The signalling in the LTM preparation procedure proceeds along expected and familiar lines: upon receiving the RRC measurement report from the UE, the source CU determines the initial list of LTM candidate cells and requests its underlying DUs and the candidate CUs to prepare the LTM candidate cells using the F1 UE Context Setup Request and the Xn Handover Request messages, respectively. Each candidate CU then requests its underlying DUs to prepare LTM candidate cells using the F1 UE Context Setup Request messages. There are some differences in the LTM preparation procedure compared to the case when the RRC and PDCP anchor is moved when the UE moves to another CU, and these are discussed below in the contribution. For example, since the UE remains connected to the source CU in our proposed solution in which the source gNB serves as an anchor, the source CU always determines the upper layer configuration, e.g., the radioBearerConfig containing, e.g., the PDCP configuration, and the RRC configuration, of a candidate cell. During LTM preparation, the candidate CU thus forwards the CellGroupConfig of a prepared candidate cell provided by an underlying DU, to the source CU.

After UE performs LTM cell switch to a candidate cell of a candidate CU, the forwarding of Control Plane signalling between the UE and the source CU can reuse the Xn RRC Transfer procedure as has been discussed in see our companion contribution [2].

Observation 1. For Inter-CU LTM with Anchor PDCP/RRC solution, during the LTM preparation phase, source CU uses legacy Xn Handover signalling messages and F1-C messages to prepare candidate DU/Cell configuration. Candidate CU will retrieve CellGroupConfig from candidate DU and will provide to source CU via legacy Xn Handover signalling messages.

Observation 2. Once LTM Cell Switch procedure is done, Source CU and Candidate DU exchange RRC signalling via RRC Transfer Procedure and Candidate CU will do routing of control plane signalling messages between candidate DU and Source CU, i.e., UE specific RRC messages are transparent to candidate CU.

For the remainder of this section, we discuss the basic aspect of LTM candidate cell configuration during LTM preparation.

Following Rel-18 LTM principles, the source CU initiates inter-CU LTM preparation based on L3 RRC measurement report received from the UE.

Proposal 1. The source CU initiates inter-CU LTM preparation based on RRC measurement report from the UE. The source CU determines the list of LTM candidate cells and transmits a Handover Request message to a candidate CU including:

- LTM indication,

- Suggested candidate cell ID,

- UE measurement results.

The preparation of multiple LTM candidate cells belonging to a candidate CU can take place in parallel (similar as CHO).

Proposal 2. Similar as in Rel-18 intra-CU LTM, the source CU uses the UE Context Setup procedure with its underlying candidate DUs for preparation of LTM candidate cells that belong to it.

Proposal 3. A candidate CU uses the UE Context Setup procedure with its underlying candidate DUs for preparation of LTM candidate cells that belong to it.

Proposal 4. Similar as in Rel-18 intra-CU LTM, a candidate DU determines whether the suggested candidate cell can be prepared, and if the suggested candidate cell can be prepared, the candidate DU determines the lower layer configuration (CellGroupConfig) for the prepared candidate cell.

In our proposed solution for inter-CU LTM in which the source gNB serves as an anchor (please see our companion contribution [2]), an underlying candidate DU of a candidate CU transmits the lower layer configuration (CellGroupConfig) of the prepared candidate cell along with the prepared candidate cell ID, to the candidate CU. The candidate CU forwards the lower layer configuration (CellGroupConfig) of the prepared candidate cell to the source CU along with the candidate cell ID over Xn in Handover Request Ack. The source CU generates the upper layer configuration, e.g., the radioBearerConfig containing, e.g., the PDCP configuration, for the candidate cell, adds it to the provided CellGroupConfig to form the RRC configuration of the candidate cell. Thus, the source CU generates all the LTM candidate configurations – for candidate cells belonging to it, as well as for candidate cells belonging to the other candidate CUs.

Proposal 5. An underlying candidate DU of a candidate CU transmits the lower layer configuration (CellGroupConfig) of the prepared candidate cell, along with the candidate cell ID to the candidate CU. The candidate CU then forwards the CellGroupConfig of the prepared candidate cell to the source CU along with the candidate cell ID over Xn in Handover Request Ack.

Proposal 6. The source CU generates the upper layer configuration, e.g., the radioBearerConfig containing, e.g., the PDCP configuration, for the candidate cell, adds it to the provided CellGroupConfig to form the RRC configuration of the candidate cell.

Similar as in Rel-18 intra-CU LTM, an underlying candidate DU of the source CU transmits the lower layer configuration (CellGroupConfig) of a prepared candidate cell, along with the prepared candidate cell ID to the source CU.

# 3 Early TA RACH configuration

In a previous RAN2 meeting (RAN2 #125-bis [3]), RAN2 agreed on the following:

- Early UL sync is supported for inter-CU LTM.

- PDCCH ordered early RACH is supported for inter-CU LTM.

- For early TA acquisition, Rel-18 option is baseline.

The above agreements indicate that similar RACH configuration and TA estimation mechanism as in Rel-18 intra-CU LTM can be used for Rel-19 inter-CU LTM. In this and the following sections, we propose extensions to the Rel-18 backhaul signalling mechanism for RACH configuration for Early TA acquisition, and the transfer of TA information from a candidate DU to the source DU.

Proposal 7. During LTM preparation, the source gNB-CU may transmit a request to a candidate gNB-CU for obtaining RACH configuration and for allocation of CFRA resources, e.g., RAP IDs, for early TA acquisition. The request uses UE-associated signaling and includes:

- LTM candidate cell ID,

- List of gNB-DU IDs of source gNB-CU.

A set of CFRA resources should be allocated for an underlying DU of the source gNB-CU, and these are to be used by the UEs served by the DU to initiate RACH for Early TA acquisition.

During LTM preparation, other candidate gNB-CUs may also initiate a request to the candidate gNB-CU for Early TA RACH related configuration. These other candidate gNB-CUs are the gNB-CUs with LTM candidate cells that may be prepared for the given UE. We plan to discuss proposals related to this in contributions in future meetings.

Proposal 8. Upon receiving the request from the source gNB-CU, the candidate gNB-CU forwards the request to its underlying candidate gNB-DU including:

- LTM candidate cell ID,

- List of gNB-DU IDs of source gNB-CU,

- The source gNB ID.

As per principles established in Rel-18 intra-CU LTM, a candidate DU determines the split of its available CFRA resources among the other DUs that are requesting for these resources for Early TA.

Proposal 9. Upon receiving the request, the candidate gNB-DU determines the split of the available CFRA resources among the indicated gNB-DUs of the source gNB, and stores this information along with the source gNB ID.

Proposal 10. In the response message, the candidate DU provides to the candidate CU:

- LTM candidate cell ID,

- List of pairs of the form (gNB-DU ID, CFRA resource set),

- The source gNB ID.

Proposal 11. The candidate CU forwards the following information to the source CU in a message over Xn:

- LTM candidate cell ID,

- List of pairs of the form (gNB-DU ID, CFRA resource set).

Proposal 12. Upon receiving the message from the candidate CU, the source CU forwards each of the set of CFRA resources to the corresponding underlying DU, indicating also the LTM candidate cell ID.

# 4 TA Information Transfer

A candidate DU determines the TA value based on the received RACH preamble from the UE. However, the candidate DU cannot determine the UE ID from the received RACH preamble since the RAP ID is from the pool of assigned RAP IDs to the serving DU, for the candidate cell.

Proposal 13. Upon successfully receiving the RACH preamble from the UE on a candidate cell, the candidate DU determines the TA value and determines the source gNB ID. The candidate DU transmits to the candidate CU the following TA information using non-UE associated signalling message:

- Determined TA value,

- RACH resource information (Detected RAP ID, RO information (RA-RNTI)),

- Source gNB ID,

- LTM candidate cell ID.

Proposal 14. The candidate CU forwards the TA information to the source CU using non-UE associated signalling over Xn.

Proposal 15. The source CU identifies the serving DU based on the provided TA information, and forwards this information to the serving DU.

The serving DU then identifies the UE based on the TA information and provides to it the TA value in the LTM Cell Switch MAC CE command during LTM execution.

# 5 LTM CSI configuration

In order to perform L1 measurements on an LTM candidate cell, the UE should be provided with the configuration of the relevant RSs transmitted on that cell, and for L1 measurement reporting, the UE should be provided with the reporting configuration.

We make the following reasonable assumptions in order to determine the signaling for LTM CSI configuration:

- Similar as in Rel-18 intra-CU LTM, each CU receives from an underlying DU the following information during F1 Setup:

- List of cells managed by the DU,

- For each such cell, the cell RS configuration, which includes the list of transmitting SSBs of the cell.

- The source CU receives from a candidate CU similar information during XN Setup:

- List of cells managed by the candidate CU,

- For each such cell, the cell RS configuration, which includes the list of transmitting SSBs of the cell.

The following principles for LTM CSI configuration established in Rel-18 intra-CU LTM should continue to hold for inter-CU LTM:

- The CSI resource configuration for measuring the LTM candidate cells is generated by the CU and is common for all the LTM candidate cells.

- The candidate DUs generate the CSI reporting configurations.

An example overall signalling flow of the CSI configuration procedure is shown in Figure 1.

Figure 1: Example signalling flow of the CSI configuration procedure

Proposal 16. Source CU generates the LTM CSI resource configuration for the candidate cells, i.e., the list of SSBs to be measured by the UE for each candidate cell, based on:

- Information received from the candidate CUs and its own underlying DUs, in XN Setup and F1 Setup procedures, as mentioned in the assumptions above,

- RRC measurement report.

Source CU provides to its own underlying candidate DU:

- Suggested candidate cell ID,

- LTM CSI resource configuration.

The underlying candidate DU responds with:

- The prepared candidate cell ID, and its associated lower layer and LTM CSI report configurations.

Proposal 17. The source CU provides to a candidate CU in Handover Request:

- Suggested candidate cell ID,

- LTM CSI resource configuration.

This information is forwarded by the candidate CU to its underlying candidate DU.

Following our proposed solution for inter-CU LTM [2], a candidate DU provides to the candidate CU the list of prepared cells of the DU and their associated lower layer and LTM CSI report configurations, which is then forwarded to the source CU.

Proposal 18. A candidate DU provides to the candidate CU the following information, which is then forwarded by the candidate CU to the source CU using Handover Request Ack:

- The prepared candidate cell ID and its associated lower layer and LTM CSI report configurations.

Proposal 19. The source CU may update the LTM CSI resource configuration based on the prepared cells of its underlying candidate DUs and those of the candidate CUs.

Proposal 20. The source CU may initiate configuration modification with the candidate CUs and its own underlying DUs, providing the updated LTM CSI resource configuration.

- The candidate DUs provide updated LTM CSI report configurations to the source CU via the candidate CU.

- The source CU forms the final RRC reconfiguration message to be provided to the UE, that includes the RRC configurations that are generated by it for all the prepared LTM candidate cells.

# 6 LTM Cell Switch Notification

The following are the proposals for the LTM Cell Switch Notification, which reuse some of the related Rel-18 messages; see Figure 2 for an example signalling flow.

Proposal 21. Upon transmitting the LTM Cell Switch Command to the UE, the source DU transmits the DU-CU Cell Switch Notification message to the source CU, including the target cell ID and target TCI state ID(s) in the message.

Proposal 22. Upon receiving the DU-CU Cell Switch Notification message, the source CU may transmit a Cell Switch Notification message to the candidate CU, including the target cell ID and target TCI state ID(s) in the message.

Proposal 23. Upon receiving the Cell Switch Notification message from the source CU, the candidate CU transmits the CU-DU Cell Switch Notification message to the candidate DU, including the target cell ID and target TCI state ID(s) in the message.

The above signalling enables the candidate DU to know the target cell and the target beam which the UE will access upon LTM cell switch, so that, e.g., the network can prepare the target beam with a dynamic UL grant for the UE to use upon access of the target cell.

Figure 2: Example signalling flow of the Cell Switch Notification procedure

# 7 Conclusion

Based on the above discussion, we recommend that RAN3 discuss the following observations and proposals.

LTM candidate cell configuration

Observation 1. For Inter-CU LTM with Anchor PDCP/RRC solution, during the LTM preparation phase, source CU uses legacy Xn Handover signalling messages and F1-C messages to prepare candidate DU/Cell configuration. Candidate CU will retrieve CellGroupConfig from candidate DU and will provide to source CU via legacy Xn Handover signalling messages.

Observation 2. Once LTM Cell Switch procedure is done, Source CU and Candidate DU exchange RRC signalling via RRC Transfer Procedure and Candidate CU will do routing of control plane signalling messages between candidate DU and Source CU, i.e., UE specific RRC messages are transparent to candidate CU.

Proposal 1. The source CU initiates inter-CU LTM preparation based on RRC measurement report from the UE. The source CU determines the list of LTM candidate cells and transmits a Handover Request message to a candidate CU including:

- LTM indication,

- Suggested candidate cell ID,

- UE measurement results.

The preparation of multiple LTM candidate cells belonging to a candidate CU can take place in parallel (similar as CHO).

Proposal 2. Similar as in Rel-18 intra-CU LTM, the source CU uses the UE Context Setup procedure with its underlying candidate DUs for preparation of LTM candidate cells that belong to it.

Proposal 3. A candidate CU uses the UE Context Setup procedure with its underlying candidate DUs for preparation of LTM candidate cells that belong to it.

Proposal 4. Similar as in Rel-18 intra-CU LTM, a candidate DU determines whether the suggested candidate cell can be prepared, and if the suggested candidate cell can be prepared, the candidate DU determines the lower layer configuration (CellGroupConfig) for the prepared candidate cell.

Proposal 5. An underlying candidate DU of a candidate CU transmits the lower layer configuration (CellGroupConfig) of the prepared candidate cell, along with the candidate cell ID to the candidate CU. The candidate CU then forwards the CellGroupConfig of the prepared candidate cell to the source CU along with the candidate cell ID over Xn in Handover Request Ack.

Proposal 6. The source CU generates the upper layer configuration, e.g., the radioBearerConfig containing, e.g., the PDCP configuration, for the candidate cell, adds it to the provided CellGroupConfig to form the RRC configuration of the candidate cell.

Early TA RACH configuration

Proposal 7. During LTM preparation, the source gNB-CU may transmit a request to a candidate gNB-CU for obtaining RACH configuration and for allocation of CFRA resources, e.g., RAP IDs, for early TA acquisition. The request uses UE-associated signaling and includes:

- LTM candidate cell ID,

- List of gNB-DU IDs of source gNB-CU.

Proposal 8. Upon receiving the request from the source gNB-CU, the candidate gNB-CU forwards the request to its underlying candidate gNB-DU including:

- LTM candidate cell ID,

- List of gNB-DU IDs of source gNB-CU,

- The source gNB ID.

Proposal 9. Upon receiving the request, the candidate gNB-DU determines the split of the available CFRA resources among the indicated gNB-DUs of the source gNB, and stores this information along with the source gNB ID.

Proposal 10. In the response message, the candidate DU provides to the candidate CU:

- LTM candidate cell ID,

- List of pairs of the form (gNB-DU ID, CFRA resource set),

- The source gNB ID.

Proposal 11. The candidate CU forwards the following information to the source CU in a message over Xn:

- LTM candidate cell ID,

- List of pairs of the form (gNB-DU ID, CFRA resource set).

Proposal 12. Upon receiving the message from the candidate CU, the source CU forwards each of the set of CFRA resources to the corresponding underlying DU, indicating also the LTM candidate cell ID.

TA Information Transfer

Proposal 13. Upon successfully receiving the RACH preamble from the UE on a candidate cell, the candidate DU determines the TA value and determines the source gNB ID. The candidate DU transmits to the candidate CU the following TA information using non-UE associated signalling message:

- Determined TA value,

- RACH resource information (Detected RAP ID, RO information (RA-RNTI)),

- Source gNB ID,

- LTM candidate cell ID.

Proposal 14. The candidate CU forwards the TA information to the source CU using non-UE associated signalling over Xn.

Proposal 15. The source CU identifies the serving DU based on the provided TA information, and forwards this information to the serving DU.

LTM CSI configuration

Proposal 16. Source CU generates the LTM CSI resource configuration for the candidate cells, i.e., the list of SSBs to be measured by the UE for each candidate cell, based on:

- Information received from the candidate CUs and its own underlying DUs, in XN Setup and F1 Setup procedures, as mentioned in the assumptions above,

- RRC measurement report.

Proposal 17. The source CU provides to a candidate CU in Handover Request:

- Suggested candidate cell ID,

- LTM CSI resource configuration.

This information is forwarded by the candidate CU to its underlying candidate DU.

Proposal 18. A candidate DU provides to the candidate CU the following information, which is then forwarded by the candidate CU to the source CU using Handover Request Ack:

- The prepared candidate cell ID and its associated lower layer and LTM CSI report configurations.

Proposal 19. The source CU may update the LTM CSI resource configuration based on the prepared cells of its underlying candidate DUs and those of the candidate CUs.

Proposal 20. The source CU may initiate configuration modification with the candidate CUs and its own underlying DUs, providing the updated LTM CSI resource configuration.

- The candidate DUs provide updated LTM CSI report configurations to the source CU via the candidate CU.

The source CU forms the final RRC reconfiguration message to be provided to the UE, that includes the RRC configurations that are generated by it for all the prepared LTM candidate cells.

LTM Cell Switch Notification

Proposal 21. Upon transmitting the LTM Cell Switch Command to the UE, the source DU transmits the DU-CU Cell Switch Notification message to the source CU, including the target cell ID and target TCI state ID(s) in the message.

Proposal 22. Upon receiving the DU-CU Cell Switch Notification message, the source CU may transmit a Cell Switch Notification message to the candidate CU, including the target cell ID and target TCI state ID(s) in the message.

Proposal 23. Upon receiving the Cell Switch Notification message from the source CU, the candidate CU transmits the CU-DU Cell Switch Notification message to the candidate DU, including the target cell ID and target TCI state ID(s) in the message.

# References

[1] Chair notes, RAN3 #124 meeting.

[2] R3-244228, RAN3 #125 meeting contribution, Inter-gNB LTM with no change of RRC/PDCP anchor, Qualcomm Incorporated, NTT DOCOMO, Vodafone, Bharti AirTel (India), Sony.

[3] Chair notes, RAN2 #125-bis meeting.