3GPP TSG-RAN WG3 Meeting #125                                                                  R3-244231

Maastricht, NL, Aug 19th – 23rd, 2024

Source:	CATT

Title:	(TP to TR 38.799) On WAB architecture and authorization

Agenda Item:	12.2

Document for:	Discussion and Decision

1 Introduction

In the last RAN3 meeting [1], following conclusions included in the chairman notes were made for WAB:

Adopt the following terminology: BH-RAN-node: This is an NG-RAN node serving the WAB-MT. BH-gNB: The gNB serving the WAB-MT. BH-AMF: The AMF serving the WAB-MT. BH-5GC: The 5GC serving the WAB-MT. BH-UPF: The UPF serving the WAB-MT for backhauling. UE´s 5GC: The 5GC connected to the WAB-gNB and serving the UEs. UE´s AMF: The AMF connected to the WAB-gNB and serving the UEs. UE´s UPF: The UPF connected to the WAB-gNB and serving the UEs. Whether a BH RAN node can be unaware of WAB needs to be further studied. It is desirable if a WAB unaware BH RAN node could be supported so long as performance is within acceptable limits and needed functionalities can be supported. Chairman suggests that the work on functionalities is carried on first before it is decided whether a BH RAN node may or may not be WAB aware. WAB Authorization RAN3 understands that authorization of the WAB-MT is different from the WAB-gNB service authorization/configuration/activation by e.g. OAM/SeGW. WAB-MT authorization provides the WAB MT with the right to support BH PDU sessions. In case MT authorization is based on slice, signaling enhancements to the AS layer for the support of WAB-MT authorization are not needed. SA2 to decide whether MT authorization based on slicing is supported. Authorization of WAB-gNB provides service authorization, i.e., the right to serve UEs. It is RAN3 understanding that WAB-gNB’s authorization status can change. The UEs served by a WAB-gNB whose authorization status changes from “authorized” to “not authorized” can either be handed over to other RAN nodes or released. The NG connection(s) of a WAB-gNB whose authorization status changes from “authorized” to “not authorized” can be removed. It needs to be further studied whether they can be suspended. Any Stage 3 decisions are to be taken during normative work.

In this contribution, we will continue to discuss the leftover issues on architecture and high-level aspects for WAB.

2 Discussion

2.1 WAB awareness issue

In (mobile) IAB, the IAB-donor-CU must be able to identify a (m)IAB-MT. That is because IAB traffic is backhauled through BAP sublayer, the IAB-donor-CU has to carry out BAP configuration, IP addresses configuration and perform necessary procedures relative with topology adaptation. As (m)IAB-MT has no PDU session, QoS of different types of IAB-node’s traffic is categorized by IAB-donor-CU and resource of BH links is managed by IAB-donor-CU itself to guarantee the QoS. So, the gNBs must be upgraded to support IAB specific functions to act as IAB-donors. Accordingly, the IAB-donor-CU must broadcast an IAB-support indicatior for differentiation with legacy gNBs and it should be aware of (m)IAB-MT’s access to carry out the special handling for the (m)IAB-MT.

However, different with (mobile) IAB, WAB utilizes the legacy Uu interface on backhaul link. The traffic of WAB-gNB is transmitted via BH PDU session of WAB-MT, which means the QoS guarantee can rely on existing QoS mechanism of PDU session. To guarantee the QoS of different types of traffic, the WAB-MT can decide the mapping between IP flows of different traffic types (such as NG-U, NG-C, OAM, Xn) of WAB-gNB and QoS flows of BH PDU session(s), then decide whether to request modifying an existing PDU session or establishing a new PDU session. That mechanism only relies on WAB-node’s implementation and does not need upgrade of legacy NG-RAN nodes. For example, the WAB-MT can request to modify an existing BH PDU session if QoS of the BH PDU session backhauling NG-U of WAB-gNB cannot satisfy the QoS of UE’s PDU sessions which are newly established.

So, the legacy NG-RAN nodes do not need specific upgrade to act as BH-RAN-nodes. And therefore, the broadcast of WAB-support indicator and awareness of WAB are not needed in BH-RAN-nodes.

Observation 1-1: Different with IAB, the legacy NG-RAN nodes do not need upgrade to act as BH-RAN-nodes. Broadcasting WAB-support indicator or awareness of WAB is not necessarily required for BH-RAN-node.

It has been captured in the latest TR 23.700-06 [2] by SA2 that

- There is no need to specify AS level indication in RRC connection establishment that the MWAB-UE intends to operate as MWAB. - There is no need to indicate to NG-RAN serving a MWAB-UE that the MWAB-UE is authorized to act as MWAB-UE.

In (mobile)IAB, the AS level indication in RRCSetupComplete message is used to indicate to RAN the terminal intends to operate as (mobile)IAB-node and help the RAN to select an AMF. Whereas, the RAN awareness of WAB-node is not necessary and the AMF selection can be done via slice mechanism in WAB.

In case some NG-RAN nodes or AMF do not allow/support access of WAB due to the deployment purpose of operator, the existing slice mechanism can be reused for availability of WAB support in part of the NG-RAN network. Dedicated slice(s) for WAB operation can be pre-configured and the allowed S-SSAIs for WAB-MT can be chosen from those dedicated slices. WAB-MT includes the dedicated WAB S-NSSAI(s) in RRCSetupComplete message. Admission or reject of WAB-MT access depends on the support or available resource for the dedicated WAB slice(s) in NG-RAN-node within a specific TA. If the NG-RAN-node admits the WAB-MT’s access, it selects AMF based on the received WAB S-NSSAI(s). Then the selected AMF can authorize the WAB-MT if it supports WAB S-NSSAI(s). Handover of WAB-MT also relies on the support of WAB S-NSSAI(s) in the target NG-RAN node and it’s handled per PDU session. If the WAB-MT is handed over to a NG-RAN-node not supporting WAB slice(s), AMF can decide to de-authorize the WAB-MT and release BH PDU sessions for WAB operation.

Nevertheless, the BH-RAN-nodes and AMFs do not need to really recognize/know what slices are used for WAB operation. There is no need to broadcast a WAB-support by NG-RAN nodes either. Then no RAN/5GC update is needed considering the support of WAB in partial area.

Observation 1-2: Supporting WAB-MT access in partial area of NG-RAN can be achieved by slice mechanism. However, awareness of the dedicated slice(s) of WAB is not required for RAN and 5GC. There is no need for NG-RAN nodes to broadcast WAB-support indicator.

Rel-19 may introduce RAN enhancements to improve the performance of WAB, such as resource multiplexing between BH link and access link which requires awareness of WAB-node, but that should not be mandatory.

WAB-MT can access to any RAN node applying PDU session, and the principle is that WAB-MT cannot be served by a WAB-gNB to prevent multi-hops.

Proposal 1: Awareness of WAB is not required for RAN, and no need for NG-RAN nodes to broadcast WAB-support indicator.

2.2 WAB-MT access restriction

SA2 has progressed Rel-19 VRM study. One out LS [3] was sent from SA2 for asking RAN to discuss several issues and feedback. The issue to restrict WAB-MT accessing a WAB-gNB cell is involved:

S2-2407345 Issue-1: SA2 has discussed the access control (to restrict MWAB-UE accessing MWAB-gNB cell) for below two cases: a) MWAB-UE accessing any MWAB-gNB in IDLE and CONNECTED mode(documented as one candidate solution in clause 6.2.3.6 of TS 23.700-06); and b) MWAB-UE accessing MWAB-gNB belonging to same MWAB. SA2 assume that those scenarios will be discussed by RAN WGs, and ask for RAN feedback.

IDLE or INACTIVE WAB-MT access restriction

For idle or inactive mode WAB-MT, case b) is invalid. Since WAB-gNB will not be started up until the WAB-MT connecting with the network via integration procedure, it’s impossible for an idle/inactive WAB-MT to camp on its co-located WAB-gNB. For case a), there are two potential solutions:

- Solution 1: BH-AMF guarantee during network integration

As BH-AMFs can recognize WAB-MT via subscription data, BH-AMF can decide not to authorize or refuses the registration request for the WAB-MT if it accesses via a WAB-node.

If WAB-MT is connecting with a WAB-gNB, it should not be authorized to operate as a WAB-node, so that multiple layers of BH PDU session can be avoided. In this case, BH PDU session for WAB should not be established when WAB is not authorized. If no PDU session is established or non-authorized WAB is not allowed to be registered, BH-AMF refuses the registration request.

- Solution 2: WAB-MT guarantee

Cells of WAB-nodes broadcast a WAB-Cell indicator.

WAB-MT does not camp on a WAB cell, or deprioritize the reselection to a WAB cell. During cell selection/reselection, a WAB-MT can consider the cell broadcasting the WAB-Cell indicator as barred so that it does not camp on a WAB cell when it is operating as WAB-node. Alternatively, the WAB-MT can consider the frequency of the cell as lowest priority to deprioritize reselection to the cell.

Per SA2’s latest conclusion [2], the WAB-gNB indicates to AMFs that it is a WAB-node when establishing NG connections to the AMFs. So, BH-AMF can realize a WAB-MT is accessing via a WAB-gNB or not. That means Solution 1 is naturally supported. In addition, the RAN enhancement as Solution 2 can be considered to reduce the possibility of WAB-MT selecting a WAB cell and therefore eliminates integration failure. Solution 2 should be support too. The usage of the WAB-Cell indicator is up to RAN2 determination.

Observation 2-1: Given WAB-gNBs send WAB-node indicator to the connected AMFs, BH-AMF can guarantee single-hop of WAB during integration procedure. For optimization, a WAB cell can broadcast a WAB-Cell indicator.

Proposal 2-1: BH-AMF can guarantee the idle/inactive mode WAB-MT not accessing to a WAB-node during WAB-node integration procedure. For optimization, a WAB cell can broadcast a WAB-Cell indicator.

CONNECTED WAB-MT access restriction

For case a) and case b), two ways to prevent handover of connected mode WAB-MT to a WAB-node:

- Solution 1: BH-AMF guarantee

BH-AMF de-authorizes the WAB-MT or refuses the handover of WAB-MT, if target RAN-node is a WAB-node.

NG based handover:

On reception of Handover Required message for WAB-MT from source node, if the target node is a WAB-node, the BH-AMF can refuse the handover requirement or turn the WAB-MT from “authorized” to “non-authorized” after handover. If WAB-MT is handed over to the WAB-node and turned to “non-authorized”, all BH PDU sessions should be release. If no PDU session is existed or non-authorized WAB-MT is not allowed to be registered, BH-AMF can deregister the WAB-MT.

Xn based handover:

BH-AMF does not know the handover is pertaining to a WAB-MT until path switch procedure. Upon receiving the path switch request, BH-AMF can turn the WAB-MT from “authorized” to “non-authorized” or deregister the WAB-MT if the target node is a WAB-node.

- Solution 2: WAB-MT guarantee

For case b), WAB-MT can be aware of the cell identity of its co-located WAB-gNB, so it does not measure the cell of co-located WAB-gNB.

For case a), WAB cell can broadcast a WAB-Cell indicator. WAB-MT does not report the measurement results of the cell(s) broadcasting the WAB-Cell indicator that the BH-RAN-node will not select the WAB cells as the target cells. To reduce the WAB-MT complexity on reading the broadcast information of neighbour cells, RAN enhancement can be considered. For example, the BH-RAN-node can indicate cell ID(s) of the neighbouring WAB cell(s) to WAB-MTs. This solution needs discussion of RAN2.

Solution 1 is supported without effort if WAB-gNBs can send the WAB-node indicator to the connected AMFs, as BH-AMF is aware of WAB-MT. Differently, solution 2 can actively avoid the handover of WAB-MT to WAB-gNB, thus it is beneficial in preventing the handover failure or de-authorization. Both solutions should be supported.

Observation 2-2: Given WAB-gNBs can indicate to AMF with a WAB-node indicator, BH-AMF can prevent WAB-MT handovers to WAB-gNBs. For optimization, a WAB cell can broadcast a WAB-Cell indicator.

Proposal 2-2: BH-AMF can prevent connected mode WAB-MT to be handed over to a WAB-node. For performance optimization, a WAB cell can broadcast a WAB-Cell indicator.

Proposal 2-3: RAN3 send response LS to SA2 on P2-1 and P2-2 as Annex B.

2.3 Support of BH-ng-eNB

According to TS 38.300 [4], NG-RAN consists of two kinds of RAN nodes, i.e., gNB and ng-eNB. Both gNB and ng-eNB have NG interface with 5GC, and they use Xn interface between each other. Ng-eNB uses E-UTRA instead of NR in Uu interface, and it is contained in 5G system with the purpose of making use of Uu interface of E-UTRAN to improve 5G coverage.

As no specific function of WAB is required and WAB-gNB traffic is backhauled via PDU session, from the basic functionality and performance aspects, awareness of WAB is not required for ng-eNB. The legacy ng-eNBs do not need upgrade for acting as BH-RAN-nodes.

Also, as analysis above, the constraint for WAB-MT’s access in specific area can be achieved by existing slice mechanism. Slice is introduced to all NG-RAN nodes in Rel-15, so ng-eNBs support slice too. E-UTRA RRCSetupComplete message included the S-NSSAI-list element as early as Rel-15. If the WAB-MT accesses a ng-eNB, the ng-eNB can admit or reject the WAB-MT’s access and select AMF based on the received WAB S-NSSAI(s). Thus, ng-eNB can also support the scenario where WAB access is available in partial area.

On WAB-MT access restriction, according to analysis above, if WAB-gNBs send WAB-node indicator to the connected AMFs, 5GC can guarantee the restriction of WAB-MT access to a WAB-node for both idle/inactive mode and connected mode WAB-MTs. Although some RAN enhancements are proposed to optimize the performance, those enhancements only target to WAB-gNB and WAB-MT. The enhancement to BH-RAN is not mandatorily required, thus it would be very limited as can be expected.

Also, ng-eNB can support establishing Xn interface with WAB-node if it acts as the BH-RAN-node of WAB-MT.

Observation 3-1: No update to ng-eNB is needed for it acting as BH-RAN-node. Although some enhancements to BH-RAN can be considered to optimize the performance, that should be optional and limited, thus the impact to E-UTRA is neglectable.

Considering ng-eNB can serve WAB-MT without update, and restricting WAB-MT to access ng-eNBs may result in out of service of WAB-node in the area served by ng-eNBs, RAN3 is proposed to introduce BH-ng-eNB as the BH-RAN-node.

Proposal 3-1: RAN3 introduce BH-ng-eNB, and define it as the ng-eNB serving the WAB-MT.

As ng-eNB is one kind of NG-RAN node, the stack protocols for NG/Xn control and user plane which are captured in the TR[5] are also applicable when ng-eNB act as the BH-RAN-node. Therefore, there should be few changes to TR for introducing BH-ng-eNB. We only need to change “NR Uu Backhual link” to “NG-RAN Backhaul link” in each figure of the TR.

Proposal 3-2: Change “NR Uu Backhual link” to “NG-RAN Backhaul link” in each figure of the TR[5].

If ng-eNB serves as BH-RAN-node, we assume it would require enhancements to E-UTRA Uu or Xn interface for the function of resource multiplexing between BH link and access link. In addition, the resource coordination mechanism introduced for IAB can only be reused in case of BH-gNB, since WAB-MT connecting to ng-eNB using E-UTRA Uu over BH link is different from the IAB scenario where NR Uu is used over BH link. The resource multiplexing between E-UTRA BH link and NR access link should be evaluated by RAN1 in the first place.

Observation 3-2: The resource multiplexing between E-UTRA BH link and NR access link would introduce impacts to E-UTRAN Uu or Xn interface, and it requires the effort of RAN1.

Nevertheless, resource multiplexing is not mandatory to support WAB. WAB-node can guarantee the NR access link does not interfere with E-UTRA BH link by internal implementation. So, Rel-19 does not need to specify the resource coordination for E-UTRA BH link and NR access link for WAB.

Proposal 3-3: RAN3 exclude the resource multiplexing between E-UTRAN BH link and NR access link from the scope of WAB.

2.4 Location of BH-UPF

According to the architecture of WAB [5], all the WAB-gNB traffic is backhauled by the WAB-MT and the BH-RAN-node via the BH link. All the WAB-gNB traffic will be transported through the BH-UPF as the BH PDU sessions are terminated at the BH-UPF. The BH-UPF takes the role of forwarding IP packets of NG/Xn connections of the WAB-gNB from the BH PDU session(s) to the core network and other NG-RAN nodes, or mapping the IP packets from core network or other NG-RAN nodes to the BH PDU session(s).

It’s beneficial to locate the BH-UPF nearby or with the BH-RAN-node for improving the transport efficiency for WAB-gNB traffic. If BH-UPF is co-located with the BH-RAN-node, GTP-U protocol stack between BH-UPF and BH-RAN-node can be omitted.

Proposal 4: The BH-UPF for BH PDU session can be located nearby or with the BH-RAN-node. If BH-UPF is co-located with the BH-RAN-node, GTP-U protocol stack between BH-UPF and BH-RAN-node can be omitted.

The U-plane and C-plane protocol stacks for NG backhauling, where BH-UPF locates in the BH-RAN-node, can be illustrated by Figure 1 and Figure 2 as below.

Figure 1 U-plane protocol stack of WAB for NG-U backhauling

Figure 2 C-plane protocol stack of WAB for NGAP backhauling

The C-plane and U-plane protocol stacks for Xn backhauling are shown in Figure 3 and Figure 4 respectively, where BH-UPF is located at the BH-RAN-node.

Figure 3 C-plane protocol stack of WAB for XnAP backhauling

Figure 4 U-plane protocol stack of WAB for Xn-U backhauling

2.5 WAB authorization

It has been agreed in the last meeting [1] that authorization of the WAB-MT is different from the WAB-gNB service authorization by, e.g., OAM/SeGW using legacy procedures, including following understandings:

Authorization of the WAB-MT provides the WAB-MT with the right to support backhauling the co-located WAB-gNB’s traffic via BH PDU session(s). When the WAB-gNB’s service authorization status is changed from “authorized” to “non-authorized”, the UEs served by the WAB-gNB are removed, then NG connection(s) of a WAB-gNB can be removed.

However, following issues are not clear:

- The exact performance for WAB-node when the WAB-MT authorization status changes.

- The actions when authorization status of WAB-MT/WAB-gNB changes from “not authorized” to “authorized”.

- Whether NG connection can be suspended when authorization status of WAB-MT/WAB-gNB changes to “not authorized”.

WAB-MT authorization status change:

If WAB-MT authorization status is changed from “authorized” to “not-authorized”, the BH PDU session(s) of the WAB-MT should be released so that the WAB-gNB’s traffic cannot be backhauling via BH PDU session(s). Since the OAM connection, NG interface and Xn interface are all provided over the IP connectivity of BH PDU session(s), the peer ends are not reachable any more after the BH PDU session(s) being teared down, though the service of WAB-gNB is still authorized. As a consequence, the WAB-gNB cannot serve the UEs, the UEs should be removed.

The issue is whether the NG connection should be removed or kept (i.e., suspended). Due to the peer ends on those interfaces/connection are not reachable during the WAB-MT de-authorized period, the WAB-gNB anyway has to re-synchronize with the peer ends when the BH PDU session(s) are available again, so there is no need to maintain the interfaces of NG/Xn including the SCTP connection. The WAB-gNB should be shut down as legacy gNB.

If the status is changed from “not-authorized” to “authorized”, the BH PDU session(s) are established again and the WAB-node performs the WAB-gNB setup, i.e., phase 2 of the integration procedure.

Proposal 5-1: For WAB-MT authorization status change,

- If the status is changed from “authorized” to “not-authorized”, the WAB-gNB removes the UEs, then it shuts down as legacy gNB, after which BH PDU session(s) of WAB-MT are released;

- If the status is changed from “not-authorized” to “authorized”, the WAB-MT’s BH PDU session(s) are established again and the WAB-gNB setup in phase 2 of integration procedure is performed.

WAB-gNB service authorization status change:

If service authorization status is changed to “not-authorized”, the WAB-gNB has no right to serve UEs. If the WAB-gNB is not authorized by SeGW, the WAB-gNB NG/XN interface cannot be backhauled via the IP connectivity provided by the BH PDU session, which means the AMF(s) and neighbour NG-RAN node are not reachable any more, thus it makes no sense for WAB-gNB to keep (or suspend) the NG/Xn interface. The WAB-gNB should turn off the NG/Xn interface including the SCTP connection, in which case NG removal procedure is not required. Before that, the UEs served by WAB-gNB have to be released or handed over.

If the WAB-gNB is not authorized by OAM, the AMF(s) and neighbour RAN nodes are still reachable because the BH PDU session providing the IP connectivity is available. The WAB-gNB can maintain the underlying SCTP connection with the AMF(s) and neighbour RAN nodes. The WAB-gNB’s NG/Xn connections can be removed. It’s FFS whether NG connections can be suspended. Due to the NG connection is not available when the WAB-gNB is not authorized, the WAB-node has to notify the AMF(s) to stop using the NG connection. And the WAB-node should inform the AMF(s) to activate the NG connection when the WAB-gNB is authorized again. That can be realized by NG removal and NG setup procedures, thus, NG suspension is not needed in case the WAB-gNB is de-authorized.

Observation 5: If the WAB-gNB is not authorized by OAM, the NG/Xn connection of the WAB-gNB can be removed with the SCTP connection maintained. NG suspension is not needed.

The WAB-gNB de-authorization should not impact the BH PDU session. Because the connection to OAM/SeGW is still backhauled over the IP connectivity of the BH PDU session. If the BH PDU session(s) are all released, OAM/SeGW will not be reachable. So, there is no need to release BH PDU session(s) when WAB-gNB service authorization status is changed to “not-authorized”.

If the status is changed from “not-authorized” to “authorized”, the WAB-gNB re-establish NG (and Xn) interface.

Proposal 5-2: For WAB-gNB service authorization status change,

- If the status is changed from “authorized” to “not-authorized”, the WAB-gNB can remove the NG/Xn interface while maintaining the SCTP connection; No need to introduce NG suspension procedure.

- If the status is changed from “not-authorized” to “authorization”, the WAB-gNB re-establish NG connection (or Xn connection as well).

3 Conclusion

In the previous sections we made the following observations and proposals:

WAB awareness issue

Observation 1-1: Different with IAB, the legacy NG-RAN nodes do not need upgrade to act as BH-RAN-nodes. Broadcasting WAB-support indicator or awareness of WAB is not necessarily required for BH-RAN-node.

Observation 1-2: Supporting WAB-MT access in partial area of NG-RAN can be achieved by slice mechanism. However, awareness of the dedicated slice(s) of WAB is not required for RAN and 5GC. There is no need for NG-RAN nodes to broadcast WAB-support indicator.

Proposal 1: Awareness of WAB is not required for RAN, and there is no need for NG-RAN nodes to broadcast WAB-support indicator.

WAB-MT access restriction

Observation 2-1: Given WAB-gNBs send WAB-node indicator to the connected AMFs, BH-AMF can guarantee single-hop of WAB during integration procedure. For optimization, a WAB cell can broadcast a WAB-Cell indicator.

Proposal 2-1: BH-AMF can guarantee the idle/inactive mode WAB-MT not accessing to a WAB-node during WAB-node integration procedure. For optimization, a WAB cell can broadcast a WAB-Cell indicator.

Observation 2-2: Given WAB-gNBs can indicate to AMF with a WAB-node indicator, BH-AMF can prevent WAB-MT handovers to WAB-gNBs. For optimization, a WAB cell can broadcast a WAB-Cell indicator.

Proposal 2-2: BH-AMF can prevent connected mode WAB-MT to be handed over to a WAB-node. For optimization, a WAB cell can broadcast a WAB-Cell indicator.

Proposal 2-3: RAN3 send response LS to SA2 on P2-1 and P2-2 as Annex B.

Support of BH-ng-eNB

Observation 3-1: No update to ng-eNB is needed for it acting as BH-RAN-node. Although some enhancements to BH-RAN can be considered to optimize the performance, that should be optional and limited, thus the impact to E-UTRA is neglectable.

Proposal 3-1: RAN3 introduce BH-ng-eNB, and define it as the ng-eNB serving the WAB-MT.

Proposal 3-2: Change “NR Uu Backhual link” to “NG-RAN Backhaul link” in each figure of the TR[5].

Observation 3-2: The resource multiplexing between E-UTRA BH link and NR access link would introduce impacts to E-UTRAN Uu or Xn interface, and it requires the effort of RAN1.

Proposal 3-3: RAN3 exclude the resource multiplexing between E-UTRAN BH link and NR access link from the scope of WAB.

Location of BH-UPF

Proposal 4: The BH-UPF for BH PDU session can be located nearby or with the BH-RAN node. If BH-UPF is co-located with the BH-RAN-node, GTP-U protocol stack between BH-UPF and BH-RAN-node can be omitted.

WAB authorization

Proposal 5-1: For WAB-MT authorization status change,

- If the status is changed from “authorized” to “not-authorized”, the WAB-gNB removes the UEs, then it shuts down as legacy gNB, after which BH PDU session(s) of WAB-MT are released;

- If the status is changed from “not-authorized” to “authorized”, the WAB-MT’s BH PDU session(s) are established again and the WAB-gNB setup in phase 2 of integration procedure is performed.

Observation 5: If the WAB-gNB is not authorized by OAM, the NG/Xn connection of the WAB-gNB can be removed with the SCTP connection maintained. NG suspension is not needed.

Proposal 5-2: For WAB-gNB service authorization status change,

- If the status is changed from “authorized” to “not-authorized”, the WAB-gNB can remove the NG/Xn interface while maintaining the SCTP connection; No need to introduce NG suspension procedure.

- If the status is changed from “not-authorized” to “authorization”, the WAB-gNB re-establish NG connection (or Xn connection as well).

4 Reference

- Draft Report of 3GPP TSG RAN3 meeting #124, MCC

- 3GPP TR 23.700-06, Study on architecture enhancements for vehicle-mounted relays - Phase 2, v 0.3.0 (2024-04)

- S2-2407345, LS on questions regarding FS_VMR_Ph2, SA2#163

- 3GPP TS 38.300, NR and NG-RAN Overall Description, Stage-2, v18.0.0 (2023-12)

- R3-243970, 3GPP TR 38.799,  Study on additional topological enhancements for NR, v0.0.1 (2024-04)

5 Annex A (TP to TR 38.799)

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>Start of Changes<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

BH-RAN-node: The NG-RAN node serving the WAB-MT.

BH-gNB: The gNB serving the WAB-MT.

BH-ng-eNB: The ng-eNB serving the WAB-MT.

BH-AMF: The AMF serving the WAB-MT.

BH-5GC: The 5GC serving the WAB-MT.

BH-UPF: The UPF serving the WAB-MT for backhauling.

UE´s 5GC: The 5GC connected to the WAB-gNB and serving the UEs.

UE´s AMF: The AMF connected to the WAB-gNB and serving the UEs.

UE´s UPF: The UPF connected to the WAB-gNB and serving the UEs.

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>Next Change<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<

# 4	Wireless Access and Backhaul (WAB)

## 4.1	General

The study is based on the following requirements:

- The WAB-node includes a gNB component (WAB-gNB) and an MT component (WAB-MT).

- The WAB-gNB is based on the gNB functionality specified in TS 38.300 [x] and TS 38.401 [y].

- The CU-DU split of the WAB-gNB is not considered in this study.

- The WAB-MT supports at least a subset of UE functionalities.

- The NR Uu is used for the radio link between WAB-gNB and the served UEs.

- The NR Uu radio link between the WAB-gNB and the served UEs does not use NTN.

- The study focuses on NR-Uu backhaul.

- In-band scenario for access and backhaul is not precluded to be studied.

- The study precludes the scenario where the access and the backhaul are in-band while the backhaul uses NTN.

- The study focuses on the use of WAB-MT´s PDU session via NG-RAN as backhaul of WAB-gNB. Other options for the backhaul (including non-3GPP radio technology) are not precluded but are not a part of the study.

- A WAB-gNB cannot serve WAB-MT(s).

- A BH-RAN-node can be unaware of WAB.

- The study includes a scenario where the WAB-gNB and the WAB-MT connect to the same PLMN or to different PLMNs.

- The WAB-MT may connect to a public PLMN or an SNPN.

- The WAB-gNB may connect to a public PLMN or an SNPN.

- Legacy UEs can connect to the WAB-gNB. There are no WAB-specific enhancements for UEs that connect to the WAB-gNB.

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>Next Change<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<

## 4.2	WAB Architecture

To improve the transport efficiency for WAB-gNB traffic, the BH-UPF can be located nearby or with the BH-RAN-node.

Figure 4.2-X shows protocol stack examples of NG Control plane and User plane transport for a UE connected to the network via a WAB-node when the BH-UPF is co-located with the BH-RAN-node, in which case the GTP-U protocol stack between the BH-UPF and the BH-RAN-node can be omitted.

Figure 4.2-X: Protocol stack examples of NG Control plane and User plane transport for a UE connected via WAB-node when BH-UPF is located at the BH-RAN-node

Figure 4.2-Y shows protocol stack examples of Xn Control plane and User plane transport for a UE connected to the network via a WAB-node when the BH-UPF is co-located with the BH-RAN-node, in which case the GTP-U protocol stack between the BH-UPF and the BH-RAN-node can be omitted.

Figure 4.2-Y: Protocol stack examples of Xn Control plane and User plane transport when BH-UPF is located at the BH-RAN-node

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>Next Change<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<

## 4.3	Operational aspects

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>Unchanged part is skipped <<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<

### 4.3.2	WAB authorization

WAB authorization includes the authorization of the WAB-MT and the service authorization of the WAB-gNB. The authorization of the WAB-MT is different from the service authorization/configuration/activation of the WAB-gNB.

Authorization of the WAB-MT provides the WAB-MT with the right to support backhauling the co-located WAB-gNB’s traffic via BH PDU session(s). The WAB-MT’s authorization status may change. In case the WAB-MT’s authorization status changes from “authorized” to “not authorized”, the UEs served by the WAB-gNB are handed over or released from the WAB-gNB. The WAB-gNB shuts down as legacy gNB, after which the BH PDU session(s) of WAB-MT are released. If the WAB-MT’s authorization status changes from “not authorized” to “authorized”, the WAB-MT’s BH PDU session(s) are re-established and the WAB-gNB setup in phase 2 of integration procedure is performed.

Authorization of the WAB-gNB provides the service authorization, i.e., the right to serve UEs. The service authorization of the WAB-gNB is performed by e.g., OAM/SeGW using legacy procedures.

The WAB-gNB’s service authorization status may change. In case the WAB-gNB’s service authorization status changes from “authorized” to “not authorized”, the UEs served by the WAB-gNB can either be handed over to other RAN nodes or they can be released, after which the NG and Xn connection(s) of the WAB-gNB can be removed with the SCTP connections with the AMF(s) and the neighbour NG-RAN nodes maintained. If the WAB-gNB’s authorization status changes from “not authorized” to “authorized”, the WAB-gNB re-establishes the NG connection or re-establish the Xn connection if needed.

### 4.3.x WAB-MT access restriction

In principle, the WAB-MT can access any NG-RAN node except WAB-node.

To restrict an idle or inactive mode WAB-MT accessing a WAB-gNB cell during integration procedure, the BH-AMF does not authorize the WAB-MT or refuses the registration request of the WAB-MT. The BH-AMF can prevent a CONNECTED mode WAB-MT being handed over to a WAB-gNB cell. If the target node of the WAB-MT is a WAB-gNB, the BH-AMF can refuse the handover or de-authorize the WAB-MT after the handover.

To prevent a WAB-node to serve WAB-MT(s), the WAB-gNB cell can broadcast a WAB-Cell indicator. The idle/inactive mode WAB-MT(s) does not camp on the cells broadcasting the WAB-Cell indicator. The connected mode WAB-MT(s) does not measure the cells broadcasting the WAB-Cell indicator.

The support of WAB in partial area of NG-RAN can rely on existing slice mechanism. Dedicated slice(s) for WAB operation can be pre-configured to WAB-MT and the WAB-MT indicates the dedicated slice(s) via the S-NSSAI list during the RRC connection establishment. Admission control of WAB-MT depends on the support or available resource for the dedicated slice(s) in BH-RAN-node within a specific TA. If the BH-RAN-node admits the WAB-MT’s access, it selects AMF based on the S-NSSAI(s) indicated by the WAB-MT. The selected AMF can authorize the WAB-MT if it supports the dedicated slice(s) for WAB operation. Handover of the WAB-MT can rely on support of dedicated slice(s) in the target NG-RAN node and it’s handled per PDU session.

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>End of Changes<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<

6 Annex B (Proposed response LS)

Title:	[DRAFT] Response LS on questions regarding FS_VMR_Ph2

Response to:	Reply LS on questions regarding FS_VMR_Ph2 (S2-2407345)

Release:	Rel-19

Work Item:	FS_NR_WAB_5GFemto

Source:	CATT (to be RAN3)

To:	SA2

Cc:	RAN2

Contact Person:

#### Name:	Yang Lu

Tel. Number:

E-mail Address:	luyang@catt.cn

Attachments:

1. Overall Description:

RAN3 thanks SA2 for providing issues on the usage of VMR after progressing the Rel-19 FS_VMR_Ph2 study and documented solution proposals in TR 23.700-06. RAN3 would like to provide the following response to SA2:

- Issue-1: SA2 has discussed the access control (to restrict MWAB-UE accessing MWAB-gNB cell) for below two cases:

- MWAB-UE accessing any MWAB-gNB in IDLE and CONNECTED mode(documented as one candidate solution in clause 6.2.3.6 of TS 23.700-06); and

- MWAB-UE accessing MWAB-gNB belonging to same MWAB.

SA2 assume that those scenarios will be discussed by RAN WGs, and ask for RAN feedback.

- RAN3 Answer: RAN3 believe that at least BH-AMF can guarantee the idle/inactive mode WAB-MT not accessing to a WAB-node during WAB-node integration procedure, and prevent connected mode WAB-MT to be handed over to a WAB-node for both case a) and b), given WAB-gNBs send a WAB-node indicator to the connected AMFs. For optimization, some solution from RAN side can be introduced, e.g., a WAB cell can broadcast a WAB-Cell indicator.

- Issue-2: SA2 has discussed and concluded that Additional ULI format will be determined by RAN WG and SA2 will align based on RAN feedback. Please refer to related conclusion in clause 8.5 of TR 23.700-06.

- RAN3 Answer: RAN3 has discussed this issue and conclude that the additional ULI includes the mapped TAC/Cell ID configured by OAM corresponding to geo-location of the WAB-node, if the PLMN serving the WAB-MT and the PLMN broadcasted by the WAB-gNB are not the same PLMN.

2. Actions:

To SA2 group:

ACTION: 	RAN3 kindly ask SA2 to take RAN3 answer into account.

3. Date of Next TSG-RAN WG3 Meetings:

TSG-RAN WG3#125	                    19th – 23rd August 2024			Maastricht, Netherlands

TSG-RAN WG3#125bis	                    14th – 18th October 2024			Hefei, China