3GPP TSG-RAN WG3 Meeting #125                                                                  R3-244232

Maastricht, NL, Aug 19th – 23rd, 2024

Source:	CATT

Title:	(TP to TR 38.799) Discussion on WAB mobility

Agenda Item:	12.2

Document for:	Discussion and Decision

1 Introduction

In the last RAN3 meeting [1], following conclusions included in the chairman notes were made for WAB mobility:

WAB mobility Support the following scenarios for intra-PLMN WAB-node mobility: - UE´s AMF/UPF remains unchanged as WAB-gNB moves inside a PLMN. - UE´s AMF/UPF changes as WAB-gNB moves inside a PLMN. - BH UPF/AMF remains unchanged as WAB-MT moves inside a PLMN. - BH UPF/AMF changes as WAB-MT moves inside a PLMN. Legacy mobility procedures can be supported for WAB-MT. RAN3 assumes that roaming scenarios for WAB-MT are supported. Establishment of Xn Connections of the WAB-gNB with BH-RAN nodes, as well as with surrounding RAN nodes, is supported and can follow legacy procedures. Addition of new information in the legacy procedures is not precluded Upon WAB-gNB mobility: If needed, the WAB-gNB may power up one or more new cells with new configuration parameters related to its current location and handover UEs between the old and new cell served by the WAB-gNB. FFS if this procedure is needed for AMF relocation. Discuss whether in case this procedure is used for AMF relocation, new cells have to belong to different logical gNBs or can belong to the same gNB.

In this contribution, we will continue to discuss the leftover issues on WAB mobility.

2 Discussion

2.1 Without change of UE’s AMF

From the last meeting [1], it was confirmed that the WAB-gNB’s configurations, e.g., the Cell ID/TAC/RANAC the WAB-gNB’s cell broadcasts, AMF address the WAB-gNB connects, are based on the location information of the WAB-node, and they are managed by OAM. The OAM can provide to the WAB-gNB those configurations when receiving location information of the WAB-MT or those configurations for different geographic locations of the WAB-MT. And it has been concluded by SA2 in the last SA2 meeting [2] that the TAC(s) and Cell ID(s) the WAB-gNB broadcasts should change when WAB-gNB changes the AMF it connects to due to mobility.

However, RAN3 didn’t confirm in the last meeting whether the TAC which the WAB-gNB broadcasts can be changed without change of AMF. In our understanding, the WAB-gNB may change the TAC it broadcasts without change of AMF during movement. As one AMF can take charge of more than one TA, the configuration of TAC for the WAB-gNB by OAM may be updated due to the new geolocation of the WAB-node while the configuration of AMF does not need to be changed for that geolocation.

The changed of TAC may trigger the Mobility Registration Update procedure of UEs which camp on or connects with the WAB-gNB’s cell(s).

Proposal 1-1: TAC the WAB-gNB cell broadcasts can be changed without the change of AMF the WAB-gNB connects with. The changed of TAC may trigger the Mobility Registration Update procedure of UEs which camp on or connects with the WAB-gNB’s cell(s).

When PCI that the WAB-gNB’s cell broadcasts is changed, intra-gNB handover should be conducted to connected UEs, so that the UEs are handed over from the cell of old PCI to the cell of new PCI. To reduce the interruption of UE’s traffic, the WAB-gNB can power up two cells corresponding to the old and new PCIs respectively during the intra-gNB handover, and shut down the cell of old PCI after all the UEs’ handover. Nevertheless, that would be a common-used method for changing the PCI of legacy gNB. And of course, alternatively the WAB-gNB can perform a hard old-to-new cell switch for conducting the intra-gNB handover. As this is pure implementation issue, it should not be considered in the study scope.

Proposal 1-2: When PCI that the WAB-gNB’s cell broadcasts changes, intra-gNB handover should be conducted to connected UEs. The implementation inside the WAB-gNB to support the intra-gNB handover is out of the study scope.

2.2 With change of UE’s AMF

When the WAB-gNB moves around over a long distance, there is a possible change of the AMF(s) the WAB-gNB connects with. The WAB-gNB may decide the change of AMF(s) when OAM provides the WAB-gNB with new AMF(s) corresponding to the new geolocation of the WAB-node, or the WAB-gNB decides the AMF(s) to connect should be changed based on the configurations for different geographic locations of the WAB-node from OAM. It has been agreed that the WAB-gNB should set up NG connection with the new AMF(s) in that case. But following issues need further check:

- Whether the gNB ID will be changed

- Whether power up two cells concurrently

- Whether a new logical WAB-gNB needs to be established to support handover of UEs

- How to handle the UEs to relocate the UE’s context to the new AMF(s)

Per SA2 conclusion [2], the TAC(s) and Cell ID(s) the WAB-gNB broadcasts should also change in that case, due to the new AMF(s) take charge of different TAs compared to the old AMF(s) and the new Cell ID(s) reflect the new geolocation of the WAB-node. The OAM may change gNB ID of WAB-gNB if the old gNB ID conflicts with the gNB ID of any gNB which has established NG connection with the new AMF(s). So, there is a possibility that the gNB ID of WAB-gNB is changed with the change of AMF(s).

Proposal 2-1: The gNB ID of WAB-gNB may be changed with the change of AMF(s).

It is also concluded by SA2 [2] that the change of Cell ID (including gNB ID) triggers NG-based handover of connected UE(s) (the old AMF decides the new AMF during the NG HO preparation). Just like the scenario of WAB-gNB mobility without AMF change, the WAB-gNB can power up two cells corresponding to the old and new configurations respectively to support an interruption-less NG-based HO, and shut down the old cell after all UEs’ handovers. In that case, the target cell can be selected based on measurement result to the cell of new configuration. However, the WAB-gNB also has the flexibility to perform a hard switch from the old configuration to the new configuration rather than broadcast two set of configurations simultaneously. If that’s the case, the target cell could not be selected based on measurement result of the new cell, and the old-to-new cell mapping inside the WAB-gNB may help.

So, it can be concluded when to activate the new cell(s) or whether to power up new cell(s) and old cell(s) concurrently is pure implementation issue, it’s up to WAB-gNB to do so.

Proposal 2-2: In case the AMF(s) change, NG-based handover should be conducted to the connected UEs. When to activate the new cell(s) and deactivate the old cell(s) is up to the WAB-gNB implementation.

The new logical WAB-gNB is supposed to broadcast the new TAC(s) as well as the new Cell ID(s), and connect with the new AMF(s). Although the WAB-gNB broadcasting the new configuration, it’s not clear what’s the necessity to implement a new logical WAB-gNB. The WAB-gNB should connect with the old AMF(s) and the new AMF(s) simultaneously during the NG handovers for UEs. While, on connecting with new AMF(s) using a new logical gNB, it is assumed based on the concept of target logical gNB-DU for mobile IAB-DU migration. The logical gNB-DU was introduced because a gNB-DU is only allowed to connect with one gNB-CU. However, one gNB can connect to more than one AMF in principle, which means there is no shown-stopper for one WAB-gNB connecting with the old AMF(s) and the new AMF(s) simultaneously. We understand present spec can support the implementation where the WAB-gNB includes different gNB-IDs (as well as TAIs) in the NG Setup Request messages during NG interface establishment towards different AMFs. And we assume no enhancement is required for the NG-based HO procedure without the new logical WAB-gNB compared to introducing the new logical WAB-gNB.

Observation: Present spec can support the WAB-gNB connects with different AMFs using different configurations. No enhancement is needed for the NG-based HO procedure without the new logical WAB-gNB compared to introducing the new logical WAB-gNB.

It’s not clear what’s the benefit and therefore there is no need to introduce the new logical WAB-gNB in case of AMF change.

Proposal 2-3: No need to install a new logical WAB-gNB in case of AMF change.

In the last meeting, RAN3 receives SA2’s LS[3] asking for the solution of changing UE’s AMF. RAN3 can response to SA2 that:

“RAN3 has discussed the solution of changing UE’s AMF during WAB-node movement. RAN3 believe that the NG-based handover procedure is needed for supporting AMF change for the UE, However, when to activate the new cell(s) and deactivate the old cell(s), i.e., whether instantiating two cells, is up to WAB-node’s implementation. The old and the new cell(s) may have different gNB-IDs, i.e., the gNB ID of WAB-gNB may be changed with the change of AMF. Nevertheless, installing different logical WAB-gNBs is not needed.”

Proposal 2-4: Send response LS to SA2 that

“RAN3 has discussed the solution of changing UE’s AMF during WAB-node movement. RAN3 believe that NG-based handover procedure is needed for supporting AMF change for the UE, However, when to activate the new cell(s) and deactivate the old cell(s), i.e., whether instantiating two cells, is up to WAB-node’s implementation. The old and the new cell(s) may have different gNB-IDs, i.e., the gNB ID of WAB-gNB may be changed with the change of UE’s AMF. Nevertheless, installing different logical WAB-gNBs is not needed.”

Main steps for handling the connected UEs by NG-based HO preparation:

- The WAB-gNB establishes NG interface with the new AMF(s).

The WAB-gNB may decide the change of serving AMF(s) when OAM provides the WAB-gNB with new AMF(s) corresponding to the new geolocation of the WAB-node, or the WAB-gNB decides the serving AMF(s) should be changed based on the configurations for different geographic locations of the WAB-node from OAM.

The WAB-gNB should serve the new AMF(s) with new cell(s). If no NG connection is set up with the new AMF(s), the WAB-gNB establishes NG connection with the new AMF(s) using the new gNB ID (if configured by OAM) as well as the new TAI(s) corresponding to the new geolocation. The WAB-gNB can instantiate the new cell(s) after the NG connection establishment, i.e., powering up the old cell(s) and new cell(s) concurrently.

- The WAB-gNB decides to trigger NG-based HOs to the connected UEs.

If there are UE(s) with the logical connection with the old AMF(s), the WAB-gNB triggers NG-based HO to those UEs to relocate context of the UEs to new AMF(s).

- The WAB-gNB sends the Handover Required message towards the old AMF of UE and the old AMF decides the new AMF.

The WAB-gNB includes one of the new TAI(s) mapping to the new geolocation, which should be served by the new AMF, as well as the new gNB ID in Handover Required message to the old AMF of UE. The new AMF is determined by the old AMF based on the new TAI corresponding to the WAB-node’s new location. The new AMF sets up context for the UE. The new AMF selects the WAB-gNB as the target node based on the (new) gNB ID.

- The new AMF selects the WAB-gNB and sends the Handover Request message to the WAB-gNB.

The new AMF decides the WAB-gNB to be the target RAN-node of the UE via the WAB-gNB’s gNB ID.

- The WAB-gNB selects the target cell of UE, formulates the RRC HO command in Handover Required ACK message and sends back to the new AMF.

Upon receiving the Handover Request message, the WAB-gNB can decide the target cell of UE via the measurement results of the UE to the candidate cells which are included in HandoverPreparationInfo of the Handover Request message. If there is no measurement result, the WAB-gNB can decide the target cell based on the old-to-new cell mapping information inside, since the source cell identity can be derived from the HandoverPreparationInfo of the Handover Request message.

Then the WAB-node can formulate RRC HO command, e.g., configuring RACH-less HO, to the UE and includes the HO command in Handover Required ACK message to the new AMF. The RRC HO command is conveyed from the new AMF to the old AMF, from the old AMF to the WAB-gNB, and delivered to the UE at last.

Figure 1 NG-based HO preparation used for changing UE’s AMF

Proposal 2-4: To do the NG-based HO preparation for changing AMF,

-	WAB-gNB establishes NG connection with the new AMF(s) using the new gNB ID (if configured) as well as the new TAI(s) corresponding to the new geolocation,

-	for old AMF to decide new AMF and new AMF to locate target RAN-node, WAB-gNB includes the new TAI corresponding to the new geolocation and the new gNB ID (if configured) in Handover Required message to old AMF,

-	target cell is selected based on UE’s measurement results and/or via old-to-new cell mapping relationship, and

-	WAB-gNB can configure RACH-less HO to UE.

2.3 NG connection management

If the AMF(s) that WAB-gNB should connect and the TAI(s) broadcasted by the WAB-gNB change, the WAB-gNB will not serve the old TAI(s) and the old AMF(s) any more. The WAB-gNB should stop using the NG connection with the old AMF(s) after all the UEs connected to the old AMF(s) are handed over to the new AMF(s).

Given the WAB-node (including WAB-MT and WAB-gNB) status is authorized, the old AMF(s) are still reachable because the BH PDU session providing the IP connection to NG connections with the old AMF(s) is available, even if the WAB-gNB should serve new AMF(s) due to the WAB-node’s movement to a new area. The WAB-gNB can keep the underlying SCTP connection with the old AMF(s). In that way, the WAB-gNB does not need to re-establish SCTP connection with the previously served AMF(s) when it moves back to areas of those AMF(s).

In the last meeting [1], some companies suggested that NG connections can be suspended. Since the WAB-node does not serve the old AMF(s) anymore, the old AMF(s) should not use the corresponding NG connections, e.g., sending paging messages to the WAB-node, the WAB-node has to notify the old AMF(s) that the NG interface is not available through either new NGAP signalling or existing NGAP signalling (i.e., implicitly). Also, the WAB-node has to inform those AMF(s) to resume the NG connection when the WAB-node moves back. Given the removal of NG connection during WAB-node’s movement has been agreed in the last meeting, there is no need to introduce NG connection suspension.

Proposal 3-1: If the NG connection is removed during WAB-node’s movement, the SCTP connection between the WAB-gNB and the AMF can be maintained.

Proposal 3-2: NG suspension procedure is not supported for WAB-node’s mobility.

2.4 Xn connection management

2.4.1 Xn setup

According to the agreements [1], the WAB-node can establish Xn interface with BH-RAN-node or surrounding NG-RAN nodes. Obviously, the WAB-gNB can get the RAN-node ID of the BH-RAN-node by reading the system information of the cell serving WAB-MT, discover the IP address of the BH-RAN-node via Xn-C TNL address discovery, then set up Xn with the BH-RAN-node. The issue is how to discover and set up Xn interface with the surrounding NG-RAN nodes.

As a legacy mechanism, UE’s ANR procedure can be used by the WAB-node to derive the RAN-node ID of the neighbour NG-RAN nodes. However, ANR is optionally supported by UE. Other method should be studied. For example, the WAB-node can detect the RAN-node ID of the neighbour NG-RAN nodes by itself.

For a cell WAB-MT can detect, the WAB-MT reads SIB1 in the system information broadcast of the cell and get RAN-node ID of the surrounding RAN-node. But this method will increase complexity of WAB-MT.

Alternatively, WAB-gNB can get the RAN-node ID of surrounding NG-RAN node via Xn interface with the BH-RAN-node, because the surrounding NG-RAN nodes are supposed neighbouring the BH-RAN-node. WAB-gNB can read global NG-RAN Node Identifier of the surrounding NG-RAN node from neighbour information, such as Neighbour Information NR IE, Neighbour Information E-UTRA IE, and Neighbour NG-RAN Node List IE in Xn SETUP REQUEST message, Xn SETUP RESPONSE message, NG-RAN NODE CONFIGURATION UPDATE message, or NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message.

After getting the RAN-node ID of the BH-RAN-node or the surrounding NG-RAN node, the WAB-node can discovery Xn-C TNL address corresponding to the RAN-node ID by existing RAN configuration update procedure and establish Xn connection with the surrounding NG-RAN node.

Proposal 4-1: For discovery and Xn setup with surrounding NG-RAN nodes, following options are proposed:

- Option 1:  WAB-MT reading the system information broadcasted by the neighbour cell(s);

- Option 2: WAB-gNB getting neighbour information of the BH-RAN-node from Xn interface.

If the target NG-RAN node is not in the same AMF as the WAB-node, Xn TNL discovery cannot obtain its IP address. Therefore, before triggering Xn TNL discovery, WAB-node needs to check whether the AMF is same as the AMF of the neighbour NG-RAN node.

Before the procedure of obtain the IP address of the neighbour NG-RAN node, WAB-gNB can check that the neighbour NG-RAN node connecting to the same AMF as it is. For example, WAB-MT can derive the information based on the OAM configurations, as the OAM can configure the mapping relationship between TAI(s) and AMF in different locations. The WAB-MT gets the TAI of the neighbour cell, e.g., by reading the TAI from SIB1 in the system information broadcast of the neighbour cell, and checks if the TAI of the neighbour node maps to an AMF with which the co-located WAB-gNB connects. If yes, the WAB-node request IP address and initiate Xn setup for the neighbour node, otherwise it does not request IP address for the neighbour node.

Proposal 4-2: Before discover Xn-C TNL address, WAB-gNB check whether the neighbour NG-RAN node connects with the same AMF as it, for example, based on OAM configuration.

2.4.2 Xn removal

Due to the mobility of WAB-node, it may move outside the serving scope of the neighbour NG-RAN nodes. That means the original NG-RAN node which WAB-node connects via Xn is no longer neighbouring the WAB-node. It’s not necessary for the WAB-node to keep Xn with it. During the WAB-node movement, it may change the serving AMF, so it would not connect with the same AMF as the NG-RAN node which it connects via Xn. In that case, the WAB-node should tear up the Xn with the NG-RAN node.

If the WAB-node moves outside the area of surrounding NG-RAN nodes, the WAB-node will be unable to continue detect those neighbour cells. Therefore, the WAB-node determines that corresponding NG-RAN nodes are not the neighbour NG-RAN nodes. WAB-gNB will trigger Xn removal procedure of the NG-RAN nodes that are no longer considered neighbour NG-RAN nodes.

Alternatively, WAB-gNB can determine whether a node is surrounding based on XnAP messages from the BH-RAN-node. If the node is no longer indicated in the neighbour information of BH-RAN-node, such as Neighbour Information NR IE, Neighbour Information E-UTRA IE, and Neighbour NG-RAN Node List IE in Xn SETUP REQUEST message, Xn SETUP RESPONSE message, NG-RAN NODE CONFIGURATION UPDATE message, or NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message. The NG-RAN node is no longer regarded as the neighbour NG-RAN node. WAB-gNB will trigger Xn removal procedure to those NG-RAN nodes.

Proposal 4-3: The WAB-node determines Xn removal with surrounding NG-RAN nodes based on WAB-MT detecting surrounding cell(s) or based on WAB-gNB getting neighbour information of the BH-RAN-node from Xn interface.

2.5 Additional ULI

According to SA2’s conclusion [2], indication of an additional ULI from the WAB-gNB to the AMF for UE is required. One LS [4] was sent from SA2 for asking RAN to discuss the issue of additional ULI and feedback:

S2-2407345 Issue-2: SA2 has discussed and concluded that Additional ULI format will be determined by RAN WG and SA2 will align based on RAN feedback. Please refer to related conclusion in clause 8.5 of TR 23.700-06.

According to clause 8.5 of TR 23.700-06 [2], SA2 has some progress on the issue of additional ULI. It was agreed that TAC/Cell ID the cell serving WAB-MT is reported as the additional ULI of UE when the PLMN serving the WAB-MT and that serving the UE is the same, whereas, 3 potential options are listed for the content of additional ULI in case the PLMN serving the WAB-MT and the PLMN serving the UE are not the same PLMN.

clause 8.5 of TR 23.700-06 - The MWAB-gNB provides to the AMF an Additional ULI. The Additional ULI is: - TAC/Cell ID of the cell serving the MWAB-UE if the PLMN serving the MWAB-UE and the PLMN broadcasted by the MWAB-gNB are the same PLMN. - If the PLMN serving the MWAB-UE and the PLMN broadcasted by the MWAB-gNB are not the same PLMN, which below option used for the Additional ULI will be determined in normative in phase. - option 1: mapped TAC/Cell ID based on geo-location of the MWAB based on input from OAM. - option 2: geo-location of the MWAB. - option 3: TAC/Cell ID of the cell serving the MWAB-UE in other PLMN. - The AMF may provide the received Additional ULI to LMF in LCS request. NOTE: If option 2 is selected in normative phase, support of passing Additional ULI to LMF needs to be reviewed.

In Rel-18, due to the cell ID/TAC broadcasted by the mobile IAB cell may kept unchanged during the IAB-node’s movement over a long distance, it may not reflect the real location of UE. So, there is problem to support the services that use cell ID/TAC as location reference, e.g., regulatory services. In Rel-18, the IAB-donor-CU can provide an Additional ULI which includes TAI/NCGI of the cell serving the mIAB-MT in addition to the ULI of UE, then the AMF can determine UE’s location according to the additional ULI.

The same problem is applied to WAB, i.e., the cell ID/TAC of the WAB-node cell may not be able to reflect the real location of UE. The method of reporting the additional ULI in mobile IAB can be reused for WAB. However, the PLMN serving the mIAB-MT should be the same as the PLMN serving the UE which connects to the mIAB cell. The mobile IAB method is only applicable to WAB when the PLMN serving the WAB-MT and that serving the UE is the same. Because the AMF of UE may not recognize the cell ID/TAC of a PLMN which is not the PLMN of UE, that the AMF would not be able to derive the UE’s location if the additional ULI pertains to cell ID/TAC of a different PLMN to the UE’s PLMN. Above 3 options are presented to address this issue.

On option 1, the mapped TAC/The OAM can be recognized by the UE’s AMF. OAM can provide to the WAB-gNB the configuration of mapped TAC/Cell ID when receiving location information of the WAB-node or multiple sets of configurations for different geographic locations. In addition, the mapped cell mechanism has been realized in NTN, so it can be reused.

On option 2, additional ULI includes geolocation of WAB-node. It seems feasible, but more normative work is observed. NGAP has to introduce the new IE of WAB-node’s geolocation in UE’s ULI. On the other hand, it consumes a lot of time to obtain UE’s geolocation before sending the additional ULI.

Option 3 is workable from RAN3 perspective, but it’s not clear how does AMF of a PLMN identify the cell ID of other PLMNs.

In summary, Option 1 is the most feasible and clear solution.

Proposal 5-1: If the PLMN serving the WAB-MT and the PLMN broadcasted by the WAB-gNB are not the same PLMN, the additional ULI includes the mapped TAC/Cell ID configured by OAM corresponding to geo-location of the WAB-node.

Proposal 5-2: RAN3 send response LS to SA2 on P5-1 as Annex B of [5].

3 Conclusion

In the previous sections we made the following observation and proposals:

Without change of UE’s AMF

Proposal 1-1: TAC the WAB-gNB cell broadcasts can be changed without the change of AMF the WAB-gNB connects with. The changed of TAC may trigger the Mobility Registration Update procedure of UEs which camp on or connects with the WAB-gNB’s cell(s).

Proposal 1-2: When PCI that the WAB-gNB’s cell broadcasts changes, intra-gNB handover should be conducted to connected UEs. The implementation inside the WAB-gNB to support the intra-gNB handover is out of the study scope.

With change of UE’s AMF

Proposal 2-1: The gNB ID of WAB-gNB may be changed with the change of AMF(s).

Proposal 2-2: In case the AMF(s) change, NG-based handover should be conducted to the connected UEs. When to activate the new cell(s) and deactivate the old cell(s) is up to the WAB-gNB implementation.

Proposal 2-3: No need to install a new logical WAB-gNB in case of AMF change.

Proposal 2-4: Send response LS to SA2 that

“RAN3 has discussed the solution of changing UE’s AMF during WAB-node movement. RAN3 believe that NG-based handover procedure is needed for supporting AMF change for the UE, However, when to activate the new cell(s) and deactivate the old cell(s), i.e., whether instantiating two cells, is up to WAB-node’s implementation. The old and the new cell(s) may have different gNB-IDs, i.e., the gNB ID of WAB-gNB may be changed with the change of UE’s AMF. Nevertheless, installing different logical WAB-gNBs is not needed.”

Proposal 2-4: To do the NG-based HO preparation for changing AMF,

-	WAB-gNB establishes NG connection with the new AMF(s) using the new gNB ID (if configured) as well as the new TAI(s) corresponding to the new geolocation,

-	for old AMF to decide new AMF and new AMF to locate target RAN-node, WAB-gNB includes the new TAI corresponding to the new geolocation and the new gNB ID (if configured) in Handover Required message to old AMF,

-	target cell is selected based on UE’s measurement results and/or via old-to-new cell mapping relationship, and

-	WAB-gNB can configure RACH-less HO to UE.

NG connection management

Proposal 3-1: If the NG connection is removed during WAB-node’s movement, the SCTP connection between the WAB-gNB and the AMF can be maintained.

Proposal 3-2: NG suspension procedure is not supported for WAB-node’s mobility.

Xn connection management

Proposal 4-1: For discovery and Xn setup with surrounding NG-RAN nodes, following options are proposed:

- Option 1:  WAB-MT reading the system information broadcasted by the neighbour cell(s);

- Option 2: WAB-gNB getting neighbour information of the BH-RAN-node from Xn interface.

Proposal 4-2: Before discover Xn-C TNL address, WAB-gNB check whether the neighbour NG-RAN node connects with the same AMF as it, for example, based on OAM configuration.

Proposal 4-3: The WAB-node determines Xn removal with surrounding NG-RAN nodes based on WAB-MT detecting surrounding cell(s) or based on WAB-gNB getting neighbour information of the BH-RAN-node from Xn interface.

Additional ULI

Proposal 5-1: If the PLMN serving the WAB-MT and the PLMN broadcasted by the WAB-gNB are not the same PLMN, the additional ULI includes the mapped TAC/Cell ID configured by OAM corresponding to geo-location of the WAB-node.

Proposal 5-2: RAN3 send response LS to SA2 on P5-1 as Annex B of [5].

4 Reference

- Draft Report of 3GPP TSG RAN3 meeting #124, MCC

- 3GPP TR 23.700-06, Study on architecture enhancements for vehicle-mounted relays - Phase 2, v 0.3.0 (2024-04)

- R3-243021, LS on FS_VMR_Ph2 solution impacts to RAN, RAN3#124

- S2-2407345, LS on questions regarding FS_VMR_Ph2, SA2#163

- R3-244231, (TP to TR 38.799) On WAB architecture and authorization, CATT, RAN3#125

5 Annex (TP to TR 38.799)

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>Start of Changes<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<

#### 4.3.4.2	WAB-gNB mobility

During WAB-node movement, establishment, and removal of the WAB-gNB’s NG and/or Xn connections may be needed. If the WAB-gNB’s NG connection is removed, the SCTP connection between the WAB-gNB and the UE’s AMF may be maintained.

Establishment of Xn connections of the WAB-gNB with BH-RAN-nodes, as well as with surrounding RAN nodes, is supported, and it can follow legacy procedures. The WAB-gNB can obtain the RAN-node ID of neighbour NG-RAN node(s) to set up Xn connections via UE’s ANR procedure or by the co-located WAB-MT detecting the broadcast information over air interface. The WAB-gNB can decide the NG-RAN node(s) to remove Xn connections by the co-located WAB-MT detecting the broadcast information over air interface or getting the neighour information from the BH-RAN-node over XnAP.

##### 4.3.4.2.1 WAB-gNB mobility without change of UE’s AMF(s)

During WAB-node movement, radio configuration parameters of the WAB-gNB may be changed (e.g., cell ID, PCI and/or TAC) without the change of the UE’s AMF(s). This change of radio configuration parameters may require the UE handling by means of, e.g., intra-gNB handover and/or Mobility Registration Update as defined in TS 23.502.

##### 4.3.4.2.2 WAB-gNB mobility with change of UE’s AMF(s)

Due to WAB-node movement, the change of UE’s AMF(s) may be needed, based on, e.g., WAB-node’s current location and/or additional criteria. When UE’s AMF(s) changes, the configuration of TAC(s) and Cell ID(s) for the WAB-gNB should also change, and the gNB-ID of the WAB-gNB may also be changed by OAM. The NG connection handling and WAB-gNB configuration update may affect the served UEs.

- The WAB-gNB may obtain the configuration parameters needed to establish the connection to the UE’s new AMF(s).

- The WAB-gNB establishes NG connection(s) towards one or more new AMF(s) using the new TAC(s) and the new gNB-ID (if configured).

- The WAB-gNB may activate one or more new cells, with new cell configuration parameters related to the WAB-gNB’s current location. The new cells may broadcast the radio parameters configured for the new AMF(s), e.g., TAC, etc. The old cell(s) can remain active.

Editor’s NOTE: It’s up to the WAB-gNB’s implementation when to activate the new cell(s) and whether to keep the new cell(s) and old cell(s) active concurrently.

- The UEs are handled as follows:

- A UE in RRC_CONNECTED state is handed over between an old cell and a new cell served by the WAB-gNB via NG-based handover with AMF relocation, as defined in TS 23.502. The WAB-gNB includes the TAI which is configured corresponding to the WAB-node geolocation and the new gNB ID (if configured) in the Handover Required message to the old AMF during the HO preparation procedure. The target cell of UE can be selected based on UE’s measurement result included in the HO preparation information or via the mapping relationship between the old cell and new cell. The UE can be configured with RACH-less HO.

- When all UEs in RRC_CONNECTED state have been handed over, the old cell(s) are removed from service. A UE in RRC_IDLE or RRC_INACTIVE state camping on the old cell(s) reselects a new cell, and legacy procedure (e.g., Mobility Registration Update procedure as defined in TS 23.502) is performed.

- The NG connection(s) between the WAB-gNB and the old AMF(s) are removed.

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>End of Changes<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<