3GPP TSG-RAN WG3 Meeting #125                                                                 R3-244233

Maastricht, NL, Aug 19th – 23rd, 2024

Source:	CATT

Title:	(TP to TR 38.799) Discussion on resource multiplexing for WAB

Agenda Item:	12.2

Document for:	Discussion and Decision

1 Introduction

In the last meeting, additional topological enhancements for NR are studied. The resource multiplexing of Wireless Access Backhaul (WAB) is discussed.

In this contribution, we will continue to discuss the resource multiplexing for WAB based on [1].

2 Discussion

According to the TR [1] from last meeting, the resource coordination mechanism introduced for IAB can be considered as the starting point to resource multiplexing for WAB. However, the TR does not specify what information to reuse from the resource coordination mechanism introduced for IAB. The issue needs to be refined.

Observation 1: According to the TR [1] from last meeting, it’s not clear what information of IAB is reused for WAB on resource multiplexing.

Similar with WAB, both access link of IAB-node (between IAB-DU and child nodes/UEs) and backhaul link of IAB-node (between IAB-MT and parent node) utilize NR Uu. IAB-MT and IAB-DU would be subject to a half-duplex constraint. The transmission of each IAB-DU’s cell depends on resource configuration or implicit determination to make sure the access link will have no impact to the BH link. The same principle should be applied to WAB, that transmission of each WAB-gNB’s cell should be constrained for potential impacts to WAB-MT transmission/reception over the BH link.

In TS 38.300 [2], the mechanism of IAB resource configuration is presented, which considers the IAB-DU and the IAB-MT of an IAB-node are subject to a half-duplex constraint.

Each symbol of IAB-DU’s cell is assigned to be hard, soft, or unavailable. When a symbol is configured as hard, the IAB-DU can use it for Tx/Rx. When a symbol is configured to be unavailable, the IAB-DU cannot use it except in some special cases. When a symbol is configured as soft, the IAB-DU can conditionally use it in cases where the parent node explicitly indicates availability via DCI format 2_5, or implicitly in cases where the IAB-node determines availability.

Similar with IAB, there can be three types for each symbol of WAB-gNB’s cell. The first type of symbol can be used without impact to WAB-MT, the second type of symbol cannot be used due to the impact to WAB-MT and the third type of symbol can be conditionally used depending on whether it has an impact to WAB-MT. The schedule on an WAB-gNB’s cell complies with the type of each symbol.

Proposal 1: The types for symbol of WAB-gNB’s cell can reuse those in IAB.

For resource multiplexing, WAB-node can send resource configurations of the WAB-node to the BH-RAN-node. The resource configuration of the WAB-gNB’s cell includes slot format and type of each symbol. The slot format includes the number and position of uplink, downlink, and flexible symbols.

The BH-RAN-node can also transport resource configurations of its cell(s) to the WAB-node. Since BH-RAN-node is legacy NG-RAN node, the resource configuration of BH-RAN-node’s cell only needs to include the slot format to indicate the number and position of uplink, downlink, and flexible symbols. The type of each symbol is not necessary.

Observation 2: The resource configuration of WAB-gNB’s cell(s) includes slot format and type of each symbol. The resource configuration of BH-RAN-node’s cell(s) includes slot format only.

Further, IAB-node may support simultaneous Tx/Rx over access and BH links, i.e., simultaneous IAB-MT Rx / IAB-DU Rx, IAB-MT Tx / IAB-DU Tx, IAB-MT Rx / IAB-DU Tx, IAB-MT Tx / IAB-DU Rx, in those cases frequency or space multiplexing would be required.

The frequency or spatial multiplexing mode between access and BH links, i.e., simultaneous operation of WAB-MT Rx / WAB-gNB Rx, WAB-MT Tx / WAB-gNB Tx, WAB-MT Rx / WAB-gNB Tx, WAB-MT Tx / WAB-gNB Rx, should also be supported by WAB. To facilitate that, the mechanism of exchanging information between IAB-node and parent node can be reused between WAB-node and BH-RAN-node.

Observation 3: The multiplexing capability between access and backhaul can be supported by WAB-node. The WAB-node can send information on the duplex capability of the WAB node to the BH-RAN-node.

In TS 38.473(Release 17) [3], there are three IE types of IAB node’s duplex capability: “supported”, “not supported”, “supported and FDM required”. In TS 38.473(Release 16) [4], there are only two IE types of that: “supported”, “not supported”. The extra IE type is “supported and FDM required”.

Since no TU is allocated for WAB in RAN1, very few TUs for WAB in RAN 2, the enhanced resource multiplexing function with the impacts to the PHY/MAC layer is not recommended for WAB. In addition, Rel-19 is the first version of WAB, it’s better the keep the function simple.

If the enhanced resource multiplexing is supported by WAB, the concurrent WAB-MT and WAB-gNB transmission without FDM must be considered, which introduces impact to PHY and MAC. So, it is not recommended to support the capability of “supported and FDM required”.

Therefore, in our view, the information of WAB node’s duplex capability includes two types, i.e., “supported” or “not supported” the concurrent WAB-MT and WAB-gNB transmission.

Observation 4: The WAB-node’s duplex capability should indicate whether the concurrent WAB-MT and WAB-gNB transmission is “supported” or “not supported”.

To achieve resource multiplexing between access link and backhaul link, resource information for each cell of BH-RAN-node and each cell of WAB-gNB is exchanged between the BH-RAN-node and the WAB-node. In addition to transport its own resource configuration to the other node, the WAB node can send the proposed slot format of the BH-RAN-node’s cell to the BH-RAN-node for BH-RAN-node to modify the slot format based on the proposed information. The BH-RAN-node can also send the proposed slot format and/or type of each symbol for each WAB-gNB’s cell to the WAB node. The WAB node can also modify the slot format and/or type of each symbol base on the proposed information from the BH-RAN-node.

This method can also achieve the purpose of resource multiplexing.

Observation 5: The WAB-node can propose slot format of the BH-RAN-node’s cell(s) to the BH-RAN-node, and the BH-RAN-node can also propose slot format and/or type of each symbol for each WAB-gNB’s cell to the WAB node.

In summary, the information from WAB-node may include the slot format and the type of each symbol for each cell, the duplex capability of the WAB node and the proposed slot format of the BH-RAN-node’s cell. And the information from BH-RAN-node may include the slot format for each cell, the proposed slot format and/or type of each symbol for each WAB-gNB’s cell.

Proposal 2: The resource coordination information of WAB-node includes slot format and symbol type for each cell, WAB-node duplex capability and proposed slot format of BH-RAN-node’s cell(s). The information of BH-RAN-node includes slot format for each cell, proposed slot format and/or proposed symbol type for WAB-gNB’s cell(s).

Proposal 3: The WAB-node duplex capability indicates whether the concurrent WAB-MT and WAB-gNB transmission is “supported” or “not supported”.

To facilitate transitioning from IAB-MT to IAB-DU operation and vice versa, guard symbols are used to overcome potentially misaligned symbol boundaries between the IAB-MT operation and the IAB-DU operation (e.g., IAB-MT Rx boundaries are not aligned with the IAB-DU Tx boundaries). An IAB-node indicates to a parent node a number of desired guard symbols, while the parent node indicates to the IAB-node the number of actually provided guard symbols for specific transitions.

Similar with IAB, guard symbols can also be used to overcome potentially misaligned symbol boundaries between the WAB-MT operation and WAB-gNB operation. The WAB-node can query the BH-RAN-node on a number of desired guard symbols, while the BH-RAN-node indicates to the WAB-node the number of actually provided guard symbols for specific transitions. However, it’s based on the time difference between WAB-gNB Tx and WAB-MT Rx, which requires the study of RAN1. It is proposed the guard symbol query and provision between the WAB-node and the BH-RAN-node is not supported.

Proposal 4: The guard symbols query and provision between WAB-node and BH-RAN-node is not supported.

The resource coordination information can be exchanged between WAB-node and its BH-RAN-node via RRC or XnAP (If Xn interface between BH-RAN-node and WAB-node is established).

As the solution of using RRC signaling can be done without the Xn connection between WAB-node and BH-RAN-node, this solution is less constraint. However, it has large impact to the RRC specification.

Using XnAP signaling to achieve resource coordination is similar with the mechanism of IAB, but the disadvantage is that it relies on the Xn connection between the BH-RAN-node and WAB-gNB. Further, it requires co-location discovery of WAB-MT and WAB-gNB.

Observation 6: When the resource coordination is done via XnAP, the BH-RAN-node needs to discover co-location of the WAB-MT and the WAB-gNB.

However, considering resource multiplexing is optional and Xn is also optional, the Xn solution is feasible. Although co-location discovery is required, limited impact is expected for XnAP. In our view, the resource multiplexing between access and backhaul links can be achieved via XnAP.

Proposal 5: Exchanging resource coordination information between BH-RAN-node and WAB-node can be achieved via XnAP.

The identity of WAB-MT is used in RRC with the BH-RAN-node and the identity of WAB-gNB is used in XnAP with the BH-RAN-node. BH-RAN-node can discover co-location of the WAB-MT and the WAB-gNB by either the identity of WAB-gNB or the identity of WAB-MT. According to the WAB-node integration procedure, the WAB-MT of a WAB-node connects to the network in the way as a UE by performing RRC connection setup procedure with the BH-RAN-node in phase 1. The WAB-gNB establishes Xn connection towards the BH-RAN-node in phase 2-3.

There are two options for co-location discovery:

- Option 1: WAB-MT reporting identity of WAB-gNB.

The WAB-MT reports the gNB ID to the BH-RAN-node via RRC message, e.g., during phase 1 for WAB-MT setup. The BH-RAN-node can discover the WAB-MT is co-located with the WAB-gNB with the gNB ID, e.g., when the WAB-gNB sets up Xn interface with it during phase 2-3.

- Option 2: WAB-gNB reporting identity of WAB-MT.

The BH-RAN-node allocates C-RNTI in the service cell for WAB-MT in phase 1. WAB-gNB sends the C-RNTI as well as the cell identity of WAB-MT to the BH-RAN-node via XnAP signaling, e.g., during the Xn setup procedure in phase 2-3. BH-RAN-node can discover co-location of the WAB-MT and the WAB-gNB via the C-RNTI and cell identity.

Comparing the two options, option 2 has an advantage over option 1 on avoiding the impact to RRC. It is proposed to adopt option 2 for co-location discovery.

Proposal 6: BH-RAN-node discovers the co-location of WAB-gNB WAB-MT via the WAB-gNB reporting the identity of the WAB-MT.

3 Conclusion

In the previous sections we made the following observations and proposals:

Observations:

Observation 1: According to the TR [1] from last meeting, it’s not clear what information of IAB is reused for WAB on resource multiplexing.

Observation 2: The resource configuration of WAB-gNB’s cell(s) includes slot format and type of each symbol. The resource configuration of BH-RAN-node’s cell(s) includes slot format only.

Observation 3: The multiplexing capability between access and backhaul can be supported by WAB-node. The WAB node can send information on the duplex capability of the WAB node to the BH-RAN-node.

Observation 4: The WAB-node’s duplex capability should indicate whether the concurrent WAB-MT and WAB-gNB transmission is “supported” or “not supported”.

Observation 5: The WAB node can propose slot format of the BH-RAN-node’s cell(s) to the BH-RAN-node, and the BH-RAN-node can also propose slot format and/or type of each symbol for each WAB-gNB’s cell to the WAB node.

Observation 6: When the resource coordination is done via XnAP, the BH-RAN-node needs to discover co-location of the WAB-MT and the WAB-gNB.

Proposals:

Proposal 1: The types for symbol of WAB-gNB’s cell can reuse those in IAB.

Proposal 2: The resource coordination information of WAB-node includes slot format and symbol type for each cell, WAB-node duplex capability and proposed slot format of BH-RAN-node’s cell(s). The information of BH-RAN-node includes slot format for each cell, proposed slot format and/or proposed symbol type for WAB-gNB’s cell(s).

Proposal 3: The WAB-node duplex capability indicates whether the concurrent WAB-MT and WAB-gNB transmission is “supported” or “not supported”.

Proposal 4: The guard symbols query and provision between WAB-node and BH-RAN-node is not supported.

Proposal 5: Exchanging resource coordination information between BH-RAN-node and WAB-node can be achieved via XnAP.

Proposal 6: BH-RAN-node discovers the co-location of WAB-gNB WAB-MT via the WAB-gNB reporting the identity of the WAB-MT.

4 Reference

- R3-243970, 3GPP TR 38.799,  Study on additional topological enhancements for NR, v0.0.1 (2024-04)

- 3GPP TS 38.300, NR and NG-RAN Overall Description, Stage-2, v18.1.0 (2024-03)

- 3GPP TS 38.473, NG-RAN, F1 application protocol (F1AP), V17.9.0 (2024-06)

- 3GPP TS 38.473, NG-RAN, F1 application protocol (F1AP), V16.18.0 (2024-06)

5 Annex (TP to TR 38.799)

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>Start of changes<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<

### 4.3.y Resource multiplexing

In scenarios where WAB-node’s access link and backhaul link mutually interfere, resource coordination may be needed to facilitate the resource multiplexing for WAB-node’s access links and backhaul link. For this purpose, the resource coordination mechanism introduced for IAB can be considered as the starting point. BH-RAN-node and WAB-node exchange the resource coordination information via XnAP. The coordination information from the WAB-node may include slot format and symbol type of each cell, duplex capability of the WAB node or proposed slot format for BH-RAN-node’s cell(s). The coordination information from the BH-RAN-node may include slot format for each cell, proposed slot format or proposed symbol type for each WAB-gNB’s cell.

For resource coordination between the access link and backhaul link, the BH-gNB discovers the WAB-gNB is co-located with the WAB-MT via the WAB-gNB reporting identity of the WAB-MT.

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>End of changes<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<