3GPP TSG-RAN WG3 #125																          R3-244234

Maastricht, NL, 19th – 23th Aug, 2024

Source:				CATT

Title:	(TP to TR 38.799) On 5G Femto architecture

Agenda item:		12.3

Document for:		Discussion and Decision

1		Introduction

In RAN3#124 meeting, the architecture for 5G Femto are discussed. In TR38.799[1], captured two options for Xn interface. i.e. option A and option B. The evaluation for the four options for NG interface also added.

This contribution will further discuss the architecture of 5G Femto based on the currently TR and give the proposal for the SI conclusion.

2		Discussion

2.1 Architecture of the 5G Femto

We agree the four options for NG interface as below. We analyzed the cons and pros for these options and captured in TR in last RAN3 meeting.

Option 1: NR Femto node connects to the 5GC directly as a gNB by means of the NG interface

Option 2: NR Femto GW to allow the NG interface between the NR Femto node and the AMF

Option 3: An SCTP concentrator acts as an IP proxy between an NR Femto node and the AMF

Option 4: NR Femto node is a gNB-DU and the gNB-CU is used as the concentration node on both control plane and user plane

For the option 1, the current specifications naturally support it. There is no impaction on the normative work. Considering the option 2 NG GW is the optional supported, the option1 is the special version is option2. Even though the cons listed as below exist in option 1, we still need support this option in the normative work.

1. Not suitable for certain deployments with large number of NR Femtos and/or 5GC not virtualized.

2. Not suitable for residential deployments with frequent switch on/off of NR Femtos

Proposal 1: Option 1 should be supported in the normative work as one special version of option 2

The option 2 is mirror version of LTE on NG interface. But the NR Femto GW only serves as a concentrator for the C-Plane i.e. the NG-C interface. The option 3 is an SCTP concentrator acts as an IP proxy between an NR Femto node and the AMF. It is option 2 likely. But it works in SCTP transport layer and it is transparent to the application layer. In option 2 and option 3, one new logical node needs to be introduced and only for the control plane. So the option 2 and option 3 is similar from the logical. Comparing the cons and pros of these two options, we propose that option 2 is selected for the normative work.

Proposal 2: Option 2 should be supported in the normative work

Proposal 2a: Option 3 can be excluded in the normative work

For option 4, the existing CU-DU split architecture is used and the CU can be as concentrator for both control plane and user plane. We may find there are lots of cons in this solution. One critical defect is the CU-UP should collocate with NR Femto node (i.e. DU) for local service access. So the triple split mode RAN node should be used in this option. To get the easy implantation on the NR femto node, we should not use this architecture.

Proposal 3: Option 4 should be excluded for the normative work

In last meeting, we add two options for Xn interface. I.e. Option A: without Xn GW; Option B: with Xn GW. The Xn GW could be benefit for a number of Xn-connectivity setup and signalling transferring. For option A, the current specification naturally support as NG option 1.

Proposal 4: Option A and Option B can be supported in normative work.

2.2 DC supporting for NR Femto

In LTE, the Dual Connectivity is supported for HeNBs. The open access and hybrid access HeNB node can act as Secondary Node (SN). The closed access HeNB node cannot be act as SN. In 5G Femto, we will use CAG mechanism for the access control. So then the principle of PNI-NPN is naturally used for the 5G Femto deployment. In TS 38.300[4] section 16.7 Public Network Integrated NPN, the DC for the PNI-NPN specified as below:

Dual Connectivity is supported and may involve both PNI-NPN and PLMN cells, according to the mobility restrictions in the UE context as described in TS 37.340.

Even though the NR Femto node use the PNI-NPN cell access mechanism, but the DC supporting cannot be simply copied from PNI-NPN rules due to the PNI-NPN node is macro node so far. Considering the NR Femto node is femto node, the Femto node cannot act as the Master node.

Proposal 5: DC supporting principle of PNI-NPN and PLMN can be used as baseline for NR Femto node DC supporting, but NR Femto node cannot act as the Master node.

2.4 The scope of NR Femto Node

In last RAN3 meeting, we agree to name the 5G femto node as NR femto node. The ng-eNB is clearly excluded in the 5G femto node because it provides E-UTRA user plane and control plane protocol terminations towards the UE. But as the definition, the en-gNB node also provide NR user plane and control plane protocol terminations towards the UE, and act as Secondary Node in EN-DC. It should be NR node. But to keep the simple of the NR femto node, we may not support femto en-gNB which act as SN in case of EN-DC.

Proposal 6: NR femto node does NOT act as SN in case of EN-DC.

3		Conclusion

In the present contribution we make the following observations and proposal:

Proposal 1: Option 1 should be supported in the normative work as one special version of option 2

Proposal 2: Option 2 should be supported in the normative work

Proposal 2a: Option 3 can be excluded in the normative work

Proposal 3: Option 4 should be excluded for the normative work

Proposal 4: Option A and Option B can be supported in normative work.

Proposal 5: DC supporting principle of PNI-NPN and PLMN can be used as baseline for NR Femto node DC supporting, but NR Femto node cannot act as the Master node.

Proposal 6: NR femto node does NOT act as SN in case of EN-DC.

4		Reference

- TR38.799 Study on additional topological enhancements for NR

- RAN3_124_agenda_20240524_eom

- TS 38.401 NG-RAN; Architecture description

- TS 38.300 NR; NR and NG-RAN Overall Description; Stage 2

5		Annex

TP for the TR 38.799 Study on Additional Topological Enhancements for NR

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>Start of TP<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<

- 5.1	General

The study is based on the following requirements:

- An NR Femto node should not act as SN in EN-DC like an en-gNB.

- Initial access control to a CAG supported by a NR Femto node is performed by the AMF, reusing current PNI-NPN functionality.

- Reusing CAG configurations and mobility functionalities specified for PNI-NPN also for NR Femto nodes deployments. FFS on Whether enhancements to legacy CAG functionalities are needed.

- No impacts on UE.

- 5.x	Conclusion

This study focused on analysis of 5G femto architecture, Access control and Local service access.

The following is concluded for the architecture for NG interface of 5G Femto:

Four architecture alternatives have been studied. Evaluation results have shown that Option 2 seems like the most promising option among the studied. Option 1 is one special case of Option 2.

Based on conducted analysis it is recommended to specify Option 2 and Option 1 in normative work.

The following is concluded for the architecture for Xn interface of 5G Femto:

Two architecture alternatives have been studied. RAN3 could give the recommendation on Option B and option A for normative work.

The following is concluded for Access Control of 5G Femto:

The CAG configurations and mobility functionalities specified for PNI-NPN is reused for NR Femto nodes deployments. Initial access control to a CAG supported by a NR Femto node is performed by the AMF, reusing current PNI-NPN functionality

The following is concluded for Local service access of 5G Femto:

The NR femto node collocated UPF can act as local GW for the local service access. The existing 5GC Connectivity Models for Edge Computing is baseline for NR Femto local service access

It is recommended to proceed with a normative work to support 5G Femto.

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>End of TP<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<