3GPP TSG-RAN WG3 #125																          R3-244235

Maastricht, NL,19th – 23th Aug, 2024

Source:				CATT

Title:	(TP to TR 38.799) On 5G Femto access control mechanism

Agenda item:		12.3

Document for:		Discussion and Decision

1		Introduction

In RAN3#124 meeting, the liaison from SA2[1] is discussed. But no agreement was made. And also there is no any conclusion captured for the NR Femto 5G access control mechanism in the TR 38.799[2].

This contribution will further discuss the issue of SA2 liaison and the 5G Femto access control mechanism.

2		Discussion

In last RAN3 meeting, we discussed the SA2 liaison on support of UEs moving between CAG cells of NR Femto Nodes and CSG cells. There are two solutions provided in the liaison[1].

Therefore, some companies proposed below solutions for this KI:

- The UE partitions CSG-CAG ID and constructs mapped CSG/CAG ID, and reports to the NG-RAN or E-UTRAN (depending on the considered mobility direction) as described in pCR (S2-2405814).

- RAN recognizes the target CSG cell (or the target CAG cell) as an open cell during the handover (e.g., via local configuration) and the core network performs access control as described in pCR (S2-2405789).

We get the primary agreement on the solution 1 in RAN3 team. This solution has both RAN and UE impacts and would expect RAN2 may provide further analysis of UE impacts. RAN2 has sent out the response liaison [3], for this issue, they answered as below:

RAN2 assumes that solution1 has RAN impact from RAN2 perspective, including enhancement of E-UTRA cell CGI reporting procedure in NR specification, enhancement of NR cell CGI reporting procedure in LTE specification, and mapping functionality at UE between CSG ID and CAG ID to be introduced for the enhanced CGI reporting.

RAN2 has the similar view as RAN3. So we may answer the solution 1 align with RAN2.

Proposal 1: For solution 1, we may align with RAN2 answer

For the solution 2, both RAN2 and RAN3 agreed there is no UE impact but identify the handover failure issue. RAN2 answer is as below:

The majority of companies assume that solution2 is NW-based solution with no UE impact, from RAN2 perspective.  However, some companies raised concerns about handover failures, and claimed that UE or RAN3 impact may be introduced to avoid such failures, but RAN2 did not have a detailed technical discussion.

In RAN3 discussion, there are two alternative liaison answers are point out as below:

Alt1: Any potential mobility issue (e.g. handover failure due to incorrect target selection) is expected to be addressed via network implementation, e.g., local OAM configuration (e.g. cell ID-CSG ID-CAG ID mapping in RAN).

Alt2: Detail description on the handover failure issues but no solution is provided

Considering the RAN2 answer and our on board the output of discussion, the handover failure issue should be there and need to be solved. But we may not get the clear solutions for this issue in the short discussion.

Proposal 2: For solution 2, we may answer as “the solution will introduce the handover failure issue and how to solve the issue may be discussed after SA2 selected the solution.”

For the 5G Femto access control mechanism in RAN SI, we agree to reuse the CAG functionalities for the NR femto nodes deployments. But up to now, there is no any description captured in the TR38.799. Also, there is one open issue left: The study should consider applicability of use cases equivalent to those in 4G, i.e. open mode, closed mode, hybrid mode. Definitions of open mode, closed mode, hybrid mode for the NG-RAN are FFS. We will discuss this issue as following and provide the proposals

In LTE, when we discussed the HeNB, we define three access modes as below for HeNB in [4]:

Closed access mode: HNB/HeNB operates as a CSG cell.

Hybrid access mode: HNB/HeNB operates as a CSG cell where at the same time, non-CSG members are allowed access.

Open access mode: HNB/HeNB operates as a normal cell, i.e. non-CSG cell.

Also, the CGS cell and CSG are defined as below:

Closed Subscriber Group (CSG): A Closed Subscriber Group identifies subscribers of an operator who are permitted to access one or more cells of the PLMN but which have restricted access (CSG cells).

CSG cell: A cell, part of the PLMN, broadcasting a specific CSG identity. A CSG cell is accessible by the members of the closed subscriber group for that CSG identity. All the CSG cells sharing the same identity are identifiable as a single group for the purposes of mobility management and charging.

From above context, we may derive the Closed access HeNB only is accessible by the UE which subscribe the CSGs. The open access mode HeNB operates as the normal cell and any UE which belong to the operator can access. The Hybrid access HeNB can be accessed by the CSG UE and non CSG UE. From the access control, the open access mode is not different from the hybrid access mode. All the UE can access the HeNB which is the open access mode and hybrid access mode. The difference may be on the resource allocation for the CSG UE and non-CSG UE.

We agree to use CAG as the access control mechanism for NR Femto cell. The CAG functionality is only used in PNI-PNP network. During the normative work, the closed access mode likely mode is specified as NPN-only as below in TS 38.331 [5]:

NPN-only Cell: A cell that is only available for normal service for NPNs' subscriber. An NPN-capable UE determines that a cell is NPN-only Cell by detecting that the cellReservedForOtherUse IE is set to true while the npn-IdentityInfoList IE is present in CellAccessRelatedInfo.

Also, the NR femto cell can be configured as normal cell. So, the closed mode and open mode is already clearly supported in 5G. We don’t need study for these two modes.

Observation 1: Closed mode and open mode are already clearly supported in 5G.

For the hybrid mode, the cellReservedForOtherUse is one optional IE, if this IE absent and npn-IdentityInfoList IE is present in CellAccessRelatedInfo, both the CAG UE and non-CAG UE can access this cell.

Observation 2: Hybrid mode is also supported in 5G.

We give the definition as below for the three modes for NR Femto node.

Closed access mode: NR Femto node operates as a NPN-only cell.

Hybrid access mode: NR Femto node operates as a NPN cell, where non-CAG and CAG members are allowed access.

Open access mode: NR Femto node operates as a normal cell, i.e. non-NPN cell.

Proposal 3: Open mode, closed mode and hybrid mode for NR femto node are defined as:

- Closed access mode: NR Femto node operates as a NPN-only cell;

- Hybrid access mode: NR Femto node operates as a NPN cell, where non-CAG members are allowed access;

- Open access mode: NR Femto node operates as a normal cell, i.e., non-NPN cell.

3		Conclusion

In the present contribution we make the following observations and proposal:

Observation 1: Closed mode and open mode are already clearly supported in 5G.

Observation 2: Hybrid mode is also supported in 5G.

Proposal 1: For solution 1, we may align with RAN2 answer

Proposal 2: For solution 2, we may answer as “the solution will introduce the handover failure issue and how to solve the issue may be discussed after SA2 selected the solution.”

Proposal 3: Open mode, closed mode and hybrid mode for NR femto node are defined as:

-	Closed access mode: NR Femto node operates as a NPN-only cell;

-	Hybrid access mode: NR Femto node operates as a NPN cell, where non-CAG members are allowed access;

-	Open access mode: NR Femto node operates as a normal cell, i.e., non-NPN cell.

4		Reference

- R3-243020 LS on Support of UE move between CAG cell of 5G Femto and CSG cell

- TR38.799 Study on additional topological enhancements for NR

- R3-244012 Reply LS on Support of UE move between CAG cell of 5G Femto and CSG cell

- TR23.830 Architecture aspects of Home NodeB and Home eNodeB

- TS38.331

5		Annex

TP for the TR 38.799 Study on Additional Topological Enhancements for NR

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>Start of TP<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<

- 5.3	Access control

Editor Note: Study how to define the 5G access control mechanism by (re-)using the existing CAG functionality and identify needed enhancements (if any)

The CAG configurations and mobility functionalities specified for PNI-NPN is reused for NR Femto nodes deployments. Initial access control to a CAG supported by a NR Femto node is performed by the AMF, reusing current PNI-NPN functionality.

The NR Femto node access modes are defined as below:

- Closed access mode: NR Femto node operates as a NPN-only cell.

- Hybrid access mode: NR Femto node operates as a NPN cell, where non-CAG members are allowed access.

- Open access mode: NR Femto node operates as a normal cell, i.e., non-NPN cell.

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>End of TP<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<