3GPP TSG-RAN WG3 Meeting #125                                             R3-244237

Maastricht, Netherlands, 19th - 23th August, 2024

Source:	CATT

Title:	On support on-demand SSB SCell operation

Agenda Item:	17.2

Document for:	Discussion and Decision

Introduction

For NES on-demand SSB SCell operation topic, the WID[1] content is as follows:

Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

In previous RAN1 meetings, some agreements are achieved on various aspects related to on-demand SSB SCell. In this contribution, we will discuss the on-demand SSB SCell operation topic, our views will be given as well.

Discussion

Issue 1: Which node decides to apply on-demand SSB to NES cell

RAN1 has discussed different scenarios for Case#1 (no always-on SSB) and Case#2 (with always-on SSB periodically transmitted) for on-demand SSB. If the cell applies on-demand SSB, the SSB transmission opportunities are reduced and energy consumed by SSB transmission can be saved.

Considering on-demand SSB is designed for low traffic load, it’s not suitable to pre-configure on-demand SSB SCells, as the traffic load is variable. Thus, the cells to apply on-demand SSB should not depend on OAM configuration, it would be gNB-DU or gNB-CU to decide.

Since SSB transmission is controlled and performed by gNB-DU, the energy used for SSB transmission is consumed by gNB-DU. Thus, the gNB-DU (instead of the gNB-CU) can be aware of energy consumed for SSB transmission. In addition, most of the energy used for base band processing are consumed by gNB-DU, so the energy efficiency of a cell can be obtained by gNB-DU. If the energy efficiency of the NES cell is low due to common signal transmission, e.g., when the user data rate is low or few UEs are connected to the cell, and the cell supports on-demand SSB operations, gNB-DU can apply on-demand SSB to the NES cell. When on-demand SSB is applied, SSB transmissions are stopped or decreased.

The gNB-DU can also decide to not apply on-demand SSB, for example, when the impact of SSB transmission on the energy efficiency can be ignored. When on-demand SSB is not applied, SSB is transmitted via legacy approach.

Observation 1: NES gNB-DU can decide whether to apply on-demand SSB (no F1 impact).

Issue 2: Which node configures on-demand SSB SCell

RAN1#117:

Agreement - For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell. - FFS: Whether to support DCI based signaling to indicate on-demand SSB transmission on the cell. - This DCI signaling does not provide SCell activation/deactivation. - If supported, details on DCI including UE-specific or group-common DCI, DCI contents, etc. - FFS: Scenarios where the above signalings are applicable Agreement - For a cell supporting on-demand SSB SCell operation, at least the following for on-demand SSB via higher layer RRC signaling is supported. - Frequency of the on-demand SSB - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst - Periodicity of the on-demand SSB - FFS: Whether more than one on-demand SSB configurations can be configured for the cell to UE - FFS: Whether the RRC is newly introduced or existing RRC is reused

Per above agreements in RAN1#117[2], on-demand SSB SCell can be configured by RRC with necessary parameters, such as frequency of on-demand SSB, SSB positions and periodicity.

In split architecture, gNB-DU is responsible for SCell configuration including configuration of SSB of the SCell to UE, so we assume the on-demand SSB SCell configuration for UE, including the configuration for on-demand SSB, should be also be carried out by gNB-DU.

Besides the on-demand SSB SCell configuration, the cell applying on-demand SSB should support the on-demand SSB operations which are realized by MAC-CE or DCI signaling. The MAC CE or DCI is transmitted by gNB-DU. gNB-CU has no knowledge about whether a cell supports on-demand SSB SCell operations or not and thus cannot know whether to configure the on-demand SSB.

Similar with SCell configuration, on-demand SSB SCell configuration has no F1 impact.

Observation 2: On-demand SSB SCell is configured by NES gNB-DU (no F1 impact).

Issue 3: Whether gNB-CU needs to allow applying on-demand SSB

Per WID description, the on-demand SSB operations target to SCell. If on-demand SSB is applied for a NES cell, the cell should not be used as PCell/PSCell, and the cell coverage is not available anymore because UE should not camp on the cell applying on-demand SSB. So, gNB-CU would need to assist gNB-DU for making decision on the cells allowed to apply on-demand SSB. Similar with SSB deactivation, where gNB-DU receives Cells Allowed to be Deactivated List IE from gNB-CU and considers that it is allowed to deactivate the SSB beams within the indicated cells for network energy saving purpose, gNB-CU can send the allowed cells list to gNB-DU for applying on-demand SSB for energy saving. If gNB-CU allows a cell to apply on-demand SSB, gNB-DU can configure the on-demand SSB SCell to UE.

A new IE can be introduced to GNB-CU CONFIGURATION UPDATE message for providing cells/SSBs allowed to apply on-demand SSB by NES gNB-CU.

Proposal 1: NES gNB-CU can indicate NES gNB-DU about the cells which are allowed to apply on-demand SSB.

Proposal 2: Introduce a new IE in GNB-CU CONFIGURATION UPDATE message, which provides the allowed cells/SSBs list to apply on-demand SSB by NES gNB-CU.

Issue 4: Whether NES gNB-CU needs to know the on-demand SSB SCell

As agreements achieved in RAN1#117 meeting, to support on-demand SSB operation, DCI or MAC CE based signaling is used to indicate on-demand SSB in the SCell. For on-demand SSB is designed for SCell, SpCell cannot support the on-demand SSB operations. If on-demand SSB is applied for a NES cell, the cell should not be configured as the special cell for UEs, as SSB is common to all the UEs in the NES cell. So, gNB-CU needs to know whether on-demand SSB for a cell is applied or not, so as to prevent configuring the NES cell which applies on-demand SSB as the SpCell of a UE.

For example, Coverage Modification Notification in GNB-DU CONFIGURATION UPDATE message is reused for NES gNB-DU to indicate gNB-CU on the cells applying on-demand SSB, maybe introducing a new cause value different with existing “network energy saving” value is needed.

It can be observed NES gNB-CU needs to know the on-demand SSB SCell. The exact impact to F1 is FFS.

Proposal 3: NES gNB-CU needs to know the cells applying on-demand SSB SCell. The F1 impact is FFS.

Issue 5: Whether NES gNB needs to inform neighbour nodes the on-demand SSB SCell

On-demand SSB SCell is operated within NES gNB, it does not need to be handled by neighbour nodes. However, when a cell applies on-demand SSB, the cell coverage is not available since the cell can only be used as SCell rather than PCell. UE cannot camp on nor handed over to an on-demand SSB cell.

So, there would be a need for NES gNB to inform the neighbour nodes on deactivation of the NES cell, that the neighbour cell can adjust the coverage to take the area of the NES cell for ensuring UE’s cell selection. But that can rely on existing Xn procedure for cell deactivation. On the other hand, when the legacy SSB transmission is resumed in the NES cell, the NES cell may be able to operate as PCell, the NES gNB would need to inform the neighbour nodes about the activation and new coverage of the NES cell. That can rely on existing Xn procedures for exchanging coverage change between NG-RAN nodes.

Observation 3: NES gNB does not need to inform neighbour nodes on on-demand SCell, but the cell deactivation or coverage changes caused by applying on-demand SSB SCell can be informed to neighbour nodes via existing Xn procedure (no Xn impact).

Conclusion

According to the analysis in section 2, we have following observations and proposals:

Observation 1: NES gNB-DU can decide whether to apply on-demand SSB (no F1 impact).

Observation 2: On-demand SSB SCell is configured by NES gNB-DU (no F1 impact).

Proposal 1: NES gNB-CU can indicate NES gNB-DU about the cells which are allowed to apply on-demand SSB.

Proposal 2: Introduce a new IE in GNB-CU CONFIGURATION UPDATE message, which provides the allowed cells/SSBs list to apply on-demand SSB by NES gNB-CU.

Proposal 3: NES gNB-CU needs to know the cells applying on-demand SSB SCell. The F1 impact is FFS.

Observation 3: NES gNB does not need to inform neighbour nodes on on-demand SCell, but the cell deactivation or coverage changes caused by applying on-demand SSB SCell can be informed to neighbour nodes via existing Xn procedure (no Xn impact).

Reference

- RP-234065, New WID: Enhancements of network energy savings for NR, RAN#103, Ericsson

- Draft Report of 3GPP TSG RAN WG1 #117 v0.3.0, RAN1#117, MCC