3GPP TSG-RAN WG3 Meeting #125                                                                  R3-244238

Maastricht, NL, Aug 19th – 23rd, 2024

Source:	CATT

Title:	Discussion on on-demand SIB1 for idle UE

Agenda Item:	17.3

Document for:	Discussion and Decision

1 Introduction

In the RAN3#123bis meeting[1], following agreement was achieved:

Scenario1: UE obtains the UL WUS configuration from Cell A, and transmits UL WUS to NES Cell. - UL WUS configuration from NES Cell to Cell A via Xn, e.g., before entering on-demand SIB1 mode in the NES Cell or when the UL WUS configuration is changed

In the last RAN3 meeting[2], following conclusion was made:

Focus on case2 in RAN3 first. Which node decides the WUS configuration and On-demand SIB1 mode? DU or CU or OAM? NES-cell gNB decides on-demand SIB1 operation activation and deactivation?

In this contribution, we will continue to discuss the on-demand SIB1 topic.

2 Discussion

Per agreements from the RAN3#123bis[1] meeting, UL WUS configuration is sent to Cell A before the NES cell entering on-demand SIB1 mode or when the UL WUS configuration is changed. If the on-demand SIB1 mode (i.g., activation/deactivation) should also be sent to Cell A, we believe that the WUS configuration provision and on-demand SIB1 mode should be delivered over Xn independently.

Proposal 0: WUS configuration provision and on-demand SIB1 mode are transmitted independently.

2.1 WUS configuration provision

Issue 1: Which node decides the WUS configuration

In RAN1#117 meeting[3], following agreements were made for on-demand SIB1:

Agreement At least for Case-2: For further study of type 0 PDCCH monitoring occasions for on demand SIB1, after UE transmits the UL WUS in idle/inactive mode, RAN1 assumes following as a starting point: - Option 1: One or more type 0 PDCCH monitoring occasions for on demand SIB1 within a time window - FFS: How the search space zero configuration is provided (e.g. from searchSpaceZero in MIB or from a new search space that is indicated by UL-WUS configuration) - FFS: Details of the time window, including at least the starting time and duration - FFS: Whether/how to support transmission of on-demand SIB1 with the association with SSB(s) based on a received UL-WUS Agreement From RAN1 point of view, the following is feasible. It is up to RAN2 to decide whether/how to support it. At least for Case 2 (Option 1+B+X) design, a unified configuration format that can support both Option 2 (i.e. a UL-WUS configuration applies to multiple NES cells) and Option 3 (i.e. a UL-WUS configuration applies to a single NES cell). Agreement For further study of on-demand SIB1 in idle/inactive mode, on the spatial relationships among PDCCH/PDSCH of on-demand SIB1, SSB, and UL WUS, as UL WUS is using dedicated PRACH resource, it is assumed that spatial relationships among PDCCH/PDSCH of on-demand SIB1, SSB and UL WUS can follow legacy mechanism. Agreement For further study of on-demand SIB1 in idle/inactive mode, use the following Table I (from R1-2405106, Ericsson) as a starting point to discuss the required parameters/contents inside the UL WUS configuration. Table I. Purpose; Parameters. To which cell does the config applies; NES-CellId; PhysCellId. ARFCN-ValueNR. WUS transmission; SIB1-RequestConfig; ss-PBCH-BlockPower. rach-OccasionsSIB1; Prach-ConfigurationIndex. msg1-FDM. msg1-FrequencyStart. zeroCorrelationZoneConfig. preambleReceivedTargetPower. preambleTransMax. powerRampingStep. ra-ResponseWindow. ssb-perRACH-Occasion. sib1-RequestPeriod. sib1-RequestResource; ra-PreambleStartIndex. ra-AssociationPeriodIndex. ra-ssb-OccasionMaskIndex. rsrp-ThresholdSSB. prach-RootSequenceIndex. msg1-SubcarrierSpacing. restrictedSetConfig. tdd-UL-DL-ConfigurationCommon. frequencyInfoUL; frequencyBandList. absoluteFrequencyPointA. offsetToCarrier. p-Max. frequencyShift7p5khz. SIB1 reception; pdcch-ConfigSIB1; ssb-SubcarrierOffset. controlResourceSetZero. searchSpaceZero. RAR Reception; pdcch-ConfigOD-SIB1-RAR; controlResourceSet. monitoringSlotPeriodicityAndOffset. Duration. monitoringSymbolsWithinSlot. aggregationLevels.

It can be observed that the UL WUS signal is based on msg1 of random access and UL WUS configuration would provide type 0 PDCCH monitoring resource for on-demand SIB1 reception after UE transmits the UL WUS. The spatial relationship among PDCCH/PDSCH of on-demand SIB1, SSB and UL WUS can follow legacy mechanism, which means the WUS (msg1) can request for the on-demand SIB1 corresponding to a specific SSB. The WUS configuration would include RACH configuration for WUS (msg1), UL frequency of WUS, type 0 PDCCH monitoring resource for SIB1 reception and PDCCH monitoring resource for RAR reception.

It can be concluded that all of the possible configuration parameters are PHY/MAC specific, which are controlled by gNB-DU. So, it should be the NES gNB-DU to decide the WUS configuration. As agreed by RAN1, the WUS configuration may apply to multiple NES cells or to a single NES cell. So, the WUS configuration can includes one or multiple identities of the NES cells. The multiple NES cells pertaining to the same WUS configuration should belong to the same gNB-DU that configures the WUS.

Proposal 1-1: NES gNB-DU should decide the WUS configuration including ID(s) of the NES cell(s).

Issue 2: How to determine the Cell A for NES cell

For the WUS configuration should be delivered to the Cell A before on-demand SIB1 mode activation or when the WUS configuration is changed, the NES gNB should be pre-configured with the Cell A for the NES cell by OAM, e.g., based on the operator network deployment. According to the network deployment, the NES cell may be mapped to multiple or a single Cell A, and a Cell A may take care of one or multiple NES cells. The Cell A would be in the same gNB or in a different gNB with the NES cell.

In principle, the OAM can configure the suitable Cell A for NES cell to either NES gNB-DU or gNB-CU. If NES gNB-CU is not configured with the Cell A information, the NES gNB-DU has to inform the NES gNB-CU about ID(s) of the Cell A so that the NES gNB-CU can deliver the WUS configuration to the Cell A. If the NES gNB-CU is aware of the Cell A, the NES gNB-DU does not need to be pre-configured with the Cell A information, or indicate the Cell A to the NES gNB-CU. It’s more beneficial for the NES gNB-CU being aware of the Cell A than the NES gNB-DU.

Proposal 1-2: Cell A is pre-configured to NES gNB-CU by OAM.

Issue 3: Does Cell A need to feedback result of the WUS configuration provision to NES cell

In the last meeting, there was an issue raised on whether Cell A has to feedback the allowance of transmission for the WUS configuration in the Cell A’s broadcast information.

As Cell A is determined based on the network deployment and pre-configured by OAM, if a cell is selected as the Cell A for the NES cell, the cell should always be able to receive the WUS configuration via Xn and broadcast the WUS configuration over the air interface. In our perspective, the refusion of WUS configuration provision by the Cell A would not be expected.

Proposal 1-3: Cell A does not feedback result for WUS configuration transmission to NES cell.

Issue 4: When WUS configuration is changed, whether NES gNB-DU indicates the change to NES gNB-CU

It has been agreed in the last meeting that the WUS configuration should be delivered to Cell A when the WUS configuration is changed, so, NES gNB-DU should indicate the WUS configuration change to NES gNB-CU. It’s reasonable that the Cell A is always provided with the latest WUS configuration, so that the Cell A can broadcast the latest WUS configuration when the on-demand SIB1 mode is activated for the NES cell. However, the Cell A does not need to broadcast the WUS configuration during the on-demand SIB1 deactivation period. So, it’s not necessary the Cell A being provided with the WUS configuration update during the on-demand SIB1 deactivation period, given the Cell A is provided with the WUS configuration before on-demand SIB1 activation.

In our opinion, the NES gNB doesn’t provide the updated WUS configuration to the Cell A during the on-demand SIB1 deactivation period. Accordingly, the NES gNB-DU doesn’t indicate the updated WUS configuration to the NES gNB-CU during the on-demand SIB1 deactivation period.

Proposal 1-4: NES gNB-DU does not indicate the change of WUS configuration to NES gNB-CU when the on-demand SIB1 mode is deactivated.

2.2 On-demand SIB1 mode

Issue 1: Which node initiates the change of on-demand SIB1 mode

Since SIB1 transmission is initiated and processed in the gNB-DU, all the energy used for SIB1 transmission in gNB is consumed by gNB-DU. Thus, the gNB-DU (instead of the gNB-CU) can be aware of energy consumed for SIB1 transmission. In addition, most of the energy used for base band processing is consumed in gNB-DU, so the energy efficiency of a cell can be obtained by gNB-DU. If the energy efficiency of the NES cell is low due to SIB1 transmission, e.g., when the user data rate is low or few UEs are connected in the cell, the gNB-DU can initiate to activate the on-demand SIB1 mode. When on-demand SIB1 mode is active, the SIB1 is not transmitted unless triggered by UE.

The gNB-DU can also decide to leave the on-demand SIB1 mode, for example, when the impact of SIB1 transmission on the energy efficiency can be ignored. When on-demand SIB1 mode is deactivated, SIB1 is transmitted as legacy approach.

It should be the gNB-DU to initiate entering or leaving the on-demand SIB1 mode, i.e., decide whether on-demand SIB1 is applied.

Proposal 2-1: NES gNB-DU decides to apply on-demand SIB1.

Issue 2: Granularity of on-demand SIB1 mode

According to the agreements in RAN1#117 meeting, it can be observed that the WUS configuration is cell level, the on-demand SIB1 mode should also be cell level in common understanding. However, the WUS would be designed in beam level, if that’s the case, the WUS can trigger the transmission of on-demand SIB1 on a specific SSB. The mapping relationship between SSB and WUS can reuse the legacy msg1 design. That means the on-demand SIB1 mode status among SSBs can be independent, e.g., SIB1 on one SSB is during the on-demand SIB1 mode activation period while the SIB1 on other SSBs is not.

If the on-demand SIB1 mode is SSB level, the mode status for a beam can be determined based on energy efficiency on the beam, that would make the control of mode status more accurate and therefore improve the energy saving effect. To support that, the on-demand SIB1 mode activation/deactivation over F1 and Xn would be indicated per SSB.

In our view, both cell level and beam level are workable for on-demand SIB1, RAN3 should discuss and decide the level of on-demand SIB1 mode.

Proposal 2-2: RAN3 discuss the granularity of on-demand SIB1 mode.

Issue 3: Whether NES gNB-CU needs to allow applying on-demand SIB1

On Issue 1, NES gNB-DU is proposed to initiate the change of on-demand SIB1 mode. The next issue is whether NES gNB-CU needs to allow the on-demand SIB1 transmission.

The activation of on-demand SIB1 mode will impact cell coverage for legacy UEs, because legacy UEs would probably not be able to camp on the NES cell without the normal SIB1 transmission, due to the cell entering on-demand SIB1 mode. For that aspect, on-demand SIB1 transmission which is different to legacy SIB1 transmission may not be allowed by gNB-CU, or on-demand SIB1 transmission may not be allowed for other reasons, e.g., there is no Cell A pre-configured for the NES cell.

So, whether a cell can apply on-demand SIB1 should be confirmed or allowed by NES gNB-CU. That is quite similar with SSB deactivation for energy saving in Rel-18 NES, where the SSB beam deactivation is initiated by gNB-DU while gNB-CU can indicate gNB-DU about the SSBs which are allowed to be deactivated.

Proposal 2-3: NES gNB-CU can indicate NES gNB-DU on the cells allowed to apply on-demand SIB1.

Issue 4: If NES cell needs to inform Cell A the change of on-demand SIB1 mode

If the on-demand SIB1 mode is activated, Cell A should cover the area of the NES cell and broadcast the UL WUS configuration for UE, so that the UEs within the area of the NES cell can obtain the UL WUS configuration. On the other hand, when the on-demand SIB1 mode is deactivated, the Cell A does not need to broadcast the WUS configuration or cover the area of the NES cell any more, thus, the Cell A would stop broadcasting the UL WUS configuration and shrink the coverage to save more energy. It’s beneficial for Cell A to be aware of NES cell’s SIB1 mode.

When the on-demand SIB1 mode change is initiated, the NES gNB-DU can indicate the mode change to the NES gNB-CU, if the mode changed is allowed. Then NES gNB-CU can inform Cell A the on-demand SIB1 mode status over Xn.

Proposal 2-4: NES cell can inform Cell A the change of on-demand SIB1 mode over Xn.

Issue 5: Whether Cell A can feedback the result of on-demand SIB1 mode change

When the NES cell informs the change of on-demand SIB1 to Cell A, the next issue is whether Cell A need to accept or refuse.

If a cell is selected as the Cell A for the NES cell, the cell should be able to afford the increased load caused by the activation of on-demand SIB1 mode in the NES cell. So, the activation or deactivation of on-demand SIB1 mode does not need being confirmed or allowed by Cell A. From this aspect, it is quite similar with the cell activation/deactivation procedure and the SSB activation/deactivation procedure in previous Releases for energy saving, where the change of cell/SSB status does not need feedback from the neighbour cells.

The feedback from Cell A on the result of on-demand SIB1 mode change should not be expected.

Proposal 2-5: Cell A should not feedback the result of on-demand SIB1 mode change.

Issue 6: Whether Cell A can request for deactivation of on-demand SIB1 mode

As the Cell A covers the area of NES cell when on-demand SIB1 mode is activated in the NES cell, the legacy UEs resides in the area of the NES cell are probably not be able to camp on the NES cell but camp on the Cell A instead. That would result in increase of load in Cell A. When traffic load in the Cell A is higher than a threshold, the Cell A may require the NES cell to leave the on-demand SIB1 mode so that some of traffic can be offloaded to the NES cell.

The Cell A can send the NES cell a request for on-demand SIB1 mode deactivation. In split architecture, the NES gNB-CU can ask the NES gNB-DU to deactivate on-demand SIB1 mode after receiving the request from the Cell A.

It is proposed that RAN3 discuss whether this scenario makes sense.

Proposal 2-6: RAN3 discuss whether Cell A can ask for deactivation of on-demand SIB1 mode.

3 Conclusion

In the previous sections we made the following observations and proposals:

Proposal 0: WUS configuration provision and on-demand SIB1 mode are transmitted independently.

WUS configuration provision

Proposal 1-1: NES gNB-DU should decide the WUS configuration including ID(s) of the NES cell(s).

Proposal 1-2: Cell A is pre-configured to NES gNB-CU by OAM.

Proposal 1-3: Cell A does not feedback result for WUS configuration transmission to NES cell.

Proposal 1-4: NES gNB-DU does not indicate the change of WUS configuration to NES gNB-CU when the on-demand SIB1 mode is deactivated.

On-demand SIB1 mode

Proposal 2-1: NES gNB-DU decides to apply on-demand SIB1.

Proposal 2-2: RAN3 discuss the granularity of on-demand SIB1 mode.

Proposal 2-3: NES gNB-CU can indicate NES gNB-DU on the cells allowed to apply on-demand SIB1.

Proposal 2-4: NES cell can inform Cell A the change of on-demand SIB1 mode over Xn.

Proposal 2-5: Cell A should not feedback the result of on-demand SIB1 mode change.

Proposal 2-6: RAN3 discuss whether Cell A can ask for deactivation of on-demand SIB1 mode.

4 Reference

- Draft Report of 3GPP TSG RAN3 meeting #123-bis, MCC

- Draft Report of 3GPP TSG RAN3 meeting #124, MCC

- Draft Report of 3GPP TSG RAN WG1 #117 v0.3.0, RAN1#117, MCC